# ACCIDENT PREVENTION

PROGRAM

for

## Poff VBA Phase II Renovation
Roanoke, VA

SOLICITATION # MFB-1810-111517

CONTRACT # 47PD0219C0004

### PRESENTED TO:	General Services Administration

### Dow Building
	100 S. Independence Mall West

### Philadelphia, PA 19106

From:                           Miami Technology Solutions, LLC
                                    1950 Roland Clarke Place, Suite 210
                                       Reston, VA 20191
                                    Phone: 609-230-4425
                                    Fax: 571-323-2102
                                    Email: hbrown@mn-e.com

TABLE OF CONTENTS

| ### Subject | #### Page |
| --- | --- |
| Drug and Alcohol Prevention | 1 |
| General Instructions | 2 |
| Company Safety Policy Letter | 3 |
| Responsibilities | 4 |
| Safety Disciplinary Policy | 5 |
| ##### Procedure for Injury or Illness on the Job | 6 |
| Basic Rules for Accident Investigation | 7 |
| Contingency for Bad Weather | 8 |
| Emergency Response, First Aid Training, Kits and Posters | 9 |
| First Aid Procedures in Construction | 10 |
| Work Crew Safety Meetings | 11 |
| Construction Safety Meeting Topic Suggestions | 12 |
| How to Hold a Good Safety Meeting | 13 |
| Walk-around Safety Inspections | 14 |
| General Safety Rules for Construction | 15 |
| Ladder Safety Rules | 17 |
| Fall Protection Safety Rules | 20 |
| Demolition | 21 |
| Scaffold Safety Rules | 22 |
| Motorized Vehicles and Equipment | 23 |
| General Materials Handling Safety | 24 |
| Lockout/Tagout Checklist | 27 |
| Welding and Cutting Safety Rules | 29 |
| Fire Protection | 33 |
| Hazard Communication Program | 34 |
| Respirator Program | 35 |
| Hearing Conservation Program | 36 |
| Heat Stress - How do you prevent heat illness? | 37 |
| Confined Space | 40 |
| Appendixes: |  |
| Job Orientation Guide | A-1 |
| Employee’s Report of Injury Form | B-1 |
| Incident Investigation Report Form | C-1-3 |
| Crew Leader Safety Meeting Form | D-1 |
| Safety Meeting Notice | E-1 |
| Fall Protection Work Plan – Sample One | F 1 |
| Fall Protection Training Guide for Employees | G 1-7 |
| Construction Self-inspection Guide | H-1 |
| Safety and Health Inspection Checklist – Sample | I 1-5 |
| Equipment Safety Inspection Checklist | J-1 |
| Job Safety Analysis Worksheet | K-1 |
| Written Hazard Communication Program | L 1-3 |
| Hazard Communication Checklist | M-1 |
| Hazardous Substances - Employee Orientation Checklist | N-1 |

ALCOHOL AND DRUG ABUSE PREVENTION PROGRAM

For A DRUG FREE WORKPLACE

It is the policy of MIAMI TECHNOLOGY SOLUTIONS, to provide a work environment that is free of alcohol, illegal drugs, and controlled substances. Illegal drugs include marijuana and all other drugs not prescribed by a licensed physician for use by the person possessing them. Unlawful manufacture, distribution, dispensing, possession or use of a controlled substance on the construction site is prohibited and will not be tolerated. As a further precaution, entry into or upon any vehicle, office or other work location of the company is conditioned upon the company's right to search the person and personal effects of any entrant for illegal drugs. When appropriate, such items discovered through these searches may be taken into custody and may be turned over to the proper law enforcement authorities. Effective January 1, 2005, all applicants for employment and employees of MIAMI TECHNOLOGY SOLUTIONS and all subcontractors may, as a condition of employment or continued employment, be required to submit to a drug screening test at the discretion of the company. At our discretion, drug/alcohol screening will be performed on all employees directly involved in job site accidents, mishaps and near misses by their actions or lack of action whether injuries requiring medical attention occurred or not.

This program is designed to notify, identify and provide penalties for those employees and applicants who, by the possession, use, or sale of an illegal drug or controlled substance, pose unnecessary and unacceptable risk to a safe, healthful and efficient operation.

We will encourage employees who are in violation of our policy to satisfactorily participate in drug abuse assistance or rehabilitation program approved for such purposes by a federal, state, or local health, law enforcement, or other appropriate agency. Refusal to submit to a drug/alcohol screening or search will be cause for disciplinary action, including termination of employment.

All employees are instructed to notify their employer of any criminal drug statute conviction for a violation occurring in the workplace no later than five (5) days after such conviction. MIAMI TECHNOLOGY SOLUTIONS will notify the contracting officer within ten (10) days after receiving notice from an employee or otherwise receiving actual notice of such conviction.

#### GENERAL INSTRUCTIONS

- Overview

Construction injuries create a no-win situation for everyone involved.  Employees experience pain, suffering and incapacitation while the company suffers from the loss of the injured person's contributions.  This document is designed to assist all personnel in assuring that such an undesirable situation will not develop in this company.  It provides information and guidance for the establishment and maintenance of an injury-free work environment.

- Procedures

This document contains guidance for safety procedures to be followed and forms to be used.  Supervisors are expected to integrate the procedures into the appropriate work activity and employees are expected to apply them on the job.  The sample forms are to be used if they apply to the job concerned.

- Dissemination

A copy of this statement will be issued to all supervisory and management personnel.  A copy of the policy statement will be posted on company safety and health bulletin boards and at the following locations:

1.  Poff VBA Bldg, 210 Franklin Road, 7th Floor AECOM Office

2.  120 Day Ave, Roanoke, VA – MTS Field Office

- Regulations

A copy of the following documents will be maintained at job site:

      - MTS Contractor’s Safety Plan

      - This Accident Prevention Program

      - MSDS Binder

#### COMPANY POLICY LETTER

SAFETY AND HEALTH POLICY FOR MIAMI TECHNOLOGY SOLUTIONS, LLC

The purpose of this policy is to develop a high standard of safety throughout all operations of Miami Technology Solutions, LLC and to ensure that no employee is required to work under any conditions, which are hazardous or unsanitary.

We believe that each employee has the right to derive personal satisfaction from his/her job and the prevention of occupational injury or illness is of such consequence to this belief that it will always be given top priority.

It is our intention here at Miami Technology Solutions, LLC to initiate and maintain complete accident prevention and safety training programs.  Each individual from top management to the working person is responsible for the safety and health of those persons in their charge and coworkers around them.  By accepting mutual responsibility to operate safely, we will all contribute to the wellbeing of our employees.

___________________________________

Harry Brown Jr., Director of Federal Construction       
                                         Miami Technology Solutions, LLC

RESPONSIBILITIES

Responsibilities for safety and health include the establishment and maintenance of an effective communication system among workers, supervisors and management officials.  To this end, all personnel are responsible to assure that their messages are received and understood by the intended receiver.  Specific safety and health responsibilities for company personnel are as follows:

- Management Officials

Active participation in and support of safety and health programs is essential.  Management officials will display their interest in safety and health matters at every opportunity.  At least one manager (as designated) will participate in the safety and health committee meetings, incident investigations and inspections.  Each manager will establish realistic goals for implementing instructions for meeting the goals.  Goals and implementing instructions shall be within the framework established by this document.  Incentives will be included as part of the instructions.

- Supervisors

The safety and health of the employees they supervise is a primary responsibility of the supervisors.  To accomplish this obligation, supervisors will:

| 1. | Assure that all safety and health rules, regulations, policies and procedures are understood and observed. |
| --- | --- |
| 2. | Require the proper care and use of all required personal protective equipment. |
| 3. | Identify and eliminate job hazards quickly through job safety analysis procedures. (See the sample Job Safety Analysis form attached to this document.) |
| 4. | Inform and train employees on the hazardous chemicals and/or procedures they MAY encounter under normal working conditions or during an emergency situation. (See the sample hazard communication program.) |
| 5. | Receive and take initial action on employee suggestions, awards or disciplinary measures. |
| 6. | Conduct crew/leader meetings the first five minutes of each work shift to discuss safety and health matters and work plans for the workday. |
| 7. | Conduct walk-around safety inspections at the beginning of each job, and at least weekly thereafter. |
| 8. | Train employees (new and experienced) in the safe and efficient methods of accomplishing each job or task as necessary. |
| 9. | Review injury trends and establish prevention measures. |
| 10. | Attend safety meetings and actively participate in the proceedings. |
| 11. | Participate in incident investigations and inspections. |
| 12. | Promote employee participation in the safety and health program. |
| 13. | Actively follow the progress of injured workers and display an interest in their rapid recovery and return to work. |

- Employees

Observe the items of responsibility established in this document as well as job safety rules which may apply to specific task assignments.

#### Safety Disciplinary Policy

Miami Technology Solutions, LLC believes that a safety and health Accident Prevention Program is unenforceable without some type of disciplinary policy.  Our company believes that in order to maintain a safe and healthful workplace, the employees must be cognizant and aware of all company, State, and Federal safety and health regulations as they apply to the specific job duties required.  The following disciplinary policy is in effect and will be applied to all safety and health violations.

The following steps will be followed unless the seriousness of the violation would dictate going directly to Step 2 or Step 3.

- A first-time violation will be discussed orally between company supervision and the employee.  This will be done as soon as possible.

- A second time offense will be followed up in written form and a copy of this written documentation will be entered in the employee’s personnel folder.

- A third time violation will result in time off or possible termination, depending on the seriousness of the violation.

##### Procedure for Injury or Illness on the Job

###### A.  Owner or lead person immediately takes charge

- Supervise and administer first aid as you wish (Good Samaritan Law applies).

- Arrange for transportation (ambulance, helicopter, company vehicle, etc.), depending on the seriousness of the injury.  Protect the injured person from further injury.

- Notify owner or top management, if not already present.

- Do not move anything unless necessary, pending investigation of the incident.

- Accompany or take injured person(s) to doctor, hospital, home etc. (depending on the extent of injuries).

- Take injured person to family doctor, if available.

- Remain with the injured person until relieved by other authorized persons (manager, EMT, doctor, etc.).

- When the injured person’s immediately family is known, the owner or supervisor should properly notify family members, preferable in person, or have an appropriate person do so.

###### B.  Documentation

- Minor injuries – requiring doctor or outpatient care:  After the emergency actions following an injury, an investigation of the incident will be conducted by the immediate supervisor and any witness to determine the causes.  The findings must be documented on our investigation form.

- Major injuries – fatality, hospitalization, non-hospitalized amputation or loss of an eye(s):  Top management must see that L&I is notified as soon as possible, but at least within 8 hours of the incident that caused the fatality or in-patient hospitalization. Also, any non-hospitalized amputation or loss of an eye(s) must be reported to L&I within 24 hours of the incident.

Call OSHA at 1-800-321-6742. Top management will then assist the Department in the investigation.

- The findings must be documented on our incident investigation report form and recorded on the OSHA 300 log, if applicable. (Sample incident investigation report form included in this document.)

###### C.  Near Misses

- All near-miss incidents (close calls) must be investigated.

- Document the finding on the company incident investigation report form.

- Review the findings at the monthly safety meetings or sooner if the situation warrants.

Sample forms for Incident investigation and Employee’s Report of Injury are available in the Appendix.

##### Basic Rules for Accident Investigation

- The purpose of an investigation is to find the cause of an incident and prevent future occurrences, not to fix blame.  An unbiased approach is necessary to obtain objective findings.

- Visit the incident scene as soon as possible – while facts are fresh and before witnesses forget important details.

- If possible, interview the injured worker at the scene of the incident and “walk” him or her through a re-enactment.  Be careful not to actually repeat the act that caused the injury.

- All interviews should be conducted as privately as possible.  Interview witnesses one at a time.  Talk with anyone who has knowledge of the incident, even if they did not actually witness the mishap.

- Consider taking the signed statements in cases where facts are unclear or there is an element of controversy.

- Graphically document details of the incident: area, tools, and equipment.  Use sketches, diagrams, and photos as needed, and take measurements when appropriate.

- Focus on causes and hazards.  Develop an analysis of what happened, how it happened, and how it could have been prevented.  Determine what caused the incident itself (unsafe equipment/condition, unsafe act, etc), not just the injury.

- How will you prevent such incidents in the future?  Every investigation should include an action plan.

- If a third party or defective product contributed to the incident, save any evidence.  It could be critical to the recovery of the claim costs.

##### Use Incident Investigation Report Form – Appendix C-1 to write up accident investigation report.

CONTINGENCY FOR BAD WEATHER

      - If building is closed due to bad weather no work will be performed.

      - Follow Building/Governing policies on early dismissal due to bad weather.

Precautions against the hazards of inclement weather can be accomplished in four ways:

1.    Move materials, equipment, and personnel.

2.   Shelter equipment, material, and personnel.

3.   Counter the effects of weather.

4.   Equip to offset weather hazards.

- EMERGENCY RESPONSE - FIRST AID TRAINING, KITS AND POSTER

A.	Purpose:  To afford the employees immediate and effective attention should an injury result, Harry Brown / Mike Main, will ensure that a certified first aider(s) will be available.

1.	To meet the above objectives, the following procedures will be followed:

a.	All supervisors or persons in charge of crews will be first aid trained unless their duties require them to be away from the jobsite.  If so, other persons who are certified in first aid will be designated as the recognized first aider.

b.	Other persons will be trained in order to augment or surpass the 						standard requirements.

c.	Valid first aid cards are recognized as ones that include both first aid and cardiopulmonary resuscitation (CPR) and have not reached the expiration date.

2.	First aid training, kits, and procedures will be in accordance with the requirements of the general safety and health standards.

      - First aid kits will be on work table at each floor.

      - Micheal Main is designated to ensure that the first aid kits are properly maintained and stocked.

      - Posters listing emergency numbers, procedures, etc., will be       strategically located, such as on the first aid kit, at telephones, and in other areas where employees have easy access.

      - Escape routes will be posted on each floor in case of fire or other evacuation emergency needs.

      - During “AFTER HOURS WORK” a building emergency contact will be listed

#### FIRST AID PROCEDURES IN CONSTRUCTION

We have first aid qualified workers here but we do not have “designated” first-aiders.  First aid at the job site is done on a Good Samaritan basis.

If first aid trained personnel are involved in a situation involving blood, they should:

- Avoid skin contact with blood/other potentially infectious materials by letting the victim help as much as possible, and by using gloves provided in the first aid kit.

- Remove clothing, etc. with blood on it after rendering help.

- Wash thoroughly with soap and water to remove blood.  A 10% chlorine bleach solution is good for disinfecting areas contaminated with blood (spills, etc.).

- Report such first aid incidents within the shift to supervisors (time, date, flood presence, exposure, names of others helping).

Hepatitis B vaccinations will be provided as soon as possible but not later than 24 hours after the first aid incident.

If an exposure incident occurs, we will immediately make available appropriate:

- Post exposure evaluation

- Follow-up treatment

#### WORK CREW SAFETY MEETINGS

We believe that hard work and perseverance are required for the prevention of injuries and illnesses, with the crew leader being the key to a successful result.

A.	Purpose:  To assist in the detection and elimination of unsafe conditions and work procedures.

B.	Procedures:

The following guidelines will be followed:

1.	These meetings are held at the beginning of each job and at least weekly thereafter, according to the various circumstances involved or when necessary to clear working procedures.  No set pattern will suit all cases.  It is important that the crew leader talk daily on injury prevention and immediately upon witnessing an unsafe act.

2.	The attendance and subjects discussed will be documented and maintained on file for one year.

3.	Copies of the minutes will be made available to the employees by posting or other means.

C.	Scope of Activities:

(certain employees, as may be designated by their supervisors, will assist)

1.	Conduct in-house safety inspections with supervisor concerned.

2.	Investigate incidents to uncover trends.

3.	Review incident reports to determine means or elimination.

4.	Accept and evaluate employee suggestions.

5.	Review job procedures and recommend improvements (Job Safety Analysis Form is available in the Appendix)

6.	Monitor the safety program effectiveness.

- Promote and publicize safety.

D.   	Documentation:  The sample form in the Appendix D-1 is available to assist in documenting activities of crew/leader meetings.  There is also a Safety Meeting Notice form that you can print out and copy to announce your next safety meeting.

##### Construction Safety Meeting Suggestions

(The crew leader’s guide)

Twelve good topics for construction safety meetings:

- Fall protection/fall prevention

- Personal protective equipment

  - Hard hats

  - Eye protection

  - Hearing protection

  - Footwear

  - Safety harness/belts

  - Respiratory protection

- Housekeeping

- Tool inspection

- Emergency procedures

- Electrical safety

- Ladder safety

- Scaffold safety

- Fire prevention/fire extinguishers

- Reporting injuries and unsafe conditions

- Confined spaces

- Lock-out procedures

- Heat Stress

##### How to hold a good safety meeting

- Be certain everyone knows the time and place of the next meeting.  You may use the sample form on the next page if you wish.

- Insist that everyone attend.  Before the next meeting, remind those who were late or failed to attend that attendance is not optional.

- Pick an appropriate topic.  If you can’t think of an appropriate topic, use one from the attached list (these usually apply to all projects).

- Start the meeting on time.

- Don’t waste time – give the meeting your undivided attention.

- Discuss the topic you have chosen and prepared.  Don’t wait until the meeting to choose your topic.

- Use handouts or posters to illustrate your topic.

- Discuss current job site safety events, injuries and close calls.

- Encourage employees to discuss safety problems as they arise.  Do not save safety concerns for the meeting.  Allow some time for employee questions or input at the end of the meeting.

- Invite managers or owners to speak.  Ask fellow employees to speak on a safety topic.

- If you prevented one injury, it is time well spent.  Your topic may be one that some employees have heard many times, but there may be one person who is new or has never been told of the safety requirement for that topic.  Repeating topics several times during the course of a project is beneficial as long as it applies to the work being done.

- Follow up on employee concerns or questions and get back to them with the answer before the next meeting.

- Be certain to document the attendance and the topics discussed.

#### WALK-AROUND SAFETY INSPECTIONS

Walk-around safety inspections will be conducted at the beginning of each job, and at least weekly thereafter.

- The inspections will be conducted jointly by one member of management and one employee, elected by the employees, as their authorized representative.

- The inspections will be documented and the documentation will be made available for inspection by representatives of the Department of Labor and Industries.

- The records of the walk-around inspections will be maintained until the completion of the job.

##### General Safety Rules for Construction

- Always store materials in a safe manner.  Tie down or support piles if necessary to prevent falling, rolling, or shifting.

- Shavings, dust scraps, oil or grease should not be allowed to accumulate.  Good housekeeping is a part of the job.

- Trash piles must be removed as soon as possible.  Trash is a safety and fire hazard.

- Remove or bend over the nails in lumber that has been used or removed from a structure.

- Immediately remove all loose materials from stairs, walkways, ramps, platforms, etc.

- Do not block aisles, traffic lanes, fire exits, gangways, or stairs.

- Avoid shortcuts – use ramps, stairs, walkways, ladders, etc.

- Standard guardrails must be erected around all floor openings and excavations must be barricaded.  Contact your supervisor for the correct specifications.

- Do not remove, deface or destroy any warning, danger sign, or barricade, or interfere with any form of protective device or practice provided for your use or that is being used by other workers.

- Get help with heavy or bulky materials to avoid injury to yourself or damage to material.

- Keep all tools away from the edges of scaffolding, platforms, shaft openings, etc.

- Do not use tools with split, broken, or loose handles, or burred or mushroomed heads.  Keep cutting tools sharp and carry all tools in a container.

- Know the correct use of hand and power tools.  Use the right tool for the job.

- Know the location and use of fire extinguishing equipment and the procedure for sounding a fire alarm.

- Flammable liquids shall be used only in small amounts at the job location and in approved safety cans.

- Proper guards or shields must be installed on all power tools before use.  Do not use any tools without the guards in their proper working condition.  No “homemade” handles or extensions (cheaters) will be used!

- All electrical power tools (unless double insulated), extension cords, and equipment must be properly grounded.

- All electrical power tools and extension cords must be properly insulated.  Damaged cords must be replaced.

- Do not operate any power tool or equipment unless you are trained in its operation and authorized by your firm to do so.

- All electrical power equipment and tools must be grounded or double insulated.

- Use tools only for their designed purpose.

##### Ladder Safety Rules

### General:

- Inspect before use for physical defects.

- Ladders are not to be painted except for numbering purposes.

- Do not use ladders for skids, braces, workbenches, or any purpose other than climbing.

- When you are ascending or descending a ladder, do not carry objects that will prevent you from grasping the ladder with both hands.

- Always face the ladder when ascending and descending.

- If you must place a ladder over a doorway, barricade the door to prevent its use and post a warning sign.

- Only one person is allowed on a ladder at a time.

- Do not jump from a ladder when descending.

- All joints between steps, rungs, and side rails must be tight.

- Safety feet must be in good working order and in place.

- Rungs must be free of grease and/or oil.

### Stepladders

- Do not place tools or materials on the steps or platform of a stepladder

- Do not use the top two steps of a stepladder as a step or stand.

- Always level all four feet and lock spreaders in place.

- Do not use a stepladder as a straight ladder.

### Straight type or extension ladders

- All straight or extension ladders must extend at least three feet beyond the supporting object when used as an access to an elevated work area.

- After raising the extension portion of a two or more-stage ladder to the desired height, check to ensure that the safety dogs or latches are engaged.

- All extension or straight ladders must be secured or tied off at the top.

- All ladders must be equipped with safety (non-skid) feet.

- Portable ladders must be used at such a pitch that the horizontal distance from the top support to the foot of the ladder is about one-quarter of the working length of the ladder.

# Fall Protection Safety Rules

Falls from elevation are a major cause of injuries and deaths in the construction industry.  We at Miami Technology Solutions are committed to eliminating injuries caused by fall hazards by instituting a program of 100% fall protection for all fall hazards 4 feet or greater.

All work sites with fall hazards of 10 feet or more will have a site-specific fall protection work plan completed before any employees begin work.  The employees on that specific job will be trained in the fall hazards and the method used to implement fall protection.  The attached training guide will be used to train employees in the inspection and maintenance of their fall protection equipment, as well as fall protection selection criteria.  All employees will use a fall protection system when there is exposure to a fall hazard of 4 feet or greater.  Employees who fail to follow this policy are subject to disciplinary action, up to and including dismissal.

The evaluation of the jobsite and the completion of the fall protection work plan will be done by a designated “competent person,” who has an understanding of OSHA fall protection requirements, the fall protection systems available for use, and has the authority to take corrective action to eliminate employee exposure to fall hazards.

Fall protection will be provided either through the use of a fall arrest system or a fall restraint system as shown below and thoroughly described in the fall protection work plan available on site for review.

|  |  |  | Fall Protection |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fall Restraint Restrained from falling |  |  |  |  | Fall Arrest Stopped after the fall |  |  |  |
| Guardrails |  |  |  |  | Full-Body Harness |  |  |  |
| Warning Line System |  |  |  |  | Safety Nets |  |  |  |
| #### OR Warning line system and Safety monitor |  |  |  |  | Catch Platforms |  |  |  |

DEMOLITION

All personnel received an overview of the basic requirements for the Project.

The method(s) of containment, storage, and handling of expected waste streams and recycling materials.

Our policy and procedures related to fences, barricades and warning devices to protect workers and the public

The correct procedures to be used/followed in the advent of an unplanned collapse of the structure.

Hazards associated with the project.

The correct procedures for donning, wearing and removal of proper personal protection equipment including inspection.

Methods for prompt, safe removal and treatment of injured workers.

##### Scaffold Safety Rules

- General

Before starting work on a scaffold, inspect it for the following:

  - Are guardrails, toe boards, and planking in place and secure?

  - Are locking pins at each joint in place?

  - Are all wheels on moveable scaffolds locked?

- Do not attempt to gain access to a scaffold by climbing on it (unless it is specifically designed for climbing – always use a ladder.

- Scaffolds and their components must be capable of supporting four times the maximum intended load.

- Any scaffold, including accessories such as braces, brackets, trusses, screw legs, ladders, etc., damaged or weakened in any way, must be immediately repaired or replaced.

- Scaffold planks must extend over their end supports not less than 6 inches nor more than 12 inches, unless otherwise specifically required.

- Scaffold platforms must be at least 18 inches wide unless otherwise specifically required or exempted.

- Where persons are required to work or pass under the scaffold, scaffolds shall be provided with a screen between the toe board and guardrail, extending along the entire opening.  The screen must be made of No. 18 gauge U.S. Standard wire, ½ inch mesh or equivalent protection.

- All scaffolds must be erected level and plumb, and on a solid footing.

- Do not change or remove scaffold members unless authorized.

- Do not allow workers to ride on a rolling scaffold when it is being moved.  Remove or secure all materials and tools on deck before moving.

- Do not alter any scaffold member by welding, burning, cutting, drilling, or bending.

##### Motorized vehicles and equipment

- Do not ride on motorized vehicles or equipment unless a proper seat is provided for each rider.

- Always be seated when riding authorized vehicles (unless they are designed for standing).

- Do not operate any motorized vehicle or equipment unless you are specifically authorized to do so by your supervisor.

- Always use your seat belts in the correct manner.

- Obey all speed limits and other traffic regulations.

- Always be aware of pedestrians and give them the right-of-way.

- Always inspect your vehicle or equipment before and after daily use.

- Never mount or dismount any vehicles or equipment while they are still in motion.

- Do not dismount any vehicle without first shutting down the engine, setting the parking brake and securing the load.

- Do not allow other persons to ride the hook or block, dump box, forks, bucket or shovel of any equipment.

- Each operator must be knowledgeable of all hand signals and obey them.

- Each operator is responsible for the stability and security of his/her load.

##### General Materials Handling Safety

General material storage safety:

- Make sure that all materials stored in tiers are stacked, racked, blocked, interlocked, or otherwise secured to prevent sliding, falling, or collapse.

- Post conspicuously the maximum safe load limits of floors within buildings and structures, in pounds per square foot, in all storage areas, except for floor or slab on grade.  Do not exceed the maximum safe loads.

- Keep aisles and passageways clear to provide for the free and safe movement of material handling equipment or employees.  Keep these areas in good repair.

- Do not store materials on scaffolds or runways in excess of supplies needed for immediate operations.

- Use ramps, blocking, or grading when a difference in road or working levels exists to ensure the safe movement of vehicles between the two levels.

- Do not place materials stored inside buildings under construction within 6 feet of any hoistway or inside floor openings, or within 10 feet of an exterior wall which does not extend above the top of the material stored.

- Anchor and brace temporary floors used in steel erection, concrete forms, and shoring and other “in-process equipment” that are to be left overnight or for longer periods of time to prevent their displacement in any direction.

- Do not place materials stored inside buildings under construction within 6 feet of any hoist way or inside floor openings, or within 10 feet of an exterior wall which does not extend above the top of the material stored.)

- Stack bagged materials by stepping back the layers and cross-keying the bags at least every ten bags high.

  - Carefully handle cement and lime delivered in paper bags to prevent the bags from bursting.

  - Do not pile cement and lime bags more than ten bags high except when stored in bins or enclosures built for the purpose of storage.

  - When bags are removed from the pile, keep the length of the pile at an even height and maintain the necessary step backs every five bags.

  - When handling cement and lime bags, wear eye protection preventing any contact with the substance (such as goggles or other sealed eye protection) and wear long sleeve shirts with close fitting collar and cuffs.

  - Do not wear clothing that has become hard and stiff with cement.

  - Make sure to report any susceptibility of skin to cement and lime burns.

  - Make sure that a hand cream or Vaseline and eyewash is provided and kept ready for use to prevent burns.

  - Store lime in a dry place to prevent a premature slacking action that may cause fire.

- Do not stack bricks more than 7 feet high.  When a loose brick stack reaches a height of 4 feet, taper it back 2 inches for every foot of height above the 4-foot level.

- Never stack bricks, for storage purposes, on scaffolds or runways.

- Always stack blocks; do not throw in a loose pile.

- When stacking masonry blocks higher than 6 feet, taper back the stack one-half block per tier above the 6-foot level.

- When stacking inside a building, distribute the piles to prevent overloading the floor.

- Do not drop or throw blocks from an elevation or deliver blocks through chutes.

- Do not stack lumber more than 20 feet high; if handling lumber manually, do not stack more than 16 feet high.

- Remove all nails from used lumber before stacking.

- Stack lumber on level and solidly supported sills, and such that the stack is stable and self-supporting.

- Stack stored lumber on timber sills to keep it off the ground.  Sills must be placed level on solid supports.

- Place cross strips in the stacks when they are stacked more than 4 feet high.

- If not racked, stack and block structural steel, poles, pipe, bar stock, and other cylindrical materials as to prevent spreading or tilting.

- Wear heavy gloves when handling reinforcing steel.

- When bending reinforcing steel on the job, use a strong bench set up on even dry ground or a floor to work on.

- Carefully pile structural steel to prevent danger of members rolling off or the pile toppling over.

- Keep structural steel in low piles, giving consideration to the sequence of use of its members.

- Stack corrugated and flat iron in flat piles, with the piles not more than 4 feet high; place spacing strips between each bundle.

- Frequently inspect stock piles of sand, gravel, and crushed stone to prevent their becoming unsafe by continued adding to or withdrawing from the stock.

  - Do not remove frozen material in a manner that would produce an overhang.

General Rigging Equipment Safety:

- Inspect rigging equipment for material handling prior to use on each shift and as necessary during its use to ensure that it is safe.  Remove defective rigging equipment from service.

- Never load rigging equipment in excess of its recommended safe working load.

- Remove rigging equipment when not in use from the immediate work area so as not to present a hazard to employees.

- Mark special rigging accessories (i.e., spreader bars, grabs, hooks, clamps, etc.) or other lifting accessories with the rated capacity.  Proof test all components to 125% of the rated load prior to the first use.  Maintain permanent records on the job site for all special rigging accessories.

Disposal of waste materials:

- Whenever materials are dropped more than 20 feet to any point lying outside the exterior walls of the building, use an enclosed chute of wood or equivalent material.

- When debris is dropped without the use of chutes, make sure that the area onto which the material is dropped is completely enclosed with barricades at least 42 inches high and 20 feet back from the projected edge of the opening above.  Post at each level warning signs of the hazard of falling materials.  Do not remove debris in this lower area until debris handling ceases above.

- Remove all scrap lumber, waste material, and rubbish from the immediate work area as the work progresses.

- Make sure to comply with local fire regulations if disposing of waste material or debris by burning.

- Keep all solvent waste, oily rags, and flammable liquids in fire-resistant covered containers until removed from the work site.

Lockout/Tagout Checklist

YES     NO	      COMPLETION DATE

1.  Equipment, machinery and personnel:

a.  A list of equipment and machines that need to

be locked out has been developed.		_____	_____	___________________________

b.  All new machinery (after Jan. 1990)

has the ability to accept a lockout device.	_____	_____	___________________________

c.  Specific written Energy Control Procedures

are developed and used for each piece of

equipment. 			 		_____	_____	___________________________

d.  A list of all authorized employees has

been developed.				_____	_____ ___________________________

e. A list of all affected employees has

been developed.				_____	_____	___________________________

2.  Energy Control Program:

a.  A written Energy Control Program

has been developed.				_____	_____	___________________________

b.  Does the written program state the methods

of compliance, including the:

- Intended use of procedures.		            _____	_____	___________________________

- Steps for shut down, isolating,

blocking and securing energy.		_____	_____ ___________________________

- Steps for placement, removal,

and transfer of lockout/tagout devices.	_____	_____	___________________________

- Requirements for testing to

verify effectiveness of lockout/tagout.		_____	_____	 ___________________________

- Compliance with energy control procedures

is verified at least annually. The results of the

inspection are certified and kept on file.		_____	_____   _________________________

d.  Lockout/tagout devices are provided.		_____	_____       _________________________

(locks, hasps, tags, etc.).

e.  Lockout devices are singularly identified,

durable, standardized, substantial and

employee identifiable.				_____	_____ _________________________

f.   Lockout devices are used only for energy

control.						_____	_____  _________________________

g.  A tagout system is used only if a isolating

device cannot be locked out.			_____	_____  _________________________

h. Tagout devices are located at the same

location as lockout devices.			_____	_____	   ________________________

i.  Tagout devices warn against hazardous

conditions such as Do Not Start, Do Not Open. _____  _____	  ________________________

j.   Energy isolation is performed ONLY by

authorized employees.				_____	_____        _________________________

k.  Affected employees are notified before and

after lockout/tagout.				_____	_____       __________________________

l.   Group lockout/tagout procedures are

used when needed. 				_____	_____     __________________________

m. Information about each other’s' lockout

program is exchanged with contractors.		_____	_____     __________________________

n.  Continuity of lockout/tagout is provided

during shift change and personnel changes.	_____	_____	_________________________

3. Training requirements:

a.  Authorized employees - recognition of energy

sources, type and magnitude of energy and

methods and procedures necessary for isolation

and control.					_____	_____    _________________________

b.  Affected employees - purpose and use of

energy control procedures.			_____	_____           _________________________

c.  Other employees - instructed on the procedures

locked or tagged out.				_____	_____          _________________________

d.  For tagout system - limitations of tags.		_____	_____	        _________________________

e.  Retraining - when change in job, assignment,

equipment, process, procedure or the result of an

inspection.					_____	_____         _________________________

f.  Training is certified with names and dates.	_____	_____         _________________________

Welding and Cutting Safety Rules

- Always follow the manufacturer’s recommendations for setting up and operating equipment, selection of tip size, and gas cylinder operating pressures.

- Always use a regulator to reduce gas cylinder pressure to the operating pressures recommended by the equipment manufacturer.  All piping and equipment must meet the standards of the Compressed Gas Association.

- Always ensure that all connections are leak tight.  Each time connections are loosened and retightened each connection should be checked with a soap and water solution (oil free soap).  Do not check with flame.

- Before “lighting up” clear out each line by letting a small amount of gas flow (separately) to remove any mixed gases that might be in the lines.

- Never use defective, worn or leaky equipment.  Repair it or take it out of service.

- Never use acetylene in excess of 15 psi pressure.  Higher pressures with acetylene are dangerous.  If the cylinder is not fitted with a hand wheel valve control, any special wrench required must be placed on the cylinder while the cylinder is in service.  On manifolds, one wrench for each manifold will suffice.

- Always have an appropriate fire extinguisher in good operating condition readily available when operating welding or cutting equipment.

- Never perform welding, cutting, brazing, or heating operations in a poorly ventilated area.  Avoid breathing fumes from these operations at all times, particularly when zinc, cadmium, or lead coated metals are involved.

- Never perform welding or cutting operations near combustible materials (gasoline cans, paints, paper, rags, etc.).

- Always protect yourself, others present, welding hoses, gas cylinders, and flammable materials in the area from hot slag and sparks from the welding and cutting operations.

- The welder and spectators must always wear goggles to protect the eyes from injurious light rays, sparks and hot molten metal during welding, cutting, and heating operations.  Eye protection must comply with the established ANSI Standards.

- Always wear clean, oil free clothing during welding and cutting operations.  Protect the hands with leather welding gloves to avoid burns from radiation and hot molten slag.  Low cut shoes and trousers with cuffs or open pockets should not be worn.

- Never use a match or cigarette lighter to light a cutting or welding torch.  Always use a spark igniter.  Fingers are easily burned by the igniting gas when a match or cigarette lighter is used.

- Ensure that the material being welded or cut is secure and will not move or fall on anyone.

- Never use a welding, cutting, or heating torch on a container that has held a flammable liquid.  Explosive vapors can accumulate and linger in closed containers for extended periods of time.

- Never use a regulator for gasses other than those for which it was designed for by the manufacturer since the diaphragm and seat materials may not be compatible with other gasses.

- Never attempt to adapt and use a fuel gas or inert gas regulator on an oxygen cylinder.  A special protective device is incorporated on the oxygen regulator to harmlessly dissipate the heat caused by the recompression when the cylinder valve is quickly opened.  Such a protective device is not furnished on fuel gas and inert gas regulators.

- Never tamper with the safety devices on cylinders, fuse plugs, safety discs, etc. and do not permit torch flames or sparks to strike the cylinder.

- Always refer to the various gasses by their proper names.  (Do not refer to oxygen as “air” or acetylene as “gas”.)

- All cylinders, particularly acetylene, should be restrained securely in an upright position to prevent accidents.  A non-vertical position for an acetylene cylinder in use would allow the discharge of acetone through the regulator and into the cutting torch, clogging the mixer passages and creating a fire hazard.  It would reduce the efficiency of the flame and contaminate the weld area.  It also can cause voids in the porous material inside the cylinder, which can lead to acetylene explosions.

- Store all gas cylinders not in use away from excessive heat sources, such as stoves, furnaces, radiators, the direct rays of the sun, and the presence of open flames.  Cylinders in storage should always be secured in an upright position.

- Keep all burning or flammable substances away from the oxygen or fuel gas storage area (at least 20 feet) and post “No Smoking” signs.

- Upon completion of a welding, heating, or cutting operation immediately inspect the surrounding areas for smoldering embers.  Allow at least one half hour to elapse before leaving the area and conduct another thorough inspection just before leaving.  Also alert other personnel of fire possibilities.

- Always have the properly fitted wrench to fasten a regulator to a cylinder.  Never tighten the regulator by hand.

- Always leave the fuel gas cylinder valve wrench in place when the cylinder valve is open so that it can be closed quickly in an emergency.  Do not open acetylene valves more than one-quarter (1/4) turn.

- Before connecting a regulator to a gas cylinder, open the cylinder valve for a moment.  Called cracking the cylinder valve, this will blow out any foreign material that may have lodged in the valve during transit.  Do not stand in front of the valve when “cracking”.

- After attaching a regulator to a gas cylinder, be sure the regulator adjusting screw is fully released (backed off in a counter clockwise direction so that it swivels freely) before the cylinder valve is opened.  Never stand in front of a regulator when you are opening a cylinder valve.

- Always open the cylinder valve slowly so that gas pressure will build up slowly in the regulator (particularly in the oxygen cylinder).  Quick opening of the cylinder valve causes a build up of heat due to recompression of the gas.  When combined with combustible materials, ignition and explosion may result.

- If a leak develops in a fuel gas cylinder that cannot be stopped by closing the valve, immediately place the cylinder outside of the building away from possible fire or ignition sources in a location that is free from wind currents that might carry the gas to an ignition source.

- Never attempt to mix gasses in a cylinder or fill an empty one from another (particularly oxygen cylinders).  Mixture of incompatible gasses and/or heat caused by recompression of the gas or gasses may result in ignition and fire.  Only the owner of a cylinder may mix gasses in it.

- When a gas cylinder is ready for return to the supplier, be certain the cylinder valve is closed to prevent internal contamination and the shipping cap is in place to protect the cylinder valve.  Identify empty cylinders.

- Never use oxygen or other gasses as a substitute for compressed air in operation of air-operated tools, blowing off parts, or for ventilation purposes.  The only exception to this rule is where oxygen is used to blow out port passages and talcum powder or dust from welding hoses when setting up new or old “dusty” equipment.

- Do not attempt to do your own repair on welding equipment.  Equipment that is improperly repaired can cause leaks and other hazardous conditions.  Repairs must be performed by qualified repair personnel.

- Never repair welding hose with tape.  Use of tape and many hose splicers can reduce the pressure to the torch and can cause hazardous conditions.  Welding hose must meet the specifications of the Compressed Gas Association.

- Use the shortest length of hose possible.  Longer hoses require higher gas pressures and can be hard to handle.

- Never use oil or grease on any part of welding or cutting equipment and never let it come into contact with oil or grease.  This includes gas cylinders, work bench, regulators, torches, tips, threads on bottles, and clothes that are worn, such as jackets, gloves, and aprons.  Oxygen and oil or grease can cause explosions and fire.

- Never use a hammer on the valve cover caps to loosen them.  Use a piece of wood to soften the impact and prevent sparks and damage to the cap.

- When moving gas cylinders always roll them on their bottom edges or in a cart designed for their movement.  Sliding or dragging them or rolling causes excessive wear and may weaken their walls by metal erosion.  Slings and electromagnets are not authorized when transporting cylinders.

- Never use cylinders as rollers to move material.  Do not let them bump into each other or let them fall.

- Fuel gas and liquefied fuels must be stored and shipped valve end up.

- Do not hammer on any cylinder.  Do not tamper with the relief valves.  If you have trouble, contact the supplier for assistance.

- Suitable eye protection must be worn for all welding and cutting operations.

- Cylinders must be secured.  Valves must be closed when unattended and caps must be on the cylinders when the regulators are not on the cylinders.

- Cylinders must be upright when they are transported in powered vehicles.

- All cylinders with a water weight of over 30 lbs. must have caps or other protection.

- All fuel gases must be used through a regulator on cylinder or manifold.

- Compressed gas cylinders must be upright except for short periods for transportation.

- Repair work on gauges and regulators must be done by qualified personnel.

- Only 4 inches of hose per foot may be covered with tape.  Defective hoses must be removed from service.

- Oxygen must not be used for ventilation.

- Oxygen regulators must be marked “Use No Oil”.  Regulators and fittings must meet the specifications of the Compressed Gas Association.

- Union nuts on regulators must be checked for damage.

- Before removing a regulator, shut off cylinder valve and release gas from regulator.  Equipment must be used only as approved by the manufacturer.

- Caps must be on cylinders unless they are transported on a special carrier.

- Hot warnings on materials are required.

- Fire is the biggest hazard in welding.  The area should be cleared for a radius of 35 feet.  Fire shields should be used.  The area should be monitored for 30 minutes or more after end of work to ensure there is no delayed ignition.

- Proper personal protective equipment must be worn by all welders and assisting personnel.

- All welding personnel should be advised of the hazards from heating zinc, lead, cadmium, and any other substances that could cause health problems from the welding activity.

(The following apply to arc welding)

- Chains, wire ropes, hoists, and elevators must not be used to carry welding current.

- Leather capes should be used for overhead welding.

- The neck and ears must be protected from the arc.

- Conduits with electrical conductors in them must not be used to complete a welding circuit.

- Welding shields must be used to protect other workers from injurious light rays.

- Welding leads must be inspected regularly for damage to insulation.  Only proper splicing will be authorized.  There should be no splices in stinger lead within 10 feet of the stinger and the leads should never be wrapped around the body.

FIRE PROTECTION – PREVENTION

Approved type and size fire extinguisher will be in the equipment repair shop and fuel service areas. They will likewise be maintained in the field office and storage areas. One 5-B-C fire extinguisher will be located on each piece of construction equipment. Gasoline or other liquid having a flash point of 100 degrees and above will not be used in lieu of approved cleaning fluids. A fire extinguisher, rated not less than 2A, shall be provided for each 3,000 square feet of the protected building area, or major fraction thereof. Travel distance from any point of the protected area to the nearest fire extinguisher shall not exceed 100 feet.

Fuel storage dispensing tanks will be properly marked as to contents. "NO SMOKING" signs will be posted at each tank in plain view. All fuel tanks, including moveable tanks on skids, will be kept grounded and bonded (multiple tanks) when not being moved. All portable containers and safety cans used for hand transporting of flammable fuels or liquids will meet the specifications of OSHA 29 CFR, Part 1910, Subpart 106 or EM385-1-1 safety standards. When applicable, flammable fuel or liquid storage tanks shall be located a minimum of 20 feet away from buildings. A fire extinguisher, rated not less than 10B, shall be provided within 50 feet of wherever more than 5 gallons of flammable or combustible liquids or 5 pounds of flammable gas are being used on the jobsite. This requirement does not apply to the integral fuel tanks of motor vehicles. Two or more portable tanks, grouped together, having a combined capacity in excess of 2,200 gallons, shall be separated by a 5-foot-clear area. Individual portable tanks exceeding 1,100 gallons shall be separated by a 5-foot-clear area. Within 200 feet of each portable tank, there shall be a 12-foot-wide access way to permit approach of fire control apparatus. Storage areas shall be kept free of weeds, debris, and other combustible material not necessary to the storage.

Hazard Communication Program

##### Purpose:

The purpose of the Hazard Communication Program is to ensure that the hazards of all chemicals produced or imported by chemical manufacturers or importers are evaluated.  Information concerning the hazards must be transmitted to affected employers and employees before they use the products.

##### Procedure:

- Inventory Lists – Know the hazardous chemicals in your workplace that are a potential physical or health hazard.  Make an inventory list of these hazardous chemicals; this list must be a part of your written program.

- MSDS – Make sure there is a material safety data sheet (MSDS) for each chemical and that the inventory list and labeling system reference the corresponding MSDS for each chemical.

- Labeling System – Each container entering the workplace must be properly labeled with the identity of the product, the hazardous warning, and the name and address of the manufacturer.

- Written Program – Develop, implement, and maintain a comprehensive written hazard communication program at the workplace that includes provisions for container labeling, safety data sheets, and an employee training program (see the editable sample in the Appendix, page M1-2).

Employees must be made aware of where hazardous chemicals are used in their work areas.  They must also be informed of the requirements of the Hazard Communication Standard, the availability and location of the written program, the list of hazardous chemicals, and the safety data sheets.

The code specifically requires employers to train employees in the protective practices implemented in their workplace, the labeling system used, how to obtain and use MSDSs, the physical and health hazards of the chemicals and the recognition, avoidance and prevention of accidental entrance of hazardous chemicals into the work environment.

#### Respirator Program

Purpose:

The purpose of the Respirator Program is to ensure that all employees are protected from exposure to respiratory hazards.  Engineering controls such as ventilation and substitution of less toxic materials are the first line of defense.  However, engineering controls are not feasible for some operations or do not completely control the identified hazards.  In these situations, respirators and other protective equipment must be used.  Respirators are also utilized for protection during emergencies.

Procedure:

This program applies to all employees who are required to wear respirators during normal work operations and during certain non-routine or emergency operations.  Employees participating in the respiratory protection program do so at no cost to them.  The expense associated with medical evaluations, training, and respiratory protection equipment will be borne by the company.

Employees who voluntarily choose to use a cartridge style respirator when the respirator is not required are subject to the medical evaluation, cleaning, maintenance, and storage elements only of this program. These individuals will also receive training covering proper procedures for cleaning, maintenance and storage of their respirators.

#### Hearing Conservation Program

Purpose:

The purpose of the Hearing Conservation Program is to ensure that all employees are protected from exposure to noise hazards.  Employers whose workers are exposed to high noise levels must have an active program for protecting their employees’ hearing.

### Procedure:

An effective hearing conservation program should first assess company wide noise exposures in order to identify any employee or group of employees exposed to noise.  Noise is measured with a sound level meter or noise dosimeters, which measure average noise levels over time.  Employees who are exposed to noise at or above an eight-hour time-weighted average of 85 dB (decibels) must be covered under a hearing conservation program.  For these employees, the employer must develop, implement, and maintain (at no cost to the employees) a program consisting of:

- Mandatory audiometric testing

- Making hearing protectors available and ensuring their use.

- Comprehensive training explaining hearing loss, hearing protective devices, and the employer’s hearing conservation program.

- Warning signs for high noise areas (115 dBA or higher).

- Keeping accurate records.

- Ensuring employee access to their records.

Additionally, the employer must post a copy of the hearing conservation standard or post a notice to affected employees or their representatives that a copy of the standard is available at the workplace for their review.

If you need assistance in noise measurements, you can contact the Consultation Section of the Department of Labor and Industries; the industrial hygiene consultants can help you free of charge.

Heat Stress - How do you prevent heat illness?

- • 	Supply adequate water and encourage workers who work in hot weather to drink regularly, even when not thirsty.  A small amount of water every 15 minutes is recommended rather that a large amount after hours of sweating.

- • 	Learn the signs and symptoms of heat-related illness.

- • 	Inform workers they should avoid alcohol or drinks with caffeine before or during work in hot weather.

- •	 Try to do the heaviest work during the cooler parts of the day.

- • 	Adjusting to work in heat takes time. Allow workers to acclimatize. Start slower and work up to your normal pace.

- • 	Wear lightweight, loose-fitting, light-colored, breathable (e.g. cotton) clothing and a hat.

- • 	Allow workers to take regular breaks from the sun. Loosen or remove clothing that restricts cooling.

- •	 Watch workers for symptoms of heat-related illness. This is especially important for non-acclimatized workers, those returning from vacations and for all workers during heat-wave events.

- • 	If exertion causes someone’s heart to pound or makes them gasp for breath, become lightheaded, confused, weak or faint, they should STOP all activity and get into a cool area or at least into the shade, and rest.

The two major heat-related illnesses are heat exhaustion and heat stroke. Heat exhaustion, if untreated, may progress to deadly heat stroke. Heat stroke is very dangerous and frequently fatal. If workers show symptoms, always take this seriously and have them take a break and cool down before returning to work. Stay with them. If symptoms worsen or the worker does not recover within about 15 minutes, call 911 and have them transported and medically evaluated. Do not delay transport.

Heat Stroke or Heat Exhaustion?

How do you tell the difference?

The telling difference is mental confusion or disorientation in ALL heat stroke victims

You can ask these 3 questions:  What is your name? What day is this? Where are we?

If a worker can’t answer these questions, assume it is heat stroke.

What are the symptoms of heat exhaustion and heat stroke?

| Heat Exhaustion | Heat Stroke |
| --- | --- |
| - • Heavy sweating - • Exhaustion, weakness - • Fainting / Lightheadedness - • Paleness - • Headache - • Clumsiness, dizziness - • Nausea or vomiting - • Irritability | - • Sweating may or may not be present - • Red or flushed, hot dry skin - • Any symptom of heat exhaustion but more severe - • Confusion / Bizarre behavior - • Convulsions before or during cooling - • Collapse - • Panting/rapid breathing - • Rapid, weak pulse - • Note: May resemble a heart attack |

What do you do if someone is suffering from heat exhaustion or heat stroke?

| Heat Exhaustion | Heat Stroke (medical emergency) |
| --- | --- |
| - • Move the worker to a cool, shaded area to rest; do not leave them alone. - • Loosen and remove heavy clothing that restricts evaporative cooling. - • Give cool water to drink, about a cup every 15 minutes. - • Fan the worker, spray with cool water, or apply a wet cloth to their skin to increase evaporative cooling. - • Recovery should be rapid. Call 911 if they do not feel better in a few minutes. • Do not further expose the worker to heat that day. Have them rest and continue to drink cool water or electrolyte drinks. | - • Get medical help immediately, call 911 and transport as soon as possible. - • Move the worker to a cool, shaded area and remove clothing that restricts cooling. - • Seconds count – Cool the worker rapidly using whatever methods you can. For example, immerse the worker in a tub of cool water; place the worker in a cool shower; spray the worker with cool water from a garden hose; sponge the worker with cool water; or, if the humidity is low, wrap the worker in a cool, wet sheet and fan them vigorously. Continue cooling until medical help arrives. - • If emergency medical personnel are delayed, call the hospital emergency room for further instruction. - • Do not give the worker water to drink until instructed by medical personnel. |

Heat Stress Check List

- Does the worksite have temperature extremes (above 85 degrees in higher humidity, above 90-95 degrees in lower humidity) that may cause heat stress?

-     Do employees do heavy labor or wear heavy protective clothing? (increases heat       stress conditions)

- Do employees have access to adequate drinking water at all times?

- Are employees allowed work breaks during prolonged heavy labor?

- Do workers have access to shade during breaks?

- Have employees been trained on the symptoms of heat-related illness (heat exhaustion and heat stroke) ?

- Are employees trained on first aid measures for heat-related illness?

.

CONFINED SPACES

Fatalities and injuries constantly occur among construction workers who, during the course of their jobs, are required to enter confined spaces.  In some circumstances, these workers are exposed to multiple hazards, any of which may cause bodily injury, illness, or death.  Workers are injured and killed from a variety of atmospheric factors and physical agents.

The construction standard requires that companies follow OSHA guidelines when working in confined spaces.  There is an exception for work on sewer systems under construction.

Employers must consult with employees and their authorized representatives on the development and implementation of all aspects of the permit required confined space entry program required by the Confined Space Standard.

All information required by the Confined Space Standard must be available to employees affected by the standard (or their authorized representatives).

You must first determine if you have any confined space situations.  A confined space has three characteristics; it must have all three characteristics to be considered a confined space:

- Large enough to get your body entirely inside to do your work

- Not designed or intended for continuous occupation

- Restricted entry or exit

If you do have any confined spaces, you must not enter them until you have carefully evaluated the hazards inside to determine what type of entry procedure may be used for each confined space you have:

- Non-permit-required confined space (NPRCS)

- Permit-required confined space (PRCS)

- Alternate Entry

APPENDIXES

Job Orientation Guide

| Company: | Miami Technology Solutions | Employee: | (Enter Employee Name |
| --- | --- | --- | --- |
| Trainer: | Micheal Main | Hire Date: | (Enter Employee's Hire Date) |
| Date |  | Position: | (Enter Employee's Job Title) |

This checklist is a guideline for conducting employee safety orientations for employees new to Miami Technology Solutions.  Once completed and signed by both supervisor and employee, it serves as documentation that orientation has taken place.

|  |  |  |  |  | Date | Initials |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Explain the company safety program, including: |  |  |  |  |  |
|  |  |  | Orientation |  | ______ | ______ |
|  |  |  | On-the-job training |  | ______ | ______ |
|  |  |  | Safety meetings |  | ______ | ______ |
|  |  |  | Incident investigation |  | ______ | ______ |
|  |  |  | Disciplinary action |  | ______ | ______ |
| 2. | Use and care of personal protective equipment (Hard hat, fall protection, eye protection, etc.) |  |  |  | ______ | ______ |
| 3. | Line of communication and responsibility for immediately reporting injuries. |  |  |  |  |  |
|  | A. | When to report an injury |  |  | ______ | ______ |
|  | B. | How to report an injury |  |  | ______ | ______ |
|  | C. | Who to report an injury to |  |  | ______ | ______ |
|  | D. | Filling out incident report forms |  |  | ______ | ______ |
| 4. | General overview of operation, procedures, methods and hazards as they relate to the specific job |  |  |  | ______ | ______ |
| 5. | Pertinent safety rules of the company and OSHA |  |  |  | ______ | ______ |
| 6. | First aid supplies, equipment and training |  |  |  |  |  |
|  | A. | Obtaining treatment |  |  | ______ | ______ |
|  | B. | Location of Facilities |  |  | ______ | ______ |
|  | C. | Location and names of First-aid trained personnel |  |  | ______ | ______ |
| 7. | Emergency plan |  |  |  |  |  |
|  | A. | Exit location and evacuation routes |  |  | ______ | ______ |
|  | B. | Use of fire fighting equipment (extinguishers, hose) |  |  | ______ | ______ |
|  | C. | Specific procedures (medical, chemical, etc.) |  |  | ______ | ______ |
| 8. | Vehicle safety |  |  |  | ______ | ______ |
| 9. | Personal work habits |  |  |  |  |  |
|  | A. | Serious consequences of horseplay |  |  | ______ | ______ |
|  | B. | Fighting |  |  | ______ | ______ |
|  | C. | Inattention |  |  | ______ | ______ |
|  | D. | Smoking policy |  |  | ______ | ______ |
|  | E. | Good housekeeping practices |  |  | ______ | ______ |
|  | F. | Proper lifting techniques |  |  | ______ | ______ |

NOTE TO EMPLOYEES:  Do not sign unless ALL items are covered and ALL questions are satisfactorily answered.

The signatures below document that the appropriate elements have been discussed to the satisfaction of both parties, and that both the supervisor and the employee accept responsibility for maintaining a safe and healthful work environment.

Date: _______________________  	Supervisor’s Signature: ___________________________________

Date: _______________________	Employee’s Signature: ___________________________________

#### Employee’s Report of Injury Form

Instructions:  Your employees may use this form to report all work related injuries, illnesses, or “near miss” events (which could have caused an injury or illness) – no matter how minor.  This helps you to identify and correct hazards before they cause serious injuries.  This form should be completed by employees as soon as possible and given to a supervisor for further action.

| I am reporting a work related: Injury Illness Near miss |  |  |
| --- | --- | --- |
| Your Name: |  |  |
| Job title: |  |  |
| Supervisor: |  |  |
| Have you told your supervisor about this injury/near miss? Yes No |  |  |
| Date of injury/near miss: |  | Time of injury/near miss: |
| Names of witnesses (if any): |  |  |
| Where, exactly, did it happen? |  |  |
| What were you doing at the time? |  |  |
| Describe step by step what led up to the injury/near miss. (continue on the back if necessary): |  |  |
| What could have been done to prevent this injury/near miss? |  |  |
| What parts of your body were injured? If a near miss, how could you have been hurt? |  |  |
| Did you see a doctor about this injury/illness? Yes No |  |  |
| If yes, whom did you see? | Doctor’s phone number: |  |
| Date: | Time: |  |
| Has this part of your body been injured before? Yes No |  |  |
| If yes, when? | Employer: |  |
| Your signature (optional): | Date: |  |

#### Incident Investigation Report Form

Instructions:  Complete this form as soon as possible after an incident that results in serious injury or illness. (Optional: Use to investigate a minor injury or near miss that could have resulted in a serious injury or illness.)

| This is a report of a: Death Lost Time Dr. Visit Only First Aid Only Near Miss |  |
| --- | --- |
| Date of incident: | This report is made by: Employee Supervisor Team Final Report |

| Step 1: Injured employee (complete this part for each injured employee) |  |  |  |
| --- | --- | --- | --- |
| Name: | Sex: Male Female |  | Age: |
| Department: | Job title at time of incident: |  |  |
| Part of body affected: (shade all that apply) | Nature of injury: (most serious one) Abrasion, scrapes Amputation Broken bone Bruise Burn (heat) Burn (chemical) Concussion (to the head) Crushing Injury Cut, laceration, puncture Hernia Illness Sprain, strain Damage to a body system: Other ___________ | This employee works: Regular full time Regular part time Seasonal Temporary |  |
|  |  | Months with this employer |  |
|  |  | Months doing this job: |  |
|  |  | (e.g.: nervous, respiratory, or circulatory systems) |  |

| Step 2: Describe the incident |  |  |  |  |
| --- | --- | --- | --- | --- |
| Exact location of the incident: |  |  |  | Exact time: |
| What part of employee’s workday? Entering or leaving work Doing normal work activities During meal period During break Working overtime Other |  |  |  |  |
| Names of witnesses (if any): |  |  |  |  |
| Number of attachments: | Written witness statements: | Photographs: | Maps / drawings: |  |
| What personal protective equipment was being used (if any)? |  |  |  |  |
| Describe, step-by-step the events that led up to the injury. Include names of any machines, parts, objects, tools, materials and other important details. Description continued on attached sheets: |  |  |  |  |

| Step 3: Why did the incident happen? |  |
| --- | --- |
| Unsafe workplace conditions: (Check all that apply) Inadequate guard Unguarded hazard Safety device is defective Tool or equipment defective Workstation layout is hazardous Unsafe lighting Unsafe ventilation Lack of needed personal protective equipment Lack of appropriate equipment / tools Unsafe clothing No training or insufficient training Other: _____________________________ | Unsafe acts by people: (Check all that apply) Operating without permission Operating at unsafe speed Servicing equipment that has power to it Making a safety device inoperative Using defective equipment Using equipment in an unapproved way Unsafe lifting by hand Taking an unsafe position or posture Distraction, teasing, horseplay Failure to wear personal protective equipment Failure to use the available equipment / tools Other: __________________________________ |
| Why did the unsafe conditions exist? |  |
| Why did the unsafe acts occur? |  |
| Is there a reward (such as “the job can be done more quickly”, or “the product is less likely to be damaged”) that may have encouraged the unsafe conditions or acts? Yes No If yes, describe: |  |
| Were the unsafe acts or conditions reported prior to the incident? Yes No |  |
| Have there been similar incidents or near misses prior to this one? Yes No |  |

Step 4: How can future incidents be prevented?

What changes do you suggest to prevent this injury/near miss from happening again? Stop this activity Guard the hazard Train the employee(s) Train the supervisor(s) Redesign task steps Redesign work station Write a new policy/rule Enforce existing policy Routinely inspect for the hazard Personal Protective Equipment Other: ____________________

What should be (or has been) done to carry out the suggestion(s) checked above?

Description continued on attached sheets:

| Step 5: Who completed and reviewed this form? (Please Print) |  |  |
| --- | --- | --- |
| Written by: Department: | Title: Date: |  |
| Names of investigation team members: |  |  |
| Reviewed by: |  | Title: Date: |

| # CREW SAFETY MEETING |  |  |  |
| --- | --- | --- | --- |
| Company/Contractor Name |  | Address |  |
| Date | Time |  | # of employees attending |
| Subjects discussed |  |  |  |
| Minutes: |  |  |  |
| Crew Leader Comments: |  |  |  |

Minutes taken by____________________________________

SAFETY MEETING NOTICE

DATE: __________________

TIME:  __________________

PLACE:

________________________

________________________

FALL PROTECTION WORK PLAN

You must develop and implement a written fall protection work plan including each area of the work place where the employees are assigned and where fall hazards of 10 feet or more exist and be available on the job site for inspection by the department.

| Company Name | Date |
| --- | --- |
| Site Address |  |

(if additional space is needed, use the back of the sheet)

Identify all fall hazards 10 feet or more above the ground level or lower level. Check all that apply.

| Open-sided floors | Wall openings | Roof openings |
| --- | --- | --- |
| Decks/Balconies | Window openings | Leading edge work |
| Floor openings | Door openings | Mobile lift work |
| Skylight openings |

Methods of fall protection to be used: (LSO = Low Slopes Only. Low Slopes = 4 x 12 or less)

| Guardrail system (LSO) | Personal fall arrest system | Safety watch system (LSO) |
| --- | --- | --- |
| Warning line System (LSO) | Personal fall restraint system | Warning line w/ safety monitor (LSO) |
| Catch platform | Positioning device system |  |
| Safety net | Horizontal life lines | Name of safety watch or monitor (if used): |
| Covers | Vertical life lines & rope grab | _________________________________ |

Overhead Hazard Protection Methods

| Hard Hats | Toe boards on Guardrails | Other: ______________ |
| --- | --- | --- |
| Overhead Hazard Signs | Screens on Guardrails | Other: ______________ |
| Debris Nets | Barricade to control Access to Area |

Describe procedures for assembly, maintenance, inspection, disassembly of fall protection system to be used.

Describe procedures for handling, storage, and securing tools, equipment, and materials.

Describe methods of overhead protection for workers who may be in, or pass through work area.

Describe methods to be implemented for prompt, safe removal of injured worker(s).

Employees who received fall protection training on the above site specific fall protection work plan.

| Name(s): | Date: |
| --- | --- |
| The competent person’s signature verifies that the fall protection work plan has been done, the employees informed of the plan and that employees have received training in the fall protection systems in use: Name: Title: Date:. |

FALL PROTECTION TRAINING GUIDE FOR EMPLOYEES

Safety Belt, Harness and Lanyard Inspection and Maintenance

| I. | ANSI Classification: |  |
| --- | --- | --- |
|  | Class II | Chest harness – used for restraint purposes (NOT for vertical free fall hazards). |
|  | Class III | Full body harness – used for fall arrest purposes. Can also be used for fall restraint. |
|  | Class IV | Suspension/position belt – used to suspend or support the worker. If a fall arrest hazard exists this must be supplemented by use of a safety harness. |

| II. | Inspection Guidelines: |
| --- | --- |
|  | To maintain their service life and high performance, all belts and harnesses must be inspected prior to each use for mildew, wear, damage and other deteriorations. Visual inspection before each use is just common sense. Periodic tests by a trained inspector for wear, damage or corrosion should be part of the safety program. Inspect your equipment daily and replace it if any of the defective conditions in this manual are found. |

Lanyard inspection:

| When inspecting lanyards, begin at one end and work to the opposite end. Slowly rotate the lanyard so that the entire circumference is checked. Spliced ends require particular attention. Hardware should be examined under procedures also detailed below, i.e., Snaps, Dee Ring, and Thimbles. |  |
| --- | --- |
| 1. | Steel While rotating the steel lanyard, watch for cuts, frayed areas, or unusual wearing patterns on the wire. Broken strands will separate from the body of the lanyards. |
| 2. | Webbing While bending webbing over a pipe or mandrel, observe each side of the webbed lanyard. This will reveal any cuts or breaks. Swelling, discolorations, cracks, and charring are obvious signs of chemical or heat damage. Observe closely for any breaks in stitching. |
| 3. | Rope Rotation of the rope lanyard while inspecting from end to end will bring to light any fuzzy, worn, broken, or cut fibers. Weakened areas from extreme loads will appear as a noticeable change in original diameter. The rope diameter should be uniform throughout, following a short break-in-period. |

FALL PROTECTION TRAINING GUIDE FOR EMPLOYEES

Fall Protection System Considerations

| 1. | Selection and use considerations: The kind of personal fall arrest system selected should match the particular work situation, and any possible free fall distance should be kept to a minimum. Consideration should be given to the particular work environment. For example, the presence of acids, dirt, moisture, oil, grease, etc., and their effect on the system, should be evaluated. Hot or cold environments may also have an adverse affect on the system. Wire rope should not be used where an electrical hazard is anticipated. As required by the standard, the employer must plan to have means available to promptly rescue an employee should a fall occur, since the suspended employee may not be able to reach a work level independently. Where lanyards, connectors, and lifelines are subject to damage by work operations such as welding, chemical cleaning, and sandblasting, the component should be protected, or other securing systems should be used. The employer should fully evaluate the work conditions and environment (including seasonal weather changes) before selecting the appropriate personal fall protection system. Once in use, the system's effectiveness should be monitored. In some cases, a program for cleaning and maintenance of the system may be necessary. |
| --- | --- |
| 2. | Testing considerations: Before purchasing or putting into use a personal fall arrest system, an employer should obtain from the supplier information about the system based on its performance during testing so that the employer can know if the system meets this standard. Testing should be done using recognized test methods. Not all systems may need to be individually tested; the performance of some systems may be based on data and calculations derived from testing of similar systems, provided that enough information is available to demonstrate similarity of function and design. |
| 3. | Component compatibility considerations: Ideally, a personal fall arrest system is designed, tested, and supplied as a complete system. However, it is common practice for lanyards, connectors, lifelines, deceleration devices, and body harnesses to be interchanged since some components wear out before others. The employer and employee should realize that not all components are interchangeable. For instance, a lanyard should not be connected between a body harness and a deceleration device of the self-retracting type since this can result in additional free fall for which the system was not designed. Any substitution or change to a personal fall arrest system should be fully evaluated or tested by a competent person to determine that it meets the standard, before the modified system is put in use. |

FALL PROTECTION TRAINING GUIDE FOR EMPLOYEES

Fall Protection System Considerations cont’d

| 4. | Employee training considerations: Thorough employee training in the selection and use of personal fall arrest systems is imperative. As stated in the standard, before the equipment is used, employees must be trained in the safe use of the system. This should include the following: Application limits; proper anchoring and tie-off techniques; estimation of free fall distance, including determination of deceleration distance, and total fall distance to prevent striking a lower level; methods of use; and inspection and storage of the system. Careless or improper use of the equipment can result in serious injury or death. Employers and employees should become familiar with this material, as well as manufacturer's recommendations, before a system is used. Of uppermost importance is the reduction in strength caused by certain tie-offs (such as using knots, tying around sharp edges, etc.) and maximum permitted free fall distance. Also, to be stressed are the importance of inspections prior to use, the limitations of the equipment, and unique conditions at the worksite which may be important in determining the type of system to use. |  |
| --- | --- | --- |
| 5. | Instruction considerations: Employers should obtain comprehensive instructions from the supplier as to the system's proper use and application, including, where applicable: |  |
|  | a. | The force measured during the sample force test; |
|  | b. | The maximum elongation measured for lanyards during the force test; |
|  | c. | The deceleration distance measured for deceleration devices during the force test; |
|  | d. | Caution statements on critical use limitations; |
|  | e. | Application limits; |
|  | f. | Proper hook-up, anchoring and tie-off techniques, including the proper dee-ring or other attachment point to use on the body harness for fall arrest; |
|  | g. | Proper climbing techniques; |
|  | h. | Methods of inspection, use, cleaning, and storage; and |
|  | i. | Specific lifelines that may be used. This information should be provided to employees during training. |
| 6. | Inspection considerations: Personal fall arrest systems must be regularly inspected. Any component with any significant defect, such as cuts, tears, abrasions, mold, or undue stretching; alterations or additions which might affect its efficiency; damage due to deterioration; contact with fire, acids, or other corrosives; distorted hooks or faulty hook springs; tongues unfitted to the shoulder of buckles; loose or damaged mountings; nonfunctioning parts; or wearing or internal deterioration in the ropes must be withdrawn from service immediately, and should be tagged or marked as unusable, or destroyed. |  |

FALL PROTECTION TRAINING GUIDE FOR EMPLOYEES

Fall Protection System Considerations cont’d

| 7. | Rescue considerations: When personal fall arrest systems are used, the employer must assure that employees can be promptly rescued or can rescue themselves should a fall occur. The availability of rescue personnel, ladders or other rescue equipment should be evaluated. In some situations, equipment that allows employees to rescue themselves after the fall has been arrested may be desirable, such as devices that have descent capability. |  |
| --- | --- | --- |
| 8. | Tie-off considerations: |  |
|  | a. | One of the most important aspects of personal fall protection systems is fully planning the system before it is put into use. Probably the most overlooked component is planning for suitable anchorage points. Such planning should ideally be done before the structure or building is constructed so that anchorage points can be incorporated during construction for use later for window cleaning or other building maintenance. If properly planned, these anchorage points may be used during construction, as well as afterwards. |
|  | b. | Employers and employees should at all times be aware that the strength of a personal fall arrest system is based on its being attached to an anchoring system which does not significantly reduce the strength of the system (such as a properly dimensioned eye-bolt/snap-hook anchorage). Therefore, if a means of attachment is used that will reduce the strength of the system, that component should be replaced by a stronger one, but one that will also maintain the appropriate maximum arrest force characteristics. |
|  | c. | Tie-off using a knot in a rope lanyard or lifeline (at any location) can reduce the lifeline or lanyard strength by 50 percent or more. Therefore, a stronger lanyard or lifeline should be used to compensate for the weakening effect of the knot, or the lanyard length should be reduced (or the tie-off location raised) to minimize free fall distance, or the lanyard or lifeline should be replaced by one which has an appropriately incorporated connector to eliminate the need for a knot. |
|  | d. | Tie-off of a rope lanyard or lifeline around an "H" or "I" beam or similar support can reduce its strength as much as 70 percent due to the cutting action of the beam edges. Therefore, a webbing lanyard or wire core lifeline should be used around the beam; or the lanyard or lifeline should be protected from the edge; or free fall distance should be greatly minimized. |

FALL PROTECTION TRAINING GUIDE FOR EMPLOYEES

Fall Protection System Considerations cont’d

|  | e. | Tie-off where the line passes over or around rough or sharp surfaces reduces strength drastically. Such a tie-off should be avoided or an alternative tie-off rigging should be used. Such alternatives may include use of a snap-hook/dee-ring connection, wire rope tie-off, an effective padding of the surfaces, or an abrasion-resistance strap around or over the problem surface. |
| --- | --- | --- |
|  | f. | Horizontal lifelines may, depending on their geometry and angle of sag, be subjected to greater loads than the impact load imposed by an attached component. When the angle of horizontal lifeline sag is less than 30 degrees, the impact force imparted to the lifeline by an attached lanyard is greatly amplified. For example, with a sag angle of 15 degrees, the force amplification is about 2:1 and at 5 degrees sag, it is about 6:1. Depending on the angle of sag, and the line's elasticity, the strength of the horizontal lifeline and the anchorages to which it is attached should be increased a number of times over that of the lanyard. Extreme care should be taken in considering a horizontal lifeline for multiple tie-offs. The reason for this is that in multiple tie-offs to a horizontal lifeline, if one employee falls, the movement of the falling employee and the horizontal lifeline during arrest of the fall may cause other employees to also fall. Horizontal lifeline and anchorage strength should be increased for each additional employee to be tied-off. For these and other reasons, the design of systems using horizontal lifelines must only be done by qualified persons. Testing of installed lifelines and anchors prior to use is recommended. |
|  | g. | The strength of an eye-bolt is rated along the axis of the bolt and its strength is greatly reduced if the force is applied at an angle to this axis (in the direction of shear). Also, care should be exercised in selecting the proper diameter of the eye to avoid accidental disengagement of snap-hooks not designed to be compatible for the connection. |
|  | h. | Due to the significant reduction in the strength of the lifeline/lanyard (in some cases, as much as a 70 percent reduction), the sliding hitch knot should not be used for lifeline/lanyard connections except in emergency situations where no other available system is practical. The "one-and-one" sliding hitch knot should never be used because it is unreliable in stopping a fall. The "two-and-two," or "three-and-three" knot (preferable), may be used in emergency situations; however, care should be taken to limit free fall distance to a minimum because of reduced lifeline/lanyard strength. |
| 9. | Vertical lifeline considerations. As required by the standard, each employee must have a separate lifeline when the lifeline is vertical. The reason for this is that in multiple tie-offs to a single lifeline, if one employee falls, the movement of the lifeline during the arrest of the fall may pull other employees' lanyards, causing them to fall as well. |  |

FALL PROTECTION TRAINING GUIDE FOR EMPLOYEES

Fall Protection System Considerations cont’d

| 10. | Snap-hook considerations: |  |  |
| --- | --- | --- | --- |
|  | a. | Required by this standard for all connections, locking snap-hooks incorporate a positive locking mechanism in addition to the spring loaded keeper, which will not allow the keeper to open under moderate pressure without someone first releasing the mechanism. Such a feature, properly designed, effectively prevents roll-out from occurring. |  |
|  | b. | The following connections must be avoided (unless properly designed locking snap-hooks are used) because they are conditions which can result in roll-out when a nonlocking snap-hook is used: |  |
|  |  | • • • • • • | Direct connection of a snap-hook to a horizontal lifeline. Two (or more) snap-hooks connected to one dee-ring. Two snap-hooks connected to each other. A snap-hook connected back on its integral lanyard. A snap-hook connected to a webbing loop or webbing lanyard. Improper dimensions of the dee-ring, rebar, or other connection point in relation to the snap-hook dimensions which would allow the snap-hook keeper to be depressed by a turning motion of the snap-hook. |
| 11. | Free fall considerations: The employer and employee should at all times be aware that a system's maximum arresting force is evaluated under normal use conditions established by the manufacturer, and in no case using a free fall distance in excess of 6 feet (1.8 m). A few extra feet of free fall can significantly increase the arresting force on the employee, possibly to the point of causing injury. Because of this, the free fall distance should be kept at a minimum, and, as required by the standard, in no case greater than 6 feet (1.8 m). To help assure this, the tie-off attachment point to the lifeline or anchor should be located at or above the connection point of the fall arrest equipment to harness. (Since otherwise additional free fall distance is added to the length of the connecting means (i.e. lanyard).) Attaching to the working surface will often result in a free fall greater than 6 feet (1.8 m). For instance, if a 6-foot (1.8 m) lanyard is used, the total free fall distance will be the distance from the working level to the body harness attachment point plus the 6 feet (1.8 m) of lanyard length. Another important consideration is that the arresting force that the fall system must withstand also goes up with greater distances of free fall, possibly exceeding the strength of the system. |  |  |

FALL PROTECTION TRAINING GUIDE FOR EMPLOYEES

Fall Protection System Considerations cont’d

| 12. | Elongation and deceleration distance considerations. Other factors involved in a proper tie-off are elongation and deceleration distance. During the arresting of a fall, a lanyard will experience a length of stretching or elongation, whereas activation of a deceleration device will result in a certain stopping distance. These distances should be available with the lanyard or device's instructions and must be added to the free fall distance to arrive at the total fall distance before an employee is fully stopped. The additional stopping distance may be very significant if the lanyard or deceleration device is attached near or at the end of a long lifeline, which may itself add considerable distance due to its own elongation. As required by the standard, sufficient distance to allow for all of these factors must also be maintained between the employee and obstructions below, to prevent an injury due to impact before the system fully arrests the fall. In addition, a minimum of 12 feet (3.7 m) of lifeline should be allowed below the securing point of a rope grab type deceleration device, and the end terminated to prevent the device from sliding off the lifeline. Alternatively, the lifeline should extend to the ground or the next working level below. These measures are suggested to prevent the worker from inadvertently moving past the end of the lifeline and having the rope grab become disengaged from the lifeline. |
| --- | --- |
| 13. | Obstruction considerations: The location of the tie-off should also consider the hazard of obstructions in the potential fall path of the employee. Tie-offs that minimize the possibilities of exaggerated swinging should be considered. |
| 14. | Other considerations: Because of the design of some personal fall arrest systems, additional considerations may be required for proper tie-off. For example, heavy deceleration devices of the self-retracting type should be secured overhead in order to avoid the weight of the device having to be supported by the employee. Also, if self-retracting equipment is connected to a horizontal lifeline, the sag in the lifeline should be minimized to prevent the device from sliding down the lifeline to a position that creates a swing hazard during fall arrest. In all cases, manufacturer's instructions should be followed. |

#### Construction Self-Inspection Guide

|  | Power lines: Minimum 10’ clearance / insulate – de-energize, under 50 kw; over 50 kw – refer to Chapter 155 |
| --- | --- |
|  | Trench/excavation: Any trench four feet or must be sloped, shored or braced |
|  | Guardrails: Any opening four feet or more above ground level must be guarded |
|  | Standard guardrail: Top rail = 39” to 45” above working surface. Midrail = halfway between top rail and floor. Toeboard = 4”. |
|  | Scaffold: Fully planked |
|  | Scaffold: Fall protection provided if fall hazards over 10 feet exist |
|  | Stairs: Four or more risers must have handrails |
|  | Fall protection: Any exposure to fall hazards of 4’ or greater must be eliminated by the use of safety harness/belt, lanyard or lifeline, horizontal lines, or cantenary lines. Positive fall restraint/protection must be utilized at all times. Two lanyards may be necessary at the beam/upright traverse points. No exposure at any time is allowed. |
|  | Fall protection work plan: Job specific, in writing; available on-site for all fall hazards above 10’. |
|  | Open belts and pulleys, chains and sprockets, points of operation must be guarded to prevent accidental contact. Air compressors and electric motor pulleys are the most common hazards. |
|  | Radial saws: Cutting head must return easily to start position when released; blade must not extend past the edge of the worktable; off/on switch should be at front of operator’s position. |
|  | Table saws: Upper hood guard; anti-kickback, push stick, belt and pulley guarded |
|  | Circular saws: Blade guard instantly returns to covering position |
|  | Never wedge or pin a guard. |
|  | Chain saw: Ballistic nylon leg protection; eye, ear, face protection; hard hat |
|  | Angle grinders: 180-degree guard required |
|  | Ladders: Extended 36” above landing and secured to prevent displacement |
|  | Articulating boomlift: Safety harness and lanyard at all times |
|  | Floor holes/openings: Covered and secured; be sure no tripping hazards in the area. |
|  | Extension cords/electric power tools: Marked/covered by Assured Grounding Program |
|  | Clothing: Minimum of short sleeve shirts, long pants, and substantial footwear; no recreational shoes |
|  | Hard hats: readily accessible at all times; worn when overhead hazard exists |
|  | Oxygen/acetylene storage areas: Cylinders chained and separated |
|  | Personal protective equipment: Head, eye, ear, respiratory, and leg protection – high visibility vests when required |
|  | Housekeeping: Workers are responsible for their own area of exposure |
|  | First aid/fire extinguishers: Available and readily accessible |
|  | First aid trained personnel: Minimum of one person on-site at all times with first aid CPR training. |
|  | Accident Prevention Program: In written format |
|  | Crew Leader Meetings: At beginning of each job and at least weekly thereafter. Documented |
|  | Chemical hazard communication program |

|  | Construction Checklist - Safety |
| --- | --- |

| Employer/Business | Date |
| --- | --- |
| Completed By (Name/Title) |  |

| Program Requirements, First Aid, and Housekeeping |  |  | Yes | No | N/A |
| --- | --- | --- | --- | --- | --- |
|  | Is the written APP tailored to hazards associated with work operations? |  |  |  |  |
|  | Are crew leader safety meetings held at the beginning of job and weekly thereafter? |  |  |  |  |
|  | Are safety meetings tailored to the operations? |  |  |  |  |
|  | Are safety meetings documented and attendance taken? |  |  |  |  |
|  | Are safety walk-around inspections conducted at the beginning of the job and weekly thereafter? |  |  |  |  |
|  | Are walk-around inspections documented and available for inspection? |  |  |  |  |
|  | Is there a first aid certified person(s) available onsite at all times and is their certification valid/current? |  |  |  |  |
|  | Are the crew leaders and supervisors first aid trained? |  |  |  |  |
|  | Is proper housekeeping maintained at the jobsite? |  |  |  |  |

| Chemicals/Hazard Communication Globally Harmonized System (GHS) |  |  | Yes | No | N/A |
| --- | --- | --- | --- | --- | --- |
|  | Is a written hazard communication program (HCP) and list of all potentially “hazardous chemicals” or substances available on site? |  |  |  |  |
|  | Is a Safety Data Sheet (SDS) for each hazardous chemical readily available at the job site for emergencies and routine use? |  |  |  |  |
|  | Are all containers of hazardous chemicals labeled properly? |  |  |  |  |
|  | Were employees provided with effective information and training about all chemical hazards, necessary safety measures and HCP requirements? |  |  |  |  |
|  | Do other on-site employers (e.g., subcontractors) know how to access your SDSs and get information about necessary safety measures and your on-site labeling system? |  |  |  |  |

| Personal Protective Equipment (PPE) |  |  | Yes | No | N/A |
| --- | --- | --- | --- | --- | --- |
|  | Are individual hard hats available on site? |  |  |  |  |
|  | Is eye/face protection worn? |  |  |  |  |
|  | Are short sleeved shirt and long pants worn? |  |  |  |  |
|  | Is leg protection used during chainsaw usage? |  |  |  |  |

| Ladders |  |  | Yes | No | N/A |
| --- | --- | --- | --- | --- | --- |
|  | Is the ladder in good repair? |  |  |  |  |
|  | Are defective ladders tagged out of service and/or removed? |  |  |  |  |
|  | Are ladders used for the purpose they were designed? |  |  |  |  |
|  | Is the weight of the ladder within the capacity of the ladder rating? |  |  |  |  |
|  | Is the ladder set up on a firm level surface or is it secured to prevent displacement? |  |  |  |  |
|  | Is the ladder set up at a 4:1 angle? |  |  |  |  |
|  | Are workers carrying tools, materials, equipment in a manner that allows the use of both hands? |  |  |  |  |
|  | Are workers facing the ladder when ascending/descending? |  |  |  |  |
|  | Is the area around top and bottom of ladder free of tripping hazards? |  |  |  |  |
|  | Do ladders extend at least 3 feet above the surface being accessed? |  |  |  |  |
|  | If ladders don’t extend at least 3 feet for persons accessing an upper level, is the top and bottom secure and have other grasping devises been provided? |  |  |  |  |
|  | Is the ladder secure at the top and bottom for the person working from the ladder? |  |  |  |  |

| Fall Protection – Regardless of Height |  |  | Yes | No | N/A |
| --- | --- | --- | --- | --- | --- |
|  | Are employees working above or adjacent to dangerous equipment/machinery? |  |  |  |  |
|  | Are there unguarded floor holes or openings? |  |  |  |  |
|  | Are there any impalement hazards? |  |  |  |  |

| Fall Distances of Four Feet or More |  |  | Yes | No | N/A |
| --- | --- | --- | --- | --- | --- |
|  | Are there unprotected walking or working surfaces? |  |  |  |  |
|  | Are employees protected from falls while constructing a leading edge? |  |  |  |  |
|  | Are there unprotected floor openings and/or skylight openings? |  |  |  |  |
|  | Are wall openings guarded, where the bottom of the opening is less than 39 inches above the working surface? |  |  |  |  |
|  | Are employees protected from falls while placing or tying reinforcing steel on a vertical surface? |  |  |  |  |
|  | Are employees protected from falls when working on steep pitch roofs (greater than 4:12)? (includes roofing work) |  |  |  |  |
|  | Are employees using fall protection while working on low pitch roof (4:12 or less) (doesn’t include roofing or leading edge work)? |  |  |  |  |
|  | Are employees using personal fall restraint or positioning device systems while working on hazardous slopes? |  |  |  |  |

| Fall Distances of Ten Feet or More |  |  | Yes | No | N/A |
| --- | --- | --- | --- | --- | --- |
|  | Is there a written fall protection work plan that addresses fall hazards of 10 feet or more? Is it available onsite |  |  |  |  |
|  | Are employees protected from falls while working on surfaces that are less than 45 inches in all directions (not applicable to ramps, runways, inclined walkways)? |  |  |  |  |
|  | Since fall protection is required for employees standing in or working in the affected area of a trench or excavation exposed to a fall hazard of ten feet or more – and the employees are not directly involved with the excavation, process or the employees are on the protective system or any other structure in the excavation – do all these employees have fall protection? |  |  |  |  |

| Electrical |  |  | Yes | No | N/A |
| --- | --- | --- | --- | --- | --- |
|  | Are extension cords with ground pins used? |  |  |  |  |
|  | Are extension cords free of improper splices? |  |  |  |  |
|  | Are multi-outlet J-Boxes waterproof? |  |  |  |  |
|  | Are ground-fault circuit interrupters or an assured equipment grounding program used? |  |  |  |  |
|  | Are strain relief devices used on flexible cords and cables? |  |  |  |  |

| Guarding Tools and Equipment |  |  | Yes | No | N/A |
| --- | --- | --- | --- | --- | --- |
|  | Are circular/skill saws properly guarded? |  |  |  |  |
|  | Are table saws guarded(hood, spreader, anti-kickback)? |  |  |  |  |
|  | Are radial-arm saws properly guarded? |  |  |  |  |
|  | Are circular power miter saws properly guarded? |  |  |  |  |
|  | Does pneumatic equipment have a safety device on the muzzle? |  |  |  |  |

| Scaffolds |  |  | Yes | No | N/A |
| --- | --- | --- | --- | --- | --- |
|  | Is a competent person qualified in erecting, moving, dismantling, or altering scaffold supervising experienced and trained employees selected by the competent person? |  |  |  |  |
|  | Are working platforms fully planked/decked? |  |  |  |  |
|  | Is safe access provided? |  |  |  |  |
|  | Are guardrails or personal fall arrest systems used to protect employees from falls greater than 10 feet to a lower level? |  |  |  |  |
|  | Are guardrails installed in accordance with the standard along open-sides and ends? |  |  |  |  |
|  | Are scaffolds set up on a firm, level foundation capable of supporting the load? |  |  |  |  |

| Stairways |  |  | Yes | No | N/A |
| --- | --- | --- | --- | --- | --- |
|  | Are permanent or temporary stairs installed in building or structures two or more stories high or twenty-four feet or more in height or depth? |  |  |  |  |
|  | Are the stairways installed in wood frame buildings before studs are raised to support the second or next higher floors? |  |  |  |  |
|  | Are stairways having 4 or more risers or rising more than 30 inches, whichever is less, equipped with at least one handrail and one stair rail system along each unprotected side or edge |  |  |  |  |
|  | When there is a break in elevation of 19 inches or more, is a stairway, ramp or ladder provided? |  |  |  |  |

| Ramps, Runways, Inclined Walkways |  |  | Yes | No | N/A |
| --- | --- | --- | --- | --- | --- |
|  | Is the ramp, runway or inclined walkway at least 18 inches wide? |  |  |  |  |
|  | Is the ramp, runway or inclined walkway slanted less than 20 degrees on the horizontal surface and is it cleated or treated to prevent a slipping hazard on the walking surface? |  |  |  |  |
|  | Is the ramp, runway, or inclined walkway equipped with a standard guardrail along each open side where there’s a fall of 4 feet or more? |  |  |  |  |

| Lead |  |  | Yes | No | N/A |
| --- | --- | --- | --- | --- | --- |
|  | Has the job site been reviewed for potential lead-containing paint and other materials that could be released in the air by grinding, sanding, welding, cutting, abrasive blasting or other work activities? |  |  |  |  |
|  | Did you determine if any employee may be exposed to lead at or above the action level (e.g. by conducting air monitoring)? |  |  |  |  |
|  | Have you notified each employee in writing of the results which represent that employees’ airborne exposure to lead? |  |  |  |  |
|  | Have you established and implemented a written compliance program? |  |  |  |  |
|  | Have you developed, implemented and maintained a written respiratory protection program? |  |  |  |  |
|  | Have you selected and provided appropriate respirators for each employee required to use one? |  |  |  |  |
|  | Have you provided, at no cost to the employee, appropriate protective work clothing and equipment? |  |  |  |  |
|  | Have you instituted a medical surveillance program for employees exposed to airborne lead above the action level? |  |  |  |  |

| Asbestos |  |  | Yes | No | N/A |
| --- | --- | --- | --- | --- | --- |
|  | Did you obtain a Good Faith Inspection report, or a written statement, from the property owner or owner’s agent that identifies any asbestos on the property? |  |  |  |  |
|  | Did you share the written report or statement with other contractors or employers hired to work on site? |  |  |  |  |
|  | If Asbestos Containing Material (ACM) and/or Presumed Asbestos Containing Material (PACM) is going to be disturbed: has a qualified company (e.g., a certified asbestos contractor) been hired? |  |  |  |  |
|  | If the work is an “asbestos project”, was L&I (and the local clean air agency) notified about the work? |  |  |  |  |
|  | Have you informed employees in proximity to any work activity where asbestos is being abated or disturbed about asbestos hazards and safety precautions? |  |  |  |  |

##### Equipment Safety Inspection Checklist

Date: __________________________________________

Project: _________________________________________

Equipment: ______________________________________

| All guards and fenders | _____ | OK | _____ | Needs Repair |
| --- | --- | --- | --- | --- |
| Brakes | _____ | OK | _____ | Needs Repair |
| Lights – front, rear, side, dash | _____ | OK | _____ | Needs Repair |
| Back-up alarm – horn | _____ | OK | _____ | Needs Repair |
| Ladders, stairs, hand holds | _____ | OK | _____ | Needs Repair |
| ROPS (Roll-over protection) | _____ | OK | _____ | Needs Repair |
| Seat belts | _____ | OK | _____ | Needs Repair |
| Fire extinguisher | _____ | OK | _____ | Needs Repair |
| Glass | _____ | OK | _____ | Needs Repair |
| Tires | _____ | OK | _____ | Needs Repair |
| Electrical cords | _____ | OK | _____ | Needs Repair |
| Ground fault circuit interrupters | _____ | OK | _____ | Needs Repair |
| Electrical hand tools | _____ | OK | _____ | Needs Repair |
| Powder actuated tools | _____ | OK | _____ | Needs Repair |
| Pneumatic condition of all hand tools | _____ | OK | _____ | Needs Repair |

Other Items Checked:

| Oil level and leaks | ___ | OK | ___ | Needs Repair | ___ | Add | ___ | Change |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hydraulic oil level and leaks | ___ | OK | ___ | Needs Repair | ___ | Add | ___ | Change |
| Anti-freeze level and leaks | ___ | OK | ___ | Needs Repair | ___ | Add | ___ | Change |
| Fuel level and leaks | ___ | OK | ___ | Needs Repair | ___ | Add | ___ | Change |
| First aid kit | ___ | OK | ___ | Needs Repair | ___ | Add | ___ | Change |

Repaired by: _________________________________________

Checked by: __________________________________________

JOB SAFETY ANALYSIS WORKSHEET

| TITLE OF JOB OPERATION: _______________________________ |  | Date: ______________ |
| --- | --- | --- |
| Title of person who does job: _____________________________________________________ |  |  |
| Employee observed: _________________________ | Location: ________________________ |  |
| Analysis made by: ___________________________ | Analysis approved by: ______________ |  |

| Sequence of basic job steps | Potential injuries or hazards | Recommended safe job procedures |
| --- | --- | --- |

Personal protective equipment required for this position:

Chemical Hazard

Communication Program

A. Company Policy

Miami Technology Solutions is committed to the prevention of exposures that result in injury and/or illness; and to comply with all applicable state health and safety rules.  To make sure that all affected employees know about information concerning the dangers of all hazardous chemicals used by  Miami Technology Solutions, the following hazardous chemical communication program has been established.

All work units of Miami Technology Solutions will participate in the hazard communication program.  This written program will be available in Miami Technology Solutions Field Office at 120 Day Street, Roanoke VA  for review by any interested employee.

B. Container Labeling

Mike Main, Project Superintendent  is responsible for container labeling procedures, reviewing, and updating.  The labeling system used at Miami Technology Solutions  is as follows: Label per OSHA regulations

It is the policy of Miami Technology Solutions  that no container will be released for use until the above procedures are followed.

C. Material Safety Data Sheets (MSDS)

Mike Main, Project Superintendent  is  responsible to establish and monitor the employer’s MSDS program.  This person will make sure procedures are developed to obtain the necessary SDSs and will review incoming MSDSs for new or significant health and safety information.  This person will see that any new information is passed on to affected employees.

Copies of MSDSs for all hazardous chemicals in use will be kept in Miami Technology Solutions Field Office at 120 Day Street, Roanoke VA. MSDSs will be available to all employees during each work shift.  If an MSDS is not available or a new chemical in use does not have an MSDS, immediately contact: 
Mike Main, Project Superintendent

Note:

If an alternative to printed Material Safety Data Sheets is used (such as computer data), provide a description of the format.

D. Employee Information and Training

Mike Main, Project Superintendent   is responsible for the employee training program

Mike Main, Project Superintendent  will make sure that before starting work, each new employee of Miami Technology Solutions   will attend a health and safety orientation that includes information and training on the following:

• An overview of the requirements contained in the Hazard Communication Standard.

• Hazardous chemicals present at his or her work places.

• Physical and health risks of the hazardous chemical.

• The symptoms of overexposure.

• How to determine the presence or release of hazardous chemicals in his or her work area.

• How to reduce or prevent exposure to hazardous chemicals through use of control procedures, work practices, and personal protective equipment.

• Steps the employer has taken to reduce or prevent exposure to hazardous chemicals.

• Procedures to follow if employees are overexposed to hazardous chemicals.

• How to read labels and review MSDSs to obtain hazard information.

• Location of the MSDS file and written hazard communication program.

• An overview of the requirements contained in the Hazard Communication Standard.

Before introducing a new chemical hazard into any section of this employer, each employee in that section will be given information and training as outlined above for the new chemical.

E. Hazardous non-routine tasks

Periodically, employees are required to perform hazardous non-routine tasks.  (Some examples of non-routine tasks are confined space entry, tank cleaning, and painting reactor vessels.)  Non-routine tasks that are performed at Miami Technology Solutions  include:

F. Multi-employer work places

It is the responsibility of Mike Main, Project Superintendent   to provide employers of any other employees at the work site with the following information:

• Copies of MSDSs (or make them available at a central location) for any hazardous chemicals that the other employer(s)’ employee may be exposed to while working.

• Inform other employers of any precautionary measures that need to be taken to protect employees during normal operating conditions or in foreseeable emergencies.

• Provide other employers with an explanation of the labeling system that is used at the work site.

It is also the responsibility of Mike Main, Project Superintendent  to identify and obtain MSDSs for the chemicals the contractor is bringing into the work place.

G. List of hazardous chemicals

The following is a list of all known hazardous chemicals used by our employees.  Further information on each chemical may be obtained by reviewing MSDSs located at  Miami Technology Solutions Field Office located at 120 Day Street Roanoke, VA

MSDS identity:

Chemical Name		Manufacturer 			Location Used

(Insert information here)          (Insert information here)          (Insert information here)

Hazard Communication checklist

| ____ | 1. | Have we prepared a list of all the hazardous chemicals in our workplace? |
| --- | --- | --- |
| ____ | 2. | Are we prepared to update our hazardous chemical list? |
| ____ | 3. | Have we obtained or developed a safety data sheet for each hazardous chemical we use? |
| ____ | 4. | Have we developed a system to ensure that all incoming hazardous chemicals are checked for proper labels and data sheets? |
| ____ | 5. | Do we have procedures to ensure proper labeling or warning signs for containers that hold hazardous chemicals? |
| ____ | 6. | Are our employees aware of the specific information and training requirements of the Hazard Communication Standard? |
| ____ | 7. | Are our employees familiar with the different types of chemicals and the hazards associated with them? |
| ____ | 8. | Have our employees been informed of the hazards associate with performing non-routine tasks? |
| ____ | 9. | Are employees trained about proper work practices and personal protective equipment in relation to the hazardous chemicals in their work area? |
| ____ | 10. | Does our training program provide information on appropriate first aid, emergency procedures, and the likely symptoms of overexposure? |
| ____ | 11. | Does our training program include an explanation of labels and warnings that are used in each work area? |
| ____ | 12. | Does the training describe where to obtain data sheets and how employees may use them? |
| ____ | 13. | Have we worked out a system to ensure that new employees are trained before beginning work? |
| ____ | 14. | Have we developed a system to identify new hazardous chemicals before they are introduced into a work area? |
| ____ | 15. | Do we have a system for informing employees when we learn of new hazards associated with a chemical? |

Hazardous Substances

###### Employee Orientation Checklist

Employee Name:  (Add Name of Employee here)

Title:  (Add title of employee here)                            Date hired:  (Add Date Hired here)

Trainer Name:  Mike Main

This checklist is to inform employees of Miami Technology Solutions of its Hazard Communication Program.  Place a check in each box to indicate that the subject has been covered.

The supervisor has reviewed the following information with the employee:

1.  The purpose of the hazard communication standard is to require chemical manufacturers or importers to assess the hazards of chemicals they produce or import.  All employers must provide information to their employees about the hazardous chemicals to which they may be exposed.

Employees must be informed about the hazard communication program, labels and other forms of warning, and safety data sheets, and they must have training on the hazardous substances they may encounter.

2.  The supervisor has reviewed the hazardous chemical list with the employee.

3.  The supervisor has shown the employee the following:

Location of hazardous chemicals within the employee’s work site.

Location of the written Hazard Communication Program.

Location of the safety data sheets for all hazardous chemicals in the employee’s assigned work area.

Location of the list of person(s) trained and authorized to handle the hazardous chemicals.

The signature below documents that the appropriate elements have been talked over to the satisfaction of both parties and that both the supervisor and employee accept responsibility for maintaining a safe and healthful work environment.

Date:  (Enter date of orientation)    Supervisor’s signature: _______________________

Date:  (Enter date of orientation)    Employee’s signature: ________________________