Hazard Waste

Management Plan

Repair Fire Evacuation – Interior Finishes

Building #410

Homestead ARB, Florida

Revision 0

Prepared for:

Toole Corp

Pembroke Park, Florida

October 2013

TABLE OF CONTENTS

PAGE

1. GENERAL OVERVIEW …………..................................................................................... 1

2. EMPLOYEE TRAINING…….……................................................................................... 1

3. HAZARD DETERMINATION....................................................................................... 2

4. MATERIAL SAFETY DATA SHEETS ………………………………......................................... 2

5. CONTAINER LABELING ….......................................................................................... 3

6.STORAGE AND HANDLING …...………………….............................................................. 3

7. TRANSPORTING …….................................................................................................. 4

8. WASTE MINIMIZATION.….…………............................................................................ 4

9. OIL MANAGEMENT …...……………………………............................................................. 5

10. TREATING WASTE TO MEET LDR …………............................................................... 6

11. NON ROUTINE TASKS ……………............................................................................... 6

12. MSDS INFO …………...……………………………................................................................ 7

LIST OF ATTACHMENTS/APPENDICES

ATTACHMENTS

HAZARD TRAINING CHECKLIST …………..………………………..................... PAGE 8

REQUEST FOR MSDS ……………………………………....................................... PAGE 9

PROGRAM/TRAINING DOCUMENT …..................................................... PAGE 10

EXPLANATION OF TERMS USED ON MSDS ………..………………………….....PAGE 11

WORK AREA HAZARD SUBTANC LIST ………………………………….............. PAGE 13

APPENDICES

INVENTORY LOG …………..………………………....................................... APPENDIX F

UNIFORM HAZARDOUS WASTE MANIFEST ………………………………..APPENDIX G

HAZARD WASTE MANAGEMENT PROGRAM

GENERAL INFORMATION

In order to comply with 29 CFR 1926.59, Hazard Communication, the following written Hazard

Waste Management Program established by Toole Corp for the above project at HARB. All work units of the company are included within this program. The written program will be available in the ofﬁce, located at 3001 W. Hallandale Bch Blvd. Ste. 316.,Pembroke Park, FL 33009 and through a superintendent for review by any interested employee.  Quality Control Manager is responsible for the implementation and ongoing compliance with the program.

EMPLOYEE TRAINING AND INFORMATION

The Job Superintendent Kelly Rivas is responsible for the employee-training program. She will ensure that all elements speciﬁed below are carried out.

Prior to starting work each new employee of Toole Corp will attend a health and safety orientation and will receive information and training on the following:

- An overview of the requirements contained in the Hazard Communication Standard.

- Chemicals present in their workplace operations

- Location and availability of our written hazard communication program,

- Physical and health effects of the hazardous chemicals.

- Methods and observation techniques used to determine the presence or release of hazardous chemicals in the work area.

- How to lessen or prevent exposure to these hazardous chemicals through usage of control/work practices and personal protective equipment.

- Steps Toole Corp has taken to lessen or prevent exposure to these chemicals.

- Emergency procedures to follow if they are exposed to these chemicals.

- How to read labels and review MSDSS to obtain appropriate hazard information.

After attending appropriate training, each employee will sign a Hazard Training Checklist form to verify that they received and understood the training, procedures and policies within Toole Corp’s Hazard Waste Management Program.

Prior to a new chemical hazard being introduced into any section of this company, each employee of that section will be given information as outlined above. Job Superintendent or other designated person is responsible for ensuring that MSDS on the new chemicals are available.

Written Hazard Waste Management Program

It is the policy of Toole Corp, that the first consideration in the performance of work shall be the protection of the safety and health of all employees. The company has developed this Hazard Waste Management Program to ensure that all employees receive adequate information relevant to the possible hazards that may be involved with the various hazardous substances used in the company's operations and processes. The following program outlines how we will accomplish this objective.

SCOPE

This policy covers all potential workplace exposures involving hazardous substances as deﬁned

by federal, state and local regulations.

HAZARD DETERMINATION

The company does not intend to evaluate any of the hazardous substances purchased from suppliers and/ or manufacturers but have chosen to rely upon the evaluation performed by the suppliers or by the manufacturers of the substances to satisfy the requirements for hazard

determination.

STEPS FOR WASTE DETERMINATION

• Check the Material Safety Data Sheet for product information.

Refer to MSDS Data Sheets

• Check with  supplier/manufacturer.

• Read the product label.

• Compare product information with the listed wastes and hazardous waste

characteristics in 40 CFR 261

LIST OF HAZARDOUS SUBSTANCES

We will compile, annually review, and update as necessary a complete inventory of all substances present in our facilities or work sites. The name of those materials determined to be hazardous are defined in applicable federal and state standards. See Appendix F for Inventory Log.

MATERIAL SAFETY DATA SHEETS (MSDS)

Each location must maintain a master MSDS ﬁle as well as a department-speciﬁc ﬁle. These Material Safety Data Sheets are available to all employees, at all times, upon request.

The Safety Committee or a designee will be responsible for reviewing all incoming MSDS for new and signiﬁcant health/safety information (the company will ensure that any new information is passed on to the employees involved).

The Quality Control Manager will review all incoming MSDS for completeness. If any MSDS is missing or obviously incomplete, a new MSDS will be requested from the manufacturer or distributor. OSHA or designated AF official is to be notiﬁed if the manufacturer or distributor will not supply the MSDS or if it is not received after 30 days from request. Any new information will be passed on to employees involved.

New materials will not be introduced into the work area until an MSDS has been received. The purchasing department will make it an ongoing part of its function to obtain MSDS for all new materials when they are first ordered. The safety coordinator or his or her designee shall coordinate with appropriate departments to make sure all MSDS are obtained, distributed and communicated.

CONTAINER LABELING

- No container or hazardous substances will be released for use unless the container is correctly labeled and the label is legible.

- All chemicals in bags, drums, barrels, bottles, boxes, cans, cylinders, reaction vessels, storage tanks, or the like will be checked by the receiving department to ensure the manufacturer's label is intact, is legible, and has not been damaged in any manner during shipment.

- Any containers found to have damaged labels will be quarantined until a new label has been installed.

- The label must contain the chemical name of the contents, the appropriate hazard warnings, manifest document number, shipping name and identification number, the name and address of the manufacturer, DOT shipping name and identification number, and any other information required.

- Each container must be marked “Hazardous Waste.”

- All secondary containers shall be labeled. The information must include details of all chemicals that are in the referenced container.

- Specific packaging and labelling requirements are listed in 49 CFR.

STORAGE AND HANDLING

STORAGE CONTAINERS

When handling and storing hazardous waste, you should establish good

housekeeping practices to avoid possible spills. Follow these requirements for

handling storage containers:

• Containers must be in good condition.

• Replace leaking containers.

• Containers must be compatible with the hazardous waste stored in them, and

must meet U.S. Department of Transportation standards.

• Containers holding hazardous waste must be closed during storage.

• Do not handle containers in a haphazard way that could cause leaking or rupture.

• Inspect containers at least weekly to check for leaks and signs of corrosion.

• Incompatible wastes must not be placed in the same container. This can cause

fires, leaks or other reactions.

• Mark each container with the date that storage began and with the words “Hazardous Waste.”

STORAGE TANKS

Rather than store hazardous waste in containers, storage tanks may be used. To safely manage waste in storage tanks follow these requirements:

• Do not place wastes in tanks if the wastes can cause the tank to fail. For example, incompatible wastes must not be placed in the same tank.

• Reactive and ignitable wastes must be stored or treated in a way that will protect them from reacting or igniting.

• Uncovered tanks must have at least two feet of freeboard or have secondary

containment to prevent spillage.

• Every day inspect discharge control equipment and data gathered from monitoring equipment, and measure the level of waste to maintain compliance with the freeboard requirement. Every week inspect the tank and the area immediately surrounding the tank to detect corrosion or obvious signs of leakage.

• Equip tanks that have an automatic waste feed with discharge control equipment.

• Mark each tank with the date storage began and the words “Hazardous Waste.”

TRANSPORTING

Disposal of hazardous waste in solid waste landfills is prohibited in Florida. Hazardous waste must be shipped to a permitted hazardous waste recycling, treatment or disposal facility. All hazardous wastes must be shipped under a Hazardous Waste Manifest (see Appendix G)

For project waste disposal and handling procedures see “Waste Management Plan”.

THE UNIFORM HAZARDOUS WASTE MANIFEST

Refer to Appendix G - Uniform Hazardous Waste Management Manifest

The Uniform Hazardous Waste Manifest is a special shipping document that must accompany hazardous waste shipments. The state of Florida requires the use of this manifest except when hazardous waste is recycled.

The following items must be completed on the Manifest:

  - Item #1 Your EPA/DEP identification number

  - Item #2 Generator name and address

  - Item #3 Transporter and its ID number

  - Item #4 Designated facility name, site address

  - Item #5 Designated facility ID number

  - Item#6  Enter the company name and site address of the facility to which you are shipping the waste listed on the manifest. The address must be the site address, which may be different from the company mailing address.

  - Item #7 U.S. Department of Transportation (U.S. DOT) description. Enter the U.S. DOT proper name, hazard class and UNNA (United Nations/North American) number for each waste identified.

WASTE MINIMIZATION

The easiest and most cost-effective way of managing any waste is not to generate it in the first place.  Decrease the amount of hazardous waste by developing “good housekeeping” habits.  Good housekeeping procedures generally save money and prevent accidents and waste.  The following are good practices for housekeeping:

- Do not mix wastes (i.e. solvents and oils, or oil and antifreeze, etc.).

- Never hose down a spill.

- Never pour hazardous substances down any type of drain.

- Change materials, processes or both.

- Recycle and reuse manufacturing materials.

- Safely store hazardous products and containers.

- No smoking or eating in areas where hazardous materials or hazardous wastes are present.

- Make a good faith effort.

OIL MANAGEMENT PROCEDURES

OPERATING RECORD AND RECORDS MANAGEMENT

A written record of hazardous waste-related operation and other records that must be retained will be collectively referred to as the “Operating Record.”  The required information will be kept with the Quality Control Manager.

PREPAREDNESS AND PREVENTION PLAN

These requirements are intended to ensure that your employees are adequately prepared to handle hazardous waste and to respond to any emergencies that might arise. Whenever hazardous waste is stored on site, potential risks from fires, explosions, and other accidents.

PREPAREDNESS AND PREVENTION PLAN REQUIREMENTS

  - Maintain and operate to minimize the possibility of fire, explosion or any unplanned release of hazardous waste into the environment.

  - Be equipped with a telephone or similar communication device to summon help.

  - Have fire extinguishers and spill-control equipment on hand. This equipment must be tested and maintained to assure proper functioning.

  - All persons involved in the handling operation of hazardous waste must have immediate access to either internal or external alarm or communication equipment.

  - Maintain sufficient aisle space in all work areas to allow the unobstructed movement of personnel and equipment in case of an emergency.

  - Familiarize police, fire departments, state emergency response teams and hospitals of the type of waste handled at the facility.

CONTINGENCY PLANS AND EMERGENCY PROCEDURE REQUIREMENTS

  - An emergency coordinator must be on-site or on call at all times.

  - Next to the telephone post the name and phone number of the emergency coordinator, the location of fire extinguishers and spill-control material and the phone number of the fire department.

  - Ensure that all employees are thoroughly familiar with proper waste handling and emergency procedures.

  - The designated emergency coordinator must respond to any emergencies that arise.

  - When an emergency occurs serious enough to warrant help from the fire department (or you have knowledge of a hazardous waste spill that could threaten the public health or the environment) the following steps are required:

- Notify Fire Department;

- Notify the Environmental Compliance Officer and Emergency Coordinator;

- Notify the Division of Emergency Management State Warning Point with Department of Community Affairs;

- File a report with the appropriate DEP district office; and

- Notify the National Response Center Document the incident onto the Emergency Response Ticket.

TREATING WASTE TO MEET LAND DISPOSAL RESTRICTIONS (LDR’S)

Most hazardous wastes may not be land disposed unless they meet “treatment standards.” The Land Disposal (LDR) program requires that the waste is treated to reduce the hazardous constituents to levels set by EPA, or that the waste treated using a specific technology.  Waste must be treated to meet LDR treatment standards before it is land disposed.

NON-ROUTINE TASKS

Infrequently, employees may be required to perform non-routine tasks that involve the use of hazardous substances. Prior to starting work on such projects, each involved employee will be given information by his or her supervisor about hazards to which they may be exposed during such an activity. This information will include:

- The specific hazards.

- Protective/safety measures that must be utilized.

- The measures the company has taken to lessen the hazards, including special ventilation, respirators, the presence of another employee, air sample readings, and emergency procedures.

PLAN ADMINISTRATION

This Hazard Communication program will be monitored by

Questions regarding this program should be directed to the

Signature

*Title

Date

*This document must be approved and signed by the senior executive on s

MSDS INFORMATION

OSHA safe work practices outline the content, but not the exact form, of every Material Safety

Data Sheet. Here is what OSHA requires each data sheet to contain:

- IDENTITY. The data sheet must contain the name of the chemicals found on the label. In

addition, subject to deletion of legitimate trade secrets, it must give the chemical and common name of the substance. If the substance is a mixture and has not been tested as such, the data sheet must give the name of each hazardous constituent.

- CHARACTERISTICS. The data sheet must recite the physical and chemical characteristics of the chemical, such as vapor pressure, ﬂash point, etc.

- PHYSICAL HAZARDS. Any potential for ﬁre, explosion or reaction must be included in the data sheet.

- HEALTH HAZARDS. Signs and symptoms of exposure must be entered, as must all medical conditions that are likely to be aggravated by exposure.

- ROUTES OF ENTRY. The data sheet must specify whether the chemical typically enters the system by ingestion, inhalation, dermal exposure or some other route.

- EXPOSURE LIMITS. If OSHA has established an exposure limit for the chemical, or if a

Threshold Limit Value has been established by the American Conference of Governmental

Industrial Hygienists, these must be entered on the data sheet, as must any exposure limit used by the authority preparing the data sheet.

- CARCINOGENS. The data sheet must indicate whether the chemical is listed as a carcinogen by the National Toxicology Program, by OSHA, or by the International Agency for Research in Cancer.

- USE AND HANDLING. The data sheet must recite any general applicable precautions for safe handling and use that are known to the ﬁrm preparing the data sheet, including hygiene practices, protective measures during repair and maintenance of contaminated equipment and procedures for clean-up of spills and leaks. Industrial chemical consumers often might add site-speciﬁc procedures to the more general information offered by the chemical manufacturer.

- EXPOSURE CONTROLS. The data sheet must include a description of special procedures to be employed in emergencies, as well as a description of appropriate first aid.

- DATES. The sheet must bear the date of its preparation or of its latest revision.

- INFORMATION SOURCE. Finally, the sheet must recite the name, address and telephone number of the person who prepared the data sheet or of some other person who can provide additional information relating to the chemical, such as citations to scientific literature or specialized emergency procedures.

HAZARD COMMUNICATION TRAINING CHECKLIST

Has the employee been informed of and trained in the following:

- Information: Has the employee been informed of the following‘?-YES—NO

  - The requirements of this section.;--

  - Any operation in the work area where hazardous substances are present.;--

  - The location of the written Hazard Communication Program;-—

  - Availability of the written program.;--

  - Location and availability of hazardous substances list(s).;--

  - Location and availability of Material Safety Data Sheets.;--

- Training: Has the employee been trained in the following?;--

  - Methods and observations that may be used to detect the presence or release of hazardous substances in the work areas.;--

  - The physical and health hazards of the substances in the work areas.;--

  - How employees can protect themselves from these hazards.;—-

  - Procedures the employer has implemented for employee protection.---

  - Appropriate work practices.---

  - Emergency procedures.--- .

  - Personal protective equipment to be used.---

  - Explanation of labeling systems.---

  - Explanation of material safety data sheets.---

  - How employees can obtain and use appropriate hazard information.---

  - Personal hygiene when working with substances.---

  - General first aid for Contact with hazardous substances.---

Employee’s Signature 							Date

Manager's Signature 							Date

REQUEST FOR MATERIAL SAFETY DATA SHEETS

Date of Request

Department

To

From

I hereby request that I be given the Material Safety Data Sheets on the following hazardous

substance(s):

Date Received

Acknowledged by _________                                 (Requesting Employee)

Dept. Manager

Date

cc: Corporate Safety Department

PROGRAM/TRAINING DOCUMENT

TRAINING ACKNOWLEDGMENT

I have received information on the Hazard Communication Standard 29 CFR l910.1200 or the appropriate state standard and understand how to interpret and to use the labelling systems and

Material Safety Data Sheets (MSDSS) that are in use and accessible to me in my work area. I agree to observe and follow the safe work practices as presented to me in the training sessions I attended on											 at

Employee Signature 			Date

The above named employee has been informed and instructed by				    work

practices, chemical hazards recognition, interpretation and use of chemical labels, MSDSS, the

CFR 29, 191 O. 1200 (e) or appropriate state standard and the location at which these items are

accessible to the employee.

Supervisor 				Date

EXPLANATION OF TERMS USED ON MATERIAL SAFETY DATA SHEETS

SECTION I

Chemical Name and Synonyms—The product identiﬁcation. The chemical or generic name of single elements and compounds.

Trade Names and Synonyms—The name under which the product is marketed and the

common commercial name of the product.

Chemical Family—Refers to a grouping of chemicals that behave and react with other

chemicals in a similar manner.

Formula—The chemical formula or single elements or compounds.

CAS Number—The Chemical Abstracts Service number, if applicable.

EPA—The code number assigned by the Environmental Protection Agency, if applicable.

DOT Classification——The appropriate classiﬁcation as determined by the regulations of the

Ofﬁce of Hazard Material, Department of Transportation.

SECTION II

Hazardous Ingredients—The major components as well as any minor one(s) having potential for harm that are considered when evaluating the product.

TLV—Threshold Limit Value (TLV) indicates the permissible exposure concentration, a limit established by a government regulatory agency, or an estimate if none has been established.

SECTION III

Physical Data

Boiling Point (°F)—The temperature in degrees Fahrenheit at which the substances will boil.

Vapor Pressure—The pressure of saturated vapor above the liquid expressed in mm Hg at 20°

C.

Vapor Density—The relative density or weight of a vapor or gas (with no air present) compared with an equal volume of air at ambient temperature.

Solubility in Water—The solubility of a material by weight in water at room temperature. The terms negligible, less than 0.1 percent, 0.1 to 1 percent; moderate 1 to 10 percent, applicable 10 percent or greater.

Appearance and Odor—The general characterization of the material, i.e., powder, colorless

liquid, aromatic odor, etc.

Specific Gravity (H20=1)—The ratio of the weight of a volume of the material to its weight of

an equal volume of water.

Percent, Volatile by Volume (%)—The percent by volume of the material that is considered

volatile. (The tendency or ability of a liquid to vaporize.)

Evaporation Rate-—The ratios of the time required to evaporate a measured volume of a liquidto the time required to evaporate the same volume of a reference liquid (ethyl ether) under ideal test conditions. The higher the ratio, the slower the evaporation rate.

SECTION IV

Flash Point (Method Used)—The temperature in degrees Fahrenheit at which a liquid will give off enough ﬂammable vapor to ignite in the presence of a source of ignition.

SECTION V

Conditions to Avoid—Conditions that, if they exist with the substance present, could cause it to become unstable.

Incompatibility (Materials to Avoid)—Materials that will react with the substance.

Hazardous Decomposition Products—Refers to that reaction that takes place at a rate that

releases large amounts of energy. Indicates whether or not it may occur and under what storage

conditions.

SECTION VI

Health Hazard Data—Possible health hazards as derived from human observation, animal

studies or from the results of studies with similar products.

Threshold Limit Value (TLV)—The value for airborne toxic material that are to be used as

guides in the control of health hazards and represent concentrations to which nearly all workers

may be exposed eight hours per day over extended periods of time without adverse effects.

Effects of Overexposure—The effects on or to an individual who has been exposed beyond the speciﬁed limits.

Emergency and First-Aid Procedures—Gives ﬁrst-aid and emergency procedures in case of eye and/or skin contact, ingestion and inhalation.

SECTION VII

Stability—Whether the substance is stable or unstable, an unstable substance is one that will

vigorously polymerize, decompose, condense, or will become self—reactive under conditions of

shock, pressure, or temperature.

A copy of the form you may want to use to list your hazardous substances by work area follows

this page. This information would be based on the initial survey and subsequent hazard

determination.

SECTION VIII

Spill or Leak Procedures—Steps to be taken if material is released or spilled. Method and

materials to use to clean up or contain.

Waste Disposal Method—Method and type of disposal site to use.

SECTION IX

Special Protection Information

Respiratory Protection—Speciﬁc type should be speciﬁed, i.e., dust mask, NIOSH-approved

cartridge respirator with organic-vapor cartridge.

Ventilation—Type of ventilation recommended, i.e., local exhaust, mechanical, etc.

Protective Gloves—Refers to the glove that should be worn when handling the product, i.e.,

cotton, rubber.

Eye Protection—Refers to the type of eye protection that is to be worn when handling or around the product.

Flammable Limits—The range of gas or vapor concentration (percent by volume in air) that

will burn or explode if an ignition source is present. (Lel) means the lower explosive limits and

(Uel) the upper explosive limits given in percent.

Extinguishing Media—Speciﬁes the ﬁre-ﬁghting agent(s) that should be used to extinguish

ﬁres.

Special Fire-Fighting Procedures/Unusual Fire and Explosion Hazards—Refer to special procedures required if unusual fire or explosion hazards are involved.

WORK AREA HAZARDOUS SUBSTANCE LIST

Work Area:

Chemical Identity

Label/Special Information 		CAS # or Serial # 		Vendor

MATERIAL SAFETY DATA SHEETS

(Place into this section those that apply to your company.)