The Safety and Health Policy and Plan

TOOLE CORP

Phone: 786.339.8089

Fax: 305.397.0355

Primary NAICS: 236220

Reviewed and Revised

| Date: | Signature: |
| --- | --- |

TABLE OF CONTENTS

PAGE

1. SAFETY POLICY ……....................................................................................................... 4

2. RESPONSIBILITIES ....................................................................................................... 5

3. SAFETY WORK PRACTICES ......................................................................................... 10

4. EMERGENCY ACTION PLAN ........................................................................................ 12

5. HAZARD COMMUNICATION PROGRAM….................................................................... 19

6. LOCKOUT/TAGOUT ……………….................................................................................... 24

7. PERSONAL PROTECTIVE EQUIPMENT........................................................................ 40

8. FALL PROTECTION SAFETY PROGRAM....................................................................... 45

9. SAFETY COMMITTEE.................................................................................................... 57

LIST OF APPENDICES

APPENDIX

CONSTRUCTION INSPECTION FORM……………………........................................ B-1

OSHA INJURY AND ILLNESS REPORT............................................................. B-2

TOOLE CORP  					Safety Policy

The management of this organization is committed to providing employees with a safe and healthful workplace. It is the policy of this organization that employees report unsafe conditions and do not perform work tasks if the work is considered unsafe. Employees must report all accidents, injuries, and unsafe conditions to their supervisors. No such report will result in retaliation, penalty, or other disincentive.

Employee recommendations to improve safety and health conditions will be given thorough consideration by this company. Management will give true attention to and provide the financial resources for the correction of unsafe conditions. Management will promote and influence safe behavior. This will be accomplished by both positive reinforcement of correct and safe activity, and by disciplinary action for those who wilfully or repeatedly work in an unsafe manner.

Disciplinary action will take the form of

1) Written warning or,

2) Written warning and suspension without pay or,

3) Termination of employment.

Management reserves the right to terminate the employment of any employee at any time for violation of company policies.

Management will participate in establishing and maintaining an effective safety program. This will include the following:

- Holding all management and supervisory staff accountable for their safety responsibilities in their respective departments, jobs, crews or workplaces;

- Providing safety and health education and training as needed; and

- Reviewing and updating workplace safety policies, practices and performances.

This policy statement serves to express this company’s commitment to and involvement in providing our employees a safe and healthy workplace. This workplace safety and health program will be incorporated as the standard of practice for this organization. Compliance with these safe practices and those of any regulatory agency will be required of all employees as a condition of continued employment.

Curtis Morrow									October 1, 2013

Signature of Principal									 Date

TOOLE CORP  						Safety Plan

Responsibilities

Senior Managers / Managers

- Ensure that safety is adequately budgeted for the department, job, etc.

- Communicate safe work practices regularly within the department.

- Attend departmental and company-wide safety meetings.

- Formally recognize outstanding safety performance by any/all personnel.

- Assist the Supervisor/Superintendent or any other personnel with the safety process as needed or as requested. This can include formal worksite periodic inspections.

- Uphold and enforce all known safe work practices.

- Maintain all OSHA 300 and 300A forms

Supervisors / Superintendents

- Ensure new-hire orientation is given to new employees, or is followed up at the work level o Ensure employees are given training that includes safe work practices on equipment, tools, machines, processes, etc.

- Personally conduct--or designate a qualified personnel to conduct-- regular inspections of the workplace

- Conduct frequent (daily) work discussions prior to the start of work that include safe work practices

- Uphold and enforce safe work practices. This includes influencing safe behavior by positive reinforcement such as recognition of worker's safe work performance, and/or monetary or gift awards for safe behavior. Enforcement action can also influence safe behavior when applied towards workers who blatantly perform unsafe acts, or who continually perform in an unsafe manner

- Investigate all incidents and take immediate corrective action to prevent re-occurrence

- Provide safety meetings on a regular basis and require attendance of all workers

- Report any and all accidents immediately as per OSHA form 301 (See attachment A-2)

All Employees

- Are to follow safe work practices, and if they are unsure of what is the correct/safe way to perform a task or a job, they are to ask their foreman, supervisor or manager

- Must immediately report all unsafe equipment or tools to their foreman, supervisor or manager. This includes reporting unsafe behavior of other workers, if these workers are approached and remain unwilling to correct their unsafe actions or conditions.

- Are to uphold the safe work practices this company has established

- If injured on the job, or become ill, immediately inform their supervisor, foreman or manager

WORKSITE ANALYSIS

- All work areas, departments, and jobs need to be inspected on a regular basis to ensure safe work practices and safe and healthy conditions. For the most part, these inspections are to be conducted by the Supervisor/Superintendent or his/her qualified and designated worker. Each inspection may not be required to be formal (written) although regular written completed inspections will be expected.

- This includes the purchase of new equipment or tools, or the re-working or retrofitting of workstations or equipment so as to ensure that safety and health is considered.

- This can include the assessment of a workstation or process that may need to be fitted to

the worker (ergonomics) so as to avoid injury or illness.

- If approached by workers who appear to have a true concern regarding a safety or health

issue, supervisors or managers need to act accordingly and give attention to the matter.

- All incidents (this includes property damage, equipment damage, incidents involving injury I or illnesses, and near-miss type incidents) need to be investigated. In most cases, the department, job foreman or supervisor will complete this investigation. Managers will be involved as necessary or when requested.

- Incidents that involve injury and illnesses will be evaluated and analyzed for trends,

common causes, and patterns so as to prevent further incidents.

HAZARD PREVENTION AND CONTROL

- If feasible, engineering controls will be used first, rather than immediately providing

personal protection equipment (PPE).

- Safe work practices will be developed and employees will be trained on using these safe

work practices to avoid injury and illnesses. This may include the implementation of task or job hazard analyses.

- PPE will be provided as necessary, and its use enforced by Supervisory and Management

staff.

- If feasible, administrative controls, such as reducing the duration of exposure can be

Implemented.

- Equipment, tools, machines, trucks, vehicles, and structures/facilities etc., need to be

Maintained in good working order by a continued preventative maintenance process.

- All workers will be made aware of workplace emergency procedures. Training on this Process will begin at orientation. Drills will be conducted periodically to assist in making all workers aware of the procedures in the event of an emergency such as fire or explosion.

SAFETY AND HEALTH TRAINING

Safety and Health Orientation

Workplace safety and health orientation begins on the first day of initial employment or job transfer. Each employee should have access to a copy of the written safety program, through his or her supervisor, for review and future reference, and will be given a personal copy of any safe work practices, policies, and procedures pertaining to his/ her job. Supervisors should question employees and should answer employees‘ questions to ensure knowledge and understanding of safe work practices, policies, and job-specific procedures. Supervisors are responsible to inform all employees that compliance with the safe work practices is required.

Job-Specific Training

- Managers, Supervisors and Foremen should receive basic safety and health training as it

relates to their positions.

- Supervisors will initially train employees on how to perform assigned job tasks safely.

- Supervisors will carefully review with each employee any specific safe work practices,

policies, and procedures that are applicable.

- Supervisors will observe employees performing the work. If necessary, the supervisor will provide a demonstration using safe work practices, or remedial instruction to correct

training deficiencies before an employee is permitted to do the work without supervision.

- All employees will receive safe operating instructions on seldom-used or new equipment before using the equipment.

- Supervisors will review safe work practices with employees before permitting the

performance of new, non—routine, or specialized procedures.

Periodic Retraining of Employees

All employees will be retrained periodically on safe work practices, policies and procedures, and

when changes are made to the written safety program.

If necessary, individual employees will be retrained after the occurrence of a work-related injury

caused by an unsafe act or work practice, or when a supervisor observes employees displaying

unsafe acts, practices, or behaviors.

FIRST AID AND MEDICAL ASSISTANCE

There will be adequate first aid supplies and /or an adequate first aid kit available at each workplace. Where required, or in the case of an emergency where the workplace is located in a remote location and emergency medical assistance can not arrive within a few minutes, there will be a designated certified first aid (and possibly CPR) trained employee who can assist in first aid emergency cases. Employees who receive work related injuries or illnesses will be given immediate attention in regards to the nature of their injury or illness.

INCIDENT INVESTIGATION

Incident Investigation Procedures

The supervisor at the location where the incident occurred will perform an incident investigation. Incidents can include property damage, near misses and workplace injuries and illnesses. These investigations are to assess the nature and the cause of the incident, not to place blame on personnel. Supervisors need to investigate incidents using procedures that include:

- Implement temporary control measures to prevent any further injuries to employees or

damage to equipment or property or the public.

- Review the equipment, operations, and processes to gain an understanding of the accident situation.

- Identify and interview each witness and any other person who might provide clues to the

causes.

- Investigate causal conditions and unsafe acts; make conclusions based on existing facts.

- Complete the incident investigation report.

- Provide recommendations for corrective actions.

- Indicate the need for additional or remedial safety training, if needed.

Incident investigation reports must be submitted to the designated management personnel as soon as possible after the incident.

Incident Report Form

The incident report form should be a simple format for the supervisor to complete in a timely manner. It can be similar to the OSHA 301 “Injury and Illness Incident Report” form. To correctly assess the nature and causes of the incident, the form should contain questions such as

- What was the employee doing just prior to the incident?

- Were there any witnesses? What were their names? Did the witnesses provide statements of the incident?

- What happened? (“Ladder kicked out and employee fell to floor”, “forklift struck wall, wall collapsed.")

- What part of the body was affected by the incident? (eye, arm, leg, fingers, hand, etc.) And what was the nature of the injury? (object in eyes, fractured arm, sprained leg, lacerated finger, cut in right hand, etc.).

- What was the object or substance that directly harmed the employee (if substance/object is known).

- Was the injury fatal?

RECORD KEEPING PROCEDURES

The company will control and maintain all employee accident and injury records. Records are

maintained for a minimum of five (5) years following the end of the year to which they relate. The data on the Injury and Illness log and posting of the Summary of Work-related injuries and

illnesses will be in accordance with government regulations. The following will be included in the record keeping process:

- Log of Work-related Injuries and Illnesses (OSHA form 300)

- Summary of Work-related Injuries and Illnesses (OSHA form 300A

- Incident investigation reports (OSHA form 301 or similar)

- Workers‘ Compensation Notice of Injury

TOOLE CORP.

GENERAL CONSTRUCTION

Additional Safety and Health Work Practices, 
Procedures, Including Hazard Communication

SAFETY WORK PRACTICES AND PROCEDURES

(Including use of Personal Protective Equipment)

These safety work practices and procedures are developed to assist in achieving job safety by having no employee accidents. Some of these are OSHA requirements. For these reasons, every employee is expected to abide by our safety work practices and procedures at all times.

Cell Phones: - NO CELL PHONE CALLS ON CONSTRUCTION SITE WITHIN 200 FEET

Hard Hats –All employees are required to wear a hard hat on every job site at all times.

Eye Protection - All employees are required to wear safety glasses on the job site at all times.

Fall Protection – All employees exposed to falls over 6ft are required to be trained and use proper fall protection equipment and/or be protected by adequate guardrail systems or adequate safety nets.

Hand Protection - All employees involved in operations exposing hands to cuts, chemicals, burns, etc. are required to wear gloves that are appropriate to the exposure. The material safety data sheet (MSDS) should be used if in doubt about what type of glove to use.

Rubber Boots - Employees involved in operations exposing the feet/legs to such hazards as concrete burns during placing uncured concrete are required to wear rubber boots in good condition.

Work Boots - All Employees must have OSHA Complied Work Boots.

TIE OFF – Must be 100% Tie Off over 6’ OSHA Requirement – 1 Warning Only and Termination Immediately after write up by Safety Coordinator.

Other - Specific jobs may cause the need for other personal protective equipment. When this occurs, the employee is expected to utilize this equipment. It is the Job Superintendent's responsibility to see that equipment in use be appropriate and in good condition.

GENERAL:

Employees must follow the safety policy, safe work practices and procedures established by Toole Corp. Violations may result in disciplinary action, as described in the Safety Policy section.

Employees should report any equipment or condition considered to be unsafe, as well as what they consider to be unsafe work practices. This type of information should be reported to the Superintendent or to the person in charge of the job.

Be courteous. Avoid distracting others as distractions may cause or contribute to accidents. Do not engage in horseplay on the job.

When lifting, bend your knees, grasp the load firmly, and then raise the load with your legs, keeping the back as straight as possible. Don't twist body with a load, move your feet. GET HELP for HEAVY LOADS.

When in doubt about the safety of a situation that is out of the norm, contact the Superintendent to find the proper procedure.

Good housekeeping practices improve safety for everyone. When you create clutter, clean it up.

When clutter is left in the work area by someone else, clean it up and report this to the

Superintendent.

The possession or consumption of alcohol, drugs or any control substance is against policy and

violators are subject to dismissal.

VEHICLE OPERATIONS:

Employees driving company vehicles, their personal vehicle on company business must have a current driver's license and an acceptable driving record. Employees driving their personal vehicle on company business must provide proof of liability insurance to Construction

Management Associates, LLC.

When driving a company vehicle or their personal vehicle on company business, all traffic laws

must be obeyed and driver and any passengers in the front seat must wear a seat belt.

SAFETY INSPECTIONS:

The Job Superintendent will complete an inspection from a safety stand- point at the start of each new job and on Monday morning of on-going jobs. Areas to check would include, but should not be limited to, proper tools on the job site to do the job safely, any unusual hazards, such as stumbling hazards or fall exposure, any overhead objects that could fall on, any special personal protective equipment needed or special procedures due to job location, areas-operations known to have contributed to employee accidents in the past, and other items that may be peculiar to the job or location. The results of each internal inspection will be recorded on the CONSTRUCTION INSPECTION FORM (on next page).

Emergency Action Plan

SCOPE

The following Emergency Action Plan applies to all situations where a particular OSHA Standard specifies that a plan be established.

ELEMENTS

A.	Emergency Escape Procedures and Routes

Emergency escape procedures and route assignments have been posted in each work area and all employees have been trained by supervision in the correct procedures to follow. New employees are trained when assigned to the work area. A sample escape procedure and route sheet that is posted in work areas is attached.

B.	Procedures for Employees Who Remain to Operate Critical Operations Before They Evacuate

The attached sheet describes those operations, procedures, and personnel required for critical operations before the assigned personnel evacuate during emergency situations. A description of the special training provided is also included.

C. 	Employee Accountability Procedures After Evacuations

Each company supervisor is responsible for accounting for all their assigned employees by the supervisor or his or her designee by reporting go to a predetermined, designated rally point and conduction a head count. Each assigned employee will be accounted for by name. All supervisors are required to report their head count (by name) to the Emergency Evacuation Coordinator. A summary of the evacuation ally points and the supervisors and their assigned employees who must report to the designated rally point is attached.

D.	Rescue and Medical Duties

Specific rescue and medical duties have been assigned to designated company individuals. These personnel have received special training and instructions to properly carry out these assignments. A list of individuals assigned and a summary of their training is attached for review.

E. 	Preferred Means of Reporting Fire and Emergencies

All company fires and emergencies will be reported by:

F.          Emergency and Fire Protection Coordinator

The Company's Emergency and Fire Protection Plan Coordinator:

Name						Title

Department:

Telephone No.:

The Coordinator may be contacted for further information or explanation of the Company's Emergency and Fire Protection Plans.

G.	Alarm System

Company employee alarm systems for notifying all employees in case of an emergency are:

When required by specific OSHA standards, the company will comply with OSHA standard 1910.165 - Employees Alarm Systems. These requirements are shown on the attached sheet.

H.	Fire Protection and Prevention Assignments

Appropriate company personnel have been assigned specific fire protection and prevention responsibilities. Fire prevention equipment must be routinely inspected and tested. Systems that can increase the likelihood or severity of a fire must be inspected and maintained.

EMPLOYEE ACCOUNTABILITY

FOLLOWING AN EMERGENCY EVACUATION

Each company supervisor is responsible for accounting for each of his or her assigned employees following an emergency evacuation. This will be accomplished by following the procedures shown below.

Employee Accountability

- Rally points have been established for all company evacuation routes and procedures. These points are designated on each posted work area escape route.

- All work area supervisors and employees must report to their designated rally points immediately following an evacuation.

- Each employee is responsible for reporting to his or her supervisor so an accurate headcount can be made. Supervisors will check off all those reporting and report hose not checked off as missing to the Emergency Evacuation coordinator.

- The Emergency Evacuation coordinator will be located at one of the following locations:

- Primary Location:

B.	Secondary Location:

- The Emergency Evacuation Coordinator will determine the method that will be utilized to locate missing personnel.

EMERGENCY ESCAPE PRODECURES AND ESCAPE ROUTE ASSIGNMENT

WORKAREA:

SKETCH:

Supervisor and Employee Rally Points

SPECIAL INSTRUCTIONS:

EMERGENCY PLAN AND FIRE PROTECTION COORDINATOR:

_________________________________

Signature:						Date:

EMPLOYEE ALARM SYSTEM REQUIREMETNS

REF: OSHA Standard 1910.165

Requirement						Reference	        Meets the Requirement

1.	Provides warning for safe escape		(b)(1)

2.	Can be perceived by all employees			(b)(2)

3.	Alarm is distinctive and recognizable			(b)(3)

4.	Employees properly trained				(b)(4)

5.	Emergency telephone numbers posted.	(b)(4)

6.	Emergency alarms have priority		(b)(4)

7.	Alarm procedures established.		(b)(5)

8.	All alarm components are approved.			(c)(1)

9.	Alarms restored promptly after testing.	(c)(2)

10.	Spare alarm devices available.		(c)(2)

11.	Alarm system maintained property.			(d)(1)

12.	Effective alarm tests conducted every 2 months	(d)(2)

13.	Power supplies maintained and back-ups provided.	(d)(3)

14.	Supervised systems prove positive notification of

any defect and are tested annually.			(d)(4)

15.	Alarms maintained properly by trained personnel.	(d)(5)

16.	All manually operated devices must not be

Obstructed  and readily accessible.			(e)

Note: The above requirements apply to all emergency employee alarms installed to meet a particular OSHA standard.

Procedures for Employees Who Remain to Operate Critical Operations Before They Evacuate

This document describes those operations, procedures, and personnel required for critical operations before the assigned personnel evacuate during emergency situations. A description of the special training provided is also included.

Critical Operation and Procedures

Personnel Remaining in the Critical Operation

Special Training Provided

TOOLE CORP.

HAZARD COMMUNICATION PROGRAM

I.	General

The purpose of this instruction is to ensure that TOOLE CORP is in compliance with the Occupational Safety and Health Administration's Hazard Communication Standard (HCS) 29 CFR 1910.1200

(Coordinators name or title) is designation as the Hazard Communication Program Coordinator and as such acts as the representative of the President of TOOLE CORP, who has overall responsibility.

In general, each employee in the facility will be apprised of the substance of the HCS, the hazardous properties of chemicals they work with, and measures to take to protect themselves from these chemicals.

II.	List of Hazardous Chemicals

The Hazard Communication Coordinator will maintain a list of all hazardous chemicals used in the facility and update the list as necessary. The hazardous chemical list will be updated upon receipt of hazardous chemicals at the facility. The list of hazardous chemicals is maintained in (enter place(s) list is kept).

III. 	Material Safety Data Sheets (MSDS's)

The Hazard Communication Coordinator will maintain an MSDS on every substance listed on the hazardous chemical list in. The MSDS will consist of a fully completed OSHA Form 174 or equivalent. The Hazard Communication Coordinator will ensure that all MSDS's are kept in the (enter location where MSDS's are to be kept). All MSDS's will be readily available to all employees.

The HazCom Coordinator is responsible for acquiring and updating MSDS's. The Coordinator will review each MSDS for accuracy and completeness. All new procurements for the facility must be cleared by the Hazard Communication Coordinator. Whenever possible, the least hazardous substance will be procured. MSDS's that meet the requirements of the HCS must be

Fully completed and received at the facility either prior to or at the time of receipt of the first shipment of any potentially hazardous chemical purchased from a vendor. It may be necessary to discontinue procurements from vendors failing to provide approved MSDS's in a timely manner.

IV.	Labels and Other Forms of Warning

The Site Safety Officer is designated to ensure that all hazardous chemicals in the facility are properly labeled. Labels should list at least the chemical identity, appropriate  hazard warnings, and the name and address of the manufacturer, importer, or other responsible party. The Site Safety Officer will refer to the corresponding MSDS to verify label information. Immediate use containers, small containers in which materials are poured for use on that shift by the employee drawing the material, do not require labeling. To meet the labeling requirements of HCS for other in-house containers, refer to the label supplied by the manufacturer. All labels for in-house containers will be approved by (enter name and/or title) prior to their use.

The Site Safety Officer will check on a monthly basis to ensure that all containers in the facility are labeled and that the labels are up to date.

V.	Training

Each employee who works with or is potentially exposed to hazardous chemicals will receive initial training on the HCS and the safe use of those chemicals. Additional training will be provided for employees whenever a new hazard is introduced into their work areas. Hazardous chemical training will be conducted by (enter name and/or title).

The training will emphasize these elements:

- A summary of the standard and this written program;

- Hazardous chemical properties including visual appearance and odor and methods that can be used to detect the presence or release of hazardous chemicals;

  - Physical and health hazards associated with potential exposure to workplace chemicals;

- Procedures to protect against hazards, e.g., personal protective equipment, work practices, and emergency procedures;

- Hazardous chemical spill and leak procedures; and,

  - Where MSDS's are located, how to understand their content, and how employees may obtain and use appropriate hazard information.

The Hazard Communication Coordinator will monitor and maintain records of employee training and advise the facility manager on training needs.

VI.	Contractors and Other "Outside" Employers

The Hazard Communication Coordinator, upon notification from management, will advise outside contractors of any chemical hazards which may be encountered in the normal course of their work on the premises. Likewise, contractors and other outside employers will be required provide information to the Hazard Communication Coordinator regarding any hazardous materials they will introduce into our facilities. This information may be conveyed by providing MSDS's to the appropriate personnel.

VII.	Non-Routine Tasks

Supervisors contemplating a non-routine task, will consult with the Hazard Communication Coordinator and ensure that employees are informed of chemical hazards associated with the performance of these tasks and appropriate protective measures. This will be accomplished by a meeting of supervisors and the Hazard Communication Coordinator with affected employees before such work is begun.

VIII.	Additional Information

Further information on this written program, the Hazard Communication Standard, and applicable MSDS's is available by contacting the Corporate Safety Director.

LOCKOUT/TAG OUT:

THE CONTROL OF HAZARDOUS ENERGY

SAFETY PROGRAM

I.	OBJECTIVES

To establish a means of positive control to prevent the accident starting or activating of machinery or systems while they are being repaired, cleaned and/or serviced.

A.  To establish a safe and positive means of shutting down machinery, equipment 	and systems.

B.  To prohibit unauthorized personnel or remote control systems from starting 	machinery or equipment while it is being serviced.

C.  To provide a secondary control system (tagout) when it is impossible to positively lockout the machinery or equipment.

D.  To establish responsibility for implementing  and  controlling  lockout/tagout 	procedures.

E.  To ensure that only approved locks, standardized tags and fastening devices 	provided by the company will be utilized in the lockout/tagout procedures.

II.	AREAS OF RESPONSIBILITY

- The Site Safety Officer will be responsible for implementing the lockout/tagout program.

- The Site Safety Officer are responsible to enforce the program and insure compliance with the procedures in their department.

- The Site Safety Officer is responsible for monitoring the compliance of this procedure and will conduct the annual inspection and certification of the authorized employees.

D.   Authorized employees (those contained in attachment #A-1) are responsible to follow established lockout/tagout procedures.

E.   Affected employees (all other employees in the facility) are responsible for insuring they do not attempt to restart or re-energize machines or equipment which are locked out or tagged out.

PROCEDURES

PREPARATION FOR LOCKOUT OR TAGOUT

Employees who are required to utilize the lockout/tagout procedure (see attachment #A-1) must be knowledgeable of the different energy sources and the proper sequence of shutting off or disconnecting energy means.

The four types of energy sources are:

(1) Electrical 	(most common form)

(2) Hydraulic or pneumatic

(3) Fluids and gases

(4) Mechanical

More than one energy source can be utilized on some equipment and the PROPER procedure must be followed in order to identify energy sources and lockout/tagout accordingly.  See Attachment D for specific procedure format.

ELECTRICAL

A.   Shut off power at machine and disconnect.

B.   Disconnecting means must be locked or tagged.

C.   Press start button to see that correct systems are locked out.

D.   All controls must be returned to their safest position.

E.   Points to remember:

1. If a machine or piece of equipment contains capacitors, they must be drained of stored energy.

2.Possible disconnecting means include the power cord,  power panels (look for primary and secondary voltage), breakers, the operator's station, motor circuit, relays, limit switches, electrical interlocks.

NOTE:

- Some equipment  may  have  a  motor  isolating  shut-off and  a  control isolating shutoff.

2. If the electrical energy is disconnected by simply unplugging the power cord, the cord must be kept under the control of the authorized employee or the plug end of the cord must be locked out or tagged out.

HYDRAULIC/PNEUMATIC

A.   Shut off all energy sources (pumps and compressors). If the pumps and compressors supply energy to more than one piece of equipment, lockout or tagout the valve supplying energy to this piece of equipment.

B. Stored  pressure from hydraulic/pneumatic lines shall be drained/bled when release of stored energy could cause injury to employees.

C. Make sure controls are returned to their safest position (off, stop, standby, inch, jog, etc.).

FLUIDS AND GASES

A.   Identify the type of fluid or gas and the proper

B.   Close valves to prevent flow, lockout/tagout.

C.   Determine the isolating device, close, and lockout or tagout.

D.   Drain and bleed lines to zero energy state.

NOTE:  Some systems may have electrically controlled valves; if so, they must 			be shut off, locked or tagged out.

E.   Check for zero energy state at the equipment.

MECHANICAL ENERGY (Gravity activation, or stored in springs, etc)

A.   Block out or use die ram safety chain.

B.   Lockout or tagout safety device.

C.   Shut off, lockout or tagout electrical system.

D.   Check for zero energy state.

E.   Return controls to safest position.

RELEASE FROM LOCKOUT/TAGOUT

A.   Inspection	- - Make certain the work is completed and inventory tools and equipment used.

B.   Clean-up - - Remove all towels, rags, work-aids, etc.

C.   Replace guards - - Replace all guards possible.  Sometimes a particular guard 	may have to be left off until the start sequence is over due to possible 	adjustments, however, all other guards should be put back into place.

D.   Check controls - - All controls should be in their safest position.

E. The work area shall be checked to ensure that all employees have been safely 	positioned or removed and notified that the lockout/tagout devices are being 	removed.

F.    Remove locks/tags - - Remove only your lock or tag.

PROCEDURE INVOLVING MORE THAN ONE PERSON

When servicing and/or maintenance is performed by more than one person, each authorized employee shall place his own lock or tag on the energy isolating source. This shall be done by utilizing a multiple lock scissors clamp if the equipment is capable of being locked out.   If the equipment cannot be locked out, then each authorized employee must place his tag on the equipment.

PROCEDURE FOR THE REMOVAL OF AN AUTHORIZED EMPLOYEE'S LOCKOUT/TAGOUT BY THE COMPANY

Each location must develop written procedures under the above heading that complies with OSHA 1910.147(e)(3) that can be utilized at that location.  Your procedures should include the following:

1.	Verification by employer that the authorized employee who applied the device is not in the facility.

2.	Make reasonable efforts to advise the employee that his device has been removed.(This can be done when he returns to the facility).

3.	Ensure that the authorized employee has this knowledge before he resumes work at the facility.

PROCEDURES FOR SHIFT OR PERSONNEL CHANGES

Each facility must develop their own written procedures based on their need and capabilities.  However, your procedure must specify how you will ensure the continuity of lockout or tagout protection during that time.  See 1910.147(e) (4).

PROCEDURES FOR OUTSIDE PERSONNEL/CONTRACTORS

Outside personnel/contractors shall be advised that the company has and enforces the use of lockout/tagout procedures.  They will be informed of the use of locks and tags and notified about the prohibition relating to attempts to restart or re-energize machines or equipment that are locked out or tagged out.

The company will obtain information from the outside personnel/contractor about their lockout/tagout procedures and advise affected employees of this information.

The outside personnel/contractor will be  required to sign a certification form 	(see attachment C).  If outside personnel/contractor has previously signed a certification that is on file, their is no need to have them sign a new certification.

TRAINING AND COMMUNICATION:

Each authorized employee who will be utilizing the lockout/tagout procedure will be trained in the recognition of applicable hazardous energy sources, type and magnitude of energy available in the work place, and the methods and means necessary for energy isolation and control.

Each affected employee (all employees other than authorized employees utilizing the lockout/tagout procedure)  shall  be  instructed   in  the  purpose  and   use  of  the lockout/tagout procedure and the prohibition relating to attempts to restart or re-energize machines or equipment which are locked out or tagged out.

Training will be certified using attachment #A-2 (Authorized personnel) or #A-3 (Affected Personnel). The certification will be retained in the employee's personnel file.

PROCEDURES FOR PERIODIC INSPECTION

A periodic inspection (at least annually) will be conducted of each authorized employee under  the  lockout/tagout  procedure.  This  inspection  shall  be  performed  by  the (Responsible person) provided they are not the ones utilizing the energy control procedure being inspected.

The  inspection  will  include a  review between  the  inspector and  each  authorized employee, of that employee's responsibilities under the energy control (lockout/tagout) procedure.  The inspection will also consist of a physical inspection of the authorized employee while performing work under the procedures.

The	(Responsible person)	shall certify in writing that the inspection has been performed.   The written  certification	(see attachment	#B) shall be  retained  in the

individual's personnel file.

ATTACHMENT A-1

LIST OF AUTHORIZED PERSONNEL

FOR

LOCKOUT/TAGOUT PROCEDURES

NAMES				JOB TITLE

ATTACHMENT A-2

CERTIFICATION OF TRAINING

(AUTHORIZED PERSONNEL)

I CERTIFY THAT I RECEIVED TRAINING AS AN AUTHORIZED EMPLOYEE UNDER	__________________________ LOCKOUT/TAGOUT PROGRAM.  I FURTHER CERTIFY THAT I UNDERSTAND THE PROCEDURES AND WILL ABIDE BY THOSE PROCEDURES.

____________________________________________________________

AUTHORIZED EMPLOYEE SIGNATURE			DATE

ATTACHMENT A-3

CERTIFICATION OF TRAINING

(AFFECTED PERSONNEL)

I CERTIFY THAT I RECEIVED TRAINING AS AN AFFECTED EMPLOYEE UNDER ______________________LOCKOUT/TAGOUT PROGRAM.  I FURTHER CERTIFY AND UNDERSTAND THAT I AM PROHIBITED FROM ATTEMPTING TO RESTART OR RE-ENERGIZE MACHINES OR EQUIPMENT WHICH ARE LOCKED OUT OR TAGGED.

____________________________________________________________

AUTHORIZED EMPLOYEE SIGNATURE			DATE

ATTACHMENT B

LOCKOUT/TAGOUT INSPECTION CERTIFICATION

I CERTIFY THAT ________________WAS INSPECTED ON THIS DATE UTILIZING LOCKOUT/TAGOUT PROCEDURES. THE INSPECTION WAS PERFORMED WHILE WORKING ON _________________________.

____________________________________________________________

AUTHORIZED EMPLOYEE SIGNATURE			DATE

____________________________________________________________

INSPECTOR SIGNATURE					DATE

ATTACHMENT C

OUTSIDE PERSONNEL/CONTRACTOR CERTIFICATION

I CERTIFY THAT ____________________ AND _______________________ (OUTSIDE PERSONNEL/CONTRACTOR) HAVE INFORMED EACH OTHER OF OUR

RESPECTIVE LOCKOUT OR TAGOUT PROCEDURES.

NAME (PRINTED)						DATE

SIGNATURE							DATE

OUTSIDE PERSONNEL/CONTRACTOR (PRINTED)	DATE

SIGNATURE							DATE

ATTACHMENT D

EQUIPMENT SPECIFIC PROCEDURE

FOR

(Date)

Machine Identification

General Description:

Manufacturer:

Mod& Number:

Serial Number:*

* If more than one piece of same equipment, list all serial numbers.

Location of equipment:

Operator Controls

The type of controls available to the operator need to be determined. This should help identify energy sources and lockout capacity for the equipment.

List types of operator controls:

Energy Sources

The energy sources present on this equipment are: (electrical, steam, hydraulic, pneumatic, natural gas, stored energy, etc.)

| ENERGY SOURCE | LOCATION | Lockable Yes N0 |  | Type lock or block needed |
| --- | --- | --- | --- | --- |

Shutdown Procedures

List the steps in order necessary to shut down and de-energize the equipment. Be specific.  For stored energy, be specific about how the energy will be dissipated or restrained.

Procedure:

Lock Type & Location:

De-energized State To Be Verified?  How?

NOTIFY ALL AFFECTED EMPLOYEES WHEN THIS PROCEDURE IS IN APPLICATION

Start Up Procedures

List the steps in order necessary to reactivate (energize) the equipment. Be specific.

Procedure:

Energy Source Activated:

NOTIFY ALL AFFECTED EMPLOYEES WHEN THIS PROCEDURE IS IN APPLICATION

Procedures For Operations and Service/Maintenance

List those operations where the procedures above do not apply. [See 29CFR 1910.147

(a)(2)] Alternate measures which provide effective protection must be developed for these operations. Job Safety Analysis is one method of determining appropriate measures.

Operation Name:

Affected and Authorized Employees

List each person affected by this procedure and those authorized to use this procedure.

| AFFECTED EMPLOYEES Name Job Title |  |
| --- | --- |

| AUTHORIZED EMPLOYEES Name Job Title |  |
| --- | --- |

Approved by (& date):

Approved by (& date):

Personal Protective Equipment (PPE)

For

TOOLE CORP

Written Hazard Assessment for

Selecting Personal Protective Equipment

•   Identifying and evaluating equipment and processes

•   Reviewing injury/accident/incident records

•   Reviewing previously selected PPE

Date of Evaluation:

Workplace Evaluated By: 	________________________________________

Evaluator Title:  _________________________________________________

| HAZARD TYPE [impact, penetration, chemical--(corrosive, reactive, toxic, irritant, flammable, etc), heat, harmful (or nuisance) dust, light / radiation, electrical, biohazard, noise, other] | LOCATION/ SOURCES/ TASKS | ANALYSIS OF RISK (Low/ Medium / High) Level of Risk Seriousness of Potential Injury |  | PPE (REQUIRED) | PPE (OPTIONAL) |
| --- | --- | --- | --- | --- | --- |

Personal Protective Equipment

Training Meeting Record

Company Name : Toole Corp

Company Address: 3001 W. Hallandale Bch. Blvd., Ste. 316, Pembroke Park, 33009

Date of Training                                         Training Provided By

TRAINING TOPIC COVERED

(1) A summary of the standard and our written program

(2) When PPE is necessary

(3) What PPE is necessary

(4) How to properly don, doff, adjust, and wear PPE

(5) Limitations of the PPE

(6) Proper care, maintenance, useful life, and disposal of the PPE

| EMPLOYEE NAME | DEPARTMENT | TITLE | SIGNATURE |
| --- | --- | --- | --- |

Employee Acknowledgement

of

Personal Protective Equipment Training

I,                                         , have been trained in the company's personal protective equipment program. The protective equipment required in my work area has been explained and I am aware of the company's policy and requirement.

Employee's Signature 				Date

Supervisor's Signature 			Date

Fall Protection Safety Program

for

TOOLE CORP

This Fall Protection Plan is specific for the following project:

Location of Job: Homestead Air Reserve Base Building 410

Erecting Company: Cast Development

Date Plan Prepared or Modified: October 2, 2013

Plan Prepared By: Eric Coffie

Plan Prepared By: Charlet Morris

Plan Approved By: Curtis Morrow

Plan Supervised By: n/a

The following Fall Protection Plan is prepared for the prevention of injuries associated with falls. A Fall Protection Plan must be developed and evaluated on a site by site basis.

I.	STATEMENT OF COMPANY POLICY

TOOLE CORP is dedicated to the protection of its employees from on-the-job injuries. All employees of TOOLE CORP. Have the responsibility to work safely on the job. The purpose of this plan is:

(a) To supplement our standard safety policy by providing safety standards specifically designed to cover fall protection on this job and; (b) To ensure that each employee is trained and made aware of the safety provisions which are to be implemented by this 	plan prior to the start of the erection.

This Fall Protection Plan addresses the use of the other than conventional fall protection at a number of areas on the project, as well as, identifying specific activities that require nonconventional means of fall protection. These areas include:

- Connecting activity (point of erection)

- Controlled Decking Zone

- Unprotected sides or edges

This plan is designed to enable employers and employees to recognize the fall hazards on this job and to establish the procedures that are to be followed in order to prevent falls to lower levels or through holes and openings in walking/working surfaces. Each employee will be trained in these procedures and strictly adhere to them except when doing so would expose the employee to a greater hazard. If, in the employee's opinion, this is the case, the employee is to notify the foreman of the concern and the concern addressed before proceeding.

Safety policy and procedure on any one project cannot be administered, implemented, monitored and enforced by any one individual. The total objective of a safe, accident free work environment can only be accomplished by a dedicated, concerted effort by every individual involved with the project from management down to the last employee. Each employee must understand their value to the company; the costs of accidents, both monetary, physical, and emotional; the objective of the safety policy and procedures; the safety rules that apply to the safety policy and procedures; and what their individual role is in administering, implementing, monitoring, and compliance of their safety policy and procedures. This allows for a more personal approach to compliance through planning, training, understanding and cooperative effort, rather than by strict enforcement. If for any reason an unsafe act persists, strict enforcement will be implemented.

It is the responsibility of (Charlet Morris)   to implement this Fall Protection Plan. (Charlet Morris)    is responsible for continual observational safety checks of their work operations and to enforce the safety policy and procedures. The foreman also is responsible to correct any unsafe  acts or conditions immediately. It is the responsibility of the employee to understand and adhere to the procedures of this plan and to follow the  instructions of the foreman. It is also the responsibility of the employee to bring to management's attention any unsafe or hazardous conditions or acts that may cause injury to either themselves or any other employees. Any changes to this  Fall Protection Plan must be approved by	(Charlet Morris)

II	FALL PROTECTION SYSTEMS TO BE USED ON THIS PROJECT

Where conventional fall protection is infeasible or creates a greater hazard at the leading edge and during initial connecting activity, we plan to do this work using a safety monitoring system and expose only a minimum number of employees for the time necessary to actually accomplish the job. The maximum number of workers to be monitored by one safety monitor is six (6). We are designating the following trained employees as designated erectors and they are permitted to

enter the controlled access zones and work without the use of conventional fall protection.

Safety monitor: Charlet Morris

Designated erector: Anamael Nunez

Designated erector:

Designated erector:

Designated erector:

The safety monitor shall be identified by Charlet Morris. Only individuals with the appropriate experience, skills, and training will be authorized as designated erectors. All employees that will be working as designated erectors under the safety monitoring system shall have been trained and instructed in the following areas:

1. Recognition of the fall hazards in the work area (at the leading edge and when making initial connections-point of erection).

2.  Avoidance of fall hazards using established work practices which have been

made known to the employees.

3. Recognition of unsafe practices or working conditions that could lead to a fall,

such as windy conditions.

4.  The function, use, and operation of safety monitoring systems, guardrail systems, body belt/harness systems, control zones and other protection to be used.

5.  The correct procedure for erecting, maintaining, disassembling and inspecting

the system(s) to be used.

6. Knowledge of construction sequence or the erection plan.

A conference will take place prior to starting work involving all members of the erection crew, crane crew and supervisors of any other concerned contractors. This conference will be conducted by the supervisor in charge of the project. During the pre-work conference, erection procedures and sequences pertinent to this job will be thoroughly discussed and safety practices to be used throughout the project will be specified. Further, all personnel will be informed that the controlled access zones are off limits to all personnel other than those designated erectors specifically trained to work in that area.

Safety Monitoring System

A safety monitoring system means a fall protection system in which a competent person is responsible for recognizing and warning employees of fall hazards. The duties of the safety monitor are to:

- Warn by voice when approaching the open edge in an unsafe manner.

2.  Warn by voice if there is a dangerous situation developing which cannot be

seen by another person involved with product placement, such as a member

getting out of control.

3. Make the designated erectors aware they are in a dangerous area.

4.  Be competent in recognizing fall hazards.

5.  Warn employees when they appear to be unaware of a fall hazard or are

acting in an unsafe manner.

6.  Be on the same walking/working surface as the monitored employees and

within visual sighting distance of the monitored employees.

7.  Be close enough to communicate orally with the employees.

8.  Not allow other responsibilities to encumber monitoring. If the safety monitor 		becomes too encumbered with other responsibilities, the monitor shall (1) 		stop the erection process; and (2) turn over other responsibilities to a

designated erector; or (3) turn over the safety monitoring function to another

designated, competent person.

The safety monitoring system shall not be used when the wind is strong enough to cause loads with large surface areas to swing out of radius, or result in loss of control of the load, or when weather conditions cause the walking-working surfaces to become icy or slippery.

Control Zone System

A controlled access zone means an area designated and clearly marked, in which

leading edge work may take place without the use of guardrail, safety net or personal fall arrest systems to protect the employees in the area. Control zone systems shall comply with the following provisions:

1. When used to control access to areas where leading edge and other

operations are taking place the controlled access zone shall be defined by a control line or by any other means that restricts access.

When control lines are used, they shall be erected not less than 6 feet (1.8 m) nor more than 60 feet (18 m) or half the length of the member being erected, whichever is less, from the leading edge.

2. The control line shall extend along the entire length of the unprotected or leading edge and shall be approximately parallel to the unprotected or leading edge.

3. The control line shall be connected on each side to a guardrail system or wall.

4. Control lines shall consist of ropes, wires, tapes, or equivalent materials, and supporting stations as follows:

5. Each line shall be flagged or otherwise clearly marked at not more than 6-foot (1.8 m) intervals with high-visibility material.

6. Each line shall be rigged and supported in such a way that its lowest point (including sag) is not less than 39 inches (1 m) from the walking/working surface and its highest point is not more than 45 inches (1.3 m) from the walking/working surface.

7.  Each line shall have a minimum breaking strength of 200 pounds (.88 kN).

Holes

All openings greater than 12 in. x 12 in. will have perimeter guarding or covering. All predetermined holes will have the plywood covers made in the stair fabricators yard and shipped with the member to the jobsite. Prior to cutting holes on the job, proper protection for the hole must be provided to protect the workers. Perimeter guarding or covers will not be removed without the approval of the erection foreman.

III	IMPLEMENTATION OF FALL PROTECTION PLAN

The structure being erected is a multi-story metal egress stairway consisting of columns, structural tubing, stair railing with wall panels and concrete filled stair treads.

The following is a list of the products and erection situations on this job:

Columns

For columns 10 ft to 36 ft long, employees disconnecting crane hooks from columns will work from a ladder and wear a body belt/harness with lanyard and be tied off when both hands are needed to disconnect. For tying off, a vertical lifeline will be connected to the lifting eye at the top of the column, prior to lifting, to be used with a manually operated or mobile rope grab. For columns too high for the use of a ladder, 36 ft and higher, an added cable will be used to reduce the height of the disconnecting point so that a ladder can be used. This cable will be left in place until a point in erection that it can be removed safely. In some cases, columns will be unhooked from the crane by using an erection tube or shackle with a pull pin which is released from the ground after the column is stabilized.

The column will be adequately connected and/or braced to safely support the  of a ladder with an employee on it.

Detailing

Employees exposed to falls of six (6) feet or more to lower levels, who are not actively engaged in leading edge work or connecting activity, such as welding, bolting, cutting, bracing, guying, patching, painting or other operations, and who are working less than six (6) ft from an unprotected edge will be tied off at all times or guardrails will be installed. Employees engaged in these activities but who are more than six (6) ft from an unprotected edge as defined by the control zone lines, do not require fall protection but a warning line or control lines must be erected to remind employees they are approaching an area where fall protection is required.

IV.	CONVENTIONAL FALL PROTECTION CONSIDERED FOR THE POINT OF ERECTION OR LEADING EDGE ERECTION OPERATIONS

A.  Personal Fall Arrest Systems

In this particular erection sequence and procedure, personal fall arrest systems requiring body belt/harness systems, lifelines and lanyards will not reduce possible hazards to workers and will create offsetting hazards during their usage at the leading edge of metal stair construction.

Leading edge erection and initial connections are conducted by employees who are specifically trained to do this type of work and are trained to recognize the fall hazards. The nature of such work normally exposes the employee to the fall hazard for a short period of time and installation of fall protection systems for a short duration is not feasible because it exposes the installers of the system to the same fall hazard, but for a longer period of time.

- It is necessary that the employee be able to move freely without encumbrance in order to guide the sections of metal stairs into their final position without having lifelines attached which will restrict the employee's ability to move about at the point of erection.

- A typical procedure requires 2 or more workers to maneuver around each

other as the metal stair is positioned to connect to the building structure. If they are each attached to a lifeline, part of their attention must be diverted from their main task of positioning a member weighing several tons to the task of avoiding entanglements of their lifelines or avoiding tripping over

lanyards. Therefore, if these workers are attached to lanyards, more fall

potential would result than from not using such a device.

In this specific erection sequence and procedure, retractable lifelines do not solve the problem of two workers becoming tangled. In fact, such a tangle could prevent the lifeline from retracting as the worker moved, thus potentially exposing the worker to a fall greater than 6 ft. Also, a worker crossing over the lifeline of another worker can create a hazard because the movement of one person can unbalance the other. In the event of a fall by one person there is a likelihood that the other person will be caused to fall as well. In addition, if contamination such as grout (during hollow core grouting) enters the retractable housing it can cause excessive wear and damage to the device and could clog the retracting mechanism as the lanyard is dragged across the deck. Obstructing the cable orifice can defeat the device's shock absorbing function, produce cable slack and  damage, and adversely affect cable extraction and retraction.

3. Employees tied to a lifeline can be trapped and crushed by moving structural members if the employee becomes restrained by the lanyard or retractable lifeline and cannot get out of the path of the moving load. The sudden movement of a metal stair member being raised by a crane can be caused by a number of factors. When this happens, a connector may immediately have to move a considerable distance to avoid injury. If a tied off body belt/harness is being used, the connector could be trapped. Therefore, there is a greater risk of injury if the connector is tied to the structure for this specific erection sequence and procedure.

When necessary to move away from a retractable device, the worker cannot move at a rate greater than the device locking speed typically 3.5 to 4.5 ft/sec. When moving toward the device it is necessary to move at a rate which does not permit cable slack to build up. This slack may cause cable retraction acceleration and cause a worker to lose their balance by applying a higher than normal jerking force on the body when the cable suddenly becomes taut after building up momentum. This slack can also cause damage to the internal spring-loaded drum, uneven coiling of cable on the drum, and possible cable damage.

The factors causing sudden movements for this location include:

(a)	Cranes

(1)	Operator error.

(2)	Site conditions (soft or unstable ground).

(3)	Mechanical failure.

(4)	Structural failure.

(5)	Rigging failure.

(6)	Crane signal/radio communication failure.

(b)	Weather Conditions

- Wind (strong wind/sudden gusting) - particularly a problem

with the large surface areas of metal stair members.

(2)	Snow/rain (visibility).

3)	Fog (visibility).

(4)	Cold - causing slowed reactions or mechanical problems.

(c)	Structure/Product Conditions.

(1)	Lifting Eye failure.

(2)	Bearing failure or slippage.

(3)	Structure shifting.

(4)	Bracing failure.

(5)	Product failure.

(d)	Human Error.

(1)	Incorrect tag line procedure.

(2)	Tag line hang-up.

(3)	Incorrect or misunderstood crane signals.

(4)	Misjudged elevation of member.

(5)	Misjudged speed of member.

(6)	Misjudged angle of member.

4.  Anchorages or special attachment points could be cast into the metal stair  members if sufficient preplanning and consideration of erectors' position is done before the members are cast. Any hole or other attachment must be approved by the engineer who designed the member. It is possible that some design restrictions will not allow a member to be weakened by an additional hole; however, it is anticipated that such situations would be the exception, not the rule. Attachment points, other than on the deck surface, will require removal and/or patching. In order to remove and/or patch these points, requires the employee to be exposed to an additional fall hazard at an unprotected perimeter. The fact that attachment points could be available anywhere on the structure does not eliminate the hazards of using these points for tying off as discussed above.

5. Providing attachment at a point above the walking/working surface would also create fall exposures for employees installing their devices. Final positioning of a metal stair member requires it to be moved in such a way that it must pass through the area that would be occupied by the lifeline and the lanyards attached to the point above. Resulting entanglements of lifelines and lanyards on a moving member could pull employees from the work surface. Also, the structure is being created and, in most cases, there is no structure above the members being placed.

- Temporary structural supports, installed to provide attaching points for lifelines limit the space which is essential for orderly positioning, alignment and placement of the metal stair members. To keep the lanyards a reasonable and manageable length, lifeline supports would necessarily need to be in proximity to the positioning process. A sudden shift of the metal stair member being positioned because of wind pressure or crane movement could make it strike the temporary supporting structure, moving it suddenly and causing tied off employees to fall.

- The time in man hours which would be expended in placing and maintaining temporary structural supports for lifeline attaching points could exceed the expended man hours involved in placing the metal stair members. No protection could be provided for the employees erecting the temporary structural supports and these supports would have to be moved for each successive step in the construction process, thus greatly increasing the employee's exposure to the fall hazard.

- The use of a cable strung horizontally between two columns to provide tie off lines for erecting or walking a beam for connecting work is not feasible and creates a greater hazard on this multi-story building for the following reasons:

- If a connector is to use such a line, it must be installed between the two columns. To perform this installation requires an erector to have more fall exposure time attaching the cable to the columns than would be spent to make the beam to column connection itself.

- If such a line is to be installed so that an erector can walk along a beam, it must be overhead or below him. For example, if a connector must walk along a 24 in. wide beam, the presence of a line next to the connector at waist level, attached directly to the columns, would prevent the connector from centering their weight over the beam and balancing themselves. Installing the line above the connector might be possible on the first level of a two-story column; however, the column may extend only a few feet above the floor level at the second level or be flush with the floor level. Attaching the line to the side of the beam could be a solution; however, it would require the connector to attachthe lanyard below foot level which would most likely extend a fall farther than 6 ft.

- When lines are strung over every beam, it becomes more and more difficult for the crane operator to lower a metal stair member into position without the member becoming fouled. Should the member become entangled, it could easily dislodge the line from a column. If a worker is tied to it at the time, a fall could be caused.

6.  The ANSI A10.14-1991 American National Standard for Construction and Demolition Operations - Requirements for Safety Belts, Harnesses, 	Lanyards and Lifelines for Construction and Demolition Use, states that the anchor point of a lanyard or deceleration device should, if possible, be 	located above the wearer's belt or harness attachment. ANSI A10.14 also 	states that a suitable anchorage point is one which is located as high as 	possible to prevent contact with an obstruction below should the worker fall. Most manufacturers also warn in the user's handbook that the safety block/retractable lifeline must be positioned above the D-ring (above the 	work space of the intended user) and OSHA recommends that fall arrest 	and restraint equipment be used in accordance with the manufacturer's	instructions.

Attachment of a retractable device to a horizontal cable near floor level or using the inserts in the floor or roof members may result in increased free fall due to the dorsal D-ring of the full-body harness riding higher than the attachment point of the snap hook to the cable or insert (e.g., 6 foot tall worker with a dorsal D-ring at 5 feet above the floor or surface, reduces the working length to only one foot, by placing the anchorage five feet away from the fall hazard). In addition, impact loads may exceed

maximum fall arrest forces (MAF) because the fall arrest D-ring would be 4 to 5 feet higher than the safety block/retractable lifeline anchored to the walking-working surface; and the potential for swing hazards is increased. Manufacturers also require that workers not work at a level where the point of snap hook attachment to the body harness is above the device because this will  the free fall distance and the deceleration distance and will cause higher forces on the body in the event of an

accidental fall.

Manufacturers recommend an anchorage for the retractable lifeline which is immovably fixed in space and is independent of the user's support systems. A moveable anchorage is one which can be moved around (such as equipment or wheeled vehicles) or which can deflect substantially under shock loading (such as a horizontal cable or very flexible beam). In the case of a very flexible anchorage, a shock load applied to the anchorage during fall arrest can cause oscillation of the flexible anchorage such that the retractable brake mechanism may undergo one or more cycles of locking/unlocking/locking (ratchet effect) until the anchorage deflection is dampened. Therefore, use of a moveable anchorage involves critical engineering and safety factors and should only be considered after fixed anchorage has been determined to be not feasible.

Horizontal cables used as an anchorage present an additional hazard due to amplification of the horizontal component of maximum arrest force (of a fall) transmitted to the points where the horizontal cable is attached to the structure. This amplification is due to the angle of sag of a horizontal cable and is most severe for small angles of sag. For a cable sag angle of 2 degrees the horizontal force on the points of cable attachment can be amplified by a factor of 15.

It is also necessary to install the retractable device vertically overhead to minimize swing falls. If an object is in the worker's swing path (or that of the cable) hazardous situations exist: (1) due to the swing, horizontal speed of the user may be high enough to cause injury when an obstacle in the swing fall path is struck by either the user or the cable; (2) the total vertical fall distance of the user may be much greater than if the user had fallen only vertically without a swing fall path.

With retractable lines, overconfidence may cause the worker to engage in inappropriate behavior, such as approaching the perimeter of a floor or roof at a distance appreciably greater than the shortest distance between the anchorage point and the leading edge. Though the retractable lifeline may arrest a worker's fall before he or she has fallen a few feet, the lifeline may drag along the edge of the floor or beam and swing the worker like a pendulum until the line has moved to a position where the distance between the anchorage point and floor edge is the shortest distance between those two points. Accompanying this pendulum swing is a lowering of the worker, with the attendant danger that he or she may violently impact the floor or some obstruction below.

The risk of a cable breaking is increased if a lifeline is dragged sideways across the rough surface or edge of a concrete member at the same moment that the lifeline is being subjected to a maximum impact loading during a fall. The typical 3/16 in. cable in a retractable lifeline has a breaking strength of from 3000 to 3700 lbs.

7.  The competent person, who can take into account the specialized operations being performed on this project, should determine when and where a designated erector cannot use a personal fall arrest system.

B.  Safety Net Systems

The nature of this particular metal stair erection worksite precludes the safe use of safety nets where point of erection or leading edge work must take place.

C. Guardrail Systems

On this particular worksite, guardrails, barricades, ropes, cables or other perimeter guarding devices or methods on the erection floor will pose problems to safe erection procedures.

V. 	OTHER FALL PROTECTION MEASURES CONSIDERED FOR THIS JOB

The following is a list and explanation of other fall protection measures available and an explanation of limitations for use on this particular jobsite. If during the course of erecting the building the employee sees an area that could be erected more safely by the use of these fall protection measures, the foreman should be notified.

A. Scaffolds are not used because:

1. The leading edge of the building is constantly changing and the scaffolding would have to be moved at very frequent intervals. Employees erecting and dismantling the scaffolding would be exposed to fall hazards for a greater length of time than they would by merely erecting the metal stair member.

2. A scaffold tower could interfere with the safe swinging of a load by the crane.

3. Power lines, terrain and site do not allow for the safe use of scaffolding.

- Vehicle mounted platforms are not used because:

- A vehicle mounted platform will not reach areas on the deck that are

erected over other levels.

- The leading edge of the building is usually over a lower level of the building and this lower level will not support the weight of a vehicle 	mounted platform.

3.  A vehicle mounted platform could interfere with the safe swinging of a load 		by the crane, either by the crane swinging the load over or into the 	equipment.

4.  Power lines and surrounding site work do not allow for the safe use of a 	vehicle mounted platform.

C. Crane suspended personnel platforms are not used because:

1. A second crane close enough to suspend any employee in the working 	and erecting area could interfere with the safe swinging of a load by the

crane hoisting the product to be erected.

2. Power lines and surrounding site work do not allow for the safe use of a 	second crane on the job.

VI.	ENFORCEMENT

Constant awareness of and respect for fall hazards, and compliance with all safety rules are considered conditions of employment. The jobsite Superintendent, as well as individuals in the Safety and Personnel Department, reserve the right to issue disciplinary warnings to employees, up to and including termination, for failure to follow the guidelines of this program.

VII.	ACCIDENT INVESTIGATIONS

All accidents that result in injury to workers, regardless of their nature, shall be investigated and reported. It is an integral part of any safety program that documentation take place as soon as possible so that the cause and means of prevention can be identified to prevent a reoccurrence.

In the event that an employee falls or there is some other related, serious incident occurring, this plan shall be reviewed to determine if additional practices, procedures, or training need to be implemented to prevent similar types of falls or incidents from occurring.

VIII.	CHANGES TO PLAN

Any changes to the plan will be approved by (name of the qualified person). This plan shall be reviewed by a qualified person as the job progresses to determine if additional practices, procedures or training needs to be implemented by the competent person to improve or provide additional fall protection. Workers shall be notified and trained, if necessary, in the new procedures. A copy of this plan and all approved changes shall be maintained at the jobsite.

Safety Committee

Safety Committee Organization

- A safety committee is established as a management tool to recommend improvements to workplace safety programs and to identify corrective measures needed to eliminate or control recognized safety and health hazards.

- Safety committee employer representatives will not exceed employee representatives.

Responsibilities

- The safety committee will be responsible for assisting management in communicating procedures for evaluating the effectiveness of control measures used to protect employees from safety and health hazards in the workplace.

- The safety committee will be responsible for assisting management in reviewing and updating workplace safety rules based on accident investigation findings, any inspection findings, and employee reports of unsafe conditions or work practices; and accepting and addressing anonymous complaints and suggestions from employees.

- The safety committee will be responsible for assisting management in updating the workplace safety program by evaluating employee injury and accident records, identifying trends and patterns, and formulating corrective measures to prevent recurrence.

- The safety committee will be responsible for assisting management in evaluating employee accident and illness prevention programs, and promoting safety and health awareness and co-worker participation through continuous improvements to the workplace safety program.

- Safety committee members will participate in safety training and be responsible for assisting management in monitoring workplace safety education and training to ensure that it is in place, that it is effective, and that it is documented.

- Management will provide written responses to safety committee written recommendations.

Meetings

- Safety committee meetings are held quarterly and more often if needed and each committee member will be compensated at his or her hourly wage when engaged in safety committee activities.

- Management will post the minutes of each meeting in a conspicuous place and the minutes will be available to all employees.

- All safety committee records will be maintained for not less than three calendar years.

SAFETY MANUEL POLICY

I have received and by signing I will Comply by the Safety Standard and Guideline for TOOLE CORP. The Terms agree that if in Violation of any Rules in the 90 Day Trail Period will be Grounds for Termination.

Employee Acceptance Signature

Date:

Approved By:

Date:

Curtis Morrow

www.Toolecorp.com

curtis@toolecorp.com

TOOLE CORP –

Office: 786.339.8089

Fax: 305.397.0355

Appendices

Appendix B-1

Construction Inspection Form 		Inspection By:

| Project: | Date: |
| --- | --- |

|  | Description | X | OK | Comments/Correction Date |
| --- | --- | --- | --- | --- |
|  | X = Needs Improvement/correction |  |  |  |
| 1 | Fall protection |  |  |  |
| 2 | Forklift Check Daily |  |  |  |
| 3 | Scissor Lift – Check Daily |  |  |  |
| 4 | Ladders – All Tools |  |  |  |
| 5 | Boom Lift – Check Daily |  |  |  |
| 6 | Personal protection equipment: Hard Hat, Safety Glasses, Gloves, Safety Vest Worn at all Times |  |  |  |
| 7 | Site housekeeping, trip/slip hazards, puncture hazards, clear stairways |  |  |  |
| 8 | Other |  |  |  |

Appendix B-2