Shoring and Sheeting Plan

Repair Fire Evacuation, Interior Finishes

Building 410

Homestead ARB, Florida

Revision 0

Prepared for:

Toole Corporation

Pembroke Park, Florida

October 2013

TABLE OF CONTENTS

1. HAZARDS OF EXCAVATIONS ....................................................................... 3

2. OSHA STANDARD ........................................................................................ 3

3. PROTECTIVE SYSTEMS................................................................................ 4

4. PLAN OF ACTION ........................................................................................ 4

5. DEWATERING DEFINITIONS ...................................................................... 5

6. PURPOSE ……………...................................................................................... 5

7. CRITERIA .................................................................................................... 5

8. CONCLUSION ……………………….................................................................... 5

1. HAZARDS OF EXCAVATIONS

There are many potential hazards when working in excavations and trenches. Probably the most common hazard at any worksite is the threat of cave-in. A cave-in occurs when walls of an excavation collapse.

Why do cave-ins occur? When earth is removed to create an excavation, these natural forces are no longer able to hold back the soil left behind forming the trench wall. With no support, eventually the laws of gravity take over, and the soil from the trench walls will eventually move downward and inward into the excavation. The result is a cave-in. Cave-ins are more likely to occur in unprotected excavation’s where:

- The excavation is dug in unstable or previously disturbed soil;

- Excessive vibration from construction equipment or vehicle traffic around the excavation is present;

- A surcharge of loads are present near the sides of an excavation, most frequently from equipment or the excavated material too near the edge;

- An accumulation of water in the excavation;

- Changes in weather conditions (freezing, melting, sudden heavy rain, etc.)

These conditions can be controlled by;

- Re-routing traffic when possible, and keeping only the heavy construction equipment required near the excavation;

- Keeping the spoil pile at least 2 feet back from the edge of the excavation;

- Pumping water out of the excavation before anyone enters it

- Using protective equipment systems when required

2. OSHA STANDARD

The standard covering excavation safety is Title 29 CFR Part 1926.650-652 (Subpart P), OSHA”s Rules and Regulations for Construction Employment. A trench refers to a narrow excavation made below the surface of the ground in which the depth is greater than the width-and the width does not exceed 15 feet. Trenching is common in utility work, where underground cables are being installed or repaired.

If an excavation is more than 5 feet in depth, there must be a protective system in place while workers are present in the excavation. Excavations more than 4 feet in depth must have a way to get in and out, usually a ladder, for every 25 feet of horizontal travel.

OSHA requires regardless of the depth on an excavation, a competent person must inspect conditions at the site on a daily basis and as frequently as necessary during the progress of work to assure that the hazards associated with excavations are eliminated before workers are allowed to enter.

2. PROTECTIVE SYSTEMS

Protective systems are methods of protecting workers from cave-ins of material the can fall or roll into an excavation or from the collapse of adjacent structures. If an excavation is less than 5 feet deep, OSHA does not require a protective system unless the competent person sees signs of a potential cave-in. For trenches between 5 feet and 20 feet deep, shoring and sheeting, shielding, sloping and benching are all acceptable protective measures. It is up to the planners of the construction project and the competent person on site to determine which systems are appropriate. If an excavation is greater than 20 feet deep, a registered professional engineer must design the protective system.

3. PLAN OF ACTION

We have provided the geotechnical engineer (Ardaman & Associates, Inc) with the depths and location of our excavations. We are currently awaiting them to offer a written recommendation on excavation depth vs. angle of slope for the sides of the excavations. Since all of our excavations are shallow shoring is not be required. Once we receive the recommendations of the engineer they will be added to this Shoring Plan as a part of the Work Plan.

5. DEWATERING DEFINITION

Dewatering is the removal of water from construction sites.

6. PURPOSE

The purpose of this practice is as follows:

- To facilitate construction in areas with surface water or a high water table.

- To prevent erosion and sediment transport

- To provide worksite safety

- To prevent pollution of groundwater or surface water

- To preserve down gradient natural resources and property

7. CRITERIA

Dewatering consists of the removal of surface water and/or ground water by diverting and/or removing water from construction sites, within a watershed, as needed to perform the required construction in accordance with the specifications.

The geotechnical report indicates that for the site must list the highest anticipated groundwater level. The contractor must determine the depth to groundwater immediately prior to excavation by drilling an exploratory hole to determine if the deepest excavation will be within 3 feet of the highest anticipated groundwater level. If groundwater is found within 2 feet of the deepest excavation, than a drawdown well dewatering system must be installed.

8. CONCLUSION

The geotechnical report dated February 15, 2013 by Ardaman & Associates, Inc. notes that the highest groundwater expected would be at 7.5’ below ground surface. Our bottom most excavation will not exceed 4 feet thus the groundwater will remain at least 3 foot below the bottom of all excavations. Therefore we don’t foresee the need to incorporate a dewatering system for the purposes of excavation.

However in the event of significant rainfall as a precautionary measure we require that the subcontractor have at his/her disposal a portable 2” sump pump on hand to pump out excess water buildup due to heavy rains.

In the event that this were to happen the Job Site Superintendent would notify the appropriate persons as per our CQCP of this site condition in her QC report.

The coordination of the location to discharge the excess water will be coordinated with Project Manager, Contracting Officer and engineer of record prior to removing the excess water.