Toole Corp.

Health & Safety Manual

# Table of Contents

Safety Policy Statement						Section 1

Assignment of Responsibility					Section 2

#### Aerial Lifts								Section 3

Behavior Based Safety						Section 4

Bloodborne Pathogens 						Section 5

Disciplinary Program						Section 6

Driving Safety 							Section 7

Drug Free Workplace Program				Section 8

Emergency Action Plan 					Section 9

Fall Protection 							Section 10

Fire Protection 							Section 11

Hand & Power Tools 						Section 12

Hazardous Communication					Section 13

Injury & Illness Recordkeeping				Section 14

Jobsite Clean-Up Procedures					Section 15

Ladder Safety							Section 16

Lockout Tagout							Section 17

Personal Protective Equipment				Section 18

Respiratory Protection 						Section 19

Scaffolds								Section 20

##### Safety Policy Statement

## To All Employees

Your Safety is one of the most vital aspects of our Company’s operations.  No operation at our Company is so important that it needs to be done in an unsafe or unhealthy manner. Our policy is designed to keep our workplace at the highest level of safety and accidents at the lowest possible level.

Our goal is zero accidents and injuries in our workplace.

Management Responsibilities:

To provide or make available, sound equipment, safe tools and necessary protective equipment, and cooperate in all efforts to maintain a safe working environment.  We will commit the necessary resources and time to create a safer and more efficient workplace. We are responsible for implementing new safety rules, regulations and programs.

Supervisors Responsibilities:

Every supervisor is responsible for the safety of the entire employee who works for and around them.  Employee safety is one of the supervisor’s primary responsibilities. Supervisors will be the central liaison in assuring the rules and regulations are enforced.

Employee Responsibilities:

Every employee is expected to cooperate fully in helping protect him/her and those around them.  Participation in safety meetings, following safety rules and contributing to a safe and healthy workplace is included in your duties. This will include reporting unsafe acts and/or working conditions.

_________________________________	______________________________

CEO								Date:

# Assignment of Responsibility

## Safety Organization

Safety Director

The Safety Director acts in a staff capacity to help administer policy, to provide technical information, to help train and to supply program material to supervisors and employees.  The safety director interprets, applies occupational safety and health standards and regulations through supervisors and safety committees.

Safety Board (Management)

The Safety Board meets monthly to review accident reports, review departmental safety records, discusses safety problems and to ensure a safe working environment for the employees.  They also meet representing all aspects of the Corporate/ Operations environments.  The Safety Board monitors safety conditions, through the Safety Committees, and develops policies and/or procedures to accommodate and rectify specific and general areas where safety issues need to be addressed.

Safety Committee (Hourly Employees)

Safety Committee meets monthly and represents all aspects of the Operations environment. The primary role of members of the Safety Committee is to perform departmental Plant Inspections on a monthly basis. General self-inspection of the plant will be rotated between other departments to cross audit unsafe conditions and corrections by Safety Committee members.

All safety violations, unsafe equipment, unsafe conditions and condition of housekeeping are recorded and made available to the Safety Committee, who in turn, will review the existing violations and recommend to the plant superintendent and department supervisor the proper route to expedite the prompt correction of the unsafe condition.

Department Supervisor

While top management has the ultimate responsibility for safety, it delegates authority for safe operation all the way down through all management levels.

The first line supervisor is the key element in a safety program because he or she is constantly in contact with the employees. They are thoroughly familiar with all the hazards that may develop in their department. They should be on the alert at all times to identify and correct unsafe conditions and practices.

Since the supervisors’ duties require them to give close supervision over their department, inspections should be continuous.

Each department supervisor is responsible for the safety of his or her cost center or department. Responsibility requires them to continuously monitor their department since, conditions change and unsafe conditions can arise at any time.

The department supervisor is responsible for good housekeeping and the enforcement of all safety rules and regulations.

Maintenance

The maintenance supervisor makes frequent trips through the plants.  Necessary work orders for correcting faulty equipment or observation of unsafe acts should be first corrected and secondly, written and given to the Safety Committee Member.

The Employee

Alert employees can help prevent accidents.  The safety conscious employee will always look for conditions that may cause an injury to themselves or to others.  Each employee should inspect his or her work place each day and report any hazardous condition to their supervisor.

When an individual enters the employ of this Company, his or her safety is paramount; safety will take precedence over expediency and short cuts.

The employee has a right to expect that he or she will be provided with a safe place in which to work and proper machines and tools with which to do his or her job. Only under such circumstances can the relationship between employer and employee be mutually profitable and harmonious.  It is the Company's desire to provide:

(a) A safe work place, safe equipment, proper materials,

(b) To establish and insist upon safe methods and practices at all times.

The employee has a duty to keep alert at all times,” think safety” and follow all safety rules and regulations provided by the Company.

Safety Offenses

The Safety and Health of our employees, our company and our environment are objectives we must achieve together.

By using your good judgment and the ability to seek others (superiors) for answers, we are contributing to the assurance our workplace remains accident free.

Be advised, any time an employee is observed willfully and knowingly conducting or, causing a hazard for themselves or others around them, corrective action will be enforced immediately through the employee’s superior.  All violation will be tracked through the progressive disciplinary program and could lead to termination.

All documentation drawn up noting the safety violations will be held in the employee’s file, permanently.

AERIAL LIFTS

Purpose

The purpose of this program is to define the requirements for safely operating an aerial lift device.  An aerial lift device is defined as any device, vehicle mounted or manually propelled, telescoping or articulating, or both, which is used to position personnel above six feet in height.

Scope

This policy shall cover all aerial lift devices used on company property.  All Toole Corp. employees shall operate these devices in accordance with this policy.

Key Responsibilities

### Supervisors

- Shall ensure that all aerial devices are properly operated by trained personnel.

- Shall ensure that aerial lift devices are designed and constructed in conformance with applicable requirements of the American National Standards for “Vehicle Mounted Elevating and Rotating Work Platforms”  ANSI A92.2-1969, including appendix.

### Employees

- Shall follow all aspects of this program.

Procedure

- Aerial lifts may be “field modified” for uses other than those intended by the manufacturer provided the modification has been certified in writing by the manufacturer or by an equivalent entity.

- Lift controls shall be tested each day prior to use to determine that such controls are in safe working conditions.  Tests shall be made at the beginning of each shift during which the equipment is to be used to determine that the brakes and operating systems are in proper working condition.

- Only authorized persons shall operate an aerial lift.

- Boom and basket load limits specified by the manufacturer shall not be exceeded.

- Aerial lifts shall have a working back-up alarm audible above the surrounding noise level or the vehicle is backed up only when an observer (spotter) signals that it is safe to do so.

- The minimum clearance between electrical lines and any part of the equipment (i.e. crane or load) shall be 10 feet for lines rated 50 kV or below.

- Employees shall always stand firmly on the floor of the basket, and shall not sit or climb on the edge of the basket or use planks, ladders, or other devices for a work position.

- Approved fall protection shall be worn and a lanyard attached to the boom or basket when working from an aerial lift.

All employees who operate an aerial lift device shall be trained in the safe operation of the specific device they will operate.  Training must conform to all OSHA requirements.

BEHAVIOR BASED SAFETY

Purpose

The Toole Corp. Behavior Based Safety (BBS) initiative is an education and observation process used to improve safety and reduce risk in the workplace. This process uses a proactive approach and is intended to communicate to employees the elements and the procedures of Behavior Based Safety that will assist in reducing at risk behaviors which in turn reduces injuries in our workplaces.

Scope

The Toole Corp. BBS applies to all staff.   Employees are permitted to participate in BBS initiatives already in place at customer locations if required by the customer.   Employees are requested to participate in Behavior Based Safety process and follow the process guidelines.

Requirements

Safety awareness principles are the foundation of the Toole Corp. Behavior Based Safety process.  The key concepts teach employees to recognize when they may be in one of the following states:

- Rushing

- Frustration

- Fatigue

- Complacency (which can cause or contribute to these critical errors)

- Eyes not on task

- Mind not on task

- Line of fire

- Loss of balance/traction/grip (which in turn increase the risk of injury.)

Pre-task Analysis is a process to evaluate the work environment by performing a Job Safety Analysis (JSA) of each job. The purpose of which is to eliminate or control all hazards that may be encountered to complete the job. This process is included in the Behavior Based Safety process to establish the correct habits and work procedures in order to reduce at-risk behaviors.

The observation process is designed to raise safety awareness and provide a feedback mechanism for management to make changes in design, process or procedure in order to reduce at-risk behaviors. The key to this process is raising awareness of behavior through observation and feedback.  The process has three key elements:

### Observation and Feedback

The process starts with the observation of workers - fellow employees, other contractor employees and customer employees as they perform their tasks. Observers collect information about worker performance and provide feedback via the observation card. The emphasis is not on who was observed but rather what behavior was observed. Observations provide direct, measurable information on employee work practices indentifying both safe and unsafe behaviors.  During the observation the observer records their findings on the BBS Observation Form.  Items to be observed include but are not limited to:

- Personal Protective Equipment

- Procedures / Methods

- People

- Work Environment

- Equipment

After the observation is made the observer will review the observation with the observed employee the following:

- Start with positive comments.

- Reinforce safe behaviors observed first.

- Describe and discuss what was unsafe.

- Solicit from observed employee explanation of his/her unsafe behavior with open-ended questions.

- Re-emphasize no consequence to observed employee.

Documenting feedback allows workers to assess what should be repeated and what should change to reduce risks in the workplace.

### Data Management

BBS Observation Forms are forwarded to the corporate safety manager for input into the BBS database. Reports are generated and forwarded to management.  Toole Corp. will compare measurements and track results by an acceptable method for the individual departments and organization overall so a continual trend analysis and statistical comparison of employee safety behavior can be made over time.

### Analysis and Action Planning

Once trend analysis is complete, appropriate action plans are developed by the safety manager to address unsafe behaviors.  Action planning will include:

- Evaluating unsafe behaviors from trend analysis and prioritize.

- Developing an action plan for unsafe behaviors based on comments and feedback from observations.

- Designating responsible parties and timeframes within the action plan.

- Defining who is responsible for action planning.

- Ensuring management support.

### Action Plan Follow Up

All action plans shall be arranged by a set time period.  To ensure effectiveness of the BBS follow-up is necessary to ensure the closure of all actions listed.  The follow-up process will include:

- Monthly frequency for review of action by the safety manager, senior management and employees.

- Assign accountability for closeout of action plans within Lanzo Companies.

- Document archiving of action plans with completed action items.

Responsibilities

### Oversight

The manager/supervisor has these oversight responsibilities:

- Coach observers and develop action plans to ensure continuous improvement.

- Ensure that all employees are trained on the Behavior Based Safety elements.

- Maintain communication with workforce by channeling information in a timely manner (feedback).

- Collect and review process modification change requests from employees.

- After reviewing and giving feedback the BBS/JSA cards should be forwarded to the corporate safety director for data entry.

Each employee plays a specific role in the Behavioral Based Safety process.  These roles include observee, observer, supervisor, manager and safety manager.

### Person being observed

- Be willing to be observed.

- Be open and cooperative.

- Avoid being defensive.

- Participate in problem-solving meetings.

- Be familiar with the Behavior Based Safety process.

### Person performing the observation

- Learn the Behavior Based Safety process and the benefits of reducing at-risk behaviors.

- Promote the Behavior Based Safety process.

- Make observing proactive.

- Be open to coaching.

- Be courteous and helpful.

- Assist workers by offering suggestions to safely perform a task or help them with a task if necessary.

- Communicate with the workers being observed.

- Give constructive feedback after observations.

- Stress the safe behaviors before the at-risk behaviors.

- Offer and work towards solutions of problems found.

- Record a comment for every recorded “at-risk” to include what and why. Make quality observations, concentrating on quality comments.

### Manager

- Actively promote and participate in the behavior safety process by supporting the goals and objectives of the Behavior Based Safety process.

- Ensure that all employees are aware of what is expected of them regarding the BBS process.

- Encourage employees to participate in observations so that incidents/injuries are reduced in the workplace.

- Provide necessary resources to keep process productive.

- Attend safety meetings and offer feedback on areas of improvement.

### Supervisor

- Actively promoting and participating in the Behavior Based Safety process by reviewing BBS Observation Forms turned in at least weekly and giving feedback, completing corrective actions needed, etc.

- Refraining from using data from the Behavior Based Safety process in a punitive manner.

- Assisting in problem solving and completing corrective actions in a timely manner.

- Understanding the behavior safety process and the benefits of reducing at-risk behaviors.

### Safety Manager

- Support the goals and objectives of the Behavior Based Safety process.

- Encourage, promote, provide technical support and assist in acquiring the resources needed for the Behavior Based Safety process.

- Address the concerns and suggestions of field personnel.

- Collect all observation data cards.

- Enter data into BBS database.

Training

Appropriate staff shall be trained for the BBS and the observation process.  The training program shall:

- Include managers and supervisors on a required basis and craft and support employees on a voluntary basis in all work locations.

- General employee awareness as related to how decisions effect behavior and the impact those decisions have on working safely.

- Be conducted using classroom and field settings.

Types of training shall include:

- Management training

- New employee training

- Refresher (annual) training for all participants

Training elements will include:

- Program objectives and incident statistics reviewed

- How to conduct the observation

- How to complete the observation form

- What do the behaviors mean

- Feedback training and role play (mentoring and coaching)

- Employees should be aware they may be requested to be observed at any time

- Documentation of training

BLOODBORNE PATHOGENS

Purpose

This Bloodborne Pathogen Exposure Control Plan has been established to ensure a safe and healthful working environment and act as a performance standard for all employees. This program applies to all occupational exposure to blood or other potentially infectious materials.  The content of this plan complies with OSHA Standard 29 CFR 1910.1030 (Occupational Exposure to Bloodborne Pathogens).

Scope

All employees who have or may have the potential for exposure to blood or other potentially infectious materials in the workplace.

Key Responsibilities

### Exposure Control Officer (Toole Corp. Safety Manager)

Has overall responsibility for developing and implementing the Exposure Control Procedure for all facilities.

### Site Project Manager and Supervisors

Site project manager and supervisors are responsible for exposure control in their respective areas.

### Employees

Know what tasks they perform that have occupational exposure.

Plan and conduct all operations in accordance with our work practice controls.

Develop good personal hygiene habits.

Procedure

### Training

Employees with reasonable anticipated occupational exposure to bloodborne pathogens shall participate in training before their initial assignment and within one year of any previous training.  Training shall include:

| - What bloodborne pathogens are; how to protect themselves from exposure - Methods of warnings (signs, labels, etc.) - The OSHA requirements of bloodborne pathogens - Availability of the Hepatitis B vaccine that have occupational exposure at no cost | Biohazard Label |
| --- | --- |

Availability of Procedure to Employees

The Bloodborne Exposure Control Plan is kept at all locations and shall be accessible to employees.

### Reviews and Update of the Procedure

The procedure is reviewed annually and updated whenever we establish new functional positions within our facility that may involve exposure to biohazards.

### Exposure Determination

- There are no job classifications in which some or all employees have occupational exposure to bloodborne pathogens that may result from the performance of their routine duties.

- Designated employees are trained to render first aid and basic life support.  Rendering first aid or basic life support will expose employees to bloodborne pathogens and will require them to adhere to this program.

- In addition, no medical sharps or similar equipment is provided to, or used by, employees rendering first aid or basic life support.

- This exposure determination has been made without regards to the Personal Protective Equipment that may be used by employees.

- A listing of all first aid and basic life support trained employees in this work group shall be maintained at each work site and at each first aid kit.

Methods of Compliance

### Universal Precautions

### Under circumstances in which differential between body fluids is difficult or impossible, all body fluids will be considered potentially infectious.

### Engineering Controls

Hand washing facilities (or antiseptic hand cleansers or antiseptic towelettes), which are readily accessible to all employees who have the potential for exposure.  Containers for contaminated reusable sharps that our clients provide have the following characteristics:  Puncture-resistant; Color-coded or labelled with a biohazard warning label; Leak-proof on the sides and bottom.

Secondary containers which are:  Leak-proof; Color-coded or labelled with a biohazard warning label; Puncture-resistant, if necessary.

### Work Practice Controls

Employees shall wash their hands immediately, or as soon as feasible, after removal of potentially contaminated gloves or other personal protective equipment.

Following any contact of body areas with blood or any other infectious materials, employees wash their hands and any other exposed skin with soap and water as soon as possible.

Hand washing facilities shall be available.  If hand washing facilities are not feasible Toole Corp. will provide either an appropriate antiseptic hand cleanser in conjunction with cloth/paper towels or antiseptic towelettes.

Contaminated needles and other contaminated sharps should not be handled if you are not AUTHORIZED or TRAINED to do so.  Contaminated needles and other contaminated sharps are not bent or recapped.

Eating, drinking, smoking, applying cosmetics or lip balm and handling contact lenses is prohibited in work areas where there is potential for exposure to biohazardous materials.

Food and drink is not kept in refrigerators, freezers, on countertops or in other storage areas where potentially infectious materials are present.

All equipment or environmental surfaces shall be cleaned & decontaminated after contact with blood or other potentially infectious materials.

Specimens of blood or other potentially infectious materials must be put in leak proof bags for handling, storage and transport.

### Work Practice Controls (continued)

If outside contamination of a primary specimen container occurs, that container is placed within a second leak proof container, appropriately labelled,-for handling and storage.

Bloodborne pathogens kits are located on top of first aid kits and are to be used in emergency situations by the caregiver.  Once the seal is broken on kit and any portion has been used it is not to be reused. Pathogen Kits shall be ordered and replaced promptly. Biohazard bags are identified by stickers and located in the first aid area.  Contaminated supplies are to be disposed at once.

### Personal Protective Equipment

Toole Corp. provides at no cost to our employees gloves, safety glasses, goggles, gowns, face shields/masks and other as need PPE for bloodborne pathogens response.  All PPE shall be of the proper size and readily accessible.

Our employees adhere to the following practices when using their personal protective equipment:

Any garments penetrated by blood or other infectious materials are removed immediately.

All potentially contaminated personal protective equipment is removed prior to leaving a work area.

Gloves are worn whenever employees anticipate hand contact with potentially infectious materials or when handling or touching contaminated items or surfaces.

Disposable gloves are replaced as soon as practical after contamination or if they are torn, punctured or otherwise lose their ability to function as an "exposure barrier".

Masks and eye protection (such as goggles, face shields, etc.) are used whenever splashes or sprays may generate droplets of infectious materials.

Any PPE exposed to bloodborne pathogens shall be disposed of properly.

PPE shall be used unless employees temporarily declined to use PPE under rare circumstances.

PPE should be cleaned, laundered & properly disposed of if contaminated.

Toole Corp. will repair and replace PPE as needed to maintain its effectiveness.

### Housekeeping

Our staffs employs the following practices:

All equipment and surfaces are cleaned and decontaminated after contact with blood or other potentially infectious materials.

Protective coverings (such as plastic trash bags or wrap, aluminum foil or absorbent paper) are removed and replaced.

All trash containers, pails, bins, and other receptacles intended for use routinely are inspected, cleaned and decontaminated as soon as possible if visibly contaminated.

Potentially contaminated broken glassware is picked up using mechanical means (such as dustpan and brush, tongs, forceps, etc.).

### Post-Exposure and Follow Up

Post-Exposure Evaluation & Follow-Up

If there is an incident where exposure to bloodborne pathogens occurred we immediately focus our efforts on investigating the circumstances surrounding the exposure incident and making sure that our employees receive medical consultation and immediate treatment.

The Toole Corp. Safety Manager/ Supervisor investigate every reported exposure incident and a written summary of the incident and its causes is prepared and recommendations are made for avoiding similar incidents in the future.  We provide an exposed employee with the following confidential information:

Documentation regarding the routes of exposure and circumstances under which the exposure incident occurred.

Identification of the source individual (unless not feasible or prohibited by law).

Once these procedures have been completed, an appointment is arranged for the exposed employee with a qualified healthcare professional to discuss the employee's medical status. This includes an evaluation of any reported illnesses, as well as any recommended treatment.

Information Provided to the Healthcare Professional.  We forward the following:

A copy of the Biohazards Standard.

A description of the exposure incident.

Other pertinent information.

Healthcare Professional’s Written Opinion

After the consultation, the healthcare professional provides our facility with a written opinion evaluating the exposed employee's situation. We, in turn, furnish a copy of this opinion to the exposed employee.  The written opinion will contain only the following information:

Whether Hepatitis B Vaccination is indicated for the employee.

Whether the employee has received the Hepatitis B Vaccination.

Confirmation that the employee has been informed of the results of the evaluation.

Confirmation that the employee has been told about any medical conditions resulting from the exposure incident which require further evaluation or treatment.

All other findings or diagnoses will remain confidential and will not be included in the written report.

### Record Keeping

All records shall be made available upon request of employees, OHSA’s Assistant Secretary and the Director of OSHA for examination and copying.  Medical records must have written consent of employee before released.  Toole Corp. shall meet the requirements involving transfer of records set forth in 29 CFR 1910.1020(h).

The respective Human Resources representative shall maintain Bloodborne Pathogen exposure records.

### Record Keeping (continued)

Employee medical records shall be kept confidential and are not to be disclosed without the employee's written consent, except as required by 29 CFR 1910.1030 or other law.

Medical records shall be maintained for the duration of employment plus 30 years and shall include at least the following:

- Employee's name, Social Security number and Toole Corp. employee number.

- Employee's Hepatitis B vaccination status, including vaccination dates.

- All results from examinations, medical testing and follow-up procedures, including all health care professional’s written opinions.

- Information provided to the health care professional.

- Any Hepatitis B Vaccine Declinations.

Training records shall be maintained for 3 years from the date on which the training occurred and shall include at least the following:

- Outline of training program contents.

- Name of person conducting the training.

- Names and job titles of all persons attending the training.

- Date of training.

### Labels and Signs

Biohazard warning labelling shall be used on containers of regulated waste; Sharps disposal containers; contaminated laundry bags and containers; contaminated equipment.

### Information

Information provided to our employees includes:

The Biohazards Standard itself.

The epidemiology and symptoms of bloodborne diseases.

The modes of transmission of bloodborne pathogens.

Our facility's Exposure Control Procedure (and where employees can obtain a copy).

Appropriate methods for recognizing tasks and other activities that may involve exposure.

A review of the use and limitations of methods that will prevent or reduce exposure.

Selection and use of personal protective equipment.

Visual warnings of biohazards within our facility including labels, signs and "color-coded" containers.

Information on the Hepatitis B Vaccine.

Actions to take and persons to contact in an emergency involving potentially infectious material.

The procedure to follow if an exposure incident occurs, including incident reporting.

Information on the post-exposure evaluation and follow-up, including medical consultation.

VACCINATION DECLINATION FORM

Date:

Employee Name:

Employee ID#:

I understand that due to my occupational exposure to blood or other potential infectious materials I may be at risk of acquiring Hepatitis B virus (HBV) infection. I have been given the opportunity to be vaccinated with Hepatitis B vaccine, at no charge to myself.  However, I decline the Hepatitis B vaccination at this time.  I understand that by declining this vaccine, I continue to be at risk of acquiring Hepatitis B, a serious disease.  If, in the future, I continue to have occupational exposure to blood or other potentially infectious materials and I want to be vaccinated with hepatitis B vaccine, I can receive the vaccination series at no charge to me.

Employee Signature								Date

Facility Representative Signature						Date

POST-EXPOSURE EVALUATION AND FOLLOW-UP CHECKLIST

The following steps must be taken, and information transmitted, in the case of an employee's exposure to bloodborne pathogens:

ACTIVITY                              					      	 COMPLETION DATE

Employee furnished with documentation

regarding exposure incident.

Source individual identified.

(	) Source individual

Appointment arranged for employee

with healthcare professional.

(	)

Professional's name

Documentation forwarded to healthcare professional

Bloodborne Pathogens Standard

Description of exposed employee's duties

Description of exposure incident, including routes of exposure

DISCIPLINARY PROGRAM

Purpose

The purpose of this program is to establish a firm but fair disciplinary action policy to enforce the safety system.

Scope

This document is applicable to all employees.

Responsibilities

It is the responsibility of each and every person employed by Toole Corp. to work in a safe and efficient manner.  The safety system provides guidelines and procedures to help insure that safe work practices are observed.  In the event that any employee violates provisions of the Toole Corp. safety system or works in a manner that threatens his own health and safety or the health and safety of the employees 	around him, he will be subject to disciplinary action, up to and including termination of employment.

The safety manager, operations managers, supervisors and foremen are responsible for enforcing the safety system and for issuing disciplinary action as required by this section of the safety manual.

Toole Corp. is committed to safety and senior management holds all supervisory staff responsible and accountable for safety within their respective areas.

Physical inspections by Toole Corp. officials or insurance representatives that indicate violations showing overall lack of commitment to Toole Corp. safety goals shall be under the same level of disciplinary actions.

Requirements

Safety is a core value and a condition of employment at Toole Corp.. The following actions constitute a safety violation:

- Not following verbal or written safety procedures, guideline or rules of Toole Corp. or our clients

- Horse play, failure to wear required PPE, and or abuse of PPE

- Being under the influence of drugs or alcohol during work

- Bringing weapons on the job site

- Failure to report incidents or injuries

- Attempted or actual physical force to cause injury, threatening statements or other actions to cause an employee to feel they are at risk of injury.

Procedure

The following procedures will be following after issuing a safety violation notice:

- The first offense will result in a verbal warning.  The employee is to be 	informed that he is being issued a verbal warning and informed why.  	Proper procedure will be discussed to clarify the situation and allow the employee to correct his behavior.  The person making this verbal warning will inform the operations manager of his branch that this warning has 	been issued so the operations manager may make a written record of the warning.

- The second offense will result in a written reprimand and additional training.  The reprimand will be written on the standard Safety Reprimand form (see below) and will describe the unsafe activity or behavior that needs correction.  Refer to the section of the safety program that was violated (when applicable).  The employee receiving the reprimand has the right to submit a written rebuttal to the reprimand.  The employee must sign the reprimand.  The reprimand and any rebuttal will become a part of the employee's employment records.

- The third offense will result in another written reprimand (using the standard form) and punitive layoff, the duration of which will be decided at the time of the disciplinary action and is to be weighed by the severity of the offense.  Again, the employee may submit a written rebuttal to the reprimand.  The employee must sign the reprimand.  The reprimand and any rebuttal will become a part of the employee's employment records.

- The fourth offense may result in the termination of the offending employee.

The above actions are to be placed against a sliding twelve month scale.  If an employee receives a reprimand on January 1 and commits his fourth offense on or before December 31st of the same year, he is terminated.  The employee does not have to commit the same violation each time to receive further reprimands.  He could receive a verbal reprimand for smoking in a no smoking area on his first offense and get a written reprimand for his second offense which might be a forklift violation and yet another for failing to use proper personal protective equipment.  He will be terminated upon his fourth offense in the last twelve months.

In the case of serious safety violations such as by-passing guarding or other unsafe activities that put the violator or other employees at serious risk of injury, the manager may move the violator directly to the second or third warning level.  If the violator’s actions put him or others at risk of death or dismemberment the manager has the option to terminate him with no further warning.

Safety Reprimand Form

Date: 		____________________					Reprimand # ____

Issued To: 	_____________________________________________________

Signature:	_____________________________________________________

Issued By:	_____________________________________________________

Signature:	_____________________________________________________

Violation (Describe in Detail):

Follow up Training:	______________________________________________________

Presented by:		______________________________________________________

Date of Training:		______________________________________________________

Trainee Signature:	______________________________________________________

DRIVING SAFETY

Purpose

This program is written to be in compliance with local regulatory requirements and provide directives to managers, supervisors, and employees about their responsibilities in the operations and management of Toole Corp. vehicle safety.

Key Responsibilities

### Toole Corp. Safety Manager

- The designated Safety Manager is responsible for developing and maintaining the program and related procedures.   These procedures are kept in the designated safety manager’s office.

### Site Manager

- Responsible for the implementation and maintenance of the program for their site and ensuring all assets are made available for compliance with the plan.

### Employees

- All shall be familiar with this procedure and the local workplace vehicle safety program.

- Follow all requirements, report unsafe conditions, and follow all posted requirements.

Vehicle and Transportation Related

### Driving Safety

Operators of Toole Corp. or client on or off road vehicles shall be qualified by possession of a valid, current driver’s license for the type of vehicle being driven.

Only authorized employees will drive a motor vehicle in the course and scope of work or operate a company owned vehicle.

Drivers shall have 3 years of driving experience on the vehicle he/she is licensed to drive & regularly drives.

Drivers will be appropriately assessed, licensed and trained to operate the vehicle they have been authorized to operate.

No passengers shall be on trucks used to deliver goods.

Backing is prohibited whenever practicable.  Where backing is required, drivers, when parking, should make every effort to park the vehicle in a manner that allows the first move when leaving the parking space to be forward.

Drivers must have either a reversing alarm, use a spotter or walk around the truck/trailer prior to backing.

Passenger compartments are to be free from loose objects that might endanger passengers in the event of an incident.  Any vehicle with non-segregated storage shall be equipped with a cargo net or equivalent to separate the storage area.

### Driving Safety (continued)

Vehicles (light vehicles, heavy vehicles and trailers) may not be modified without the endorsement of the manufacturer.

Signs, stickers or labels are to be fitted in such a manner that they do not obstruct the driver's vision or impede the driver's use of any controls.

Employees driving vehicles are required to follow safe driving practices:

Obey all federal and local driving laws or regulations as well as requirements of clients;

Immediately report any citation, warning, traffic violation, collision, vehicle damage or near miss associated with company or client vehicle operation or while driving on company duties to the supervisor;

Immediately report any restriction or change to their driving privileges to the supervisor.

Seat belts shall always be worn by all occupants whenever the vehicle is in motion; only seats fitted with three-point inertia-reel type seatbelts shall be used.  All vehicles capable of more than 10 mph/15 kph shall have seat belts installed.

Defensive drivers continually assess conditions and hazards and remain prepared for any challenge that may approach them;

When speaking with a passenger, always keep your eyes on the road;

Both hands on the wheel;

No use of cell phones, radios or other electronic devices while driving any vehicle - vehicle must be safely parked prior to using a mobile phone or 2-way radio.

Slow down around construction, large vehicles, wildlife, fog, rain, snow, or anything else that adds a hazard to your driving;

Drive for conditions, not just the speed limit;

Alcohol or illegal drugs are not allowed to be in a company, client or leased vehicle at any time;

Drivers shall not operate a motor vehicle while under the influence of alcohol, illegal drugs, or prescription or over-the counter medications that might impair their driving skills.

Drivers are to be prepared before leaving:

Perform 360 walk around – report new damage;

Check windshield for cracks that could interfere with vision;

Inspect for vehicle damage and immediately report any damage to the supervisor if not previously observed;

Make sure dirt or snow is removed from lights on all sides of the vehicle;

Brush or clean off snow or ice on all windows to ensure complete vision;

Check fuel level to be certain the destination can be reached;

Check to ensure the license plates and inspection tag on vehicle are current;

Ensure that there is a first aid kit and inspected fire extinguisher in the company vehicle;

Ensure driver is rested and alert for driving;

Employees are not to perform repairs or maintenance other than routine fluid additions.

### Vehicle Requirements

All vehicles shall be fit for the purpose, and shall be maintained in safe working order.

No vehicle less than 1000 kg is to be used on public roads.

Tires, including spares if full size, are to be of same type, profile and tread pattern, except when the vehicle or tire Manufacturer recommends a different type for certain axles.

Tire type and pattern is to be recommended by the vehicle or tire manufacturer for use on the vehicle in the area of operation.

Vehicles are to be fitted with a spare wheel and changing equipment to safely change a wheel, or a suitable alternative.

Vehicles longer than 6 meters/20 feet or with restricted rear view (i.e. pickup trucks that are fully loaded) are to be fitted with an audible reversing alarm.

All seats are to be fitted with headrests

All light duty vehicles (including buses) are to be equipped with an adjustable left, right and central rear view mirrors

Loads shall be secure and shall not exceed the manufacturer’s specifications and legal limits for the vehicle.

All vehicles are to be equipped with a multipurpose fire extinguisher with a capacity of at least 0.9 kg/2 lb.  The fire extinguisher shall be securely mounted on a bracket and located so that it is easily accessible in an emergency without becoming a hazard in case of an incident.

All light vehicles shall be equipped with a securely stowed first aid kit.

All drivers of light vehicles shall carry a high visibility jacket for use in case of emergency stops.

All light duty vehicles carry a minimum of one collapsible hazard warning triangle.

Rollover protection will be installed in any vehicle to address high risk environments.  The rollover protection engineered will conform to recognized regulatory standard and industry preferred practices.

All light equipment vehicles shall be outfitted with two red high-intensity lights located as high, as far apart, and as far back as practical, wired to the headlight switch, but also with an override switch, if permitted by local regulations.

### Transportation

If workers are required to travel in a worker transportation vehicle Toole Corp. must ensure that reasonable measures are taken to evaluate road, weather and traffic conditions to ensure the safe transit of the workers.

The operator of a worker transportation vehicle must ensure that the worker transportation vehicle has been inspected by a qualified person before first use on a work shift.

Seated workers must wear seat belts while being transported in a vehicle equipped with seat belts.

A worker must not ride in a vehicle in a standing position, unless protected from being thrown off balance.

A worker must not ride in a vehicle with any part of the body outside the vehicle unless essential to the work process and then only if the worker is adequately restrained.

Materials, goods, tools or equipment carried in a portion or compartment of a vehicle in which workers are riding must be located and secured to prevent injury to the operator or workers.

Any enclosed portion or compartment of a vehicle in which workers are transported must have:

- effective ventilation, independent of doors, providing clean air,

- adequate lighting and means for heating and cooling,

- an effective means of communication between the operator and passengers, and

- more than one means of exit.

### Traffic Control

Toole Corp. shall develop, in writing, and implement a traffic protection plan for its workers at a worksite if any of them may be exposed to a hazard from vehicular or pedestrian traffic that may endanger the safety of any worker.  It shall include the following control measures:

- Effective means of traffic control shall be provided whenever the unregulated movement of vehicular traffic constitutes a hazard to workers.

- Traffic control shall include barricades and cones as the primary control and, where required, signs, flagmen or other techniques and devices made necessary by the prevailing circumstances.

- Operations or equipment, encroaching on the traveled way, shall be protected by barricades and cones as the primary control and, where required other effective devices.

- Toole Corp. must train workers in the traffic control safe work procedures.

- Toole Corp. will ensure that before a worker is designated as a flag person, the worker is trained in the safe work procedures for the safe control of traffic operations and wears the appropriate high visibility outer clothing and/or equipment.

- If a worker at a project on a highway may be endangered by vehicular traffic unrelated to the project, the project shall make use of as many measures as necessary to adequately protect the worker.

- A worker who is required to set up or remove traffic control measures on a roadway or a shoulder of a roadway shall be a competent worker, shall be equipped with the appropriate high visibility apparel, shall not perform any other work while setting up or removing the measures and shall be given adequate written and oral instructions in a language that he or she understands, with respect to setting up or removing the measures.

### ATV Vehicles

If an Toole Corp. work site utilizes ATV vehicles then the following shall apply:

- If the manufacturer has not set limits for operation of the ATV on sloping ground, 5% is the maximum allowable slope unless Toole Corp. has developed and implemented written safe work procedures appropriate for any steeper slope on which the equipment is to be used.

- Toole Corp. must ensure that each ATV operator is properly licensed and trained in the safe operation of the vehicle.  The training program for an ATV operator must cover:

  - the operator's pre-trip inspection,

  - use of personal protective apparel,

  - operating skills according to the ATV manufacturer's instructions,

  - basic mechanical requirements, and

  - loading and unloading the vehicle, if this is a job requirement.

- An ATV operator and any passenger on an ATV must wear approved eye and hearing protection as required by local regulatory requirements and the Toole Corp. PPE Program.  An ATV operator and any passenger on an ATV must wear clothing suitable for the environmental conditions and when necessary to protect against the hazards presented at the worksite, suitable gloves and clothing which covers the ankles and legs and the arms to the wrists and appropriate footwear.

- Toole Corp. requires that approved helmets shall be worn by the operator and passenger.

- Loading and unloading of an ATV onto or off a carrier vehicle must be done in a safe manner.  If ramps are used when loading or unloading an ATV they must be placed at a suitable angle, be sufficiently wide and have a surface finish which provides an adequate grip for the ATV's tires.

beginning work at the specific site.

DRUG FREE WORKPLACE PROGRAM

Drug Free Workplace

The unlawful manufacture, distribution, dispensation, possession or use of a controlled substance is prohibited in the Toole Corp. workplace. A “controlled substance” means any controlled substance in schedules I through V of Section 202 of the Controlled Substances Act (21U.S.C. Section 812). Employees violating such prohibition will be subject to: 1) disciplinary action up to and including discharge in accordance with the Toole Corp. policies and procedures; 2) satisfactory participation in and completion of a drug abuse assistance or rehabilitation program as required by the Toole Corp. Drug Free Workplace policy, if any.

Each employee, as a condition of employment, will abide by the terms of this statement and notify the personnel coordinator of any criminal drug statute conviction for a violation of this prohibition occurring in the workplace no later than five days after such conviction. Failure to timely provide the required notice of a drug statute conviction occurring in the workplace will result in discipline, up to and including discharge.

Dangers of Drug Abuse in the Workplace

Drug abuse in the workplace is one of the most common health and safety hazards in the workplace. Many drugs impair the physical and mental abilities of employees. One study has found that 40% of workplace deaths and 47% of all workplace accidents are drug related.

Drug abuse in the workplace also adversely impacts on productivity. Substance abusers work at only two-thirds of their capacity, are absent and tardy more often than non-abusers, and cause quality control problems. It is estimated that substance abuse in the workplace cost U.S. employers $100 billion dollars per year in productivity losses.

Finally, drug abuse in the workplace cause theft and employee morale problems. Studies have shown that 10% of drug abusers steal from their employers and co-workers or attempt to sell drugs to their co-workers to support their drug abuse habits. The real victims of drug abuse in the workplace are the drug abusers’ co-workers who must work overtime and otherwise cover for the loss productivity associated with drug abuse in the workplace.

EMERGENCY ACTION PLAN

Purpose

Each Toole Corp. location shall have a written Emergency Action Plan, appropriate to the hazards of the workplace, in order to respond to an emergency that may require rescue or evacuation.

Each Emergency Action Plan shall be prepared to reflect all known probable emergency conditions which may arise from within the workplace and from adjacent workplaces, the minimum of which will include fire or other emergencies.

An emergency action plan must be in writing, kept in the workplace and available to employees for review. However, if a site has 10 or fewer employees the plan may be orally to employees.

Emergency Response Planning, Issuing and Annual Review Guidelines

Emergency Procedures shall be issued and discussed with all new/transferred personnel upon arrival for assignment.

Emergency Action Plans shall be established, implemented, reviewed, maintained and updated annually in conjunction with:

- Client emergency services department requirements.

- Toole Corp. safety staff and management.

- The requirement to ensure the plan is up to date to reflect current circumstances at the workplace.

The plan is to be reviewed before the job and when conditions warrant and should be used for routine and non-routine emergencies as well as changes in operation, and products or services which warrant new emergencies situations.

Additionally, a review of the emergency action plan should occur with employees:

- When the plan is developed or the employee is assigned initially to a job.

- When the employee's responsibilities under the plan change.

- When the plan is changed.

Evacuation Procedures Planning

Procedures for emergency evacuation shall include type of evacuation and exit route assignments. The individual site evacuation procedure shall be appropriate to the risk must be developed and implemented to:

- Notify staff, including the first aid attendant, of the nature and location of the emergency,

- Evacuate employees safely,

- Check and confirm the safe evacuation of all employees,

- Notify the fire department or other emergency responders, and

- Notify adjacent workplaces or residences which may be affected if the risk of exposure to a substance extends beyond the workplace. Notification of the public must be in conformity with the requirements of other jurisdictions, including provincial and municipal agencies.

List of Potential Emergencies

Each location shall conduct a risk assessment for hazards posed by potential hazardous substances from accidental release, fie or other such emergencies that could cause an evacuation or rescue and list the potential emergencies for Toole Corp. operations.  Procedures for each of these potential emergencies shall be contained within the Emergency Action Plan.  Examples include:

- Fire

- Gas Leaks/Chemical Spills

- Bomb Threats

- Medical Emergencies

- Explosion

- Workplace Violence

### Guidance Procedures for Potential Emergencies

### Fire

- Warn others in the immediate area.  Notify the appropriate emergency response personnel by phone or radio and pull the nearest fire alarm if present.

- If nearby staff have been trained, and it is safe to do so, fight the fire using a portable fire extinguisher. Remember, if in doubt get out.

- Evacuate the premises via the nearest exit and proceed to the nearest Emergency Assembly Area.

- Re-enter only after the Emergency Coordinator has given an ALL CLEAR.

### Gas Leaks/Chemical Spills - Upon smelling or noticing a gas leak or unusual vapors, or a chemical spill:

- Pull fire alarm (if present) or sound warning and evacuate the premises via the nearest exit

- Proceed to the Emergency Assembly Area

- Contact local emergency response personnel by phone or radio

- Re-enter only after the Emergency Coordinator has given an ALL CLEAR.

If employees are required to control a release of a hazardous substance, to perform cleanup of a spill, or to carry out testing before re-entry, Toole Corp. shall provide:

- Adequate written safe work procedures and documented training.

- Appropriate personal protective equipment which is readily available to employees and is adequately maintained, and

- Material or equipment necessary for the control and disposal of the hazardous substance.

### Bomb Threats

- If a threat is received by phone, mail or other means, get as much information as possible.

- If the threat is received by phone, try to keep the person on the line for as long as possible.  Do not hang up the phone, even after the call has been terminated.

- Contact local emergency response personnel by phone or radio.

- If a suspicious device is identified, evacuate the immediate area and notify local emergency response personnel.

### Guidance Procedures for Potential Emergencies (continued)

### Medical Emergencies

- Call for assistance by phone or radio.  Give the exact location and details of the medical emergency.

- If qualified, provide basic first aid, and keep the person comfortable.  Do not move the person.  Do not leave him/her unattended.

Arrange for emergency medical transportation based on the medical planning portion of the site’s Emergency Action Plan.

### Explosions

Get down on the floor, take shelter under tables or desks, and protect your face and head against flying glass and debris.

Once it is safe to do so, evacuate the premises via the nearest exit and proceed to the nearest Emergency Assembly Area.

Re-enter only after the Emergency Coordinator has given an ALL CLEAR.

### Workplace Violence

Notify security immediately by phone or radio and report the occurrence.

Do NOT attempt to physically intervene.  Protect yourself first at all costs.

Emergency Response Equipment

### Listing of Types of Emergency Equipment

Each site Emergency Action Plan shall identify, list the locations of and provide operational procedures for types of emergency equipment.  For off-site locations, available emergency equipment should be identified and reviewed with workers prior to commencing work activities.  Examples include:

Living areas with an audible alarm and a fire hose cabinet.

Emergency lighting, exit doors, dampers and fire stop flaps.

First aid kits located throughout the facility and in vehicles.

Portable fire extinguishers being located throughout the facility and clearly marked.

Only authorized and trained personnel will operate emergency equipment.

### Inspection & Maintenance Records

Maintenance records must be kept, including but not limited to the name of manufacturer, the type of equipment, the date put into service, when and for what purpose the equipment has been used, the date of the last inspection and name of the inspecting person, any damage suffered, and the date and nature of any of maintenance on emergency response equipment.

Ropes and associated equipment must be inspected visually and physically by qualified employees after each use for rescue, evacuation or training purposes.

The Toole Corp. designated representative will perform and maintain the Toole Corp. Emergency Inspection Checklist Form on a monthly basis.  The checklist shall be maintained for retention in active files for two years and in on site archives for seven years.

Media Response Plan

Toole Corp. employees must not be interviewed by anyone unless the Legal Department has given prior approval. In most cases the Legal Department will have an attorney present for such interviews.

Note: If after Toole Corp. personnel have received approval for an interview from the Legal Department and another party’s attorney appears unannounced, you should politely adjourn the interview until the Toole Corp. Legal Department can be contacted.  Personnel must not give any work related interviews, affidavits, written or recorded statements, or depositions without the express approval from the Toole Corp. Legal Department.

In the case of interviews of Toole Corp. employees by non-attorneys, (law enforcement, government officials, media, etc) you must inform the Legal Department before the interview.  If the interview is taped or videotaped, you must request a copy of the tape. If the interview is reduced to writing, you must ask for a copy of any notes or statements taken.  This procedure is to avoid information being misrepresented.

All media requests should be referred to the Toole Corp. Chief Operating Officer.  Unless requested to do so by the Legal Department, other company personnel are not to give interviews or make statements to the media.  Management prefers that families of personnel involved in an incident receive initial notification from a Toole Corp. representative and not the media.

Training

Toole Corp. shall ensure training for Emergency Action Plan is delivered, documented and prepares the staff and facility for emergency conditions.  Toole Corp. will designate and train employees to assist in a safe and orderly evacuation of other employees.  Requirements include:

All employees must be given adequate instruction in the fire prevention and emergency evacuation procedures applicable to their workplace.

The designated site representative shall provide the Emergency Action Plan orientation to all new/transferred personnel before they begin work.

All personnel shall receive a review/update orientation at least annually, or whenever any new/revised information is to be provided.

The Emergency Action Plan Orientation Check List shall be completed after orientation and the record maintained in the individual’s training records.

Toole Corp. management shall ensure that contractors/consultants working in areas under the supervision of Toole Corp. also receive the Emergency Action Plan orientation upon arrival to the area.

Employees expected to perform duties under the Emergency Action Plan will be trained prior to assuming their roles.   This will include simulated rescue or evacuation exercises and regular retraining, appropriate to the type of rescue or evacuation being provided, and training records must be kept.

A list of trained staff responders shall be posted and maintained indicating their name, response function, their work location and what type of equipment they have been trained for.

Location and Use of Emergency Facilities

Toole Corp. shall ensure each Emergency Action Plan lists the location and how to use emergency facilities for each work site.   For off-site locations, outside services that can provide assistance in the event of an emergency should be identified and reviewed with workers prior to commencing work activities. A list shall be posted in a conspicuous area showing local emergency facilities and how to contact.  Examples include:

- Client Emergency Response Department (Initial Responder for All Emergencies If Applicable)

- Local Police, Local Hospital, Poison Center (Poison Response)  1-800-332-1414, etc.

Fire Protection & Response

Toole Corp. shall ensure each Emergency Action Plan provides fire protection and response planning within each site Emergency Action Plan and is utilized during all phases of work.  As a minimum, all shall include the following:

### Protection

Smoking is not permitted except in designated ‘SMOKING” areas.

Facilities shall be designed and maintained in accordance with local fire code and regulations.

Portable fire extinguishers shall be stationed, inspected and maintained in accordance with local fire code and regulations.  Toole Corp. personnel shall be trained in their use.

Flammable and combustible liquids shall be properly stored.

Employees shall report all fire safety issues to their immediate supervisor.

Facilities shall be inspected by use of the Toole Corp. Emergency Inspection Checklist

### Response

In the event of a fire, personnel working in facility will adhere to the following procedure for their work area:

Warn others in the immediate area.  Notify the appropriate emergency response personnel by phone or radio and pull the nearest fire alarm if present.

If nearby staff have been trained, and it is safe to do so, fight the fire using a portable fire extinguisher. Remember, if in doubt get out.

Evacuate the premises via the nearest exit and proceed to the nearest Emergency Assembly Area.

Re-enter only after the Emergency Coordinator has given an ALL CLEAR.

Roads are designated as fire lanes.  Vehicles can stop there for unloading, but no parking will be allowed.

Alarm & Emergency Communication

### Each Emergency Action Plan for Toole Corp. shall contain methods to address alarms and communications in case of an emergency.  For off-site locations, the method of emergency notification should be identified and reviewed with workers prior to commencing work activities.

### Alarm System

Each work location must have and maintain a system to alert employees of emergencies. The alarm system shall be distinctive and recognizable as a signal to evacuate the work area or perform actions designated under the emergency action plan. For sites with 10 or fewer employees in a particular workplace, direct voice communication is an acceptable procedure for sounding the alarm provided all employees can hear the alarm. Each Emergency Response plan will describe how to activate an alarm and what to do after either activating or hearing an alarm.

Personnel responding to any alarm shall avoid complacency.  Every alarm should be treated as an actual incident until proven otherwise.  Treating and responding to alarms as a routine happening can result in injuries, fatalities and destruction of property.

### Communications

Toole Corp. responders and security use telephones, cell phones and radios in conjunction with emergency response.

Rescue and Evacuation Procedures

### Procedures for Rescue and Medical Services

Each site Emergency Action Plan shall address who performs recue services when required.  It is the position of Toole Corp. that all rescue and medical duties are performed by client emergency responders or local governmental responders when on their location.  For off-site locations, evacuation procedures and methods of rescue shall be identified and reviewed with workers prior to commencing work activities.

At least one member of a rescue team must be a first aid attendant trained to immobilize an injured employee.

Effective communications must be maintained between the employees engaged in rescue or evacuation and support persons.

### Procedure for Evacuation

### Preparation for Evacuation

Each site Emergency Action Plan shall contain a procedure for evacuation if required.

The Toole Corp. designated Emergency Coordinator will maintain an active list of all Toole Corp. and contract emergency responders.

### Critical Plant Operations Personnel

Staff designated to remain in the facility to shut down or supervise critical operations or equipment will be specifically trained and authorized by management to perform their duties before any evacuation may occur.

### Procedure for Evacuation (continued)

### Evacuation Drills

Evacuation drills shall be conducted at least annually.  Before conducting an evacuation drill a pre-drill assessment of the evacuation routes and assembly points shall be conducted. The pre-drill assessment is intended to verify that all egress components (stairs, doors, etc.) are in proper order and that occupants can use them safely.

### Coordination Within a Facility

Emergency training and drills should also be coordinated within a Toole Corp. facility so that key staff are involved in the planning process and are aware of their responsibilities in an emergency as well as during the drill.

Facility management also needs to be informed of the potential for the interruption in productivity and business operations. Alternatives for the continuity of critical operations need to be considered.

### Emergency Evacuation Notification and Routes

In the event of an emergency occurring within or affecting the work site, the Emergency Coordinator makes the following decisions and ensures the appropriate key steps are taken:

Advise all personnel of the emergency.

Activate the emergency notification sequence to alert the appropriate responders and initiate emergency notification within the building.

Evacuate all persons to the identified assembly area and account for everyone including visitors and clients.

All personnel will proceed to the primary safe area immediately located at the identified emergency assembly area for their location.

A copy of escape routes shall be posted in all offices, at all alarm stations and at all exits.

### Sweep Check by Toole Corp. Designated Responders

Toole Corp. trained responders will establish a pattern that will permit covering the area in the shortest time, with a minimum of backtracking.

When the evacuation alarm rings, stop work immediately, and conduct a sweep of the area.  Ask everyone to leave the premises immediately and proceed to the identified emergency assembly area for their location.

If you encounter smoke or flame, leave that section immediately, finish your sweep and evacuate the building by activating fire alarm pull stations.  Remember, if in doubt get out.

If anyone refuses to leave, note their name and location, and advise the client emergency services personnel.

Meet the client emergency services personnel and advise them of your sweep or an area of smoke or flame that you were unable to check.  Assist with head count and evacuation if required.

Ensure that everyone stays at the emergency assembly area until the Emergency Coordinator has given an all clear to re-enter the building.

In the event of inclement weather, the client will make arrangements to have buses either as temporary shelter or to transport personnel to another location.

### Procedure for Evacuation (continued)

### Evacuation or Drill Evaluation

Following an evacuation or drill a response review shall be conducted and documented by the Toole Corp. Emergency Coordinator and lessons learned share with the appropriate responders and staff using the Toole Corp. Evacuation Report.

Emergency Response Program Management

The Toole Corp. site manager will have the overall accountability for administering the Emergency Action Plan. This is the person who may be contacted by employees who need more information about the plan or an explanation of their duties under the plan.

For the purpose of this Emergency Action Plan guidance the Emergency Coordinator will be designated by the Toole Corp. site manager.  His/her alternate will be the Toole Corp. Site Safety Supervisor or otherwise designated by the site manager.

Employees performing rescue or evacuation must wear personal protective clothing and equipment appropriate to the hazards likely to be encountered.

### Duties

### Toole Corp. Emergency Coordinator

The Toole Corp. Emergency Coordinator ensures that:

Evacuation drills are conducted on an annual basis.

Inspections of facilities are performed monthly.

All necessary repairs of components for evacuation paths are completed.

Plans for the modification of any part of an evacuation path are reviewed.

An up to date list of Fire Wardens is maintained.

Radios and reflective vests and other response equipment are available.

During an evacuation or evacuation exercise, the Toole Corp. Emergency Coordinator:

Coordinates activities in accordance with either local authorities or the client Security and ERT as required.

Coordinates Fire Wardens and informs them the nature of the emergency via handheld radios.

Following an evacuation or evacuation exercise, the Toole Corp. Emergency Coordinator:

Notifies Fire Wardens that it is safe to re-enter the building.

Prepares a report following an evacuation (actual or drill).

Reports to management for follow up or corrective actions.

### Toole Corp. Site Safety Supervisor

Assist the Toole Corp. Emergency Coordinator when requested.

### Fire Wardens

Be equipped with radios and reflective vests.  The equipment is to be handed into the Toole Corp. Emergency Coordinator and reissued to the next oncoming Fire Warden for the designated area.

Be familiar with exits and muster stations for their responsible area.

Direct residents safely out of the building to the designated muster station or to an alternate location.

Sweep their effected area, ensuring that the alarms are properly functioning and that residents evacuate safely.

In order to account for all employees after evacuation the fire wardens or designated personnel shall complete a head count and reconcile the evacuees with the attendance or daily housing report at the assigned muster station or alternate location.

Radio unaccounted for personnel to Security.

Notify personnel that they may re-enter the building when permission has been given by the appropriate authorities.

### Residents, Contractors & Visitors

All employees, users, contractors and visitors will follow the instructions of the Fire Wardens, Security, ERT, Safety Personnel, managers and supervisors when asked to evacuate the building.

Know the two safest and most direct evacuation routes from their work area(s).

Know the designated evacuation assembly point for the building.

Toole Corp. Emergency Inspection Checklist

Page 1

| Department: | Location: | Date of Inspection: |
| --- | --- | --- |
| Inspected by: | Title: | Ext: |

| This form is to be used monthly. | N/A | Yes | No |
| --- | --- | --- | --- |
| EGRESS |  |  |  |
| Is every means of egress arranged and clearly marked, so that the way to safety is unmistakable at all times? |  |  |  |
| Are exits signs lit? |  |  |  |
| Are there sufficient exits for the prompt escape of all employees in case of fire or other emergencies? |  |  |  |
| Are doors that aren’t exits that could be mistaken as one, clearly marked “Not an Exit”? |  |  |  |
| Do exit doors swing out? |  |  |  |
| Are means of egress at least 28 inches at any point and adequate width for the number of people? |  |  |  |
| Are egresses kept clear of obstructions and materials at all times? |  |  |  |
| Is there proper lighting for emergency exiting? (i.e. during a power failure) |  |  |  |
| Are at least two exits by separate ways of travel available for each occupant? |  |  |  |
| Is the minimum width of any exit way no less than 28 inches? |  |  |  |
| Are furnishings and decorations so placed that they will not obstruct the exits, the access thereto, or the egress there from, or the visibility thereof? |  |  |  |
| Are explosive and highly flammable furnishings or decorations prohibited? |  |  |  |
| EMERGENCIES/EVACUATION |  |  |  |
| Are evacuation maps posted in readily accessible places? |  |  |  |
| Do employees know where their muster point is located? |  |  |  |
| Do employees know area hazards, the nearest exit and alternate routes of escape? |  |  |  |
| Do employees know the preferred means of reporting emergencies? |  |  |  |
| Do employees know the site emergency number(s)? |  |  |  |
| Is the site emergency number posted on or by the phone? |  |  |  |
| Do employees know what signal indicates evacuation? |  |  |  |
| Can all personnel perceive the employee alarm? |  |  |  |
| Do employees with special assistance needs been addressed? |  |  |  |
| Employees questioned know where the emergency shut off is for the natural gas |  |  |  |
| FIRE PROTECTION |  |  |  |
| Are fire hydrants accessible? |  |  |  |
| Are fire hydrants inspected yearly and records maintained to show the date? |  |  |  |
| Are control and operating valves locked open or electronically supervised? |  |  |  |
| Are fire hoses maintained and periodically tested? |  |  |  |
| Are combustible materials kept away from ignition sources? |  |  |  |
| Are standpipe and hose system components visually inspected quarterly? |  |  |  |
| Is the accumulation of flammable and combustible materials controlled so they do not contribute to fire emergency? |  |  |  |
| All product, supplies, merchandise etc not piled within 18” of Sprinkler heads |  |  |  |
| No Combustibles within three feet of Hot Water Tank, Space Heaters and/or Electrical panels |  |  |  |
| All Compressed Gas Cylinders tied or chained to eliminate tipping |  |  |  |

Toole Corp. Emergency Inspection Checklist

Page 2

| Department: | Location: | Date of Inspection: |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Inspected by: | Title: | Ext: |  |  |  |  |
| DETECTION AND ALARM SYSTEMS |  |  |  |  |  |
| Are detection systems installed and maintained? |  |  |  |  |  |
| Are all trouble alarms and fire signals investigated? |  |  |  |  |  |
| Do detection/alarm systems shut down or reverse HVAC systems for smoke control? |  |  |  |  |  |
| Do detection/alarm systems close smoke or fire doors? |  |  |  |  |  |
| Do detection/alarm systems activate local alarms? |  |  |  |  |  |
| Are alarm and PA systems periodically tested? |  |  |  |  |  |
| PORTABLE FIRE EXTINGUISHERS |  |  |  |  |  |
| Does everyone know where the nearest fire extinguisher is stored? |  |  |  |  |  |
| Has the area fire extinguisher been maintenance tested within the last year and tagged to show the date? |  |  |  |  |  |
| Are fire extinguishers accessible and the proper type for the fire hazard? |  |  |  |  |  |
| Are employees trained in how to use fire extinguishers? |  |  |  |  |  |
| Is there a fire extinguisher mounted within 75 ft of any point in an area? |  |  |  |  |  |
| Are the extinguishers clean and well cared for? |  |  |  |  |  |
| Is the seal and lock pin in place? |  |  |  |  |  |
| Clear access to extinguishers? Not blocked |  |  |  |  |  |
| Is the extinguisher location plainly marked, so as to be visible at a distance? |  |  |  |  |  |
| Is the extinguisher class marked on the extinguisher? |  |  |  |  |  |
| FIRST AID / MEDICAL SUPPLIES |  |  |  |  |  |
| Are first aid supplies stocked, clean, accessible and sanitary? |  |  |  |  |  |
| Are there eye/body wash facilities near injurious corrosive materials? |  |  |  |  |  |
| Is a person or persons adequately trained to render first aid available in the near proximity to the workplace? |  |  |  |  |  |
| Are AEDs present and operators trained? |  |  |  |  |  |
| Condition of First Aid Kits Acceptable |  |  |  |  |  |
| Are employees/subcontractors familiar with the incident/accident reporting process? |  |  |  |  |  |
| Do employees/subcontractors know where accident/incident forms are located? |  |  |  |  |  |

Date of last inspection of sprinkler system (required yearly) _______________________

Comment/Actions:

Toole Corp. Evacuation Report

This form is to be used to record all emergency evacuations (including drills).

Building Details

Building Name ___________________________     		Number of Floors (including ground) ________

Designated Muster Station _________________  		Person Completing Form ___________________

Evacuation Details

Evacuation Date/Time: ______________/____________		Evacuation Drill	Yes 	   No

Trigger for Evacuation:	Fire Alarm Activated  ___     Drill  ___     ERT  ___     Security  ___

Emergency Situation:   __________________________________________________________________________

____________________________________________________________________________________________

Condition:   Staff Only ____    All Occupants  ____     After Hours  ____     Unoccupied  ___  Weather _________

Number of Evacuees __________________	Elapsed Time to Evacuate __________ minutes

Evacuation was orderly with no panic						Yes 	   No

Mobility-impaired persons present (sight, hearing, physical, etc.)?			Yes 	   No

The majority of evacuees went to the mustering points?				Yes 	   No

Were the building occupants notified of this drill?			Not a drill 	Yes 	   No

Emergency Control Organization

Emergency Coordinator ______________ 	Deputy Emergency Coordinator ______________

Emergency Coordinators were stationed at the proper emergency control point?	Yes 	   No

All Fire Wardens reported to the Emergency Coordinator?				Yes 	   No

If not, who did not report in? ______________________________________

All Fire Wardens were identifiable (vests, hard hats, flash lights)?			Yes 	   No

Control of external building exits achieved?						Yes 	   No

Did the Fire Wardens perform their duties correctly?					Yes 	   No

Evacuation maps and emergency procedures posters are up-to-date?			Yes 	   No

Building Fire & Emergency Equipment

Was the evacuation signal audible throughout the building?				Yes 	   No

Automatic closing fire doors closed when the fire alarm activated?			Yes 	   No

Card access doors automatically released when the fire alarm activated?		Yes 	   No

Fire doors and emergency exits unobstructed?					Yes 	   No

Emergency Response Members

Client:		Maintenance       Security       Toole Corp. Emergency Coordinator       HSE

Emergency Response Team     Fire Brigade       Ambulance       Police       Other:  __________________

Toole Corp. Action Sheet

| Issue(s) | Action(s) Required | By Who | By When | Sign Off/Date |
| --- | --- | --- | --- | --- |

Records

- Keep the original in your Emergency Response folder and monitor to ensure all action items completed as soon as possible.  Report delays to senior management.

- Copies shall be distributed in accordance with the Toole Corp. Site Emergency Action Plan.

Emergency Action Plan Orientation Check List

Employee Name __________________________		Department _______________________

Hire/Transfer Date _______________ 			Orientation Date ___________________

[   ]	Emergency Procedures

[   ]	Evacuation route(s) from assigned work area

[   ]	Evacuation from an unfamiliar area

[   ]	Location of Emergency Assembly Areas

[   ]	Receiving and following instructions during an emergency

[   ]	ALL CLEAR and re-entry procedure

[   ]	Reporting hazards and/or substandard conditions

[   ]	Advising anyone who may require assistance during an emergency evacuation

[   ]	Location of Emergency Equipment (i.e. Fire Extinguishers, etc.)

Employee Signature: ____________________________________________________

Orientation Conducted by: _______________________________________________

Job Position/Title: _____________________________________________________

Sample Emergency Action Plan Core Requirements

| Potential emergencies (based on hazard assessment) | The following are identified potential emergencies: - Fire - List others |  |
| --- | --- | --- |
| Emergency procedures | In the event of a fire occurring within or affecting the work site, the Emergency Coordinator (or deputy) makes the following decisions and ensures the appropriate key steps are taken: - advise all personnel - pull the fire alarm to alert the nearest fire station and initiate all fire alarms within the building - evacuate all persons to a safe point in the assembly area and account for everyone including visitors and clients |  |
| Location of emergency equipment | Emergency equipment is located at: - Fire Alarm – List - Fire Extinguisher – List - Fire Hose - List |  |
| Workers trained in the use of emergency equipment | (1) _____________________ (2) _____________________ (3) _____________________ (4) _____________________ |  |
| Emergency response training requirements | Type of Training - Use of fire extinguishers - Practice fire drills | Frequency - Orientation and annually - At the call of site management |
| Location and use of emergency facilities | The nearest emergency services are located at: - List facilities |  |
| Fire protection requirements | - List all site fire protection requirements. |  |
| Alarm and emergency communication requirements | - Pulling the fire alarm automatically alerts the fire department and initiates an alarm within the building - The fire alarm signal is (describe sound and pattern) |  |
| First aid | First aid supplies are located at: - List First Aiders are: - List all names Transportation for ill or injured workers is by (describe). The contact number or radio channel is (describe). |  |
| Procedures for rescue and evacuation | In case of fire: - Advise all personnel - Pull the fire alarm - Evacuate all persons to a safe point in the staff parking lot and account for everyone including visitors and clients - Assist ill or injured workers to evacuate the building - Provide first aid to injured workers if required - Call emergency response personnel to arrange for transportation of ill or injured workers to the nearest health care facility if required. |  |
| Designated rescue and evacuation workers | The following workers are trained in rescue and evacuation (or describe client rescue organization): (1)_____________________ (2)_____________________ (3)_____________________ (4) _____________________ |  |
| Completed on: ________________________ Signed: _____________________________ |  |  |

FALL PROTECTION

Purpose

The purpose of this program is to provide fall protection procedures to prevent injury to employees while performing work assignments at elevated levels.

Any changes to this Fall Protection Program must be approved by the HSE Manager, who is designated the Qualified Person.  This is based on training received in fall protection planning and has demonstrated skills and knowledge in the preparation of fall programs, plans and the hazards involved.

Scope

Applies to all Toole Corp. employees who have work assignments at work levels that exceed 6 feet in height where guardrails or nets are not utilized. This includes work near and around excavations. Guardrails, safety nets, or personal fall arrest systems shall be used where feasible. When work is performed on a non-owned or operated site, the operator’s program shall take precedence, however, this document covers Toole Corp. employees and shall be used on owned premises, or when an operator’s program doesn’t exist or is less stringent.

Definitions

"Anchorage" means a secure point of attachment for lifelines, lanyards or deceleration devices.

"Body belt (safety belt)" means a strap with means both for securing it about the waist and for attaching it to a lanyard, lifeline, or deceleration device.

"Body harness" means straps which may be secured about the employee in a manner that will distribute the fall arrest forces over at least the thighs, pelvis, waist, chest and shoulders with means for attaching it to other components of a personal fall arrest system.

"Buckle" means any device for holding the body belt or body harness closed around the employee's body.

"Carabineer” - see Snaphook

"Connector" means a device which is used to couple (connect) parts of the personal fall arrest system and positioning device systems together. It may be an independent component of the system, such as a carabineer, or it may be an integral component of part of the system (such as a buckle or D-ring sewn into a body belt or body harness, or a snap-hook spliced or sewn to a lanyard or self-retracting lanyard).

"Deceleration device" means any mechanism, such as a rope grab, rip-stitch lanyard, specially-woven lanyard, tearing or deforming lanyards, automatic self-retracting lifelines/lanyards, etc., which serves to dissipate a substantial amount of energy during a fall arrest, or otherwise limit the energy imposed on an employee during fall arrest.

"Deceleration distance" means the additional vertical distance a falling employee travels, excluding lifeline elongation and free fall distance, before stopping, from the point at which the deceleration device begins to operate. It is measured as the distance between the location of an employee's body belt or body harness attachment point at the moment of activation (at the onset of fall arrest forces) of the deceleration device during a fall, and the location of that attachment point after the employee comes to a full stop.

Definitions (continued)

"Equivalent" means alternative designs, materials, or methods to protect against a hazard which the employer can demonstrate will provide an equal or greater degree of safety for employees than the methods, materials or designs specified in the standard.

"Failure" means load refusal, breakage, or separation of component parts. Load refusal is the point where the ultimate strength is exceeded.

"Free fall" means the act of falling before a personal fall arrest system begins to apply force to arrest the fall.

"Free fall distance" means the vertical displacement of the fall arrest attachment point on the employee's body belt or body harness between onset of the fall and just before the system begins to apply force to arrest the fall. This distance excludes deceleration distance, and lifeline/lanyard elongation, but includes any deceleration device slide distance or self-retracting lifeline/lanyard extension before they operate and fall arrest forces occur.

"Guardrail system" means a barrier erected to prevent employees from falling to lower levels.

"Infeasible" means that it is impossible to perform the inspection work using a conventional fall protection system (i.e., guardrail system, safety net system, or personal fall arrest system) or that it is technologically impossible to use any one of these systems to provide fall protection.

"Lanyard" means a flexible line of rope, wire rope, or strap which generally has a connector at each end for connecting the body belt or body harness to a deceleration device, lifeline, or anchorage.

"Leading edge" means the edge of a floor, roof, or formwork for a floor or other walking/working surface (such as the deck) which changes location as additional floor, roof, decking, or formwork sections are placed, formed, or constructed. A leading edge is considered to be an "unprotected side and edge" during periods when it is not actively and continuously under construction.

"Lifeline" means a component consisting of a flexible line for connection to an anchorage at one end to hang vertically (vertical lifeline), or for connection to anchorages at both ends to stretch horizontally (horizontal lifeline), and which serves as a means for connecting other components of a personal fall arrest system to the anchorage.

"Lower levels" means those areas or surfaces to which an employee can fall. Such areas or surfaces include, but are not limited to, ground levels, floors, platforms, ramps, runways, excavations, pits, tanks, material, water, equipment, structures, or portions thereof.

"Personal fall arrest system" means a system used to arrest an employee in a fall from a working level. It consists of an anchorage, connectors, a body belt or body harness and may include a lanyard, deceleration device, lifeline, or suitable combinations of these.

"Positioning device system" means a body belt or body harness system rigged to allow an employee to be supported on an elevated vertical surface, such as a wall, and work with both hands free while leaning.

"Rope grab" means a deceleration device which travels on a lifeline and automatically, by friction, engages the lifeline and locks so as to arrest the fall of an employee. A rope grab usually employs the principle of inertial locking, cam/level locking, or both.

Definitions (continued)

"Safety Nets...Safety nets shall be provided when workplaces are higher than 25 feet above ground or water surfaces or other surfaces where the use of ladders, scaffolds, catch platforms, temporary floors, safety lines or safety belts are impractical.

Nets shall extend 8 feet beyond the edge of the work surface where employees are exposed and shall be installed as close under the work surface as practical but in no case more than 25 feet below the work surface. Nets shall be positioned in a manner to prevent the user from coming into contact with below surfaces or structures. Proper clearance positioning of nets shall be determined by impact load testing. Work procedures shall not begin until nets are in place and have been properly tested.

New nets shall meet accepted performance standards of 17,500 foot pounds minimum impact resistance as determined and certified by the manufacturers and shall bear a label of proof test. Edge ropes shall provide a minimum breaking strength of 5000 pounds.

"Self-retracting lifeline/lanyard" means a deceleration device containing a drum-wound line which can be slowly extracted from, or retracted onto, the drum under slight tension during normal employee movement, and which, after onset of a fall, automatically locks the drum and arrests the fall.

"Snaphook" means a connector comprised of a hook-shaped member with a normally closed keeper, or similar arrangement, which may be opened to permit the hook to receive an object and, when released, automatically closes to retain the object. Snaphooks are generally one of two types:  (1) The locking type with a self-closing, self-locking keeper which remains closed and locked until unlocked and pressed open for connection or disconnection; or (2) The non-locking type with a self-closing keeper which remains closed until pressed open for connection or disconnection. As of January 1, 1998, the use of a non-locking snaphook as part of personal fall arrest systems and positioning device systems is prohibited.

"Unprotected sides and edges" means any side or edge (except at entrances to points of access) of a walking/working surface, e.g., floor, roof, ramp, or runway where there is no wall or guardrail system at least 39 inches (1.0 m) high.

"Walking/working surface" means any surface, whether horizontal or vertical on which an employee walks or works, including, but not limited to, floors, roofs, ramps, bridges, runways, formwork and concrete reinforcing steel but not including ladders, vehicles, or trailers, on which employees must be located in order to perform their job duties.

"Work area" means that portion of a walking/working surface where job duties are being performed.

Drawing of Components

Figure B

Figure D

Figure C

Figure A

1.  Tie-off Point

2.  Lifeline

3.  Rope Grab

4.  Shock Absorbing Lanyard

5.  Cross-Arm Strap

6.  Retractable Lifeline

7.  Full-Body Harness

8.  Restraining Belt

9.  Restraining Lanyard

10. Carabineer

Responsibilities

### Operations Manager

It is the responsibility of the local operations manager (designated competent person) to implement this Fall Protection Program. Continual observational safety checks of work operations and the enforcement of the safety policy and procedures shall be regularly enforced.  All jobs shall be pre-planned prior to the start of work.

### Supervisor

The Supervisor shall ensure that all persons assigned to work at elevated levels, exceeding 6 feet in height or more above lower level and where guardrails or nets are not utilized, be protected by personal fall protection equipment.

- Supervisors shall make exposure determinations and shall discuss with their employees the extent to which scaffolds, ladders or vehicle mounted work platforms can be used.

- Ensure that fall protection equipment is available and in safe working condition.

- Provide for emergency rescue in the event of a fall.   Pre-plan the job to ensure that employees have been properly trained in the use, limitations, inspections and rescue procedures and that training records are on file.

### Employees

Employees shall ensure they have and use the fall protection equipment as required by this program and:

- Understand the potential hazards of working at elevated levels as well as gaining access to and from the work location.

- Understand the use and limitations of such equipment.

- Pre-plan the job with his/her supervisor to agree that the job can be done safely.

- Inspect such equipment before each use and to report defective equipment immediately to their supervisor.

Procedure

Fall protection is required whenever employees are potentially exposed to falls from heights of six feet or greater to lower levels.  This includes work near and around excavations.  Use of guard rails, safety net, or personal fall arrest systems should be used when the standard methods of protection are not feasible or a greater hazard would be created.

When purchasing equipment and raw materials for use in fall protection systems applicable ANSI, ASTM or OSHA approved equipment shall be used.

### Minimum Standards

The following are minimum standards for Toole Corp. employee personal fall protection systems:

- Connectors shall be drop forged, pressed or formed steel, or made of equivalent materials.

- Connectors shall have a corrosion-resistant finish, and all surfaces and edges shall be smooth to prevent damage to interfacing parts of the system.

- D-rings and snap hooks shall have a minimum tensile strength of 5,000 pounds.

- D-rings and snap hooks shall be proof-tested to a minimum tensile load of 3,600 pounds without cracking, breaking, or taking permanent deformation.

- Snap hooks shall be sized to be compatible with the member to which they are connected to prevent unintentional disengagement of the snap hook.  Only a locking type snap hook designed and used to prevent disengagement of the snap hook by the contact of the snap hook keeper by the connected member shall be used.

- Horizontal lifelines shall be designed, installed, and used, under the supervision of a qualified person, as part of a complete personal fall arrest system, which maintains a safety factor of at least two.

- Lanyards and vertical lifelines shall have a minimum breaking strength of 5,000 pounds.  Where vertical lifelines are used, each employee shall be attached to a separate lifeline.

- Lifelines shall be protected against being cut or abraded.

- Self-retracting lifelines and lanyards which automatically limit free fall distance to 2 feet or less shall be capable of sustaining a minimum tensile load of 3,000 pounds applied to the device with the lifeline or lanyard in the fully extended position.

- Self-retracting lifelines and lanyards which do not limit free fall distance to 2 feet or less, rip stitch lanyards, and tearing and deforming lanyards shall be capable of sustaining a minimum tensile load of 5,000 pounds applied to the device with the lifeline or lanyard in the fully extended position.

- Anchorages used for attachment of personal fall arrest equipment shall be independent of any anchorage being used to support or suspend platforms and capable of supporting at least 5,000 pounds per employee attached, or shall be designed, installed, and used as part of a complete personal fall arrest system which maintains a safety factor of at least two and under the supervision of a qualified person.

- Systems used by an employee having a combined person and tool weight in excess of 310 pounds shall be modified to provide proper protection for such heavier loads.

- The attachment point of the body harness shall be located in the center of the wearer’s back near shoulder level, or above the wearer’s head, except when climbing.

- Body harnesses and components shall be used only for employee protection and not to hoist materials.

- Personal fall arrest systems and components subjected to impact loading shall be immediately removed from service and shall not be used again for employee protection until inspected and determined by a competent person to be undamaged and suitable for reuse.

- Provide for prompt rescue of employees in the event of a fall or assure that employees are able to rescue themselves.

- Personal fall arrest systems shall be inspected prior to each use for wear, damage and other deterioration, and defective components shall be removed from service.

- Personal fall arrest systems shall not be attached to guardrail systems, nor shall they be attached to hoists unless prior approval is obtained from a competent person.

- If and when a personal fall arrest system is used at hoist areas, it shall be rigged to allow the movement of the employee only as far as the edge of the walking/working surface.

### Stopping a Fall

The arresting force on an employee stopped by a fall shall be limited to a maximum arresting force of 1,800 pounds when wearing a body harness.

The fall arrest system shall be rigged such that an employee can neither free fall more than 6 feet, nor contact any lower level.

The fall arrest system shall bring an employee to a complete stop and limit maximum deceleration distance an employee travels to 3.5 feet.

The fall arrest system shall have sufficient strength to withstand twice the potential impact energy of an employee free falling a distance of 6 feet, or the free fall distance permitted by the system, whichever is less.

### Protection From Falling Objects

When employees are required to work in the near vicinity of others working with materials, tools, or equipment at elevated levels, Barricades around the immediate area of the overhead work shall be erected to prohibit employees from entering the barricaded area.

Employees performing work at elevated levels shall keep tools, materials, and equipment away from the edge to keep potential objects from falling over the side.   Where practical, tools, etc. shall be secured with rope, wire, etc. to keep them from falling.

### Portable Ladders

Three point climbing is required while ascending/descending ladders.  While on ladders, both hands and one foot, or both feet and one hand shall always be in contact with the ladder.

Tools required to perform a task shall be transported by a mechanical carrier such as a tag line, suspended bucket or tool belt.

- Tools shall not be carried by hand while climbing.

- Hands must be free to grip the ladder.

- Tools shall not be carried in clothing pockets.

- Tools shall be pulled up to the job site only after reaching the area of work.

When work is to be performed from straight/extension ladders, fall protection shall be utilized when heights exceed 6 feet.

Straight ladders shall be tied off at the top to prevent them from moving.   A second person shall steady the ladder at the base while it is being tied off at the top by another employee.   Do not tie off fall protection equipment to the ladder.

### Storage

A dedicated storage area shall be provided for the storage of fall protection equipment and all components.  The storage area shall keep the equipment clean, dry, and free from oils, chemicals, paints, and excessive heat.

### Inspections

Fall protection equipment shall be inspected before each use for wear, damage, other deterioration, or other defects.

### Elevated Personnel Platforms

Work performed, regardless of the nature of the work, from personnel platforms raised by forklifts, cranes, scissor lifts, etc., shall require the use of a full body harness and shall be connected to the platform.

### Rescue

Prompt rescue of employees shall be provided in the event of a fall or shall assure the employees are able to rescue themselves.   The pre-planning stage prior to the beginning of each elevated work assignment shall be evaluated by the supervisor to provide rescue of employees involved in a fall.

### Fall Protection Plan

This option is available only to employees engaged in leading edge work who can demonstrate that it is infeasible or it creates a greater hazard to use conventional fall protection equipment.  The fall protection plan shall conform to the following provisions:

- The fall protection plan shall be prepared by a qualified supervisor and developed specifically for the site where the leading edge work is being performed.

- The fall protection plan shall document the reasons why the use of conventional fall protection systems (guardrail systems, personal fall arrest systems, or safety net systems) are infeasible or why their use would create a greater hazard.

- The fall protection plan shall identify each location where conventional fall Protection methods cannot be used.

- These locations shall then be classified as controlled access zones.

### Controlled Access Zones

When used to control access to areas where leading edge or other operations are taking place the controlled access zone shall be defined by a control line or by any other means that restricts access.

When control lines are used, they shall be erected not less than 6 feet (1.8 m) nor more than 25 feet (7.7 m) from the unprotected or leading edge.

The control line shall extend along the entire length of the unprotected or leading edge and shall be approximately parallel to the unprotected or leading edge.

The control line shall be connected on each side to a guardrail system or wall.

- Control lines shall consist of ropes, wires, tapes, or equivalent materials.

- Each line shall be flagged or otherwise clearly marked at not more than 6-foot (1.8 m) intervals with high-visibility material.

- Each line shall be rigged and supported in such a way that its lowest point (including sag) is not less than 39 inches (1 m) from the walking/working surface and its highest point is not more than 45 inches (1.3 m).

- Each line shall have a minimum breaking strength of 200 pounds.

Only employees engaged in the related work shall be permitted in the controlled access zone.

### Safety Monitoring System

When the use of conventional fall protection equipment is deemed infeasible or the use of this equipment creates a greater hazard a Fall Protection Plan which includes a safety monitoring system shall be implemented by the supervisor.

Supervisors shall designate a competent person to monitor the safety of other employees. The competent person shall be assigned to:

- Recognize fall hazards;

- Warn employees if they are unaware of fall hazard or are acting in an unsafe manner;

- Be on the same working surface and in visual contact of working employees;

- Stay close enough for verbal communication; and

- Not have other assignments that would take his/her attention from the monitoring function.

### Incident Investigations

All incidents and near misses must be investigated according to Toole Corp. incident investigation procedure.  Changes to the fall protection program shall be implemented if deemed appropriate from incident corrective actions.

### Training

Employees who may be exposed to fall hazards shall be trained to recognize the hazards of falling and understand the procedures to be followed in order to minimize these hazards.

The employee will be trained in the use and operation of fall arrest systems, inspections, and maintenance procedures.

Training must be conducted initially and refresher training conducted annually or as needed due to deficiencies in training, changes in the workplace, changes in fall protection systems or procedures that render previous training obsolete or inadequacies in an employee’s understanding of previous training.

Training must be documented in writing.  Training records shall include:

- Who was trained

- When and dates of training

- Signature of person providing training

- Date training was deemed adequate

Training records shall be retained in the corporate office.

FIRE PROTECTION / EXTINGUISHERS

Purpose

The purpose of this program is to provide fire extinguisher procedures to ensure equipment is operable and employees have the knowledge to safely operate in case of a fire incident.

Scope

Applies to all Toole Corp. employees and all Toole Corp. locations.

Responsibilities

The Safety Manager is responsible for developing procedures for the use and care of fire extinguishers and for developing a training program for the proper use of these devices.  The Manager is responsible for implementing fire extinguisher training at his location.  The shop foremen are responsible for enforcing the provisions of this section of the safety manual.  All employees are responsible for following these provisions.

Procedure

### Selection and Distribution

Portable fire extinguishers shall be provided for employee use and selected and distributed based on the classes of anticipated workplace fires and on the size and degree of the hazard which would affect their use. Fire extinguishers used by this company are for four classes of fires:

- Class A Fire Extinguishers. Use on ordinary combustibles or fibrous material, such as wood, paper, cloth, rubber and some plastics. Travel distance for employees to any extinguisher is 75 feet (22.9 m) or less.

- Class B Fire Extinguishers. Use on flammable or combustible liquids such as gasoline, kerosene, paint, paint thinners and propane. Travel distance from the Class B hazard area to any extinguisher is 50 feet (15.2 m) or less.

- Class C Fire Extinguishers. Use on energized electrical equipment, such as appliances, switches, panel boxes and power tools. Travel distance from the Class C hazard area to any extinguishing agent is 50 feet (15.2 m) or less.

- Class D Fire Extinguishers. Use on combustible metals, such as magnesium, titanium, potassium and sodium. Travel distance from the combustible metal working area to any extinguishing agent is 75 feet (22.9 m) or less.

### Labelling Of Fire Extinguishers

Fire extinguishers are to be mounted in easily accessible locations that are indicated by a sign that reads "Fire Extinguisher".  Fire extinguishers are to be located so that no employee will ever be more than 75 feet from an extinguisher.  No equipment, boxes or product may be placed (even temporarily) in the way of a fire extinguisher.

Each fire extinguisher will be assigned a unique number.

### Maintenance

All fire extinguishers shall be mounted no higher and no lower than four (4) feet from the floor.  All fire extinguishers shall be maintained as follows:

- Numbered to identify their proper location

- Fully charged and in operable condition

- Clean and free of defects

- Readily accessible at all times

### Inspection, Maintenance and Testing

All fire extinguishers are to be visually inspected by Toole Corp. employees monthly.  All fire extinguishers are to receive an annual maintenance check by certified personnel from a fire extinguisher dealer.  Fire extinguishers are to be inspected and re-charged by certified personnel after any use.

Any fire extinguisher that shows a loss of pressure during the monthly inspection will be inspected and re-charged by certified personnel.  Completed fire extinguisher inspection logs will be maintained in the safety files and become a part of the safety records.  They are to be maintained for 5 years.

### Use

In the event of a fire, one employee will get the nearest fire extinguisher and use it to attempt to put the fire out.  All other employees in the immediate area will prepare to evacuate if needed.  All other employees in the building need to be advised that a fire is in progress.

The employee attempting to extinguish the fire will break the safety seal on the handle and pull the pin.  He will then aim his extinguisher at the base of the fire and discharge it with a sweeping motion from side to side; continuing until the fire is out or the extinguisher is emptied.

Remember that a standard fire extinguisher will be emptied in about 10 to 15 seconds.  If the fire is not out when the extinguisher has been completely discharged, the employees must evacuate the area.

### Training and Education

The purpose of this section is to establish training procedures which are necessary for the proper use and understanding of a fire extinguisher and incipient stage fire fighting.   Training will occur prior to initial assignment and at least annually thereafter.

On even numbered years this training will be conducted by a member of the local fire department (where possible) and will include "live fire" hands on use of the extinguisher.  On odd number years this training will be conducted by the Safety Manager and will include a demonstration of the use of a fire extinguisher, without actually discharging the unit.

New employees will be given the odd  number year training upon hire.

Initial Training Outline

- General principles of a fire

- Hazards employed with an incipient stage fire(s)

- When to "back off' (evacuate) of an incipient stage fire(s)

- General fire principles of a fire extinguisher

- Hazards employed with the use a fire extinguisher

- Use of a fire extinguisher

Retraining

Retraining shall reestablish employee proficiency and introduce new or revised control methods and procedures, as necessary.   Retraining shall be provided for all authorized and affected employees whenever there is:

- An annual basis or

- A change in job assignment or

- Toole Corp. has reason to believe that there are deviations from or inadequacies in the employee's knowledge or use of fire extinguishers or fire prevention procedures.

Training Documentation

- All training will be documented and each employee's understanding will be subject to a "hands-on" test.

- Documentation will consist of; as a minimum, the employee's name, the trainer's name, the date of the training, and an outline of training provided.

HAND AND POWER TOOLS

Purpose

The purpose of this program is to provide establish requirements for the safe operation of hand and power tools and other portable tools, including proper guarding.  All hand and power tools shall be maintained in a safe condition.

This program applies to all Toole Corp. employees who use hand and power tools.

Scope

This program is applicable to all Toole Corp. employees while engaged in work at Toole Corp. facilities and/or facilities operated by others.

Responsibilities

Any tool which is not in compliance with any applicable requirement of this plan is prohibited and shall either be identified as unsafe by tagging or locking the controls to render them inoperable or shall be physically removed from its place of operation.

### Managers/Supervisors

- Ensure that all employees using portable tools have been trained and fully understand the operations and maintenance procedures of such tools, including their proper use.

- Provide and train employees with all additional PPE that may be needed for the safe operation of portable tools.

### Employees

- Shall ensure they have and properly use the correct tool for each task.

- Shall follow manufactures safety and operating instructions before using

Requirements

### General

All tools, regardless of ownership, shall be of an approved type and maintained in good condition.

- Tools are subject to inspection at any time.

- All employees have the authority and responsibility to condemn unsafe tools, regardless of ownership.

Unsafe tools shall be tagged with a “DO NOT USE OR OPERATE” tag to prevent their use.

Employees shall always use the proper tool for the job to be performed.    Makeshift and substitute tools shall not be used.

Hammers with metal handles, screwdrivers with metal continuing through the handle, and metallic measuring tapes shall not be used on or near energized electrical circuit or equipment.

### General (continued)

Tools shall not be thrown from place to place or from person to person; tools that must be raised or lowered from one elevation to another shall be placed in tool bags/buckets firmly attached to hand lines.

Tools shall never be placed unsecured on elevated places.

Impact tools such as chisels, punches, and drift pins that become mushroomed or cracked shall be dressed, repaired, or replaced before further use.

Chisels, drills, punches, ground rods, and pipes shall be held with suitable holders or tongs (not with the hands) while being struck by another employee.

Shims shall not be used to make a wrench fit.

Wrenches with sprung or damaged jaws shall not be used.

Tools shall be used only for the purposes for which they have been approved.

Tools with sharp edges shall be stored and handled so that they will not cause injury or damage.  They shall not be carried in pockets unless suitable protectors are in use to protect the edge.  They shall not be carried in pockets unless suitable protectors are in use to protect the edge.

Wooden handles that are loose, cracked, or splintered shall be replaced.  The handle shall not be taped or lashed with wire.  The handle shall not be taped or lashed with wire.

Tools shall not be left lying around where they may cause a person to trip or stumble.

When working on or above open grating, a canvas or other suitable covering shall be used to cover the grating to prevent tools or parts from dropping to a lower level where others are present or the danger area shall be barricaded or guarded.

The insulation on hand tools shall not be depended upon to protect users from high voltage shock (except approved live line tools).

### Portable Electric Tools

The non-current carrying metal parts of portable electric tools such as drills, saws, and grinders shall be effectively grounded when connected to a power source unless:

- The tool is an approved double-insulated type, or

- The tool is connected to the power supply by means of an isolating transformer or other isolated power supply.

All powered tools shall be examined prior to use to ensure general serviceability and the presence of all applicable safety devices.

Powered tools shall be used only within their design and shall be operated in accordance with manufacturer’s instructions.  The use of electric cords for hoisting or lowering tools shall not be permitted.

All tools shall be kept in good repair and shall be disconnected from the power source while repairs or adjustments are being made.

### Portable Electric Tools (continued)

Electrical tools shall not be used where there is hazard of flammable vapors, gases, or dusts without a valid Hotwork Permit.

Ground fault circuit interrupters or use of an Assured Grounding Program shall be used with portable electric tools.  This does not apply to equipment run off of portable or truck mounted generators at 5kw or less that are isolated from ground or to equipment ran directly off of secondaries.

### Pneumatic Tools

Pneumatic tools shall never be pointed at another person.

Pneumatic power tools shall be secured to the hose or whip by some positive means to prevent the tool from becoming accidentally disconnected.

Safety clips or retainers shall be securely installed and maintained on pneumatic impact (percussion) tools to prevent attachments from being accidentally expelled.

Compressed air shall not be used for cleaning purposes, except where reduced to less than 30 psi and then only with effective chip guarding and personal protective equipment.

Compressed air shall not be used to blow dust or dirt from clothing.

The manufacturers stated safe operating pressure for hoses, pipes, valves, filters, and other fitting shall not be exceeded.

The use of hoses for hoisting or lowering tools shall not be permitted.

Before making adjustments or changing air tools, unless equipped with quick-change connectors, the air shall be shut off at the air supply valve ahead of the hose.  The hose shall be bled at the tool before breaking the connection.

Compressed air tools, while under pressure, must not be left unattended.

All connections to air tools shall be made secure before turning on air pressure.

Air at the tool shall not be turned on until the tool is properly controlled.

All couplings and clamps on pressurized air hose shall be bridged (pinned) with suitable fasteners.

Hose and hose connections used for conducting compressed air to utilization equipment shall be designed for the pressure and service to which they are subjected.

Use only approved end-fitting clamps (screw type heater hose clamps are not acceptable).

While blowing down hose, do not point it toward people.

Power tools are to be operated only by competent persons who have been trained in their proper use.

Conductive hose should not be used near energized equipment.

Foot protection shall be worn while operating paving breakers, tampers, rotary drills, clay spades, and similar impactor-type tools or at other times when instructed by supervision.

All pneumatically driven nailers, staplers, and other similar equipment provided with automatic fastener feed, which operate at more than 100 psi. pressure at the tool shall have a safety device on the muzzle to prevent the tool from ejecting fasteners, unless the muzzle is in contact with the work surface.

Airless spray guns of the type which atomize paints and fluids at high pressures (1,000 pounds or more per square inch) shall be equipped with automatic or visible manual safety devices which will prevent pulling of the trigger to prevent release of the paint or fluid until the safety device is manually released.

In lieu of the above, a diffuser nut (which will prevent high pressure), high velocity release (while the nozzle tip is removed), plus a nozzle tip guard (which will prevent the tip from coming into contact with the operator), or other equivalent protection, shall be provided.

### Powder Actuated Tools (Tools actuated by an explosive charge)

Only those employees who have been certified in their use shall operate these tools.

Explosive charges shall be carried and transported in approved containers.

Operators and assistants using these tools shall be protected by means of eye, face, and hearing protection.

Tools shall be maintained in good condition and serviced regularly by qualified persons.  The material upon which these tools are to be used shall be examined before work is started to determine its suitability and to eliminate the possibility of hazards to the operator and others.

Prior to use, the operator shall ensure that the protective shield is properly attached to the tool.

Before using a tool, the operator shall inspect it to determine to his satisfaction that it is clean, that all moving parts operate freely, all guards and safety devices are in place, and that the barrel is free from obstructions.

Before using tools the operator shall read and become familiar with the manufacturers operating guidelines and procedures.

When a tool develops a defect during use, the operator shall immediately cease to use it, until it is properly repaired in accordance with the manufactures specifications.

Tools shall not be loaded until just prior to the intended firing time, nor shall an unattended tool be left loaded. Empty tools are to be pointed at any workmen.

In case of a misfire, the operator shall hold the tool in the operating position for at least 30 seconds. He shall then try to operate the tool a second time. He shall wait another 30 seconds, holding the tool in the operating position; then he shall proceed to remove the explosive load in strict accordance with the manufacturer's instructions.

A tool shall never be left unattended in a place where it would be available to unauthorized persons.

Fasteners shall not be driven into very hard or brittle materials including, but not limited to, cast iron, glazed tile, surface hardened steel, glass block, live rock, face brick, or hollow tile.

Driving into materials easily penetrated shall be avoided unless such materials are backed by a substance that will prevent the pin or fastener from passing completely through and creating a flying missile hazard on the other side.

Tools shall not be used in an explosive or flammable atmosphere.

### Hydraulic Power Tools

The fluid used in hydraulic powered tools shall be fire-resistant fluids approved under Schedule 30 of the U.S. Bureau of Mines, Department of the Interior, and shall retain its operating characteristics at the most extreme temperatures to which it will be exposed.

The manufacturer's safe operating pressures for hoses, valves, pipes, filters, and other fittings shall not be exceeded.

All hydraulic tools, which are used on or around energized lines or equipment, shall use non-conducting hoses having adequate strength for the normal operating pressures.

### Hydraulic Jacks

Loading and Marking

- The operator shall make sure that the jack used has a rating sufficient to lift and sustain the load.

- The rated load shall be legibly and permanently marked in a prominent location on the jack by casting, stamping, or other suitable means.

Operation and Maintenance

- In the absence of a firm foundation, the base of the jack shall be blocked. If there is a possibility of slippage of the cap, a block shall be placed in between the cap and the load.

- The operator shall watch the stop indicator, which shall be kept clean, in order to determine the limit of travel. The indicated limit shall not be overrun.

- After the load has been raised, it shall be cribbed, blocked, or otherwise secured at once.

- Hydraulic jacks exposed to freezing temperatures shall be supplied with adequate antifreeze liquid.

- All jacks shall be properly lubricated at regular intervals.

Each jack shall be thoroughly inspected before each use.  Jacks, which are in unsafe condition, shall be tagged accordingly, and shall not be used until repairs are made.

### Abrasive Blast Cleaning Nozzles

The blast cleaning nozzles shall be equipped with an operating valve, which must be held open manually. A support shall be provided on which the nozzle may be mounted when it is not in use.

### Fuel Powered Tools

All fuel-powered tools shall be stopped while being refueled, serviced, or maintained, and fuel shall be transported, handled, and stored in accordance with the Flammable and Combustible Liquids Program.

When fuel powered tools are used in enclosed spaces, the applicable requirements for concentrations of toxic gases and use of personal protective equipment, shall be adhered too.

### Guarding Portable Tools

Guards shall be in place and operable at all times while the tool is in use.  The guard may not be manipulated in such a way that will compromise its integrity or compromise the protection in which intended.  Guarding shall meet the requirements set forth in ANSI B15.1.

Portable Circular Saws

- All portable, power-driven circular saws having a blade diameter greater than 2 in. shall be equipped with guards above and below the base plate or shoe.

- The upper guard shall cover the saw to the depth of the teeth, except for the minimum arc required to permit the base to be tilted for bevel cuts.

- The lower guard shall cover the saw to the depth of the teeth, except for the minimum arc required to allow proper retraction and contact with the work.

- When the tool is withdrawn from the work, the lower guard shall automatically and instantly return to covering position.

- All cracked saw blades shall be removed from service.

Switches and Controls

- All hand held powered tools, circular saws, drills, tappers, fastener drivers, horizontal or vertical angle grinders, etc., shall be with a constant pressure switch or control, and may have a lock-on control provided that turnoff can be accomplished by a single motion of the same finger or fingers that turn it on.

- All hand-held powered circular saws having a blade diameter greater than 2 inches, electric, hydraulic or pneumatic chain saws, and percussion tools without positive accessory holding means shall be equipped with a constant pressure switch or control that will shut off the power when the pressure is released. All hand-held gasoline powered chain saws shall be equipped with a constant pressure throttle control that will shut off the power to the saw chain when the pressure is released.

- The operating control on hand-held power tools shall be so located as to minimize the possibility of its accidental operation, if such accidental operation would constitute a hazard to employees.

- Grounding of portable electric powered tools shall meet the electrical requirements that can be found in the Electrical Safety Program.  All electric power tools shall be equipped with a three-prong plug.

### Portable Abrasive Wheels

Safety Guards Exceptions

- Wheels used for internal work while within the work being ground.

- Mounted wheels used in portable operations 2 inches and smaller in diameter.

- Types 16, 17, 18, 18R, and 19 cones, plugs, and threaded hole pot balls where the work offers protection.

- Guards shall be made of steel or other material with adequate strength.

- A safety guard shall cover the spindle end, nut and flange projections.  The safety guard shall be mounted so as to maintain proper alignment with the wheel, and the strength of the fastenings shall exceed the strength of the guard.

- Exception: safety guards on all operations where the work provides a suitable measure of protection to the operator may be so constructed that the spindle end, nut and outer flange are exposed. Where the nature of the work is such as to entirely cover the side of the wheel, the side covers of the guard may be omitted.

- Exception: the spindle end, nut, and outer flange may be exposed on portable machines designed for, and used with, type 6, 11, 27, and 28 abrasive wheels, cutting off wheels, and tuck pointing wheels.

Mounting and Inspection of Abrasive Wheels

- Immediately before mounting, all wheels shall be closely inspected and a ring test performed, to make sure they have not been damaged in transit, storage, or otherwise.

- Ring test – “tap" wheels about 45 degrees each side of the vertical centerline and about 1 or 2 inches from the periphery; then rotate the wheel 45 degrees and repeat the test; a sound and undamaged wheel will give a clear metallic tone - If cracked, there will be a dead sound and not a clear "ring.”

- The spindle speed of the machine shall be checked before mounting of the wheel to be certain that it does not exceed the maximum operating speed marked on the wheel.

- Grinding wheels shall fit freely on the spindle and remain free under all grinding conditions.

- A controlled clearance between the wheel hole and the machine spindle (or wheel sleeves or adaptors) is essential to avoid excessive pressure from mounting and spindle expansion.

- The machine spindle shall be made to nominal (standard) size plus zero minus .002 inch, and the wheel hole shall be made suitably oversize to assure safety clearance under the conditions of operating heat and pressure.

- All contact surfaces of wheels, blotters, and flanges shall be flat and free of foreign matter.

- When a bushing is used in the wheel hole it shall not exceed the width of the wheel and shall not contact the flanges.

### Portable Grinders

Special "revolving cup guards" which mount behind the wheel and turn with it shall be used.  They shall be made of steel or other material with adequate strength and shall enclose the wheel sides upward from the back for one-third of the wheel thickness. It is necessary to maintain clearance between the wheel side and the guard. The clearance shall not exceed one-sixteenth inch.

Vertical portable grinders, also known as right angle grinders, shall have a maximum exposure angle of 180 degrees and the guard shall be located between the operator and the wheel during use.  Adjustment of the guard shall ensure that pieces of an accidentally broken wheel will be deflected away from the operator.

Other Portable Grinders

The maximum angular exposure of the grinding wheel periphery and sides for safety guards used on other portable grinding machines shall not exceed 180 degrees and the top half of the wheel shall be enclosed at all times.

### Personal Protective Equipment

Employees using hand and power tools and exposed to the hazard of falling, flying,  abrasive, and splashing objects, or exposed to harmful dust, fumes, mists, vapors or gases shall be provided with the particular PPE necessary to protect them from  the hazard.

HAZARD COMMUNICATION - (HAZCOM)

Purpose

The purpose of this program is to ensure that the hazards of all chemicals and substances are evaluated and the information concerning their hazards is communicated to employees, including emergency response organizations, state and federal agencies, other employers and contractors, as necessary. This hazard information will be communicated, and displayed in accordance with this Hazard Communication Program.

Toole Corp. is firmly committed to providing each of its employees a safe and healthy work environment. It is recognized that workers may use chemicals or substances that have potentially hazardous properties. When using these substances, workers must be aware of the identity, toxicity or hazardous properties of a chemical or substance, since an informed employee is more likely to be a safe employee. To this end, Toole Corp. has established a written Hazard Communication Program.

Scope

This program is applicable to all Toole Corp. employees who may be exposed to hazardous chemicals. When work is performed on a non-owned or operated site, the operator’s program shall take precedence, however, this document covers Toole Corp. employees and contractors and shall be used on owned premises, or when an operator’s program doesn’t exist or is less stringent.

Definitions

Chemical - any element, chemical compound, or mixture of elements and/or compounds.

Chemical Inventory List - a list of chemicals used at this facility, or by personnel that report to this facility.

Electronic Access – using electronic media (telephone, fax, internet, etc.) to obtain Material Safety Data Sheets or health information.

Facility - an establishment at one geographical location containing one or more work areas.

Hazardous chemical - any chemical that is a physical hazard, a health hazard, or has a Permissible Exposure Limit established for it.

Hazardous substance - see hazardous chemical.

Hazard Communication Program Coordinator - the person who has overall responsibility at a facility for that facility's Hazard Communication Program.

Health hazard - a substance for which there is statistically significant evidence based on at least one study conducted in accordance with established scientific principles that acute or chronic adverse health effects may occur in exposed employees.

IDLH - immediately dangerous to life and health.

Immediate Use - the chemical will be under the control of and used only by the person who transfers it from a labeled container and only within the work shift in which it is transferred.

Jobsite - an area remote from a Toole Corp. facility where hazardous chemicals are stored or used and employees are present for the purpose of Toole Corp. business.

(MSDS) Material Safety Data Sheet - a written or printed document containing chemical hazard and safe handling information, prepared in accordance with the OSHA Occupational Safety & Health Standards, Section 1910.1200, paragraph (g).

(NFPA) National Fire Protection Association Labeling - a common industry labeling method developed by the National Fire Protection Association to identify the hazards associated with a particular chemical.

(PEL) Permissible Exposure Limit - the maximum eight-hour time weighted average of any airborne contaminant to which an employee may be exposed.

Readily available - when an employee has access during the course of his/her normal work shift.

Substance - see Chemical.

(TLV) Threshold Limit Value - the airborne concentration of a substance that represents conditions under which it is believed that nearly all normal workers may be repeatedly exposed day after day without adverse effect.

Work area - a room or defined space in a facility where hazardous chemicals are stored or used and where one or more employees are present.

Workplace - see Facility.

Workplace Chemical List - see Facility Chemical List.

Responsibilities

A written hazard communication program shall be developed, implemented and maintained at each Toole Corp. workplace.  The program shall describe how labels, other forms of warning and material safety data sheets shall be communicated to employees.

The Safety Manager is responsible for developing and implementing the Hazard Communications Program.  Managers are responsible for maintaining Material Safety Data Sheets and the Chemical Inventory List for their locations.   The Safety Manager reviews the MSDS files and Chemical Inventory List at each location at least annually to ensure that they are complete and up to date.

Employees are responsible for following the requirements in the Hazard Communication Program, to use proper personal protective equipment, to report containers without labels immediately and to not deface any label.

Any employee who transfers any material from one container to another is responsible for labeling the new container with all required information.

All employees are responsible for learning the requirements of this section and for applying them to their daily work routine.

Requirements

### Introduction

This Hazard Communication Program was prepared for use by Toole Corp. to explain how Toole Corp. meets the requirements of the federal Occupational Safety and Health Administration's Hazard Communication Standard (29 CFR 1910.1200).  It spells out how Toole Corp. will inventory chemicals stored and used, obtain and use material safety data sheets, maintain labels on chemical substances, and train employees about the hazards of chemicals they are likely to encounter on the job.

Preparation of this program indicates our continuing commitment to safety among our employees in all of our locations.

- Each facility is expected to follow this program and maintain its work areas in accordance with these requirements.

- Employees, their designated representatives, and government officials must be provided copies of this program upon request.

- In addition to the program, other information required as part of our hazard communication effort is available to workers upon request.

- Asking to see this information is an employee's right.

- Using this information is part of our shared commitment to a safe, healthy workplace.

### List of Hazardous Chemicals

Toole Corp. maintains a listing of all known hazardous chemicals known to be present used at, or by this facility by using the identity that is referenced on the material safety data sheet (MSDS). This identity is often a common name, such as the product or trade name (i.e., Lime-A-Way).

The Chemical Inventory List is updated as necessary and at least annually by the Hazard Communication Program Coordinator or their designee.

The facility Chemical Inventory List must be available for review upon request.

### Material Safety Data Sheets

Chemical manufacturers are responsible for developing MSDSs. Toole Corp. shall have a MSDS for each chemical used with the exception of consumer products.

Material Safety Data Sheets, for chemicals used in this facility or by personnel reporting to this facility, shall be maintained, readily accessible in each work area and be made available, upon request, to employees, their designated representatives and regulatory officials in accordance with the requirements of 29 CFR 1910.1020(e).

Material Safety Data Sheets are filed alphabetically, by material classification, in the MSDS Book.  A Chemical Inventory List is provided in the front of the MSDS Book, listing all MSDS' contained therein.  This inventory serves as the index of the MSDS Book.  The MSDS Book shall be displayed in a prominent location in the work area where it is accessible to all employees.

A copy of a MSDS request form is located in the first section of the MSDS Book.  An employee may use a copy of this form to request an MSDS or he may ask the Manager for one.  In either case the requested MSDS must be given to the employee within 24 hours.

MSDS’ must be obtained for each required chemical from the chemical manufacturer, supplier or vendor.  The purchasing of any potentially hazardous chemical products from any supplier that does not provide an appropriate Material Safety Data Sheet in a timely fashion is prohibited.

The Material Safety Data Sheet must be kept in the MSDS library for as long as the chemical is used by the facility.

Electronic access (telephone, fax, internet, etc.) may be used to acquire and maintain MSDS libraries and archives.

The Manager is responsible for seeing that the Chemical Inventory List	inventory is maintained, is current and is complete.  He will review the inventory and the MSDS Book at least annually.  When a hazardous material has been permanently removed from the work place, its MSDS is to be removed from the MSDS Book and the Chemical Inventory List.  A file copy is to be maintained in a "dead file".

MSDS' for hazardous materials to which Toole Corp. employees have been exposed must be maintained after the employee leaves the employment of Attest Inspection, LLC.   Before any non-routine task is performed, employees will be advised of special precautions and the hazards associated with chemicals contained in unlabeled pipes in their work areas, if present.  In the unlikely event that such tasks are required, the Manager will provide MSDS for involved chemical. Employees have the right to request MSDS on any chemical and it must be provided without any issues.

### Labels, Labelling and Warnings:

The Manager will ensure that all hazardous chemicals used or stored in the facility are properly labelled.

- Damaged labels or labels with incomplete information shall be reported immediately.

- Damaged labels on incoming containers of chemicals shall not be removed.

- New labels shall be provided as needed so that all containers are properly labelled.

- Only containers into which an employee transfers a chemical for their own immediate use will not require labelling.

- Employees who are unsure of the contents of any container, vessel or piping must contact their supervisor for information regarding the substance including:

  - The name of the substance

  - The hazards related to the substance

  - The safety precautions required for working with the substance.

Labels, tags or markings on containers shall list as a minimum:

- Words, pictures, symbols or combinations thereof may be used.

- The trade name of the product as listed on the Material Safety Data Sheet.

- Appropriate hazard warnings to help employees protect themselves from the hazards of the substance.

- Labels provided by chemical manufacturers, distributors, and importers must also list the name and address of the manufacturer, importer, or vendor responsible for the chemical, and from whom more information about the chemical can be obtained.

- Labels shall be legible, in English.  However, for non-English speaking employees, information shall be presented in their language as well.

- Toole Corp. or employees shall not remove or deface labels on incoming containers of hazardous chemicals.

All containers must be labelled.  When an employee transfers the contents of one container to another, he must label the new container with all required information.  This information can be obtained from the labelling of the original 	container or from the material's MSDS.  Any container of a potentially hazardous material that will not be emptied during one shift must be labelled, without exception.

Personnel in the Shipping and Receiving Departments are responsible for proper labelling of all containers shipped by Toole Corp. and for the inspection of all incoming materials to ensure correct labelling.  Chemicals received from vendors that are not properly labelled must be rejected.

NFPA Standard 704 labels shall be the preferred hazard identification method used in Toole Corp. facilities and on materials containers used on client sites.  All employees, clients, subcontractors and visitors who may come in contact with a Toole Corp. hazardous substance must be briefed to ensure understanding of the NFPA 704 labelling system.

### Training

Toole Corp. shall provide employees and new hires at their initial assignment effective information and training on hazardous chemicals in their work area.

Additional training will be provided whenever a new chemical hazard is introduced into the work area.  To reinforce the importance of handling chemicals properly when performing new or non-routine tasks, Supervision will conduct supplementary training as needed.

Formal training will be conducted by facility employees or individuals who are knowledgeable in the Hazard Communication program.

The Manager shall ensure records of employee training are maintained.

When an outside contractor, such as a pest control worker or a carpenter enters a Toole Corp. site to perform a service for the company, he must first present MSDS' for any and all hazardous chemicals he will use.  These MSDS’ will be treated as above with the same training requirements.  The Manager will be responsible for contacting each contractor before work is started to gather and disseminate any information concerning chemical hazards the contractor is bringing into the work place.

The Hazard Communication Program documented training shall, as a minimum, include:

- Requirements, details and rights of the employee as contained in the Hazard Communication regulation

- Operations and work areas where hazardous chemicals are present.

- Location of the written Hazard Communication Program, MSDSs and the Chemical Inventory List.

- How to access MSDS’ or MSDS information.

- How to read and an explanation of labels and Material Safety Data Sheets for pertinent hazard information and how employees can obtain and use the appropriate hazard information.

- Methods and observations that may be used to detect the presence or release of hazardous chemicals by use of monitoring devices, visual appearance or odor.

- The physical & health hazards of chemicals in the work area.

- Protection measures to be utilized to prevent exposure.

- Appropriate work practices.

- Emergency procedures.

- Proper PPE to be used.

### Multi-Employer Job Sites/Multi-Work Site

Multi-Work Sites

Where employees must travel between work places during a work shift, the written HAZCOM Program shall be kept at a primary job site.  If there is no primary job site, then the program shall be sent with employees.

The program shall be made available, upon request, to employees, their designated representatives, the Assistant Secretary and the Director in accordance with requirements of 29 CFR 1910.1020(e).

A pre-job briefing shall be conducted with the contractor prior to the initiation of work on the site.

- During this pre-job briefing, contractors shall notify Toole Corp. and present current copies of Material Safety Data Sheets and label information for every hazardous substance brought on-site.

- Toole Corp. shall notify and provide MSDS’  and label information for all hazardous materials the contractor may encounter on the job.

- The facilities labelling system and any precautionary measures to be taken by contractor during normal conditions and emergencies shall be addressed.

- By providing such information to other employers, Toole Corp. does not assume any obligations that other employers have for the safety of their employees.

- In this regard, other employers working on Toole Corp. property or for Toole Corp. on client’s property remain fully responsible for developing and implementing their own compliant hazard communication programs.

### Hazard Warnings / NFPA 704

The NFPA 704 Diamond is a means of disseminating hazard warning and information for a material. The diamond is divided into four sections. Each of the first three colored sections has a number in it associated with a particular hazard. The higher the number is, the more hazardous a material is for that characteristic. The fourth section includes special hazard information. The four sections and an explanation of the numbers in them are provided below:

INJURY & ILLNES RECORDKEEPING

Purpose

To facilitate reporting of recordable fatalities, injuries and illnesses that are work related, and to reduce the work related recordable injuries and illnesses

Scope

All divisions and projects are to complete a “Notice of Recordable Injury Report" (referred to as report) within 24 hours of any recordable or suspected recordable injury. The report is to be faxed to the Toole Corp. officer or designee.

All injuries are to be recorded in the OSHA 300 logs immediately after the injury.

Responsibilitie

The Injury and Illness Prevention Program (IIP Program) administrator, who is the Toole Corp.  Safety Manager, has the authority, responsibility and overall accountability for implementing the provisions of this program for Toole Corp. .

All managers and supervisors are responsible for implementing and maintaining the IIP Program in their work areas and for answering worker questions about the IIP Program.  A copy of this IIP Program is available from each manager and supervisor.

Responsibilities

- Supervisory personnel is responsible for ensuring that the report is completed on time, that it is complete and accurate and that the report is faxed to the designated person.

- Supervisory personnel is responsible for investigating the accident and completing the necessary forms.

- The Human Resource department will insure the record keeping of the OSHA logs and is signed by a company official before posting.

- Human Resources will maintain OSHA 300 log records for five years.

Compliance

Management is responsible for ensuring that all safety and health policies and procedures are clearly communicated and understood by all employees. Managers and supervisors are expected to enforce the rules fairly and uniformly. All employees will comply with safe and healthy work practices by incentives, training, re-training programs and disciplinary programs.  Our system of ensuring that all employees comply with the rules and maintain a safe work environment include:

- Informing employees of the provisions of our IIP Program.

- Evaluating the safety performance of all employees.

- Recognizing employees who perform safe and healthful work practices.

- Providing training to employees whose safety performance is deficient.

- Disciplining employees for failure to comply with safe and healthful work practices.

Procedure

- All accidents and near misses are to be investigated and all incidents leading to a recordable injury are to have a Notice of Recordable Injury Report completed.

- The project name, project number, name of injured, and the date-time of injury are self explanatory and are mainly for identification purposes only.

- The name of injured employee's supervisor must be on the record.  They are the individuals who are responsible for the actions of the injured employee.

- Lost time.  Yes is to be checked if the employee has or is expected to lose time from work due to the injury.  This can be changed, if needed, at a later date.

- Written records will be kept in Human Resources. The records will address work related injuries , fatalities, injuries and illnesses.

- All recordable injuries or illnesses must be recorded on the OSHA 300 log by the HR department within seven calendar days from receiving information that the injury has occurred.

- The OSHA 300A log will be posted annually (February 1st- April 30) in a visible location.

- Brief injury description is to be simple: a broken leg, laceration on the arm, a foreign body imbedded in the eye, etc.

- Medical treatment is a description of why the injury is recordable, i.e., sutures, a positive x-ray of a fracture, multiple doses of a prescription medication, etc.

- How the injury occurred is a step by step description of what happened including actions before, during and after the accident.  This can and should include actions of others (Foreman, other employees).

- History of employee's training is to indicate whether the training was proper, complete and adequate.

- Steps to prevent reoccurrence are the planned actions used to prevent any similar injuries.  These steps are to be detailed and actually used on the project. Telling the employee to 'be careful' is not adequate.

- Home office assistance in training will be provided when needed.

- Cost of injury are the estimated direct costs linked to the injury.  Indirect cost range for 4 to 20 times the direct costs.

- A copy of the completed "Notice of Recordable Injury Report" form is to be faxed to the home office Project Manager or designee within 24 hours of the recordable injury or suspected recordable injury.

Communication

We recognize that open, two-way communication between management and staff on health and safety issues is essential to an injury-free, productive workplace. The following system of communication is designed to facilitate a continuous flow of safety and health information between management and staff in a form that is readily understandable and consists of one or more of the following checked items:

- New employee orientation including a discussion of safety and health policies and procedures.

- Review of our IIP Program, workplace safety and health training programs.

- Regularly scheduled safety meetings.

- Effective communication of safety and health concerns between employees and supervisors, including translation where appropriate.

- Posted or distributed safety information.

- A system for employees to report safety and health hazards/problems effectively and anonymously without fear of reprisal or reprimand.

- Where required, a labor/management safety and health committee that meets regularly, prepares mitten records of the safety and health committees meetings, reviews results of the periodic scheduled inspections, reviews investigations of accidents and exposures and makes suggestions to management for the prevention of future incidents, reviews investigations of alleged hazardous conditions, and submits recommendations to assist in the evaluation of employee safety suggestions.

Hazard Assessment

Periodic inspections to identify and evaluate workplace hazards shall be performed by the following competent observer(s) in the following areas of our workplace:

| Inspector | Inspection | Location | Frequency |
| --- | --- | --- | --- |
| Project Mgr or Safety Representative | Safety Review | Project Site | Daily |
| Safety Group / Committee | Safety Evaluation | Project Site | As Needed |

Periodic inspections are performed according to the following schedule:

- When new substances, processes, procedures, or, equipment which present potential new hazards are introduced into our workplace.

- When new, previously unidentified hazards are recognized

- When occupational injuries and illnesses occur.

- When we hire and/or reassign permanent or intermittent employees to processes, operations, or tasks for which a hazard evaluation has not been previously conducted.

- Whenever workplace conditions warrant an inspection. Periodic inspections consist of identification and evaluation of workplace hazards utilizing applicable documentation and any other effective methods to identify and evaluate workplace hazards.

ACCIDENT/EXPOSURE INVESTIGATIONS

Procedures for investigating workplace accidents and hazardous substance exposures include:

- Visiting the accident scene as soon as possible.

- Interviewing injured workers and witnesses.

- Examining the workplace for factors associated with the accident/exposure.

- Determining the cause of the accident/exposure.

- Taking corrective action to prevent the accident/exposure from recurring.

- Recording the findings and corrective actions taken.

TRAINING AND INSTRUCTION

All employees, including managers and supervisors, shall have training and instruction on general and job-specific safety and health practices provided prior to or at the time of initial job assignment. Training and instruction shall be provided as follows:

- When the IIP Program is first established.

- To all new employees, except for those in construction who are provided training through a OSHA approved construction industry occupational safety and health training program.

- To all employees given new job assignments for which training has not been previously provided.

- Whenever new substances, processes, procedures or equipment are introduced to the workplace and. represent a new hazard.

- Whenever the employer is made aware of a new or previously unrecognized hazard.

- To supervisors to familiarize them with the safety and health hazards to which workers under their immediate direction and control may be exposed.

- TRAINING AND INSTRUCTION (continued)

- To all employees with respect to hazards specific to each employee's job assignment.

- Workplace safety and health training practices for all industries include, but arc not limited to, the following:

- Explanation of the employer's IIP Program, emergency action plan and lire prevention plan, and measures for reporting any unsafe conditions. work practices, and injuries.

- Use of appropriate clothing including gloves, footwear, and personal protective equipment.

- Information about chemical hazards to which employees could be exposed and other hazard communication program information.

- Availability of toilet, hand-washing and drinking water facilities.

- Provisions for medical services and first aid including emergency procedures. In addition, we provide specific instructions to all employees regarding hazards unique to their job assignment, to the extent that such information was not already covered in other training.

RECORDKEEPING

We have taken the following steps to implement and maintain our IIP Program:

- Records of hazard assessment inspections, including the person(s) conducting the inspection, the unsafe conditions and work practices that have been identified and the action taken to correct the identified unsafe conditions and work practices, are recorded on a hazard assessment and correction form.

- Documentation of safety and health training for each employee, including the employee's name or other identifier, training dates, type(s) of training, and training providers are recorded on an employee training and instruction form.

- We also include the records relating to employee training provided by a construction industry occupational safety and health training program approved by OSHA.

- Inspection records and training documentation will be maintained according to the following checked schedule:

- For one year, except for training records of employees who have worked for less than one year which are provided to the employee upon termination of employment.

HAZARD CORRECTION

Unsafe or unhealthy work circumstances, practices or procedures shall be corrected in a timely manner based on the severity of the hazards. Hazards shall be corrected according to the following procedures:

- When observed or discovered.

- When an imminent hazard exists which cannot be immediately abated without endangering employee (s) and/or property, we will remove all exposed workers from the area except those necessary to correct the existing condition.

- Workers necessary to correct the hazardous condition shall be provided with the necessary protection.

All such actions taken and dates they are completed shall be documented on the appropriate forms.

JOBSITE CLEAN-UP PROCEDURE

Clean-Up Procedures

- The matter of clean-up on the job site is a problem for the construction industry in that a satisfactory method of handling it in a manner equitable to both the general contractor and the subcontractor must be established.

- First, Toole Corp. recognizes that a clean job site is conducive to increase quality of work and general efficiency on the part of trades’ people, but is also essential to the safety of all those present on the site. It also recognizes that a Project Superintendent is ultimately responsible for establishing a clean work site, monitoring subcontractors accordingly for removal of their respective rubbish and debris.

- Second, the removal of bulk rubbish and debris, such as crating cartons, and general packaging materials, is the responsibility of each subcontractor, unless otherwise covered in the contract documents, in which special arrangements must be made.

- Third, all rubbish and debris incidental to the work of each subcontractor is also his or her responsibility to remove. Included in this category is miscellaneous trash produced by the individual trade person.

- Fourth, final clean-up of the site will be the responsibility of Toole Corp.. Therefore, it is in the interest of Toole Corp. and is the responsibility of the job superintendent that the preceding conditions are continually adhered to by all employees and subcontractors over the course of the project.

- Process: Because of the numerous variations in project scope, sequence and logistical considerations, it is not feasible to dictate a uniform “clean-up” policy, which could be applied to all job sites. Therefore, it is the responsibility of the Safety Rep. in conjunction with the project manager to establish and maintain a set of procedures that produce a constantly safe and clean environment.

- The following sections constitute a framework for such dynamic policy, but may be modified, omitted, or suspended as the job superintendent sees fit.

- Preliminary work: Before letting subcontract, Toole Corp. shall determine that the subcontractor has the cost of his own clean-up included in his quotations in accordance with the General Conditions Documents. In addition, Toole Corp. shall explicitly state that it is the subcontractors’ responsibility to maintain a clean and safe working environment for the duration of the work involved. Furthermore, the subcontractor shall be informed that he would be held responsible for subsequent costs if he fails to comply with these conditions.

- A clear understanding between Toole Corp. and the various subcontractors is required, in regards to the extent of clean-up necessary to facilitate job progress. When it comes to enforcing the clean-up policy on the job site, it is the superintendent’s duty.

- Site Management: It is the responsibility of the Safety Rep. to monitor the general cleanliness and overall safety of the site on a daily basis. Serious deviations from acceptable conditions should be duly noted in the daily report and the foreman for the responsible subcontractor should be notified of the deviation.

- Continual deviation from the established policies should warrant written notification by the Safety Rep. to either the supervisor or project manager for the subcontractor(s) involved.

- Job Program/Coordination Meeting: Review of the project’s clean-up policy should be an ongoing (and “open”) issue at all job/coordination meeting held with the various subcontractors’ representatives.

- Gross negligence on the part of one or more subcontractors in regards to this policy should be duly noted, recorded and corrective action should be agreed upon by Toole Corp. and all involved parties.

As the demands of each project dictate, it will be the responsibility of the project superintendent to create, implement and maintain a clean-up policy, which is both fair and equitable to Toole Corp. and all subcontractors while establishing a constantly safe job site for the duration of the project.

LADDER SAFETY

Purpose

The purpose of the program is to prescribe rules and establish minimum requirements for the construction, care, and use of the common types of ladders.

All ladders that are purchased and placed into service; or, any ladders that are engineered, manufactured and installed on any of Toole Corp. equipment shall follow the requirements set forth by this program.

Scope

This program is applicable to all employees who may utilize ladders. When work is performed on a non-owned or operated site, the operator's program shall take precedence, however, this document covers Toole Corp. employees and contractors and shall be used on owned premises, or when an operator's program doesn't exist or is less stringent.

Definitions

Ladder - an appliance usually consisting of two side rails joined at regular intervals by cross- pieces called steps, rungs, or cleats, on which a person may step in ascending or descending.

Stepladder - a self-supporting portable ladder, nonadjustable in length, having flat steps and a hinged back. Its size is designated by the overall length of the ladder measured along the front edge of the side rails.

Single ladder - a non-self-supporting portable ladder, nonadjustable in length, consisting of but one section. The overall length of the side rail designates its size.

Extension ladder - a non-self-supporting portable ladder adjustable in length. It consists of two or more sections traveling in guides or brackets so arranged as to permit length adjustment. Its size is designated by the sum of the lengths of the sections measured along the side rails.

Fixed ladder - a ladder permanently attached to a structure, building, or equipment.

Individual-rung ladder - a fixed ladder each rung of which is individually attached to a structure, building, or equipment.

Cage - a guard that may be referred to as a cage or basket guard, which is an enclosure that is fastened to the side rails of the fixed ladder or to the structure to encircle the climbing space of the ladder for the safety of the person who must climb the ladder.

Key Responsibilities

### Managers and Supervisors

- Managers and supervisors are responsible for ensuring that all employees, and/or contractors have been trained in the use and inspection of ladders in accordance to the manufactures guidelines.

- Managers and supervisors are responsible for ensuring that all employees and contractors are aware that if an inspection discovers a defect, the ladder shall not be used and taken out of service.

### Employees

- Employees shall inspect ladders prior, during and at the completion of each use to ensure the condition of the ladder and the safety of its occupants.

- Employees are responsible for following this program and reporting any damage or repairs that may be needed to their supervisor.

Procedure

### Inspection, Care and Safe Work Practices of Ladders

Inspection

Ladders shall be inspected by a competent person for visible defects on a periodic basis and after any occurrence that could affect their safe use.

- Ladder rungs, cleats and steps shall be parallel, level and uniformly spaced when the ladder is in position for use.

- Any ladder has developed defects, shall be withdrawn from service for repair or destruction and tagged or marked as "Dangerous, Do Not Use."

- If a ladder is tipped over, it shall be inspected by a competent person for side rail dents or bends, or excessively dented rungs; check all rung to side rail connections; check hardware connections; check rivets for shears.

- Ladders with broken or missing steps, rungs, or cleats, broken side rails, or other faulty equipment shall not be used; improvised repairs shall not be made.

- All wood parts shall be free from sharp edges and splinters; sound and free from accepted visual inspection from shake, or other irregularities.

Care

Ladders shall be maintained in good condition at all times, the joint between the steps and side rails shall be tight, all hardware and fittings securely attached, and the movable parts shall operate freely without binding or undue play.

Metal bearings of locks, wheels, pulleys, etc., shall be frequently lubricated.

Frayed or badly worn rope shall be replaced. Safety feet and other auxiliary equipment shall be kept in good condition to ensure proper performance.

Rungs shall be kept free of grease and oil.

Ladders shall be stored in a well-ventilated area in a manner to prevent sagging and warping.

Safe Work Practices

Ladders shall be used only for the intended purpose for which they were designed.

The ladder shall be secured at the top or held by another person at the base.

The footing of the ladder shall be placed on a stable and level surface.

Safe Work Practices (continued)

Ladders shall extend 3 feet above the top of an upper landing surface and extension ladders shall be placed at a 4:1 ratio.

When ladders are not able to be extended then the ladder shall be secured at its top to a rigid support that will not deflect.

Ladders shall not be placed on boxes, barrels, or other unstable bases to obtain additional height.

Ladders shall not be used in a horizontal position as platforms, runways, or scaffolds.

Ladders shall not be used by more than one man at a time.

Ladders shall not be placed in front of doors opening toward the ladder unless the door is blocked open, locked, or guarded.

If a ladder is used in a high traffic area, barricades shall be placed to avoid accidental displacement due to collisions.

Do not stand on the top two rungs or top of step ladders.

On two-section extension ladders the minimum overlap for the two sections in use shall be as follows:

| Size of Ladder (feet) | Overlap (feet) |
| --- | --- |
| Up to and including 36’ | 3 |
| Over 36 up to and including 48’ | 4 |
| Over 48 up to and including 60’ | 5 |

No ladder shall be used to gain access to an elevated surface unless the top of the ladder extends at least 3 feet above the point of support, and properly tied off.

The employee shall maintain a three (3)-point grip on the ladder at all times and carry tools/equipment on a belt or hoist up.  Do not carry anything in the hands that could cause injury in case of fall.

The employee shall face the ladder while ascending or descending.

The bracing on the back legs of stepladders is designed solely for increasing stability and not for climbing.

The ladder shall not be moved while occupied.

### Portable Ladders

Stepladders shall not be longer than 20 feet.

Single ladders shall not be longer than 30 feet.

A two-section extension ladders shall not be longer than 60 feet. All ladders of this type shall consist of two sections, one to fit within the side rails of the other, and arranged in such a manner that the upper section can be raised and lowered.

Keep all ladders at least ten (10) feet away from power lines.

Ladders shall not be loaded beyond the maximum intended load for which they were built or in excess of the manufacturer’s rated capacity.  Weight includes the combined weight of the climber and his tools/equipment. Ladders are rated as the following:

- I (holds 250 Ibs)

- I-A (holds 300 Ibs)

- II (holds 225 Ibs)

- III (holds 200 Ibs)

### Fixed Metal Ladders

Ladders shall be constructed to withstand a minimum of 200 pounds.

All metal rungs shall have a minimum diameter of 3/4 inches and wooden rungs shall have a minimum diameter of 1 1/8 inches.

Rungs shall not be more than 12 inches apart and shall be uniform throughout the length of the ladder.

Rungs shall be a minimum length of 16 inches and provide protection so a foot cannot slip off the end.

Rungs shall have a minimum of 7 inches between itself and the structure behind it.

A fall restraint system must be provided for all fixed ladders greater than six feet in length.

- A Cage is required when the fixed ladder is at least twenty feet tall.

- Cages on fixed ladders shall not begin at a point less than 7 feet nor greater than 8 feet from the walking surface below the cage.

- Cages shall provide a clear width of 15 inches in each direction of the rung's centerline.

- Cages shall not extend less than 27 inches, but not greater than 28 inched from the centerline of the rung.

- A climbing fall restraint system may be substituted for a ladder cage.

Lockout / Tagout

Purpose

The purpose of this program is to establish procedures for affixing appropriate lockout/tagout equipment to energy isolating devices and to otherwise disable machines or equipment to prevent unexpected energization, start up or release of stored energy to prevent injury or incident.

Scope

This program covers the servicing and maintenance of machines and equipment where the unexpected energization or start up of the machine or equipment, or the release of stored energy could cause an incident. This program establishes minimum performance requirements for the control of such hazardous energy. When work is performed on a nonowned or operated site, the operator’s program shall take precedence, however, this document covers Toole Corp. employees and contractors and shall be used on owned premises, or when an operator’s program doesn’t exist or is less stringent.

Definitions

Affected employee - An employee whose job requires them to operate or use a machine or equipment on which servicing and maintenance is being performed under lockout/tagout, or whose job requires the employee to work in an area in which such servicing or maintenance is being performed.

Authorized employee - A person that performs lockout/tagout procedures on machines or equipment in order to perform servicing or maintenance on that machine or equipment.  An affected employee becomes authorized when that employee’s duties include performing servicing or maintenance covered under this program.

Capable of being locked out - An energy isolating device is capable of being locked out if it has a hasp or other means of attachment to which, or through which, a lock can be affixed, or it has a locking mechanism built into it. Other energy isolating devices are capable of being locked out if lockout can be achieved without the need to dismantle, rebuild or replace  the energy isolating device or permanently alter its energy control capability.

Energized - Connected to an energy source or containing residual or stored energy.

Energy isolating device - A mechanical device that physically prevents the transmission or release of energy including, but not limited to, the following:

- A manually operated electrical circuit breaker, a disconnect switch, a manually operated switch by which the conductors and no pole can be operated independently, a line valve, a block and any similar device used to block or isolate energy.

- Push buttons, selector switches and other control circuit type devices are not isolating devices.

Energy source - Any source of gas, electrical, mechanical, hydraulic, pneumatic, chemical, thermal, or other energy sources.

Hot tap - A procedure used in the repair, maintenance and service activities that involves welding on a piece of equipment (pipelines, vessels or tanks) under pressure, in order to install connections or other appurtenances (note: 1910.147 (2) (iii) [B] [1] [2] [3]).

Lockout - The placement of a lockout device on an energy isolating device in accordance with an established procedure, ensuring that the energy isolating device and the equipment being controlled cannot be operated until the lockout device is removed.

Lockout device - A device that utilizes a positive means, such as either a key or combination type lock, to hold an energy isolating device in the safe position and prevent the energizing of a machine or equipment. Included are blank flanges and bolted slip blinds.

Normal operation - The utilization of a machine or equipment to perform its intended operation.

Servicing and/or maintenance - Workplace activities such as constructing, setting up, adjusting, inspecting, modifying and maintaining and/or servicing machines and equipment, where the employee may be exposed to an unexpected energization or startup of the equipment or release of a hazardous energy source.

Setting up - Any work performed to prepare a machine or equipment for performing its normal operation.

Tagout: - The placement of a tagout device on an energy isolating device, in accordance with an established procedure, to indicate that the energy isolating device and the equipment being controlled may not be operated until the tagout device is removed.

Tagout device - A prominent warning device, such as a tag and a means of attachment, which can be securely fastened to an energy isolating device in accordance with an established procedure, to indicate that the energy isolating device and the equipment being controlled may not be operated until tagout device is removed.

Key Responsibilities

### Managers and Supervisors

- Responsible to control and enforce this plan and to see that all their employees and contractors that are affected by lockout/tagout procedures, have the knowledge and understanding required for safe application, usage, and removal of all energy controls and devices.

- Ensure employees are trained and comply with the requirements of this program.

### Employees

- Employees who are affected by this program are required to attend training on an annual basis.

- Are required to follow the provisions of this program.

Procedure

### General

Only an authorized employee or employees performing the servicing or maintenance shall perform lockout or tagout.

### Devices

Lockout Device - If an energy source can be locked out a device that utilizes a lock to hold an energy isolating device in a safe position shall be used.  Each site shall have the same type of lock as specified by Toole Corp. .

Tagout Device – If an energy source cannot be locked out with a lockout device then a tagout device shall be used.  Tagout devices are a warning only level of protection and shall be weather and chemical resistant, standardized in color with clear written warning of hazardous energy; i.e. Do Not Operate, Do Not Start, Do Not Energize, etc.  Each site shall have the same style of tags specified by Toole Corp. .

### Specific Energy Control Procedures

Each manager or supervisor is responsible for developing specific step-by-step shutdown and startup procedures for a particular machine or piece of equipment in their respective area.

- A written, step-by-step isolation procedure for shutdown and startup shall be prepared for each type of machine or piece of equipment.

- This procedure shall include:

  - Equipment number if assigned.

  - Equipment location.

  - Energy Source(s) (i.e. electrical, hydraulic, gas pressure, etc.)

  - Location of isolating controls (i.e. breaker switches, valves, etc.)

  - Quantity of isolating controls

  - Quantity of locks required to isolate the equipment

  - Other hardware required to isolate the equipment (i.e. chains, valve covers, blocks, etc.)

  - List any residual energy required to be dissipated before work begins.

### Specific Sequence for Application of Energy Control

### 1. Notification

Authorized employees must notify all other affected employees of the application and removal of lockout/tagout devices.  Notification shall be given before the controls are applied and before they are removed from the machine or equipment.

### 2. Preparation for Shutdown

Before an authorized or affected employee shuts down a machine or equipment, the authorized employee shall have the knowledge of the type and magnitude of the energy, the hazards of the energy to be controlled, and the method or means (locks) to control the energy sources.

### 3. Machine or Equipment Shutdown

The machine or equipment shall be shut down using the procedures established for that machine or piece of equipment.  The shutdown shall be orderly to avoid any addition hazards to employees as a result of the stoppage.

### 4. Machine or Equipment Isolation

All energy isolating devices that are needed to control the energy to the machine or equipment shall be physically located and operated in such a manner as to isolate the machine or equipment from the energy source(s).

### 5. Lockout/Tagout Devices and Application

- Each authorized employee shall have the proper number of locks and devices to be able to perform proper lockout/tagout procedures for machines or equipment that they may be working on.

- Lockout or tagout devices shall be affixed to each energy isolating device by authorized employees.

- Each lockout and tagout devices shall include the name of the individual placing the device.

- Lockout devices shall be affixed in a manner to hold the energy isolating devices in a safe or off position.

- Tagout devices shall be affixed in a manner that will clearly indicate that the operation or movement of isolating devices from the safe or off position.

- Tagout devices used with energy isolating devices with the capability of being locked out shall be fastened at the same point at which the lock would have been attached.  If a tag cannot be directly attached to the energy isolation device it shall be located as close as safely as possible to the device in a position that will be immediately obvious to anyone attempting to operate the device.

- Each energy source shall be locked out completely isolating the equipment.

- Isolating machines or equipment shall include, but are not limited to:

- Pumps, compressors, generators, electric distribution, storage tanks, etc.

- Each type of equipment to be isolated shall have specific procedures for isolation, i.e. for compressors: suction, discharge, power, starting, fuel, dumps shall be closed, locked and tagged out properly. The blow-down valve shall be opened, locked and tagged out properly. (NOTE): If compressor has a side stream hooked up, the side stream shall be closed, locked and tagged out properly.

### 6. Stored Energy and the Possibility of Reaccumulation

Following the application of lockout or tagout devices to energy isolating devices, all potentially hazardous stored or residual energy shall be relieved, disconnected, restrained and otherwise rendered safe.

If there is a possibility of re-accumulation of stored energy, verification of isolation shall be continued until the servicing or maintenance operation is completed, or until the possibility of such accumulation no longer exists.

### 7. Verification of Isolation

The authorized employees performing the lockout procedure verifies/ensures that the equipment is isolated or disconnected from the energy source(s) by first checking that no personnel are exposed, then verify the isolation of the machine or equipment by operating the control(s) or by testing to make certain the equipment will not operate.

### Multiple Workers

A crew of authorized employees may use a group lockout or tagout device.    This will afford the group of employees a level of protection equal to that provided by a personal lockout or tagout device.

- A tailgate meeting shall be conducted to review the lockout procedures and other information as required for safe work to continue – all crafts and effected departments shall be involved.

- An authorized employee will isolate the equipment and ascertain the exposure status of individual group members.

- All workers will then place their individual locks on the device’s group lockout or tagout device after they have verified the procedure.

- The crew leader or an assigned authorized employee shall be responsible of assuring the continuity of the lockout procedures including documenting lockout information passed along during a shift or personnel changes.

### Release from Lockout/Tagout

When servicing or maintenance is completed or when Lockout / Tagout devices must be temporarily removed, the equipment requires testing and the machine or equipment is ready for testing or to return to normal operating conditions, the following steps shall be taken, in this order:

- Check the machine or equipment and the immediate area surrounding the machine or equipment to ensure that all nonessential items such as tools have been removed and that the machine or equipment components are operationally intact.

- Check the work area to ensure that all personnel have been safely positioned or removed from the area.

- Remove the Lockout/Tagout device

- Energize and proceed with testing

- Deenergize and reapply control methods including Lockout / Tagout devices

- Document the procedure by use of the completed isolation log and provide to supervisor for filing.

### Removal of Locks

The authorized employee who applied the lock shall be the one to remove their lock. However, after all work has been completed, certain conditions may arise which prohibit this person from being present to remove the lock.

The following procedures shall be followed to allow for the removal of a lock that another person has applied:

- Every effort shall be made to contact the authorized employee who applied the lock to obtain the key(s).

- If the key(s) cannot be made available, the employee who requests removal of the lock shall contact their supervisor.

- The supervisor shall verify that every effort was made to contact the original authorized employee who applied the lock and to obtain the key(s).

- The employee removing the lock shall note on the Service Report that the lock(s) were removed with permission by supervisor.

- All reasonable efforts will be made by supervisor to notify that employee their lock has been removed, ensuring that the authorized employee has this knowledge before they return to work.

- If the equipment is client owned, the supervisor or employee requesting to remove the lock(s) shall contact the client to get the lock removed. Clients must remove their lock(s).

- NOTE: Toole Corp. employees shall not remove any client locks.

### Shift or Personnel Changes

In the event shift or personnel changes occur during maintenance and/or repair activities, the designated Toole Corp. employee in charge shall take the necessary steps to maintain the continuity of the lockout/tagout protection.  This includes maintaining that all provisions in this procedure are adhered to and the transfer of lockout/tagout devices between authorized employees is accomplished.

### Contractors

Contractors performing lockout procedures on Toole Corp. property shall comply with this procedure.  Contractors shall supply their own locks.

Toole Corp. shall initially lockout Toole Corp. machines and equipment before the contractor will be allowed to apply their own lock in addition to the Lanzo Companies.

### Annual Audits

Each year the manager or supervisor, or his representative, will perform an inspection of the Lockout Program in their respective areas to verify the effectiveness of the program. An authorized employee other than the one(s) utilizing the energy control procedure being inspected shall perform the audit and shall verify that:

- Each authorized and/or affected employee has been trained as required.

- Any new equipment added has specific lockout procedures developed and documented.

- Current procedures are adequate for performing complete isolation of equipment and resulting in a zero energy state.

- The annual audit will be certified in writing and a copy of the audit maintained on file at the managers/supervisors office.

TRAINING

Toole Corp. shall provide training to ensure that the purpose and function of the energy control program are understood by authorized employees and that the knowledge and skills required for the safe application, usage, and removal of the energy controls are acquired by employees. The training shall include the following:

- The recognition of applicable hazardous energy (lockout/tagout) sources, the type and magnitude of the energy available in the workplace, and the methods and means necessary for energy isolation and control.

- The purpose and use of energy control procedures.

- When tagout systems are used, employees shall also be trained in the following limitations of tags:

  - Tags are essentially warning devices affixed to energy isolating devices, and do not provide the physical restraint on those devices that is provided by a lock.

  - When a tag is attached to an energy isolating means, it is not to be removed without authorization of the authorized person responsible for it, and it is never to be bypassed, ignored, or otherwise defeated.

  - Tags must be legible and understandable by all authorized employees, affected employees, and all other employees whose work operations are or may be in the area, in order to be effective.

  - Tags and their means of attachment must be made of materials which will withstand the environmental conditions encountered in the workplace.

  - Tags must be securely attached to energy isolating devices so that they cannot be inadvertently or accidentally detached during use.

  - Tags may evoke a false sense of security, and their meaning needs to be understood as part of the overall energy control program.

All other employees whose work operations are or may be in an area where energy control procedures may be utilized, shall be instructed about the procedure, and about the prohibition relating to attempts to restart or reenergize machines or equipment which are locked out or tagged out.

Retraining

Retraining shall be conducted whenever a periodic inspection reveals, or whenever Toole Corp. has reason to believe that there are deviations from or inadequacies in the employee's knowledge or use of the energy control procedures.

The retraining shall reestablish employee proficiency and introduce new or revised control methods and procedures, as necessary.

Training Documentation

Toole Corp. shall certify that employee training has been accomplished and is being kept up to date. The certification shall contain each employee's name and dates of training.

Specific Equipment Lockout Procedures

Department__________________________________________________________

Equipment No.________________________________________________________

Energy Source_______________________________________________________

Procedure for Shutdown and Isolation:

(List number of steps required to isolate machine or equipment - write N/A on lines not used or add additional steps if necessary)

STEP NO.

1.______________________________________________________________________ 2.______________________________________________________________________ 3.______________________________________________________________________ 4.______________________________________________________________________ 5.______________________________________________________________________ 6.______________________________________________________________________ 7.______________________________________________________________________ 8.______________________________________________________________________ 9.______________________________________________________________________ 10._____________________________________________________________________

Additional Information:____________________________________________________ _______________________________________________________________________ _______________________________________________________________________ _______________________________________________________________________

Prepared By:__________________________________  Date:__________________

(This procedure to be communicated to all authorized and affected employees and kept on file at location of machine or equipment)

SAMPLE TAG

WARNING

MINIMUM LOCK/OUT – TAG/OUT PROCEDURES

Inlet Suction Block Valve Discharge Block Valve

Fuel Gas Valve Start Gas Valve

Liquid Dump Line Blow Down (Lock Open)

When working on this compressor package the following items must be LOCKED OUT & TAGGED OUT. Residual pressure must be blown down. Open all valves on surge bottles and piping to relieve any pressure that may be trapped.

Side Stream (For Units Set up with Side Streams)

When working on the compressor each person must lock and tag the compressor package!

Isolation Log

Date of Isolation:

Description of Work:

List of Equipment out of Service:

Necessary Requirements of Clear Isolation:

Authorized Employee Signature:  		_____________________________________

Person Continuing Work Signature:  	_____________________________________

Locks/Tags for GROUP LOCKOUT or Multiple Locks/Tags

| Lock # or Tag | Date Installed | Date Removed | Print Name (for Group Lockout) | Signature |
| --- | --- | --- | --- | --- |

(If additional space is needed, please attach an additional page)

ANNUAL AUDIT OF THE CONTROL OF HAZARDOUS ENERGY PROGRAM

I certify that an audit of the Toole Corp. “Control of Hazardous Energy” Program was conducted and that each employee has been trained in the recognition and procedures to lockout equipment they may be required to work on or may be affected by.

I further acknowledge that the current procedure is adequate to safely lockout equipment in this department for servicing and maintenance.

Department:  _____________________________________________

Manager (or representative):  ________________________________

Date:  ______________________

Original to file:  ___________________________

PERSONAL PROTECTIVE EQUIPMENT

Purpose

The purpose of the Personal Protective Equipment section is to set forth the procedures for the use, care, and maintenance of personal protective equipment required to be used by employees for the prevention of injuries.

Scope

Applies to all Toole Corp. employees. When work is performed on a non-owned or operated site, the operator’s program shall take precedence, however, this document covers Toole Corp. employees and contractors and shall be used on owned premises, or when an operator’s program doesn’t exist or is less stringent.

Key Responsibilities

### HSE Manager

- Assists in the selection of appropriate PPE.  If a task exposes an employee to hazards which cannot be eliminated through engineering or administrative controls, the HSE Manager assists the supervisor and project manager to identify and select PPE suitable for the specific task performed, conditions present, and frequency and duration of exposure.  Employees need to give feedback to the supervisor about the fit, comfort, and suitability of the PPE being selected.  Employees are provided reasons for selection of PPE.

- Assists supervisor and site managers in assuring all PPE obtained meets regulatory and this procedure’s requirements.

- Performs Worksite Hazard Assessments - The hazard assessment must indicate a determination if hazards are present or are likely to be present, which necessitate the use of PPE.  Sources of hazards include, but are not limited to:  hazards from impact/motion, high/low temperatures, chemicals, materials, radiation, falling objects, sharp objects, rolling or pinching objects, electrical hazards, and workplace layout.

- Certifies in writing the tasks evaluated, hazards found and PPE required to protect employees against hazards and ensures exposed employees are made aware of hazards and required PPE before they are assigned to the hazardous task.    Certificate shall include certifier’s name, signature, dates and identification of assessment documents.

### Managers and Supervisors

- Supervisors and managers shall regularly monitor employees for correct use and care of PPE, and obtain follow-up training if required to ensure each employee has adequate skill, knowledge, and ability to use PPE.

- Supervisors and managers shall enforce PPE safety rules following the guidance of the Toole Corp. progressive disciplinary procedures and ensure Required PPE Poster is posted properly.

### Employees

- Complying with the correct use and care of PPE.

- Reporting changes in exposure to hazardous conditions that might require a follow-up assessment of the task for PPE.

- Reporting and replacing defective PPE, which shall not be used.

- Wearing of required PPE is a condition of employment.

Procedure

### General

Employee owned equipment is NOT permitted, except for safety toe footwear and prescription safety glasses.   Toole Corp. is still responsible for the assurance of its adequacy, maintenance and sanitation of those two items.

All PPE issued shall be at no cost to the employee and PPE shall be used and maintained in a sanitary and reliable condition.

All employees will know and follow the procedures outlined in this Program.

### Eye Protection

Employees must use appropriate eye or face protection when exposed to eye or face hazards from flying particles, molten metal, liquid chemicals, acids or caustic liquids or chemical gases or vapours.  Eye and Face PPE must comply with ANSI Standard Z87.1-2003 (Z87+), Occupational and Educational Personal Eye and Face Protective Devices.

Safety Glasses

Safety glasses, with side shields, that meet ANSI Z-87.1-2003 standards with “high Impact lenses” are required to be worn by all employees, subcontractors, and visitors while on Toole Corp. property, at all times, as described below:

- At field locations, in shops and warehouses, except in approved, designated, striped safety zones.

- In all yard work zones or by everyone when in the vicinity of loading or unloading equipment, performing mechanic or maintenance work, test stand operations, operating equipment such as forklifts, welding, or any type of work which has the potential to inflict an eye injury.

- In any office, restroom, or any other building while performing any type of work where a potential eye injury may be present.

- Visitors will be provided with visitor glasses.  In the absence of approved prescription safety glasses, “Over the glass” type safety glasses or goggles, must be worn over the nonsafety glasses until approved prescription safety glasses are obtained.

- Workers assisting welders must wear absorbent safety glasses that protect the wearer from ultra-violet (UV) and/or infrared rays (IR).

- Dark shaded lens (sunglasses) darker than a # 1 shade is prohibited to be worn indoors unless welding or assisting a welder.

- A doctor must support “exceptions for medical reasons” in writing to exempt safety eyewear requirements.

- Safety glasses are not required:

  - Inside offices.

  - Parking lots when traveling from vehicles to and from office buildings by way of main doors that do not pass through shops.

Goggles

- Chemical splash proof goggles shall be worn when handling or mixing liquid chemicals, solvents, paints, etc., and/or as recommended on the Material Safety Data Sheet of the material being handled.

- Dust proof goggles shall be worn when blowing equipment down with air or while performing other jobs where safety glasses are not adequate to prevent airborne particles from entering the openings around the lenses and side shields.

Face Shields

- Full face shields shall be worn over safety glasses when operating hand held or stationery grinders with abrasive or wire wheels, while chipping paint or concrete or, performing jobs where there is the potential for flying objects striking the face and safety glasses or goggles would not provide adequate protection.

### Head Protection

Employees must wear protective helmets when working in areas where there is a potential for injury to the head from employee initiated impact or impact from falling or other moving objects.  Helmets must comply with ANSI Standard Z89.1-1997 Class E, American National Standard for Industrial Head Protection for Type II head protection or be equally effective.

- Employees must wear protective helmets when working in areas where there is a potential for injury to the head from falling objects.

- Hardhats are to be worn at all field, shop and warehouse locations, or where deemed necessary as per each location’s PPE Hazard Assessment.

- Hardhats will not be altered in any way.

- Do not paint or apply unauthorized stickers, name plates, etc.

- Do not drill, cut, bend, or apply heat.

- Do not alter the suspension system.  H

- Hardhats will be inspected by the employee regularly for cracks, chips, scratches, signs of heat exposure (sun cracks), etc.

- Defective hardhats will be replaced immediately.

- Hardhats shall not be placed in rear windows of vehicles where they will be exposed to the sun or become projectiles during an accident.

- A supply of hardhats must be made available to visitors.

- Toole Corp. shall provide hardhats.

- Employees will be trained in the use, care and maintenance of head protection equipment.

### Hearing Protection

Hearing protection is required to be worn by all employees, subcontractors, and visitors while in posted “High Noise” areas.  Refer to the Toole Corp. Hearing Conservation Program for more information.

Warning signs will be posted in areas known or suspected to have noise levels exceeding 85 dBA either constantly or intermittently.

When signs are not posted, employees shall wear hearing protection when noise caused by machinery, tools, etc., prevents normal conversations to be heard clearly.

Rule of thumb: If you have to yell to be heard, hearing protection is required

Types

- Molded Inserts (ear plugs)

- Canal Caps (head band type)

- Muff, either headband or hard hat mounted Earmuffs and earplugs shall be provided to the employee in sizes and configurations that will be comfortable to the employee.

Care and Maintenance

- Inspect hearing protection prior to each use.

- Hearing protection must be kept clean to prevent ear infections.

- Most earplugs used today are disposable and must be discarded when they become dirty, greasy, or cracked.

- Earmuffs that have deteriorated foam inserts, cracked seals or are defective must be replaced.

Fit

- Due to individual differences, not everyone can wear the same type of hearing protection. A variety of styles may have to be tried before one is found to be comfortable and provide adequate protection.

- Employees shall be instructed how to obtain the proper fit.

Hand Protection

Gloves

- Gloves are required to be worn when performing work, which may expose the hands to extreme temperatures, cuts and abrasions, or exposure to chemicals.

- Welding: Welding gloves made of leather or other heat resistant materials shall be worn when performing arc welding or oxy/gas cutting.

- Chemical: Impervious (chemical resistant) gloves shall be worn when handling chemicals that specify gloves as personal protection equipment when handling.

- Refer to the specific chemical’s Material Safety Data Sheet for the correct glove type.

- Persons assigned to working with chemicals, i.e., solvent vats, shall be issued their own individual gloves for hygiene purposes.

- Leather: Leather gloves should be worn when working with sharp materials or when handling rigging equipment.

- Cloth: Cloth gloves should be worn when handling objects or materials, which could cause blisters, splinters, cuts, etc.

- Heat Resistant: Heat resistant gloves shall be worn when handling hot bearings, races, or other materials or objects that have been heated beyond ambient temperatures.

- Insulated: Insulated gloves shall be worn to prevent frostbite in extreme cold climates.

- Glove Inspections

  - Gloves shall be inspected before each use for holes, tears, and worn areas.

  - Chemical gloves shall be periodically air tested for pinholes by twisting the cuff tightly, apply low air pressure to expand the glove, and then submersing in water to check for bubbles.

  - Defective gloves shall be discarded immediately.  Exception: machinists are exempted from wearing gloves while working with rotating machinery.

### Foot Protection

Safety footwear shall be worn by all employees with regularly assigned duties at field locations, in shops and warehouses.

- Office workers and visitors who enter these areas on an infrequent basis will not be required to wear foot protection provided they stay clear of the work being performed.

- If required to be in the close proximity of the work, the work will be stopped while visiting the area or safety footwear will be worn.

Shops, Field Locations, Warehouses and Parts Departments: Leather or equivalent boots, either lace up or pull up, shall be worn.

- The boot must provide ankle protection and have soles designed to protect from punctures with defined heels for climbing ladders.

- Metatarsal guards will be worn when duties present a hazard of equipment or material crushing the foot.

- All safety footwear must meet ANSI Z41-1999 standards.

- Client locations may require safety footwear to be worn by everyone; check with the local supervisor for client requirements before visiting field locations.

### Fall Protection

Personal fall protection is required when performing certain elevated jobs in excess of six feet.  Consult the Toole Corp. Fall Protection Program.

### Electrical Protection

Consult the Toole Corp. Electrical Safety Program.

### Worksite Hazard Assessment

During a hazard assessment the following sample hazard sources will be identified:

- High or low temperatures; Chemical exposures (use MSDS for guidance)

- Flying particles, molten metal or other eye, face, or skin hazards

- Falling objects or potential for dropping objects; employee falling from a height of 6’ or more

- Sharp objects; Rolling or pinching that could crush the hands or feet;

- Electrical hazards

Where these hazards could cause injury to employees, personal protective equipment must be selected to substantially eliminate the injury potential.   Employees will be notified for the selection and reason.

The results of this assessment shall be communicated to each affected employee and kept at the local office.

Selected/identified PPE shall be fitted to each affected employee.  Exemptions for use of PPE must be supported by the PPE hazard assessment.

### Monitoring

Supervisors and site managers monitor worksite tasks for changes in, or the introduction of new hazards.  If new hazards are discovered, they advise the HSE Manager who then conducts a hazard assessment for appropriate PPE.   The HSE Manager monitors the effectiveness of the PPE Procedure and makes recommendations to management to improve the procedure.

Training

Each employee who requires PPE shall be properly trained.  Training shall include:

- When PPE is necessary.

- What PPE is necessary.

- How to properly don, doff, adjust and wear PPE.

- The limitations of PPE.

- How to maintain PPE in a sanitary and reliable condition.

### Retraining

Retraining is required when:

- The workplace changes, making the previous training obsolete.

- The type of PPE changes.

- When the employee demonstrates lack of use, improper use, or insufficient skill or understanding in PPE selection, necessity, use and limitations.

### Documentation

Training shall be documented and records kept at the local office.  The training documentation shall include:

- Name of employee(s) trained;

- The dates of training; and

- The training subject.

RESPIRATORY PROTECTION

Purpose

It is the intention of Toole Corp. to provide a respirator protection program that meets or exceeds all federal standards. Toole Corp. will attempt to engineer potential harmful vapors and oxygen deficient atmosphere exposure hazards out of the work environment.  If engineering control measures are not feasible or during emergency situations with high exposure then respirators shall be provided which are applicable and suitable for purpose intended.

Scope

This program applies to all Toole Corp. projects and operations.

Respiratory Program Administrator

Overall responsibility for the respiratory protection program is assigned to the Toole Corp. Safety Manager in order to ensure that specific requirements are followed.

The Administrator must be knowledgeable of the complexity of the program, conduct evaluations, and be properly trained.

This assignment is made, however, with the understanding that individual supervisors will have to implement and enforce major portions of the program. It is understood that the Program Administrator will report performance problems to the appropriate manager for resolution. The person who will have responsibility for administering all the aspects of this program will be the Project Manager or their designee.

The responsibilities of the Program Administrator will include, but are not limited to:

- Conducting an annual written evaluation of the program. The program evaluation should be completed no later than December, 31, of each year.

- Ensuring an adequate supply of respirators, cartridges, and repair/replacement parts. The Program Administrator may delegate this duty but will retain overall responsibility. The person(s) to whom this duty has been delegated is the Project Manager and/or Field Supervisor.

- Ensuring that only respirators that have been approved by the Corporate Health and Safety office are ordered and used. Under no circumstances will respirators be used that have not been approved by NIOSH/MSHA.  The selected designated respirator manufacturer for Toole Corp. is 3M.

- Ensuring that all respirator users have been trained in the use, selection and limitations of the type of respirators they will be using prior to the first time the respirator must be used. While the duty of conducting the training may be delegated, the Program Administrator retains final responsibility for seeing that all employees are appropriately trained.

- Ensuring that all respirator users have been medically evaluated and found fit to use the type of respirators that will be required in their job. The medical evaluation must be completed prior to assigning any employee to a task that requires use of a respirator.

- Ensuring that all respirator users are fit-tested at least annually and more often if other federal requirements apply.

- Ensuring that respirators are individually issued, are cleaned and sanitized on a regular basis, and respirators are stored in a clean and accessible location. This duty may also be delegated but the Program Administrator retains final responsibility for seeing that it is done.

- Ensuring that respirators are selected based on the hazard that will be encountered. This program describes the basic respirators that will be used at this site and the tasks for which they will be required. In special circumstances, the Program Administrator will contact the corporate health and safety staff for guidance in selecting the correct respirator.

- Ensuring that employee exposure is monitored to assure correct respirator type is used. Exposure monitoring may be delegated to others; however, the Program Administrator has final responsibility of monitoring completion and to request assistance when necessary.

- Ensuring surveillance of employees who wear respirators shall leave the area to wash, change cartridges or if they detect break through or resistance.

- Ensuring that the elements of the Respiratory Protection Program for the selection, use, cleaning/maintenance, storage and fit-testing of respirators are followed.

- Ensuring that respirator parts are not exchanged between brands of respirators.

- Ensuring medical evaluations, respirators and required training are provided at no cost to the employee.

Medical Requirements

### General

Toole Corp. shall provide a medical evaluation to determine the employee's ability to use a respirator, before the employee is fit tested or required to use the respirator in the workplace.   Toole Corp. may discontinue an employee's medical evaluations when the employee is no longer required to use a respirator.

### Medical Evaluation Procedures

Toole Corp. shall identify a physician or other licensed health care professional (PLHCP) to perform medical evaluations using a medical questionnaire or an initial medical examination that obtains the same information as the medical questionnaire.  The medical evaluation shall obtain the information requested by the Medical Questionnaire in Forms section (or equivalent).

The medical evaluation prior to fit-testing will be confidential, conducted during normal working hours, be at a convenient time and location, be understandable and the employee will be given a chance to discuss the results with the PLHCP.

### Supplemental Information for the PLHCP

The following information must be provided to the PLHCP before the PLHCP makes a recommendation concerning an employee's ability to use a respirator:

- The type and weight of the respirator to be used by the employee;

- The duration and frequency of respirator use (including use for rescue and escape);

- The expected physical work effort;

- Additional protective clothing and equipment to be worn; and

- Temperature and humidity extremes that may be encountered.

### Supplemental Information for the PLHCP (continued)

Toole Corp. shall provide the PLHCP with a copy of the Toole Corp. Respiratory Protection Program.

Note:  When Toole Corp. replaces a PLHCP, Toole Corp. must ensure that the new PLHCP obtains this information, either by providing the documents directly to the PLHCP or having the documents transferred from the former PLHCP to the new PLHCP. However, OSHA does not expect employers to have employees medically re-evaluated solely because a new PLHCP has been selected.

### Medical Determination

In determining the employee's ability to use a respirator, Toole Corp. shall obtain a written recommendation regarding the employee's ability to use the respirator from the PLHCP.   The recommendation shall provide only the following information:

- Any limitations on respirator use related to the medical condition of the employee, or relating to the workplace conditions in which the respirator will be used, including whether or not the employee is medically able to use the respirator;

- The need, if any, for follow-up medical evaluations; and

- A statement that the PLHCP has provided the employee with a copy of the PLHCP's written recommendation.

All recommendations are to be sent to Lanzo Companies’s Safety Manager.

Additional Medical Evaluations

At a minimum, Toole Corp. shall provide additional medical evaluations that comply with the requirements of this program if:

- An employee reports medical signs or symptoms that are related to ability to use a respirator;

- A PLHCP, supervisor, or the respirator Program Administrator informs Toole Corp. that an employee needs to be re-evaluated;

- Information from the respiratory protection program, including observations made during fit testing and program evaluation, indicates a need for employee re-evaluation; or

- A change occurs in workplace conditions (e.g., physical work effort, protective clothing, and temperature) that may result in a substantial increase in the physiological burden placed on an employee.

Work Site Procedures

Each work site where respirators are required to protect the health of the worker shall have work site procedures that follow the guidelines of this program.   Specific procedures may also be required by our client which will be followed.  The following areas shall be included:

- Identification of specific hazard requiring respiratory protection

- The selection of the appropriate respiratory protection equipment based on the specific hazard and concentration levels, characteristics, etc.  Specific brand and models of respiratory equipment to be used shall be identified in the procedures.

- Verification that each user of respiratory protection is qualified (medical approval, current fit test, annual training and demonstrates competency.

Respirator Selection Criteria

The selection of the respiratory equipment is based on the hazards the employee is exposed to.  Toole Corp. shall:

- Perform hazard identification,

- Select and provide respirators based on those hazards and factors affecting performance,

- Establish brands and models to be used, and

- Estimate exposures and contaminant information.

### Hazard Identification

Due to the many varied work locations Lanzo Companies’s identification of respiratory hazards will be contained in the various work site specific safety plans.  However, common respiratory hazards that will be encountered include:

- Dust

- Fumes

- Gases

- Chemical particles

- Oxygen Deficiency

Characteristics of Hazardous Operation or Process

- Hot operations: welding, chemical reactions, soldering, melting, melding and burning

- Liquid operations: painting, degreasing, dipping, spraying, brushing, coating, etching, cleaning, pickling, plating, mixing, galvanizing and chemical reactions

- Solid operations: pouring, mixing, separations, extraction, crushing, conveying, loading, bagging and demolition.

- Pressurized spraying: cleaning parts, applying pesticides, degreasing, sand blasting and painting

- Shaping operations: cutting, grinding, filing, milling, melding, sawing and drilling

Gaseous Contaminants

- Inert gases (helium, argon, etc.), which do not metabolize in the body but displace air to produce an oxygen deficiency.

- Acid gases (SO2, H2S, HCl, etc.) which are acids or produce acids by reaction with water.

- Alkaline gases (NH3, etc.), which are alkalies or produce alkalies by reaction with water.

- Organic gases (butane, acetone, etc.), which exist as true gases or vapors from organic liquids.

- Organometallic gases (tetraethyl lead, organo-phosphates, etc.), which have metals attached to organic groups.

Particulate contaminants

- Dusts are mechanically generated solid particulates (0.5 to 10µm)

- Fumes are solid condensation particles of small diameter (0.1 to 1.0 µm)

- Mists are liquid particulate matter (5 to 100 µm)

- Smoke is chemically generated particulates (solid and liquid) of organic origins (0.01 to 0.3 µm)

### Selection of Respirator

The following factors shall be taken into account when selecting the proper respirator:

Concentration and Type of Contaminant

The concentration and type of contaminant will determine the model and type of respirator and cartridges/filters or filters to be used.  The concentration is based on a sampling of the atmosphere.

Location of Hazardous Area

(Confined Space, nearby contaminants, etc.)

Worker Activity

(Extreme heat, cold, welding hood requirement, etc.)

Types of Respirators

Air-purifying respirators can be either full-face or half masks with mechanical or chemical cartridges to filter dusts, mists, fumes, vapors or gases.

Powered air-purifying respirators use a blower to pass the contaminated air through a filter. The purified air is then delivered into a mask or hood. They filter dusts, mists, fumes, vapors and gases, just like ordinary air-purifying respirators.

Air-purifying respirators cannot be used in oxygen-deficient atmospheres, which can result when another gas displaces the oxygen or consumption of oxygen by a chemical reaction occurs. Oxygen levels below 19.5% require either a source of supplied air or supplied-air respirator protection. Levels below 16% are considered to be unsafe and could cause death.  To determine the proper cartridge for air-purifying respirators contact the Toole Corp. Safety Manager or a qualified on-site safety representative of the client.  You should also consult the Material Safety Data Sheet of the substance that needs to be filtered.

All cartridges are assigned a color designating the type of contaminant they will filter:

White: 		Acid gas

Black: 		Organic vapors

Green: 		Ammonia gas

Yellow: 		Acid gas and organic vapors

Purple: 		Radioactive materials

Orange:		Dust, fumes and mists

Olive: 		Other gases and vapors

Once the wearer of the respirator can detect an odor, irritation, or taste of the contaminant, the cartridge should be replaced.  All cartridges and/or filters shall be changed at the beginning of each shift.

Supplied-air respirators provide the highest level of protection against highly toxic and unknown materials. Supplied air refers to self-contained breathing apparatuses (SCBAs) and air-line respirators. SCBAs have a limited air supply that is carried by the user, allowing for good mobility and fewer restrictions than air-line respirators.

Air-line respirators have an air hose that is connected to a fresh air supply from a central source. The source can be from a compressed air cylinder or air compressor that provides at least Grade D breathing air.

Emergency Escape Breathing Apparatuses (EEBAs) provide oxygen for 5, 10 or 15 minutes depending on the unit. These are for emergency situations in which an employee must escape from environments immediately dangerous to life or health (IDLH).

SCBA (Self Contained Breathing Apparatus)

Toole Corp. does NOT allow employees to work in an Immediately Dangerous to Life and Health (IDLH) environment.

In order to maintain the NIOSH/MSHA approval of any respirator, mixing parts from other respirator manufacturers is prohibited. This includes airline hoses, valves, gaskets, cartridges, etc.  For example, do not use 3M cartridges or valve gaskets with an MSA product.

### Brand and Models

Toole Corp. has selected 3M as its NIOSH-certified respirator.   Only this brand of respirator shall be used in compliance with the conditions of the certification of its Respiratory Protection Program (fit testing model, no mixing of different manufacturer parts, cartridges, filters, etc.).

The specific model will be based on the hazard, concentration of contaminant, oxygen level, work environment and type of work being performed.  To aid in the selection process the following will be used to identify the proper 3M respiratory equipment for the work being performed and hazard that is present.

- NIOSH Pocket Guide to Chemicals

- 3M Cartridge Selection Guide

- 3M Respirator Selection Guide

### Estimate of Exposures and Contaminant Information

- No employee shall enter an IDLH environment.

- Normal oxygen levels shall be maintained.

- No employee shall be exposed to an atmosphere containing concentrations that would exceed the STEL or PEL for the identified atmospheric hazard.

Respirator Fit Testing

Before an employee may be required to use any respirator with a negative or positive pressure tight-fitting face piece, the employee must be fit tested with the same make, model, style, and size of respirator that will be used.  This section specifies the kinds of fit tests allowed, the procedures for conducting them, and how the results of the fit tests must be used.

All respirator users are fit-tested at least annually and more often if other federal requirements apply.

Supplied Air Respirators are required to be fit tested as well.

Toole Corp. shall ensure that employees using a tight-fitting face piece respirator pass an appropriate qualitative fit test (QLFT) or quantitative fit test (QNFT) as stated in this program.

Toole Corp. shall ensure that an employee using a tight-fitting face piece respirator is fit tested prior to initial use of the respirator, whenever a different respirator face piece (size, style, model or make) is used, and at least annually thereafter.

Toole Corp. shall conduct an additional fit test whenever the employee reports, or Lanzo Companies’s PLHCP, supervisor, or Program Administrator makes visual observations of, changes in the employee's physical condition that could affect respirator fit. Such conditions include, but are not limited to, facial scarring, dental changes, cosmetic surgery, or an obvious change in body weight.

If after passing a QLFT or QNFT, the employee subsequently notifies Lanzo Companies, Program Administrator, supervisor, or PLHCP that the fit of the respirator is unacceptable, the employee shall be given a reasonable opportunity to select a different respirator face piece and to be retested.

The fit test shall be administered using an OSHA-accepted QLFT or QNFT protocol. The OSHA-accepted QLFT and QNFT protocols and procedures are contained in this section.

QLFT may only be used to fit test negative pressure air-purifying respirators that must achieve a fit factor of 100 or less.  Half face air filtering respirators may be fit tested with irritant smoke while full face air filtering respirators require Portacount fit testing.

If the fit factor, as determined through an OSHA-accepted QNFT protocol, is equal to or greater than 100 for tight-fitting half face pieces, or equal to or greater than 500 for tight-fitting full face pieces, the QNFT has been passed with that respirator.

Fit testing of tight-fitting atmosphere-supplying respirators and tight-fitting powered air-purifying respirators shall be accomplished by performing quantitative or qualitative fit testing in the negative pressure mode, regardless of the mode of operation (negative or positive pressure) that is used for respiratory protection.

Qualitative fit testing of these respirators shall be accomplished by temporarily converting the respirator user's actual face piece into a negative pressure respirator with appropriate filters, or by using an identical negative pressure air-purifying respirator face piece with the same sealing surfaces as a surrogate for the atmosphere-supplying or powered air-purifying respirator face piece.

Quantitative fit testing of these respirators shall be accomplished by modifying the face piece to allow sampling inside the face piece in the breathing zone of the user, midway between the nose and mouth. This requirement shall be accomplished by installing a permanent sampling probe onto a surrogate face piece, or by using a sampling adapter designed to temporarily provide a means of sampling air from inside the face piece.

Any modifications to the respirator face piece for fit testing shall be completely removed, and the face piece restored to NIOSH-approved configuration, before that face piece can be used in the workplace.

Fit Test Procedures

The requirements in this section apply to all OSHA-accepted fit test methods, both QLFT and QNFT.

The test subject shall be allowed to pick the most acceptable respirator from a sufficient number of respirator sizes so that the respirator is acceptable to, and correctly fits, the user.

Prior to the selection process, the test subject shall be shown how to put on a respirator, how it should be positioned on the face, how to set strap tension and how to determine an acceptable fit. A mirror shall be available to assist the subject in evaluating the fit and positioning of the respirator. This instruction may not constitute the subject's formal training on respirator use, because it is only a review.

The test subject shall be informed that he/she is being asked to select the respirator that provides the most acceptable fit. Each respirator represents a different size and shape, and if fitted and used properly, will provide adequate protection.

The test subject shall be instructed to hold each chosen face piece up to the face and eliminate those that obviously do not give an acceptable fit.

The more acceptable face pieces are noted in case the one selected proves unacceptable; the most comfortable mask is donned and worn at least five minutes to assess comfort. Assistance in assessing comfort can be given by discussing the following points:

- If the test subject is not familiar with using a particular respirator, the test subject shall be directed to don the mask several times and to adjust the straps each time to become adept at setting proper tension on the straps.

- Position of the mask on the nose

- Room for eye protection

- Room to talk

- Position of mask on face and cheeks

The following criteria shall be used to help determine the adequacy of the respirator fit:

- Chin properly placed;

- Adequate strap tension, not overly tightened;

- Fit across nose bridge;

- Respirator of proper size to span distance from nose to chin;

- Tendency of respirator to slip;

- Self-observation in mirror to evaluate fit and respirator position.

Use the Fit Test form.

User Seal Check

Before conducting the negative and positive pressure checks, the subject shall be told to seat the mask on the face by moving the head from side-to-side and up and down slowly while taking in a few slow deep breaths.   The test subject shall conduct a user seal check, either the negative or positive pressure seal checks described below:

Positive Pressure Check

Close off the exhalation valve and exhale gently into the face piece. The face fit is considered satisfactory if a slight positive pressure can be built up inside the face piece without any evidence of outward leakage of air at the seal. For most respirators this method of leak testing requires the wearer to first remove the exhalation valve cover before closing off the exhalation valve and then carefully replacing it after the test.

Negative Pressure Check

Close off the inlet opening of the canister or cartridge(s) by covering with the palm of the hand(s) or by replacing the filter seal(s), inhale gently so that the face piece collapses slightly, and hold the breath for ten seconds. The design of the inlet opening of some cartridges cannot be effectively covered with the palm of the hand. The test can be performed by covering the inlet opening of the cartridge with a thin latex or nitrile glove. If the face piece remains in its slightly collapsed condition and no inward leakage of air is detected, the tightness of the respirator is considered satisfactory.

The test shall not be conducted if there is any hair growth between the skin and the face piece sealing surface, such as stubble beard growth, beard, moustache or sideburns which cross the respirator sealing surface. Any type of apparel which interferes with a satisfactory fit shall be altered or removed, including glasses.

If a test subject exhibits difficulty in breathing during the tests, she or he shall be referred to a physician or other licensed health care professional, as appropriate, to determine whether the test subject can wear a respirator while performing her or his duties.  If the employee finds the fit of the respirator unacceptable, the test subject shall be given the opportunity to select a different respirator and to be retested.

Prior to the commencement of the fit test, the test subject shall be given a description of the fit test and the test subject's responsibilities during the test procedure. The description of the process shall include a description of the test exercises that the subject will be performing. The respirator to be tested shall be worn for at least 5 minutes before the start of the fit test.

The fit test shall be performed while the test subject is wearing any applicable safety equipment that may be worn during actual respirator use which could interfere with respirator fit.

Test Exercises

Each test exercise shall be performed for one minute except for the grimace exercise which shall be performed for 15 seconds. The test subject shall be questioned by the test conductor regarding the comfort of the respirator upon completion of the protocol. If it has become unacceptable, another model of respirator shall be tried.  If due to medical or health conditions the employee cannot perform the test exercises the fit test shall not be performed and the employee not allowed to use a respirator until all elements of the fit test can be achieved.

The respirator shall not be adjusted once the fit test exercises begin. Any adjustment voids the test, and the fit test must be repeated.

The following test exercises are to be performed for all fit testing methods prescribed in this procedure:

- Normal breathing. In a normal standing position, without talking, the subject shall breathe normally.

- Deep breathing. In a normal standing position, the subject shall breathe slowly and deeply, taking caution so as not to hyperventilate.

- Turning head side to side. Standing in place, the subject shall slowly turn his/her head from side to side between the extreme positions on each side. The head shall be held at each extreme momentarily so the subject can inhale at each side.

- Moving head up and down. Standing in place, the subject shall slowly move his/her head up and down. The subject shall be instructed to inhale in the up position (i.e., when looking toward the ceiling).

- Talking. The subject shall talk out loud slowly and loud enough so as to be heard clearly by the test conductor. The subject shall read from the Rainbow Passage

Rainbow Passage

“When the sunlight strikes raindrops in the air, they act like a prism and form a rainbow. The rainbow is a division of white light into many beautiful colors. These take the shape of a long round arch, with its path high above, and its two ends apparently beyond the horizon. There is, according to legend, a boiling pot of gold at one end. People look, but no one ever finds it. When a man looks for something beyond reach, his friends say he is looking for the pot of gold at the end of the rainbow.”  Continue to read for one minute.

- Grimace. The test subject shall grimace by smiling or frowning. (This applies only to QNFT testing; it is not performed for QLFT)

- Jogging in place. The test subject shall jog in place being careful to be aware of their surroundings.

- Normal breathing. Same as exercise (1).

### Qualitative Fit Test (QLFT) Protocols

General

Toole Corp. shall ensure that persons administering QLFT are able to prepare test solutions, calibrate equipment and perform tests properly, recognize invalid tests, and ensure that test equipment is in proper working order.  Toole Corp. shall ensure that QLFT equipment is kept clean and well maintained so as to operate within the parameters for which it was designed.

Irritant Smoke (Stannic Chloride) Protocol

This qualitative fit test uses a person's response to the irritating chemicals released in the ``smoke'' produced by a stannic chloride ventilation smoke tube to detect leakage into the respirator.

General Requirements and Precautions.  The respirator to be tested shall be equipped with high efficiency particulate air (HEPA) or P100 series filter(s).

Only stannic chloride smoke tubes shall be used for this protocol.  No form of test enclosure or hood for the test subject shall be used.

The smoke can be irritating to the eyes, lungs, and nasal passages. The test conductor shall take precautions to minimize the test subject's exposure to irritant smoke. Sensitivity varies, and certain individuals may respond to a greater degree to irritant smoke. Care shall be taken when performing the sensitivity screening checks that determine whether the test subject can detect irritant smoke to use only the minimum amount of smoke necessary to elicit a response from the test subject.

The fit test shall be performed in an area with adequate ventilation to prevent exposure of the person conducting the fit test or the build-up of irritant smoke in the general atmosphere.

The person to be tested must demonstrate his or her ability to detect a weak concentration of the irritant smoke.

- The test operator shall break both ends of a ventilation smoke tube containing stannic chloride, and attach one end of the smoke tube to a low flow air pump set to deliver 200 milliliters per minute, or an aspirator squeeze bulb. The test operator shall cover the other end of the smoke tube with a short piece of tubing to prevent potential injury from the jagged end of the smoke tube.

- The test operator shall advise the test subject that the smoke can be irritating to the eyes, lungs, and nasal passages and instruct the subject to keep his/her eyes closed while the test is performed.

- The test subject shall be allowed to smell a weak concentration of the irritant smoke before the respirator is donned to become familiar with its irritating properties and to determine if he/she can detect the irritating properties of the smoke. The test operator shall carefully direct a small amount of the irritant smoke in the test subject's direction to determine that he/she can detect it.

### Irritant Smoke Fit Test Procedure

- The person being fit tested shall don the respirator without assistance, and perform the required user seal check(s).

- The test subject shall be instructed to keep his/her eyes closed if wearing a half face respirator.

- The test operator shall direct the stream of irritant smoke from the smoke tube toward the face seal area of the test subject, using the low flow pump or the squeeze bulb. The test operator shall begin at least 12 inches from the face piece and move the smoke stream around the whole perimeter of the mask. The operator shall gradually make two more passes around the perimeter of the mask, moving to within six inches of the respirator.

- If the person being tested has not had an involuntary response and/or detected the irritant smoke, proceed with the test exercises.

- The exercises identified in the Test Exercises of this procedure shall be performed by the test subject while the respirator seal is being continually challenged by the smoke, directed around the perimeter of the respirator at a distance of six inches.

- If the person being fit tested reports detecting the irritant smoke at any time, the test is failed. The person being retested must repeat the entire sensitivity check and fit test procedure.

- Each test subject passing the irritant smoke test without evidence of a response (involuntary cough, irritation) shall be given a second sensitivity screening check, with the smoke from the same smoke tube used during the fit test, once the respirator has been removed, to determine whether he/she still reacts to the smoke.  Failure to evoke a response shall void the fit test.

- If a response is produced during this second sensitivity check, then the fit test is passed.  The glass tube shall be disposed of properly.

Quantitative Fit Test (QNFT) Protocols

Using controlled negative pressure and appropriate instrumentation to measure the volumetric leak rate of a  face piece to quantify the respirator have been demonstrated to be acceptable to OSHA.

Toole Corp. shall ensure that persons administering QNFT are able to calibrate equipment and perform tests properly, recognize invalid tests, calculate fit factors properly and ensure that test equipment is in proper working order.

Toole Corp. shall ensure that QNFT equipment is kept clean, and is maintained and calibrated according to the manufacturer's instructions so as to operate at the parameters for which it was designed.

Portacount Fit Test Requirements

- Check the respirator to make sure the respirator is fitted with a high-efficiency filter and that the sampling probe and line are properly attached to the face piece.

- Instruct the person to be tested to don the respirator for five minutes before the fit test starts. This purges the ambient particles trapped inside the respirator and permits the wearer to make certain the respirator is comfortable. This individual shall already have been trained on how to wear the respirator properly.

- Check the following conditions for the adequacy of the respirator fit: Chin properly placed; Adequate strap tension, not overly tightened; Fit across nose bridge; Respirator of proper size to span distance from nose to chin; Tendency of the respirator to slip; Self-observation in a mirror to evaluate fit and respirator position.

- Have the person wearing the respirator do a user seal check. If leakage is detected, determine the cause. If leakage is from a poorly fitting face piece, try another size of the same model respirator, or another model of respirator.

- Follow the manufacturer's instructions for operating the Portacount and proceed with the test.

- The test subject shall be instructed to perform the exercises in Test Exercises section of this procedure.

- After the test exercises, the test subject shall be questioned by the test conductor regarding the comfort of the respirator upon completion of the protocol. If it has become unacceptable, another model of respirator shall be tried.

Portacount Test Instrument

The Portacount will automatically stop and calculate the overall fit factor for the entire set of exercises. The overall fit factor is what counts. The Pass or Fail message will indicate whether or not the test was successful. If the test was a Pass, the fit test is over.  Since the pass or fail criterion of the Portacount is user programmable, the test operator shall ensure that the pass or fail criterion meet the requirements for minimum respirator performance.

A record of the test needs to be sent to the Safety Manager and kept on file, assuming the fit test was successful. The record must contain the test subject's name; overall fit factor; make, model, style, and size of respirator used; and date tested.

Use, Maintenance and Care of Respirators

This section requires Toole Corp. to provide for the use, cleaning and disinfecting, storage, inspection, and repair of respirators used by employees.  Appendix B - Respirator Cleaning Procedures (Mandatory) shall be followed.

### Use

- Items that can affect the face to mask seal are prohibited.  This includes facial hair, glasses, clothing, etc.

- Each time a respirator is put on a positive and negative pressure check shall be performed.

### Cleaning and Disinfecting Requirements

Toole Corp. shall provide each respirator user with a respirator that is clean, sanitary, and in good working order. Toole Corp. shall ensure that respirators are cleaned and disinfected using the procedures in this Respiratory Protection Program, or procedures recommended by the respirator manufacturer, provided that such procedures are of equivalent effectiveness.  The respirators shall be cleaned and disinfected at the following intervals:

- Respirators issued for the exclusive use of an employee shall be cleaned and disinfected by the employee as often as necessary to be maintained in a sanitary condition,

- Respirators used in fit testing and training shall be cleaned and disinfected after each use by the Safety Manager or designated person.

- Each individual who is assigned a cartridge respirator is responsible for seeing that the respirator is cleaned, inspected and properly stored.

### Cleaning Procedures

- Remove filters, cartridges, or canisters. Disassemble face pieces by removing speaking diaphragms, demand and pressure-demand valve assemblies, hoses, or any components recommended by the manufacturer. Discard or repair any defective parts.

- Wash components in warm water with a mild detergent or with a cleaner recommended by the manufacturer. A stiff bristle (not wire) brush may be used to facilitate the removal of dirt.

- Rinse components thoroughly in clean, warm, preferably running water. Drain.

- When the cleaner used does not contain a disinfecting agent, respirator components should be immersed for two minutes in commercially available cleansers of equivalent disinfectant quality.  Another alternative is to use wipes containing alcohol that are intended for use with respirators.

- Rinse components thoroughly in clean, warm, preferably running water. Drain. The importance of thorough rinsing cannot be overemphasized. Detergents or disinfectants that dry on face pieces may result in dermatitis. In addition, some disinfectants may cause deterioration of rubber or corrosion of metal parts if not completely removed.

- Components should be hand-dried with a clean lint-free cloth or air dried.  Reassemble face piece, replacing filters, cartridges, and canisters where necessary.  Test the respirator to ensure that all components work properly.

### Storage and Inspection

- Respiratory equipment shall be stored in a manner to protect it from damage, contamination, temperature extreme, etc.

- Respiratory equipment intended for emergency use shall be stored in an area that is readily accessible and be clearly marked.

Toole Corp. shall ensure that respirators are inspected as follows:

- All respirators used in routine situations shall be inspected by the employee before each use and during cleaning;

- A check by the employee of respirator function, tightness of connections, and the condition of the various parts including, but not limited to, the face piece, head straps, valves, connecting tube, and cartridges, canisters or filters; and

- A check of elastomeric parts for pliability and signs of deterioration.

- Emergency respiratory equipment will be inspected at least monthly, and before and after each use.

- Escape only respiratory equipment will be inspected before being carried into workplace.

### Breathing Air Quality and Use

Toole Corp. shall ensure that compressed air accords with the following specifications:

- Compressed breathing air shall meet at least the requirements for Type 1-Grade D breathing air described in ANSI/Compressed Gas Association Commodity Specification for Air, G-7.1-1989, to include:

- Oxygen content (v/v) of 19.5-23.5%;

- Hydrocarbon (condensed) content of 5 milligrams per cubic meter of air or less;

- Carbon monoxide (CO) content of 10 ppm or less;

- Carbon dioxide content of 1,000 ppm or less; and

- Lack of noticeable odor.

- Toole Corp. shall ensure that oxygen is not used in compressed air units.

- Toole Corp. shall ensure that oxygen concentrations greater than 23.5% are used only in equipment designed for oxygen service or distribution.

- Toole Corp. shall ensure that cylinders used to supply breathing air to respirators meet DOT requirements and that:

- Cylinders are tested and maintained as prescribed in the Shipping Container Specification Regulations of the Department of Transportation (49 CFR part 173 and part 178);

- Cylinders of purchased breathing air have a certificate of analysis from the supplier that the breathing air meets the requirements for Type 1--Grade D breathing air; and

- The moisture content in the cylinder does not exceed a dew point of -50 deg. F (-45.6 deg. C) at 1 atmosphere pressure.

- Toole Corp. shall ensure that compressors used to supply breathing air to respirators are constructed and situated so as to:

- Prevent entry of contaminated air into the air-supply system;

- Minimize moisture content so that the dew point at 1 atmosphere pressure is 10 degrees F (5.56 deg. C) below the ambient temperature;

- Have suitable in-line air-purifying sorbent beds and filters to further ensure breathing air quality. Sorbent beds and filters shall be maintained and replaced or refurbished periodically following the manufacturer's instructions.

- Have a tag containing the most recent change date and the signature of the person authorized by Toole Corp. to perform the change. The tag shall be maintained at the compressor.

- For compressors that are not oil-lubricated, Toole Corp. shall ensure that carbon monoxide levels in the breathing air do not exceed 10 ppm.

- For oil-lubricated compressors, Toole Corp. shall use a high-temperature or carbon monoxide alarm, or both, to monitor carbon monoxide levels. If only high-temperature alarms are used, the air supply shall be monitored at intervals sufficient to prevent carbon monoxide in the breathing air from exceeding 10 ppm.

- Toole Corp. shall ensure that breathing air couplings are incompatible with outlets for nonrespirable worksite air or other gas systems. No asphyxiating substance shall be introduced into breathing air lines.

### Repairs

Toole Corp. shall ensure that respirators that fail an inspection or are otherwise found to be defective are immediately removed from service, and are discarded or repaired or adjusted in accordance with the following procedures:

- Repairs or adjustments to respirators are to be made only by persons appropriately trained to perform such operations and shall use only the respirator manufacturer's NIOSH-approved parts designed for the respirator;

- Repairs shall be made according to the manufacturer's recommendations and specifications for the type and extent of repairs to be performed; and

### Voluntary Use

If an employee chooses to voluntarily wear a respirator when not required by this Program (contaminants do not meet protection standards, odors, etc.)  they will be advised of the following in their training:

Respirators are an effective method of protection against designated hazards when properly selected and worn. Respirator use is encouraged, even when exposures are below the exposure limit, to provide an additional level of comfort and protection for employees.

However, if a respirator is used improperly or not kept clean, the respirator itself can become a hazard to the employee. Sometimes, employees may wear respirators to avoid exposures to hazards, even if the amount of hazardous substance does not exceed the limits set by OSHA standards. If your employer provides respirators for your voluntary use, of if you provide your own respirator, you need to take certain precautions to be sure that the respirator itself does not present a hazard.

You should do the following:

- Read and heed all instructions provided by the manufacturer on use, maintenance, cleaning and care, and warnings regarding the respirators limitations.

- Choose respirators certified for use to protect against the contaminant of concern. NIOSH, the National Institute for Occupational Safety and Health of the U.S. Department of Health and Human Services, certifies respirators. A label or statement of certification should appear on the respirator or respirator packaging. It will tell you what the respirator is designed for and how much it will protect you.

- Do not wear your respirator into atmospheres containing contaminants for which your respirator is not designed to protect against. For example, a respirator designed to filter dust particles will not protect you against gases, vapors, or very small solid particles of fumes or smoke.

- Keep track of your respirator so that you do not mistakenly use someone else's respirator.

Workplace Monitoring

A program of monitoring potential employee exposures has been implemented through the corporate health and safety department. Project personnel may also be assigned with the task of conducting air monitoring. Direct-reading instruments will also be used in the characterization of potential exposures. All the data collected is used to determine the appropriateness of the respiratory equipment.

Recordkeeping

Toole Corp. will establish and retain written information regarding medical evaluations, fit testing and the respirator program.  Records of medical evaluations required by this section must be retained and made available in accordance with 29 CFR 1910.1020. Toole Corp. shall provide the employee with an opportunity to discuss the questionnaire and examination results with the PLHCP.

Records will be treated confidentially and maintained on file in the Toole Corp. corporate office by the Safety Manager.

Program Evaluation

Toole Corp. shall conduct evaluations of the workplace as necessary to ensure that the provisions of the current written program are being effectively implemented and that it continues to be effective.

Toole Corp. shall regularly consult employees required to use respirators to assess the employees' views on this program’s effectiveness and to identify any problems. Any problems that are identified during this assessment shall be corrected. Factors to be assessed and verified include, but are not limited to:

- Respirator fit (including the ability to use the respirator without interfering with effective workplace performance); Appropriate respirator selection for the hazards to which the employee is exposed;

- Proper respirator use under the workplace conditions the employee encounters; and

- Proper respirator maintenance.

Training

All employees will receive respirator training during their initial health and safety training class and on at least an annual basis, if required for their job classification.  Training shall address employee knowledge of respirators, fit, use, limitations, emergency situations, wearing, fit checks, maintenance & storage, medical signs and symptoms of effective use and general requirements of the OSHA standard.   The training must be provided before requiring the employee to use the respirator.

Retraining

Retraining shall be administered annually, and when the following situations occur:

- Changes in the workplace or the type of respirator render previous training obsolete;

- Inadequacies in the employee's knowledge or use of the respirator indicate that the employee has not retained the requisite understanding or skill; or

- Any other situation arises in which retraining appears necessary to ensure safe respirator use.

Toole Corp. Qualitative Respiratory Fit Test Record Sheet

Note:  Employee Must Have Completed Respiratory Protection Training and Passed Airway Exam Prior To Fit Testing

Test Date:    __________________

Employee Name: _______________________________	SS# _________________

Test Agent:	Irritant Smoke (Stannic Chloride)

Respirator Identification:

Model:  3M 6000 Series Half Mask		Size (circle one):     Small    Medium   Large

Manufacturer:	3M Products		Approval No:   42 CFR 95

Additional Information:	Respirator must be equipped with 3M HEPA filters

Fit Test Protocol (Test Subject Initials indicate steps were performed):

___ Told to keep eyes closed during smoke exposure

___Test subject smelled irritant smoke before fit test		___Wore respirator 5 minutes before fit test

___Protocol reviewed before fit test 			___Test subject did not have hair in fitting area

___Shown how to wear respirator 				___Performed positive pressure & negative fit

___Mirror available for use by subject		      	       check successfully after seating respirator

___Must wear PPE (hard hat, etc.) if needed

Fit Test Steps (1 minute each except Grimace = 15 seconds)

___  Breath normally	 	___   Breathe deeply           		___   Turned head side to side

___  Nod up and down		___   Talking (Read Rainbow Passage)        	___   Grimace

___  Jog in place			___   Breath normally

“When the sunlight strikes raindrops in the air, they act like a prism and form a rainbow.  The rainbow is a division of white light into many beautiful colors.  These take the shape of a long round arch, with its path high above, and its two ends apparently beyond the horizon.  There is, according to legend, a boiling pot of gold at one end.  People look, but no one ever finds it.  When a man looks for something beyond his reach, his friends say he is looking for the pot of gold at the end of the rainbow”.

Fit Test Results:	_____ Pass		_____ Fail

Test Subject Signature: _____________________________ Date: ________________

Examiner's Name: ___________________  	Examiner's Signature:____________________   Date:_____________

Distribution:	Employee Local File - Toole Corp. Safety & Training Dept

SCAFFOLDS

Purpose

The purpose of this program is to prevent injuries do to falls from elevated work areas and ensure employees and contractors are able to inspect scaffolding materials and erected scaffolds.

Scope

This program is applicable at every work area where scaffolding is erected. When work is performed on a non-owned or operated site, the operator's program shall take precedence, however, this document covers Toole Corp. employees and contractors and shall be used on owned premises, or when an operator's program doesn't exist or is less stringent.

Definitions

Bearer - A horizontal member of a scaffold upon which the platform rests and which may be supported by ledgers.

Brace - A tie that holds one scaffold member in a fixed position with respect to another member.

Coupler - A tie that holds one scaffold member in a fixed position with respect to another member.

Double pole or independent pole scaffold - A scaffold supported from the base by a double row of uprights, independent of support from the walls and constructed of uprights, ledgers, horizontal platform bearers, and diagonal bracing.

Guardrail - A rail secured to uprights and erected along the exposed sides and ends of platforms.

Heavy Duty Scaffold - A scaffold designed and constructed to carry a working load not to exceed 75 pounds per square foot.

Ledger (stringer) - A horizontal scaffold member which extends from post to post and which supports the putlogs or bearer forming a tie between the posts.

Light Duty Scaffold - A scaffold designed and constructed to carry a working load not to exceed 25 pounds per square foot.

Manually Propelled Mobile Scaffold - Manually propelled mobile scaffold.

Maximum intended load - The total of all loads including the working load, the weight of the scaffold, and such other loads as may be reasonably anticipated.

Medium duty scaffold - A scaffold designed and constructed to carry a working load not to exceed 50 pounds per square foot.

Mid-Rail - A rail approximately midway between the guardrail and platform, used when required, and secured to the uprights erected along the exposed sides and ends of platforms.

Putlog - A scaffold member upon which the platform rests.

Runner - The lengthwise horizontal bracing or bearing members or both.

Definitions (construction)

Scaffold - Any temporary elevated platform and its supporting structure used for supporting workmen or materials or both.

Toe board - A barrier secured along the sides and ends of a platform, to guard against the falling of material.

Tube and coupler scaffold - An assembly consisting of tubing, which serves as posts, bearers, braces, ties, and runners, a base supporting the posts, and special couplers which serve to connect the uprights and to join the various members.

Tubular welded frame scaffold - A sectional, panel, or frame metal scaffold substantially built up of prefabricated welded sections that consist of posts and horizontal bearer with intermediate members. Panels or frames shall be braced with diagonal or cross braces.

Working Load - Load imposed by men, materials, and equipment.

Key Responsibilities

### Managers and Supervisors

- Responsible for ensuring that scaffolds are erected by a qualified person, that set up inspections are performed, and all daily inspections are performed before work starts for the day.

- Responsible for ensuring that all employees, and/or contractors have been trained in the use and inspection methods for scaffolds.

- Responsible for ensuring that all employees and contractors are aware that if an inspection discovers a defect, the scaffold cannot be used until repairs are made.

### Employees

- Responsible for following this program by inspecting the scaffolds daily and report any damages or repairs that may be needed to their supervisor.

Procedure

### General Requirements

Scaffolds shall be furnished and erected in accordance with applicable standards for persons engaged in work that cannot be done safely from the ground or from solid construction.  Except that ladders used for such work shall conform to ladder safety standards.

Scaffolds shall only be erected by a qualified third party, who is competent to certify the scaffolding safe to use.

The footing or anchorage for scaffolds shall be sound, rigid, and capable of carrying the maximum intended load without settling or displacement.  Unstable objects such as barrels, boxes, loose boards shall not be used to support scaffolds or planks.

Procedure (continued)

Scaffolds and their components shall be capable of supporting without failure at least four times the maximum intended loads.  Scaffold components must meet OSHA requirements 29 CFR 1910.28 and 29 CFR 1926.451.

Wood scaffold planks must be cross-supported every 8 feet.  Scaffold deck boards shall be cleated, wired or nailed into place.

All working levels of scaffolds will be floored completely except where internal ladders require space for ladder openings.

Scaffolds and other devices mentioned or described in this program shall be maintained in safe condition.  Scaffolds shall not be altered or moved horizontally while they are occupied.

Any scaffold damaged or weakened from any cause shall be immediately repaired and shall not be used until repairs have been completed.

Scaffolds shall not be loaded in excess of the working loads for which they are intended.

Bolts used in the construction of scaffolds shall be of adequate size and in sufficient numbers at each connection to develop the designed strength of the scaffold.

All platforms shall be overlapped (minimum 12 inches) and secured from any movement.

An access ladder or equivalent safe access shall be provided.

Scaffold planks shall extend over their end supports not less than 6 inches or more than 18 inches.

The poles, legs, or uprights of scaffolds shall be plumb, and securely and rigidly braced to prevent swaying and displacement.

Materials being hoisted onto a scaffold shall have a tag line.

Overhead protection shall be provided for workers on a scaffold exposed to overhead hazards.

Toe boards and guardrails shall be installed if a scaffold or platform is erected to a height of 6 feet or more.  Scaffolds shall be provided with a screen between the toe board and the guardrail, extending along the entire opening, consisting of No. 18 gauge wire one-half inch mesh or the equivalent, where workers are required to work or pass under the scaffolds.

Work shall not be performed on a scaffold during storms or high winds.

Work shall not be performed on scaffolds that are covered with snow or ice, unless all snow and ice has been removed and all planking has been sanded to prevent slipping.

Tools, material, and debris shall not be allowed to accumulate in quantities to cause a hazard.

### Inspections

Scaffolding shall be inspected, by a qualified person, in conjunction with the manufactures required recommendations. The Competent Person must insure scaffolds are safe prior to and during scaffold use.

Procedure (continued)

- At a minimum, the following shall be inspected after erection, before the start of the day or beginning of a shift change:

  - Ground or surface footing shall be inspected to ensure that there is no settling.

  - All main supports and cross braces shall be inspected for any signs of damage, missing pins, bolts and any locks and/or safety keepers.

  - All walking surfaces and/or planks shall be inspected for damage and proper placements and any possible movement.

  - All walkways and planks must be secure to prevent any movement.

- Inspection shall be made to ensure that the scaffold is stable and any movement is prevented.

- If during the inspection, a defect or damage to the scaffold is discovered, the scaffold shall be tagged out and use prohibited until needed repairs are made.

### Mandatory Signs and Tags

Signs and tags shall be visible at all times when work is being performed, and shall be removed or covered promptly when the hazards no longer exist.

Defective or unsafe equipment or conditions shall be tagged out by the competent person using a weather resistant tag secured to the scaffolding structure on all four sides and must be complied with.

Danger signs shall be used only where an immediate hazard exists. Danger signs must be posted around the immediate area of the scaffold, to alert other workers of possible danger from falling objects from the scaffold.

Caution Signs and/or barricade tape shall be used to mark off a larger area around scaffolding warning other workers to use caution.

### Modifications

Modification and repairs shall be performed by a qualified person, who is competent to certify the scaffolding safe to use.

Employees shall not perform any modifications or repairs, unless they have been trained and certified, failure to comply may result in disciplinary action and or termination.

Training Requirements

The supervisor shall have each employee who performs work while on a scaffold trained by a person qualified in the subject matter to recognize the hazards associated with the type of scaffold being used and to understand the procedures to control or minimize those hazards.  The training shall occur before use and include the following areas, as applicable:

- Basic safety information.

- The nature of any electrical hazards, fall hazards and falling object hazards in the work area.

- The proper use of the scaffold, and the proper handling of materials on the scaffold.

- The correct procedures for dealing with electrical hazards and for erecting, maintaining, and disassembling the fall protection systems and falling object protection systems being used.

- The maximum intended load and the load-carrying capacities of the scaffolds used.

The supervisor shall have each employee who is involved in erecting, disassembling, moving, operating, repairing, maintaining, or inspecting a scaffold trained by a competent person to recognize any hazards associated with the work in question.

- The training shall include the following topics, as applicable:

- The nature of scaffold hazards.

- The correct procedures for erecting, disassembling, moving, operating, repairing, inspecting, and maintaining the type of scaffold in use.

- The design criteria, maximum intended load-carrying capacity and intended use of the scaffold.

When the employer has reason to believe that an employee lacks the skill or understanding needed for safe work involving the erection, use or dismantling of scaffolds, the employer shall retrain each employee so that the requisite proficiency is regained.

- Retraining is required in at least the following situations:

- Where changes in scaffolding at the worksite present a hazard about which an employee has not been previously trained.

- Where changes in the types of scaffolds, fall protection, falling object protection, or other equipment present a hazard about which an employee has not been previously trained.

- Where inadequacies in an affected employee's work involving scaffolds indicate that the employee has not retained the requisite proficiency.

Tube & Coupler Scaffolds - Light Duty

| Uniformly distributed load |  | Not to exceed 25 p.s.f. |  |
| --- | --- | --- | --- |
| Post Spacing (longitudinal) |  | 10 ft. 0 in. |  |
| Post Spacing (transverse) |  | 6 ft. 0 in. |  |
| Working Levels | Additional Planked Levels |  | Maximum Height |
| 1 | 8 |  | 125 ft. |
| 2 | 4 |  | 125 ft. |
| 3 | 0 |  | 91 ft. 0 in. |

Tube & Coupler Scaffolds - Medium Duty

| Uniformly distributed load |  | Not to exceed 50 p.s.f |  |
| --- | --- | --- | --- |
| Post spacing (longitudinal) |  | 8 ft. 0 in. |  |
| Post spacing (transverse) |  | 6 ft. 0 in. |  |
| Working Levels | Additional Planked Levels |  | Maximum Height |
| 1 | 6 |  | 125 ft. |
| 2 | 0 |  | 78 ft. 0 in. |