# CORPORATE HEALTH AND SAFETY PLAN / ACCIDENT PREVENTION PLAN

SOLICITATION No. : TBD

CONTRACT No. : TBD
NAME OF PROJECT

LOCATION OF PROJECT

PRESENTED TO:			Name and address of Agency / Client

FROM:		MIAMI Environmental & Energy Solutions, LLC

1950 Roland Clarke Place, Suite 210B

Reston, VA 20191

Phone: 571-323-5674

Fax: 571-323-2102

Email: kking@mn-e.com

Table of Contents

Safety Organizational Chart	1

Name of Project - Supervisory Personnel	2

Scope of Work and Work Schedule	2

Safety Organization	2

Safety Training and Motivation	2-4

Enforcement and Discipline	4

Employee’s Protective Equipment	4-5

Equipment Inspections and Tests	5

Housekeeping and Sanitation	6

First Aid and Medical Treatment	6-7

Accident Investigation and Reporting	7-8

Specific Job Hazards	8-10

Prevention of Falls	10

Fire Protection - Prevention	11

Construction Equipment	12-13

Hand Tools and Power Tools	13

Poisonous and Harmful Substances	13

Protection of The Public	13-14

Traffic Control	14

Electrical and Lighting	14

Safe Clearance Procedures	14-15

Temporary Structures	15

Contingency and Evacuation Plan for Severe Weather	15-16

Subcontractors Participation	16

Excavations	16

Reporting Forms	16

Emergency Numbers (Hospital, Ambulance, and Doctor)	Attachment 1

Alcohol and Drug Abuse Prevention	Attachment 2

Activity Hazard Analysis	Attachment 3

Hazardous Substance Communication Program	Attachment 4

Signature Sheets	Attachment 5

Subcontractors and Suppliers	Attachment 6

Hot and Cold Stress Monitoring	Attachment 7

Safety Policy for Subcontractors	Attachment 8

Safety Training and Rules	Attachment 9

Personal Protective Equipment	Attachment 10

Hearing Conservation Program	Attachment 11

Safe Access, Fall Protection, Work Platforms	Attachment 12

Hazard Communication Program	TAB 1

Excavation; Trenching; Shoring	TAB 2

Machinery and Mechanized Equipment	TAB 3

Pressured Cylinders and Equipment	TAB 4

Blasting Plan	TAB 5

Respirator Protection Plan	TAB 6

Welding, Cutting, and Hot Work	TAB 7

Lead Program	TAB 8

Asbestos Program	TAB 9

Hand and Power Tools	TAB 10

Confined Space Entry Program	TAB 11

Hazardous Energy Control/LOCKOUT TAGOUT	TAB 12

Critical Lift/Crane and Rigging Operation	TAB 13

Severe Weather Contingency Plan	TAB 14

Access and Haul Road Plan	TAB 15

Project Name - Location

SAFETY ORGANIZATIONAL CHART

MIAMI MEES

1

- NAME OF PROJECT - SUPERVISORY PERSONNEL

- Contract Number:    TBD

B. Personnel:	Program Manager – Harry Brown

Site Superintendent/SSHO/CQC Manager (TBD)

- SCOPE OF WORK AND WORK SCHEDULE

  - Perform all work identified in the Contract Proposal. (TBD)

  - Determined by individual Task Order. (TBD)

  - Number of employees: Variable

  - Scope:  This site-specific Health and Safety Plan (HSP) has been written for use by MIAMI MEES, LLC and its subcontractors only.  MIAMI MEES, LLC claims no responsibility for its use by others unless that use has been specified and defined in project or contract documents. The plan is written for the specific site conditions and identified scope(s) of work in the Contract Proposal and must be amended in writing if those conditions or scope(s) of work change.

- SAFETY ORGANIZATION

The SAFETY OFFICER will be directly responsible for the safety program. Foremen will, at all times, be obligated to assist the SAFETY OFFICER in fulfilling the safety program. The PROJECT SUPERINTENDENT will assist in preparation of reports of injury and for regular reports.

  - The insurance company carrying the liability for injuries on the job site may visit the job intermittently and give suggestions for avoiding injuries to contractor's personnel. If desired, copies of any reports will be sent to the owner. All employees, prior to starting work, will be given safety indoctrination by the Quality Control Manager or someone designated by the Project Manager who is familiar with the safety program. He will be told what to do when he is injured, where to find first aid supplies, and who will attend to his needs. The orientation will be given to each employee before he does any work. The importance of reporting all accidents and having first aid treatment will be stressed to each employee. A copy of the orientation certification will be on file at the contractor's office. Orientation will be in accordance with EM385-1-1, Paragraph 01.B, Indoctrination and Training.

  - This safety program will comply with OSHA 29 CFR Pt 1910, and Pt 1926, and Corps of Engineers Standards, EM385-1-1.

- SAFETY TRAINING AND MOTIVATION

A meeting between Project Superintendent and foremen will be held at least once per month to discuss safety, company policy on safety and observations on past

2

experience. Company officials will participate. The Client Representative shall be

notified of these meetings so they may attend or speak if they so elect.

  - On the first workday of each week, prior to beginning work, a group safety meeting will be held at a designated site. Attendance by all employees and subcontractors on site is mandatory. The Safety Officer or Project Superintendent shall conduct the meeting. There will be a different safety topic each week, in addition to a site-specific overview of hazards, emergency contacts, and emergency action procedures (EAP).  After the specified topic has been discussed, the meeting will be opened for questions, comments observations, and discussion from all attendees. The open session is intended to involve all employees and subcontractors in direct participation in project safety. The topics may vary from safety history to specific reminders of possible hazards in ongoing and future work. When the meeting is finished, all attendees are required to sign a roster indicating their attendance. This document will be kept on file at the contractor’s office.

  - Every employee will receive an opportunity for training in the form of safety meeting and literature during progress of the work. After each employee injury, where lost time is involved, a special safety meeting will be held to discuss the injury and to determine how it could have been avoided.  In addition, after each mishap or near miss where injuries or lost time is not involved a special safety meeting as described above will be held. Each foreman who has workmen assigned to him will have a weekly safety meeting at his toolbox location or if two or more foremen are working close together, the meeting will be combined. All employees and subcontractors will be required to attend these meetings.

  - A review of the past week's occurrences will be made and ways of avoiding similar injuries in the future will be discussed. Any hazardous situations anticipated for future work will be discussed with a goal of preventing any injuries or accidents that might occur. Employees at all safety meetings will be encouraged to participate in the discussion and urged to speak up if they have suggestions to make a possible method of involving employees and subcontractors, is to make Activity Hazard Analysis of future work a meeting topic. This will directly involve them and get them thinking about potential hazards and safety.  If a special safety condition is required in a certain work area, this work area and working condition will be discussed with the superintendent, foreman and crew working in this particular area each morning at their morning gang box safety review.    Ordinarily, the foremen will be in charge of the safety meetings, unless we have visitors representing the contractor.  Representatives of the owner will be welcomed.

  - New- or temporary/short term-employee site specific indoctrination will be given to each employee or visitor to the site, regardless of duration of visit.  The Project Superintendent will provide this orientation.

3

- Mandatory training and certifications for this project are:

    - Appropriate License for commercial truck operation

    - First Aid and CPR-certified person on each shift

    - OSHA-trained and certified safety officer

- ENFORCEMENT AND DISCIPLINE

  - If an employee fails to heed the safety indoctrination given him, he will be reprimanded in writing on the first and second occurrences. On the third occurrence, he will be discharged with the reason given in writing. The foreman should initiate the employee's discharge through the superintendent and timekeeper.

  - The Safety Officer shall be in charge of enforcing the safety program. Delegating some of the responsibility to the foremen on the job will be his prerogative.

  - The Safety Officer will be responsible for coordination of subcontractor's safety program.

  - If a Life-Threatening condition exists, or any other condition with which employee(s) are uncomfortable/unfamiliar, work shall be stopped until condition is corrected. Employee(s) may enact “Stop Work Authority” without fear of reprimand or retaliation.

  - The Safety Officer will provide immediate notification of major accidents by phone.

- EMPLOYEE’S PROTECTIVE EQUIPMENT

  - It will be the superintendent's responsibility and duty, or anyone designated by him, to see that employees have AND wear required protective equipment. Type A or B Hard Hats will be worn everywhere and at all times on the job site. The hardhat bill will face forward.  As a minimum, leather boots or shoes, in good condition, will be mandatory at all times. The wearing of "TENNIS SHOES" WILL NOT be acceptable. Hearing protection, safety glasses, goggles, electric rubber gloves, ear plugs and full-face shields, and other specialty PPE will be worn as the hazard dictates and the use of such PPE will be enforced by the project manager, project superintendent, corporate safety officer or his representative, and foremen on site. Trouser-length pants in good repair are mandatory. Shirts with sleeves are minimum required wear for upper body, and will be free of any derogatory messages including alcohol or substance abuse slogans, weapons, or anatomical references.

  - Protective footwear, such as rubber boots, protective covers, ice clamp-ons and steel-toed safety boots in accordance with ANSI F2413, shall be worn by all persons exposed to hazards to the feet (including, but not limited to impact,

4

puncture, slipping, electrical or chemical hazards).  Activity Hazard Analyses

developed for each segment of the work will include activities and specific protective measures to be enforced for foot hazards.

  - For all activities in which C.O.E., Contractor personnel or official visitors are potentially exposed to foot hazard, the applicable job/activity hazard analysis, accident prevention plan, or project safety plans shall include an analysis of, and prescribe specific protective measures to be enforced for foot hazards.

  - Footwear providing protection against impact and compressive forces, conduction hazards, electrical hazards, and sole puncture SHALL MEET the applicable requirements of ANSI F2413.

  - When respiratory protective equipment is required (Confined Space Entry, or any other example of a respiratory hazard exists), a respiratory protection program shall be developed and implemented. Respiratory equipment specified shall be selected, provided, fit tested, used, maintained, stored and personnel shall be medically evaluated by a Licensed Health Care Provider, properly trained in accordance with the requirements contained in the latest edition of ANSI Z88.2, and the Joint NIOSH/OSHA (29 CFR 1910.134 and 29 CFR 1926.103) Standard Completion Program Respirator Decision Logic and Appendix N BEFORE the use of any respirator is authorized/allowed. In the event of conflicts between these and referenced standards or regulations, the more stringent shall prevail.

  - Protection against the effects of noise exposure shall be provided for all employees whenever the sound level exceeds 85 dB continuous or 140 dB impulse, regardless of the duration of exposure. A dB meter will be used whenever sound levels are questionable. Earplugs and/or muffs will be provided.

- EQUIPMENT INSPECTIONS AND TESTS

  - Equipment will not be operated on the jobsite until it has been inspected, approved, and recorded on Form 1192. Thereafter, daily and pre-shift inspections will be made by the operators of the equipment, by the maintenance mechanic and supervisory personnel. Inspections will be made at the job site. Commercial equipment shops and agencies will be utilized to keep the equipment in running order.  Most needed repairs will be made at the job site.

  - It will be the duty and responsibility of the superintendent to have eye examinations for such operators as required by regulations, typically ASME B30.2-2001, Overhead and Gantry Cranes. This requirement applies only to crane and dragline operators at this time.

  - All operators will be tested and certified prior to operating equipment.

  - All operators of hoisting equipment used for hoisting personnel shall have an annual physical examination by a licensed physician in accordance with EM385-1-1

5

- HOUSEKEEPING AND SANITATION

  - It is our policy to clean up the jobsite daily, and to keep the offices, grounds and work areas clean of rubbish and trash. Waste containers will be located at strategic points and at water cans to receive waste materials. The contents of waste containers will be disposed of satisfactorily.

  - Flush type water closets or chemical toilets will be maintained for the workers at the job site. These will be serviced at regular periods, or as often as necessary. Toilets will be available for each 20 employees and as contractually required.

  - Potable drinking water will be obtained and furnished for all personnel. If the water is obtained from a source outside of municipal water supply, or which is not regularly tested, the contractor will have physical, chemical and bacteriological test made before using any of the water. If there is no suitable water obtainable at the job site, arrangements will be made to transport water from a suitable source. Water will be served from covered vessels provided with faucets and single serve, paper drinking cups. Trash cans will be placed at each water station.  The vessels will be covered at all times, except for filling and adding ice. Salt tablets and other heat stress mitigation measures will be available on site as environmental conditions warrant.

- FIRST AID AND MEDICAL TREATMENT

  - First aid will be available at all times. One sixteen-unit first aid station will be provided for each 25 employees on the job site. The station will be in accordance with ANSI Z308.1-1978A.  A first aid log will be maintained for all treatment not otherwise reported. When, in terms of time and distance, a medical facility or physician is not reasonably accessible to a group of two or more employees for the treatment of injuries, at least one employee on each shift shall be qualified to administer first aid and CPR.

In the event it is necessary to take an injured patient to a hospital, he will be transported by a designated passenger vehicle, the use of which is limited exclusively to that purpose of transporting injured personnel. An ambulance will be brought in to transport injured employees to the hospital when necessary.

A supervisor will accompany all injured personnel to the hospital. The names and location of these facilities will be made available for emergency purposes ONLY.

In the event of an employee being sent to a doctor for treatment, a release will be obtained from the doctor on date of treatment stating either (1)

employee not fit for duty; (2) employee fit for light duty; or (3) employee fit for

duty.  A copy of this release will accompany the accident report.

EMERGENCY telephone numbers and reporting instructions for ambulance,

physician, hospital, fire and police shall be posted on the project bulletin

board, adjacent to any landline telephones, and in conspicuous locations

6

across the job site. These numbers are listed in Supplement No. 1.

  - As stated elsewhere in this program, each employee before performing any work will be given a safety indoctrination which will include information about where and when he can receive first aid.

- ACCIDENT INVESTIGATION AND REPORTING

  - The cause of all accidents will be investigated and the responsibility placed therefore. A safety meeting and taproot investigation of the incident will be held for affected employees, Safety Officer, Project Manager, and Project Superintendent and corrective measures will be taken to prevent a recurrence of the same accident or one of like nature. If findings indicate negligence on the behalf of any employee personally, the employee will be given a chance to explain his actions and if found unsatisfactory, the employee may be subject to disciplinary action up to and including termination.

  - The Safety Officer will prepare required paperwork involved as a result of an accident. All accidents and property damage will be reported using forms attached. The complete investigation of the accident will be the responsibility of the Safety Officer, however, at his discretion; he may appoint a representative to conduct the investigation.  ALL incidents/accidents will be reported using the attached OSHA 300/301(series) forms.

  - After the accident report has been prepared and ready for transmittal, it will be reviewed and signed by the superintendent. In some cases, an officer of the company may review the accident report.

  - In compliance with the general provisions of the contract reportable accidents occurring on contract operations will be investigated by the contractor and reported to the C.O.E. Representative.       All reports will be forwarded to the

C.O.E. Representative within 24 hours. The inspector at the site or the owner's representative will assist the contractor when necessary to assure that the accident report is accurate, complete, and timely.

A REPORTABLE ACCIDENT IS

- Any accident that results in the following:

  - Any lost time accident (day(s) away from work, restricted duty or transfer to another position), any injury or illness suffered at or created by the workplace that requires medical attention from a Licensed Health Care

Provider (even if it doesn’t result in lost time), any fatality, three or more people admitted to the hospital, loss of consciousness (regardless of duration), amputation, loss of eye/eyesight, or any other significant accident, injury, or illness.

  - Property damage in excess of $ 700.00 to Government or Contractor's

property.

7

  - Any	property	damage	to	private	property	shall	be	reported immediately.

  - Any injury to non-Government or non-Contractor personnel.

  - Non-lost time accidents will be reported within 24 hours.

  - ALL first aid-only accidents will be reported only to local supervisors immediately.

- Damage of $700.00 or more to contractor property when resulting from contractor activities or when occasioned by fire, flood, earthquake, navigation, wind, ice, mechanical or structural failure, or similar circumstances, including motor and marine vehicle accidents.

- It is essential that all accidents be investigated promptly and all pertinent facts of each accident be carefully considered. The principle purpose of an accident investigation is to obtain information through which recommendations for corrective action can be developed.

- This information will be used for the prevention of similar or other accidents in the area affected. All accidents are potentially serious; therefore, regardless of the degree of seriousness of any injury or property damage, each accident must be investigated. Accidents must be investigated as soon as possible after the occurrence. Delays, even those of a few hours, can permit important information to be removed, destroyed or forgotten.

- The contractor will also furnish the project supervisor with a completed copy of the insurance carrier's standard form, First Report of Injury within three (3) days following an accident.

- Direct OSHA Reporting criteria is as follows:

  - Any fatality- report to OSHA via telephone or in person within 8 hours

  - Hospitalization of 3 or more employees-report to OSHA via telephone or in person within 8 hours

  - In-patient hospitalization of one or more employees or an employee's amputation or an employee's loss of an eye- report to OSHA via telephone or in person within 24 hours

  - OSHA telephone: 1-800-321-OSHA (1-800-321-6742).

  - New Jersey Public Employees Occupational Safety and Health (PEOSH): (609) 984-1863.

- SPECIFIC JOB HAZARDS

In addition to the general rules governing heavy construction operations in the contract specifications and in the OSH Act, 29 CFR 1926, the following additional rules will be enforced:

8

GENERAL

  - The Safety Officer will inspect all tools and equipment before being placed into use.  Daily inspections of tools and equipment may be conducted by the Project Superintendent thereafter. Defective tools and equipment will be taken out of service immediately, and either replaced (preferred) or repaired by a qualified repair person (electrician, authorized repair facility, etc.) and re- inspected by the Safety Officer before being returned to service. Tools and equipment will be repaired and maintained by qualified mechanics.  All portable power tools will be equipped with proper guards and electrically grounded.

  - All mechanical equipment will be provided with substantially constructed canopies, shield or guards for the protection for the operator.

  - All persons will be required to stand clear of moving machines, equipment, equipment and lines when in strain.  A qualified signalman will be provided where deemed necessary by the superintendent for the protection of personnel working nearby, to signal operators or mechanized equipment.

  - Each foreman will hold a "tool box" safety meeting for his entire crew at least once each week. Such safety meetings will not be less than ten minutes in length.

  - New employees will be instructed in company safety policies and basic job safety requirements before beginning work.

  - All persons will be required to wear Type A or B hard hats.

  - Operations will be organized so that crews are dispersed in such a manner that operations of one crew will not present a hazard to members of nearby crews.

  - All obstructions or hazards that will interfere with construction operations will be removed before work begins.

  - Climbing while carrying unguarded sharp edged tools is prohibited.  3 points of contact will be in place during climbing or working at heights.  Appropriate fall protection equipment will be used while working at heights.

  - Axes, brush hooks, saws and other cutting tools will be kept out of the path of workmen.

  - Working in or on trees during high winds is prohibited.  Additional precautions for lifting and rigging during periods of high winds will be addressed on an “as-needed” basis, depending on environmental conditions.

  - When using chain saws, the chain will not be started until the saw is in cutting position. Hard-hats, gloves, eye protection and face shields, hearing protection and logger's chaps will be worn while operating chain saws.

9

  - Burning operations, if necessary, will be kept under strict control and will not be left unattended, except where express written permission of the C.O.E. Representative has been obtained. Contractor will provide suitable fire protection

equipment and measures.

  - Firing, punching and placing of material for burning will be done from the windward side.

  - The use of flammable liquids such as gasoline, kerosene, fuel oil number 1 and fuel oil number 2 with a flash point less than 125 degrees in burning operations will be prohibited. Fuel oil numbers 3,4,5,6 and other higher flash point liquids only will be used.

  - The itemized requirements for the handling of hot substances and equipment to be used will be in accordance with OSHA rules.

  - Nails will be removed or bent over safely from all scrap lumber.

  - All stairways, passageways, gangways, and access ways will be kept clear of supplies, materials, and obstructions at all times.

  - All welding, cutting, and grinding of machinery will be in accordance with EM385-1-1 and OSH Act 29 CFR Part 1926, Subpart J standards.

  - Safety lashing shall be provided at connection between tool and hose, and at all quick makeup type connections.

  - The use of powder-actuated tools will only be by manufacturer-trained personnel who have been certified in their safe and proper use.

- PREVENTION OF FALLS

  - Approved barricades will be furnished and installed around shafts and excavations when and where required.

  - Danger signs will be placed at sliding banks and other hazardous locations.

  - Scaffolds shall be furnished and erected in accordance with OSH Act 29 CFR, Part 1926, Subpart L, and EM385-1-1.

  - Guard Rails will be constructed as per OSH Act 29 CFR, Part 1926, Subpart M Standards and EM385-1-1.

  - Where falling hazards exist from building tops or other structures, prevention measures such as safety belts & lanyards, guardrails, scaffolding, or a combination shall be implemented.

10

- FIRE PROTECTION - PREVENTION

Approved type and size fire extinguisher will be in the equipment repair shop and

fuel service areas. They will likewise be maintained in the field office and storage areas. One 5-B-C fire extinguisher will be located on each piece of construction equipment. Gasoline or other liquid having a flash point of 100 degrees and above will not be used in lieu of approved cleaning fluids.  A fire extinguisher, rated not less than 2A, shall be provided for each 3,000 square feet of the protected building area, or major fraction thereof. Travel distance from any point of the protected area to the nearest fire extinguisher shall not exceed 100 feet.

Fuel storage dispensing tanks will be properly marked as to contents. "NO SMOKING" signs will be posted at each tank in plain view. All fuel tanks, including moveable tanks on skids, will be kept grounded and bonded (multiple tanks) when not being moved. All portable containers and safety cans used for hand transporting of flammable fuels or liquids will meet the specifications of OSHA 29 CFR, Part 1910, Subpart 106 or EM385-1-1 safety standards. When applicable, flammable fuel or liquid storage tanks shall be located a minimum of 20 feet away from buildings. A fire extinguisher, rated not less than 10B, shall be provided within 50 feet of wherever more than 5 gallons of flammable or combustible liquids or 5 pounds of flammable gas are being used on the jobsite. This requirement does not apply to the integral fuel tanks of motor vehicles. Two or more portable tanks, grouped together, having a combined capacity in excess of 2,200 gallons, shall be separated by a 5-foot-clear area. Individual portable tanks exceeding 1,100 gallons shall be separated by a 5-foot-clear area.  Within 200 feet of each portable tank, there shall be a 12-foot-wide access way to permit approach of fire control apparatus.  Storage areas shall be kept free of weeds, debris, and other combustible material not necessary to the storage.

11

- CONSTRUCTION EQUIPMENT

No construction equipment will be placed in operation before the designated inspector has inspected the equipment and approved it for use. This applies to subcontractors as well as the prime contractor. During the operation, if the inspector requests use of equipment be suspended due to needed repairs or changes, the equipment will be repaired as required and placed in operation again when the inspector so approves. Such items as steel cable, operating mechanisms, tracks, tires, etc. will be inspected daily either by the operator or in such a way as not to endanger other employees. Installation of boom stops, as required, will be one of the items to be taken care of prior to placing the equipment in operation. Cranes will be load tested to safely perform designated work.  Backup horns will be used on all equipment where required.

No lifting rigs and aerial equipment such as cranes, dragline, man-lift/man-basket, cherry picker, etc., will be operated within 10 feet of power lines.

  - All load drums on loading-hoisting equipment shall be equipped with at least one positive holding device. This device should be applied directly to the motor shaft or some part of the gear train. It is not necessary that the positive holding device utilize shearing of metal to meet this requirement. Friction surfaces are acceptable.

  - During personnel handling operations load and boom hoist drum brakes, swing brakes, and locking devices such as pawls or dogs shall be engaged when the occupied platform is in a stationary working position.

  - During personnel handling operations the load hoist drum shall have a system or device on the power train other than the load hoist brake, which regulates the lowering rate of speed of the hoist mechanism (controlled load lowering). Free fall is prohibited.

  - All telescopic boom cranes engaged in standard lift operations (including concrete bucket) should be equipped with a two-block warning feature, a two- block damage prevention feature, or an anti-two-block device for all points of two-blocking (i.e., jibs, extension, etc.). In addition, all new telescopic boom cranes shall be equipped with an anti-two-blocking device or a two-block damage prevention feature for all points of two-blocking. Cranes that are used exclusively as duty cycle machines (clamshell, dragline, grapple, pile driving operations) are exempt from this requirement.

  - All lattice boom cranes engaged in standard lift crane operations (including

concrete bucket) should be equipped with a two-block warning feature which

functions for all points of two-blocking. Cranes that are used exclusively as duty

cycle machines (clamshell, dragline, grapple, pile driving operations) are

12

exempt from this requirement but will meet the requirements of ANSI/ASME- B30.5-1982 (as revised). In all cases where cranes are utilized without these safeguards, equivalent protection shall be established and documented and then approved by the designated authority.

  - During personnel handling operations all telescopic and lattice boom cranes shall be equipped with a device which when activated disengages all functions whose movement can cause contact between the load block or overhaul ball and the boom tip (anti-two-block device) or a system shall be used which deactivates the hoisting action before damage occurs in the event of a two-blocking situation (two-block damage prevention feature).  The device or system must be installed for all points of two-blocking (i.e., jib or boom points) and in the case of the anti-two-block device the crane must be equipped with automatic brakes on each hoist line; hoist lines not so equipped must be taken out of service while personnel lifts are being made.

  - All ropes, slings and chains shall be inspected prior to their use each day and as necessary during their use by a competent craftsman for suitability, excessive wear or damage, and current working load certification. All hooks used to support personnel loads or loads that pass over personnel will have safety latches.

  - All critical, compound or heavy lifts (75% of rated capacity) will not be made until Project Manager, Project Superintendent, Safety Officer, and owner representative has approved the method.

- HAND TOOLS AND POWER TOOLS

The itemized requirements for installing and operating hand tools and portable power tools will be in accordance with OSHA 29 CFR Part 1926, Subpart I regulations as well as any other local requirements that may be deemed appropriate for safety.

- POISONOUS AND HARMFUL SUBSTANCES

Poisonous and harmful substances will be handled and stored in accordance with OSHA 29 CFR Part 1926, Subpart D along with owner’s safety regulations.

A copy of Material Safety Data Sheets (MSDS) for hazardous materials and substances on site shall be provided to the Government Representative and will be available to all workers.

- PROTECTION OF THE PUBLIC

The public will be protected from physical harm in so far as possible by aid of warning signs and signals, developed as the work progresses and the need

13

appears. One large sign will be located at the entrance of the work area stating that

all visitors are prohibited from continuing further into the work area. Authorization can only be obtained from the project office for outside people who have business in the work area.   They will be accompanied by an employee or will not be allowed near an excavation or in the area where heavy equipment is being operated. Local authorities will be contacted to determine requirements for use of public roads, right of ways, easements and noise and dust abatement. Hardhat areas shall be clearly marked with signs. Temporary gates, fencing or barricades will be used as necessary for protection and control of the site.  Unauthorized persons will not be allowed to enter the work site.  Authorized persons may enter after check-in with the Project Superintendent and reviewing site specific hazards.

- TRAFFIC CONTROL

Detailed plans for traffic control will be submitted as necessary for all roads, highway intersections, railroads, utilities, ramps, bridges, restricted areas, etc. The plans will include proposed road widths, travel directions, road markings, sign posts, curves, grades and sign distances.

- ELECTRICAL AND LIGHTING

No night operations will be performed without proper lighting. Portable light plants will be used when it is not practical to use stationary floodlights. Night lighting will be grounded.

ELECTRICAL WORK PROTECTION:

- Make sure power is off before installing equipment.  The use of Lockout/Tagout will be enforced at all times to isolate hazardous energy and electrical power sources.

- All power tools will be grounded and ground fault receptacle outlets will be used.

- Only qualified personnel will be allowed to perform electrical work.

- SAFE CLEARANCE PROCEDURES

In construction and maintenance equipment check-out, assume that all equipment is under power and implement these procedures before attempting to work on the equipment.

  - The superintendent, General Foreman or Foreman of any craft wishing to have equipment "Locked-Out" will contact the Safety Officer and a lock and tag will be installed on the equipment switch or gear with both men present. The danger tag will bear the signatures of the Safety Officer and the Foreman/Authorized person requesting the lock-out.  Affected persons will be notified of the lockout/tagout.

  - To unlock and remove a tag from the switch or gear, the craft Foreman or

14

Authorized person must contact the Safety Officer. The craft Foreman/Authorized person will remove his signature from the tag in the presence of the Safety Officer

who will then remove his own signature and the tag and lock from the switch or gear.

  - In the case where more than one trade requests a lock-out on the same piece of equipment, the same procedure is to be followed with each craft Foreman/Authorized person signing the tag in the presence of the Safety Officer.   The Safety Officer will not unlock any switch until each Foreman has removed their signature in his presence.

  - These same procedures will be applied when lock-out is needed on any types of work or in the control of the release of any hazardous energy, including: electrical equipment or lines, mechanical equipment, pressure systems, and vessels and lines or equipment containing dangerous activated or released remotely or inadvertently.

- TEMPORARY STRUCTURES

All subcontractor temporary buildings and facilities shall have their location approved by Prime Contractor and anchoring shall be inspected and approved by the Safety Officer.

Trailers or other temporary structures used as field offices, personnel housing or storage building shall be anchored with rods and cable or steel straps to ground anchors. The anchor system shall be designed to withstand winds and will be supplied by the manufacturer, as part of his standard design, whenever possible. The anchor system will meet applicable state and local requirements for the structure being anchored.

- CONTINGENCY AND EVACUATION PLAN FOR SEVERE WEATHER

Precautions against the hazards of inclement weather can be accomplished in four ways:

- Move materials, equipment, and personnel.

- Shelter equipment, material, and personnel.

- Counter the effects of weather.

- Equip to offset weather hazards.

These four precautions can be applied according to the condition:

High Winds

- Move personnel off of high workstations.

- Secure ALL aerial work or lifting processes.

- Shelter materials, which could be damaged by wind.

15

- Counter winds by bracing new walls.

- Equip work platforms with plywood windbreaks.

Precipitation

- Move personnel off of high workstations, materials to shelter.

- Shelter materials with plastic, canvas, or store them inside.

- Counter runoff with drainage swales.

- Equip walkways with non-slip treading.

Low Temperatures

- Move personnel to most sheltered work areas possible.

- Cover material / work, which will freeze, cover walkways.

- Use portable heaters, remove ice from walkways.

- Equip walkways with non-slip tread.

- During extreme low temperatures, put in place max/min time allowances for work/warming periods.

Floods

- Move material, personnel, and equipment to high ground.

- Counter with drainage ditches.

- Equip project with communication system.

- SUBCONTRACTORS PARTICIPATION

Any subcontractor engaged in work under this contract will be furnished with copies of this accident prevention program and will be required to abide by all of the Safety stipulations contained therein. They will be required to conform to the same contract requirements in performance of their work as the prime contractor. Weekly progress meetings will be held with all subcontractors to coordinate jobsite activities and maintain a safe working environment.

- EXCAVATIONS

All excavations shall conform to subpart P "Excavations, Trenching, and Shoring" of the OSHA 29 CFR Parts 1926/1910) and EM385-1-1.  These are minimum standards and on site conditions may dictate additional safe guards.

16

- REPORTING FORMS

SUPERVISOR'S ACCIDENT INVESTIGATION

MIAMI MEES

Contract No: FA448420C0006

WHEN:

ACCIDENT DATE: 	TIME:	 REPORT TO SUPERVISOR OR FIRST AID DELAYED? YES [   ] NO [  }

IF YES, WHY?

SPEAK ENGLISH? 	MARRIED? 	MINOR CHILDREN:

WHO:

INJURED PERSON:

OCCUPATION:

HOW LONG AT THIS JOB? 	LENGTH OF EMPLOYMENT: 	AGE:

INJURY/LOSS:

NATURE/EXTENT OF INJURIES OR PROPERTY DAMAGE:

WHAT IS ACTUAL OR EXPECTED LENGTH OF DISABILITY:  	  CHECK ONE:  HOSPITAL CARE: [  ] DOCTORS CARE: [   ] JOB SITE FIRST AID: [  ]

WHERE:

EXACT LOCATION WHERE ACCIDENT OCCURRED:

FOREMAN: 	WITNESSES:

WHAT:

TYPE OF ACCIDENT (see next page-enter #) 	WAS EMPLOYEE DOING SOMETHING OTHER THAN REQUIRED DUTIES AT TIME OF ACCIDENT?

IF SO WHAT AND WHY:

DESCRIPTION OF ACCIDENT (Detail of what employee was doing and what physical objects, tools, machines, structures or equipment were involved)

WHY:

CHECK ACCIDENT CAUSES ON NEXT PAGE OF THIS REPORT AND COMMENT FULLY HERE

PREVENTION:

WHAT SHOULD BE DONE AND BY WHOM TO PREVENT RECURRENCE OF THIS TYPE OF ACCIDENT?

WHAT ACTION ARE YOU TAKING TO SEE THAT THIS IS DONE?

SUPERVISOR'S SIGNATURE:  	DATE OF THIS REPORT: _____________________________________________________

ALL ACCIDENT MUST BE REPORTED TO THE MAIN OFFICE. THIS FORM MUST BE COMPLETED AND SENT TO THE MAIN OFFICE NO LATER THAN 12:00 NOON THE DAY FOLLOWING THE ACCIDENT.  REPORT IMMEDIATELY WHEN AN EMPLOYEE RETURNS TO WORK AFTER LOSING TIME.

##### TYPE OF ACCIDENT

INJURY	PROPERTY DAMAGE

1.) Fall from elevation; Fire or explosion 2.) Fall same level; Collapse

3.) Struck by; Rupture or bursting

4.) Caught in, under or between; Collision or overturn 5.) Overexertion; Other (describe)

Push, Pull, Lift, Lower, Carry, Hold 6.) Electrical contact

7.) Fumes, dust, gas, caustics, etc. 8.) Motor vehicle

9.) Other (describe)

ACCIDENT CAUSE ANALYSIS

ENVIRONMENTAL

1.) INADEQUATE SAFEGUARDS

Lack of handling or safety devices; unguarded machinery

2.) IMPROPER OR DEFECTIVE EQUIPMENT

Poorly maintained equipment: worn, cracked, broken, rough, or slippery agencies.

3.) HAZARDS OF LOCATION

Poor layout; congestion; insufficient storage; poor lighting; poor footing; heights; poor guardrails.

4.) POOR HOUSKEEPING

Improper piling or placing; clutter; spillage or breakage 5.) OTHERWISE CLASSIFIED

PERSONAL

1.)  BODILY CONDITIONS

Overweight; emotional upset; fatigue; intoxication; illness; age; poor eyesight; lack of strength; other physical handicaps, etc.

2.)  LACK OF SKILL OR KNOWLEDGE

Improperly trained; inexperienced; unaware, etc.

3.)  ADEQUATE SKILL/KNOWLEDGE BUT LACK OF EXECUTION

Chance taking; failure to use or unauthorized use of safety equipment

4.)  IMPROPER APPAREL

Rundown shoes; lack of personal protective gear; loose sleeves; torn clothing

5.)  NOT OTHERWISE CLASSIFIED

##### WEEKLY SAFETY MEETING ROSTER MIAMI MEES

Contract No. FA448420C0006

FOR THE WEEK OF:

MEETING TOPIC:

ATTENDEES: (Please Print Legibly and Indicate Company)

13.

1.

2.

3.

4.

5.

6.

7.

8.

9.

10.

11.

12.

14.

15.

16.

17.

18.

19.

20.

21.

22.

23.	 24.

COMMENTS/SUGGESTIONS:

SAFETY OFFICER

Attachment 1

EMERGENCY NUMBERS

# HOSPITAL DIAL - 911

TBD

Name, address, and Phone No.

Of closest Hospital to Job Site

AMBULANCE DIAL 911

TBD

POLICE DIAL - 911

TBD

FIRE

DIAL - 911

TBD

# Attachment 2

# ALCOHOL AND DRUG ABUSE PREVENTION PROGRAM

For A

DRUG FREE WORKPLACE

It is the policy of MIAMI MEES, to provide a work environment that is free of alcohol, illegal drugs, and controlled substances. Illegal drugs include marijuana and all other drugs not prescribed by a licensed physician for use by the person possessing them. Unlawful manufacture, distribution, dispensing, possession or use of a controlled substance on the construction site is prohibited and will not be tolerated. As a further precaution, entry into or upon any vehicle, office or other work location of the company is conditioned upon the company's right to search the person and personal effects of any entrant for illegal drugs.  When appropriate, such items discovered through these searches may be taken into custody and may be turned over to the proper law enforcement authorities. Effective January 1, 2005, all applicants for employment and employees of MIAMI MEES and all subcontractors may, as a condition of employment or continued employment, be required to submit to a drug screening test at the discretion of the company. At our discretion, drug/alcohol screening will be performed on all employees directly involved in job site accidents, mishaps and near misses by their actions or lack of action whether injuries requiring medical attention occurred or not.

This program is designed to notify, identify and provide penalties for those employees and applicants who, by the possession, use, or sale of an illegal drug or controlled substance, pose unnecessary and unacceptable risk to a safe, healthful and efficient operation.

We will encourage employees who are in violation of our policy to satisfactorily participate in drug abuse assistance or rehabilitation program approved for such purposes by a federal, state, or local health, law enforcement, or other appropriate agency. Refusal to submit to a drug/alcohol screening or search will be cause for disciplinary action, including termination of employment.

All employees are instructed to notify their employer of any criminal drug statute conviction for a violation occurring in the workplace no later than five (5) days after such conviction. MIAMI MEES will notify the contracting officer within ten (10) days after receiving notice from an employee or otherwise receiving actual notice of such conviction.

I HAVE READ, UNDERSTAND AND AGREE TO ABIDE BY THIS POLICY.

Print full name:

Signature:

Date:

Supervisor's Signature:

# Attachment 3

# ACTIVITY HAZARD ANALYSIS

##### PREPARATORY INSPECTION CHECKLIST - MIAMI MEES

Contract No.: FA448420C0006

DATE:

DEFINABLE SEGMENT OF WORK:

SPECIFICATION SECTION:

CONTRACTOR'S/SUBCONTRACTOR'S INSURANCE Certificates on file and in order? YES [	] NO [	]

SHOP DRAWINGS / TRANSMITTALS INVOLVED

NUMBER AND ITEM CODE: APPROVAL DATE

1.

2.

3.

4.

5.

6.

ITEMS NOT YET APPROVED

ITEM STATUS

1.

2.

3.

4.

5.

6.

MATERIAL STATUS

[	] ALL MATERIALS ARE ON HAND AND HAVE APPROVED SHOP DRAWINGS.

[	] THE FOLLOWING ITEMS ARE NOT ON HAND OR ARE NOT IN ACCORDANCE WITH THE SHOP DRAWINGS. ITEM DEFICIENCY SCHEDULED DELIVERY

1.

2.

3.

4.

#### CONTROL TESTING REQUIRED

TEST REQUIRED FREQUENCY RESPONSIBILITY SPEC SECTION

1.

2.

3.

4.

#### BRIEF DESCRIPTION OF WORK INVOLVED

SAFETY

[	] ACTIVITY HAZARD ANALYSIS IS ATTACHED. [	] ACTIVITY HAZARD ANALYSIS IS AS FOLLOWS.

ACTIVITY POTENTIAL HAZARD CONTROL MEASURES

1.

2.

3.

4.

5.

6.

#### EQUIPMENT REQUIRED

[	] SAFETY CHECKLIST COMPLETED	[	] SAFETY CHECKLIST ATTACHED

1.

2.

3.

4.

5.

6.

Activity Hazard Analysis (AHA)

| ACTIVITY/WORK TASK: | Demolition |  | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  |  | M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SIGNATURES |  | Activity # |  |  |  | AHA # |  |  |  |  | 001 |
| PWD/OICC/ROICC OFFICE |  |  | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| NAME & DATE ACCEPTED BY GDA: |  |  |  |  |  |  |  |  |  |  |  |  |
| CONTRACT NUMBER: |  |  | Severity |  |  | Probability |  |  |  |  |  |  |
| TASK ORDER/DELIVERY #: |  |  |  |  |  |  |  |  |  |  |  |  |
| PRIME CONTRACTOR: | Miami MEES |  |  |  |  | Frequent |  | Likely | Occasional | Seldom | Unlikely |  |
| SUBCONTRACTOR: | All |  |  |  |  |  |  |  |  |  |  |  |
| DATE OF PREPARATORY MEETING: | TBD |  | Catastrophic |  |  | E |  | E | H | H | M |  |
| DATE OF INITIAL INSPECTION: | TBD |  | Critical |  |  | E |  | H | H | M | L |  |
| CONTRACTOR COMPETENT PERSON: | TBD |  | Marginal |  |  | H |  | M | M | L | L |  |
| SITE SAFETY and HEALTH OFFICER | TBD |  | Negligible |  |  | M |  | L | L | L | L |  |
| ACCEPTANCE BY GOVERNMENT DESIGNATED AUTHORITY (GDA) |  |  | Review each “Hazard” with identified safety “Controls” and determine (RAC) |  |  |  |  |  |  |  |  |  |
| E = EXTREMELY HIGH (PWO/OICC/ROICC) |  |  | Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” .Place the highest RAC at the top of AHA. This is the overall risk assessment code for this activity |  |  |  |  |  |  |  |  |  |
| H = HIGH RISK (FEAD DIRECTOR) |  |  | “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic, Critical, Marginal, or Negligible after controls are in place |  |  |  |  |  |  |  |  |  |
| M = MODERATE RISK (CM or ET or PAR) |  |  |  |  |  |  |  |  |  |  |  |  |
| L = LOW RISK (ET or PAR) |  |  | “Probability” is the likelihood to cause an incident, near miss, or accident did occur and identified as: Frequent, Likely, Occasional, Seldom, or Unlikely after controls are put in place. |  |  |  |  |  |  |  |  |  |
| Job Steps |  | Hazards |  | Controls |  |  |  |  |  |  | RAC |  |
| 1. General Safety Precautions 2. Mobilization |  | a. Abrasions b. lacerations c. sight hazards d. noise/hearing hazards e. crushing/impact hazards f. struck by/pinch point hazards g. dust/respiratory hazards h. injury to hands i. dehydration j. exposure to hazardous chemicals a. slips, trips, falls during mobilization b. physical injury while lifting during staging/unloading c. noise/hearing hazard |  | a. Wear long sleeves and long pants of durable material b. Same as above, avoid sharp objects and protruding rebar/steel c. wear appropriate safety glasses and eye protection (cutting steel) d. wear ear protection/hearing protection e. stay clear of moving equipment, wear high vis clothing, all motorized moving equipment must have back-up alarms. f. wear hard hat g. wear appropriately fitted, proper APR respirator in dusty or toxic atmospheres h. wear appropriately sized, task-appropriate gloves i. drink plenty of fluids, primarily water, especially in cold weather j. post SDS in conspicuous location, ensure HAZCOM “right to know” policy is in place a. wear non-slip footwear, maintain 3 points of contact while climbing on/off equipment b. utilize proper lifting techniques, use lifting assistance (crane, PIT, etc) to unload c. wear appropriate hearing protection |  |  |  |  |  |  | M M M M M M M M L M M M M |  |

| Job Steps | Hazards | Controls | RAC |
| --- | --- | --- | --- |
| 3. Commence Demolition 3a. heavy equipment operation 3b. basic demolition tasks 3c. hotwork during demolition 4. Complete Demolition/cleanup/de-mobilization 4a. secure jagged and hazardous leading edges of material/structures 4b. cleanup 4c. de-mobilization | 3a. crush hazard, physical hazard 3b. noise/hearing hazard, cuts/abrasions/physical injury from hand tools and demolished materials, falls from heights, material handling/debris removal 3c. burns, vision damage, flash burns, smoke inhalation, torch oxyacetylene hose cuts, plasma cutter burns, fire, storage of compressed gas cylinders 4a. cuts/ lacerations/impalement on sharp objects 4b. dust, falls into open pits/holes, exposure to hazardous dust (concrete dust), crushing hazard by heavy equipment, engulfment by materials 4c. crushing hazard by equipment, general safety precautions for construction site, lifting injury, disposal of waste, potential fuel spill | 3a. all personnel wear high visibility safety attire. Stay clear of heavy equipment, and use spotters front and rear while moving machinery. Always ensure appropriate equipment guards and alarms are in place. (If you can see the operator, He can’t see you!) 3b. wear appropriate hearing protection at all times, wear appropriate PPE for tasking, including gloves, hard hat, steel toed boots, use appropriate fall protection devices/systems (full body harness and arrest system capable of 3000 # tensile strength) when working at height of 6’ or above, wear respirator or employ dust control measures for grinding or cutting concrete, and while removing debris 3c. personnel will wear appropriate fire retardant clothing, wear appropriate shaded goggles/welding helmet, wear respirators as required by ventilation available, keep areas around and below cuts clear of material and hoses, use extreme care while using plasma cutter, station fire watches and have extinguishing equipment standing by during hotwork and cutting, store compressed gas cylinders IAW local, state and federal guidelines, including tethering and chaining 4a. wear appropriate PPE, treat all sharp and jagged edges to round or remove the hazard entirely, cap all rebar and dowel penetrations 4b. wear appropriate respirator, wear fall protection near pit openings/employ barricades near open pits, stay clear of moving machinery, use spotters front and rear while moving machinery 4c. stay clear of moving and lifting equipment, use tag lines if cranes are in use, wear appropriate PPE for tasking, utilize appropriate lifting techniques, dispose of waste properly, ensure any spills are reported and cleaned up immediately | M M M M M M |

| Equipment to be Used | Training Requirements and Competent or Qualified Personnel name(s) | Inspection Requirements | RAC |
| --- | --- | --- | --- |
| Specific to Demolition 1. General hand tools 2. PIT/lifting/heavy equipment 3. Fall protection systems 4. Fire extinguishers 5. Plasma cutter/oxyacetylene cutting equipment | 1. Minimum 2 years experience 2. Current forklift certification, 2 year minimum experience on all other heavy equipment. 3. Minimum 3000 lb. tensile strength on fall arrest systems. 4. Inspected monthly 5. minimum 2 years experience, fire watch required Competent person on location | 1. Daily, prior to use. 2. Daily, prior to use 3. Daily, prior to use 4. Monthly 5. Prior to use | M M M M M |

| Equipment to be Used | Training Requirements/Competent or Qualified Personnel name(s) | Inspection Requirements |
| --- | --- | --- |

Instructions for completing Contractor Activity Hazard Analysis

- Activity/Work Task – Insert work/task this AHA is written for i.e. excavation, scaffold building, foundation preparation.

- PWO/OICC/ROICC – Insert name of Public Works Office, Officer In Charge of Construction Office or Resident Officer in Charge of Construction (PWD/OICC/ROICC

- Enter name & date AHA accepted by Government Designated Authority (GDA)

- Enter contract number

- Enter Task order or Delivery order number

- Enter Prime Contractors name

- Enter Subcontractors name

- Enter date preparatory meeting was held

- Enter date initial inspection was performed

- Enter name of contractor competent person on site for this activity

- Enter name of Prime Contractor Site Safety and Health Officer

- Level of government person responsible for accepting the AHA, progressive signatures as level of risk increases.

- Overall Risk Assessment code is highest code assigned to any Job step after Hazards are assessed and Controls have been assigned

- Schedule number is activity number from production daily reports

- AHA number is the sequential number of all AHA’s for this contract.

- Job steps is the complete sequence of work, not general statements to complete the entire activity

- Hazards is the known safety risks associated with completing the task

- Controls is the safety measures in place to reduce the hazard to the lowest level possible

- Risk Assessment code is where Severity and Probability intersect, place that letter E, H, M, or L in the RAC column

- List all equipment to be used to complete this activity i.e. crane, backhoe, vehicle, all heavy equipment

- List the training requirements required by EM 385, Safety Spec 01356 or OSHA that apply to this task. List competent person(s) required for specific tasks in EM 385

List qualified person(s) required for specific tasks in EM 385 List CPR/First Aid training and qualification dates

- List all inspection requirements of EM 385, Governmental Safety Requirements Specifications or OSHA 29 CFR 1926

##### Attachment 4

##### HAZARDOUS SUBSTANCE COMMUNICATION PROGRAM

##### TO: ALL PERSONNEL

RE:  HAZARDOUS SUBSTANCE COMMUNICATION PROGRAM

- PURPOSE

MIAMI MEES, Hazardous Substance Communication Program is established to provide employees with information regarding hazardous substances that may be encountered at our shop and project sites. It is our policy to comply with applicable safety and health regulations established for the construction industry. This written program is available at our shop, and in the jobsite office of MIAMI MEES.

- CONTAINER LABELING

  - A Jobsite superintendent or his designated representative, will check to verify containers are clearly labeled for: 1.) Contents

2.) Appropriate hazard warning

3.) Name and address of manufacturer

  - The superintendent or his designated representative will verify labeling of secondary containers for: 1.) Contents

2.) Appropriate hazard warning

3.) Name and address of manufacturer

  - Superintendent or his designated representative will review labeling and update MSDS log book with each delivery.

- SCOPE

  - To provide our employees with information supplied by manufacturers regarding the hazards of substances they produce or furnish.

  - To notify Subcontractors and their employees, on our projects with regards to the hazardous substances they might encounter, as well as our own employees. Any substance which we use on our projects or in our shop, if used in such a manner that employees may be exposed under normal conditions of use or in a foreseeable emergency, is to be included in the program. See attachment for list of chemicals.

  - This program does not apply to: 1.) Wood

2.) Drugs that are packaged for sale to consumers or intended for personal consumption. 3.) Certain consumer products where used in and for the same manner as a consumer.

- DETERMINATION OF HAZARDOUS SUBSTANCES

We rely on the supplier to perform evaluations of the substances which they deliver to our projects and shop to determine if the substances are hazardous.  All suppliers are required to provide MSDS information with all deliveries.

- MATERIAL SAFETY DATA SHEETS (MSDS)

The superintendent or his designated representative is responsible for establishing and maintaining the jobsite Material Safety Data Sheets. These sheets will be located in the shop office and in each field office, in a well-marked, indexed three ring binder. Each employee will be informed of the location of this information and have access to the information in the MSDS.

- HAZARDOUS NON-ROUTINE TASKS

Periodically, employees are required to perform hazardous non-routine tasks.  Some examples of non-routine tasks are:

  - Confined space entry

  - Tank cleaning

  - Painting reactor vessels

Prior to starting work on such projects, each affected employee will be given information by the superintendent or his designated representative about the hazardous chemical he or she may encounter during such activity. This information will include specific chemical hazards, protective and safety measures the employee can use, and steps the company is using to reduce the hazards, including ventilation, respirators, presence of another employee and emergency procedures.

- INFORMATION TO SUBCONTRACTORS

It is the responsibility of the superintendent or his designated representative to provide subcontractors with information about hazardous chemicals their employees may be exposed to on a jobsite and suggested precautions for the subcontractor's employees. Subcontractors will be contacted before work is started to gather and distribute information concerning any chemical hazard that they may bring to our workplace, and MSDS WILL be placed in log book.

- CHEMICAL LIST

The attached is a list of all known hazardous chemicals used by our employees. Further information on each chemical may be obtained by reviewing MSDS'S located at the MIAMI MEES jobsite office and in our shop.

- EMERGENCY PLANNING AND COMMUNITY RIGHT TO KNOW

DEFINITIONS

CERCLA - Comprehensive Environmental Response, Compensation, and Liability Act CO -	Contracting Officer

CHS - CERLA Hazardous Substance EHS - Extremely Hazardous Substance FEC - Facility Emergency Coordinator HCS - Hazardous Chemical Substance RQ -	Reportable Quantity

TPQ - Threshold Planning Quantity

Whenever a CERLA HCS or EHS is brought onto the site, the contractor will submit the attached form to the FEC, the Fire Department with jurisdiction, and the CO as described below:

- Within 5 days for an EHS which equals or exceeds its TPQ or which is a solid or liquid weighing 500 pounds or more, whichever is less

or –

- Within 10 days for a HCS which equals or exceeds 10,000 pounds for a solid or 55 gallons for a liquid.

SOME POTENTIALLY HAZARDOUS SUBSTANCES COMMONLY FOUND ON CONSTRUCTION PROJECTS

ACETONE ACETYLENE GAS ADHESIVES

ALUMINUM ETCHING AGENT AMMONIA

ANTI-FREEZE

ARSENIC COMPOUNDS ASBESTOS

ASPHALT (PETROLEUM) VAPORS BENZENE (AND DERIVATIVES) BLEACHING AGENTS

CARBON BLACK

CARBON MONOXIDE (IN CYLINDERS) CAULKING SEALANT AGENTS CAUSTIC SODA (SODIUM HYDROXIDE) CHROMATE SALTS

CHROMIUM CLEANERS COAL TAR PITCH COATINGS COBALT

CONCRETE AND CURING COMPOUNDS CREOSOTE

CUTTING OIL (OIL MIST) DE-EMULSIFIER FOR OIL DIESEL GAS, DIESEL OIL DRYWALL

DUSTS (BRICK, CONCRETE BLOCK) ENAMEL

ETCHING AGENTS ETHYL ALCOHOL

FIBERGLASS, MINERAL WOOL

FOAM INSULATION

FREON 20, r20 (AND OTHERS) GASOLINE (PETROL, ETHYL) GLUES

GRAPHITE GREASES

HELIUM (IN CYLINDERS) HYDRAULIC BRAKE FLUID HYDROCHLORIC ACID HYDROGEN (IN CYLINDERS) INKS

INSULATION IRON KEROSENE LEAD

LIME (CALCIUM OXIDE) LIMESTONE LUBRICATING OILS

LYE	(SODIUM	HYDROXIDE,	POTASSIUM HYDROXIDE)

MAGNESIUM

METALS (ALUMINUM, NICKEL, COPPER, ZINC) CADMIUM, IRON ETC.

METHANOL (METHYL ALCOHOL) METHYL ETHYL KETONE (2-BUTANONE) MOTOR OIL ADDITIVES

MURIATIC ACID NAPHTHA (COAL, TAR) NITROGLYCERIN OXALIC ACID

OZONE

PAINT REMOVER

PAINT STRIPPER PAINTS & LACQUERS PARTICLE BOARD PENTACHLOROPHENOL

PHOTOGRAPHIC DEVELOPERS AND FIXERS PHOTOGRAVURE INK (COPY MACHINE) PLASTICS

POLISHES FOR FLOORS PROPANOL

PUTTY

RESINS, EPOXY/SYNTHETICS SEALERS

SHELLAC

SOLDER, FLUX (ZINC CHLORIDE, FLUORIDES, ETC.)

SOLDER, SOFT (LEAD, TIN) SOLVENTS

SULFURIC ACID

THINNER, PAINT/LACQUER TIN

TRANSITE

TURPENTINE, GUM SPIRITS, OIL OF TURPENTINE VARNISHES

WATERPROOFING AGENTS WAXES

WELDING RODS

WOOD ALCOHOL (METHANOL) WOOD PRESERVATIVES ZYLENE

ZINC

Attachment 5

# SIGNATURE SHEET

- PLAN PREPARER	Harry Brown

Project Engineer	Signature	Date

MIAMI MEES 609-230-4425

- PLAN APPROVAL	Mike Brown

Corp. Safety Officer	Signature	Date

MIAMI MEES 918-961-8755

- CONCURRENCE

GOVERNMENT REPRESENTATIVE

Signature	Date

Signature	Date

Signature	Date

Attachment 6

# SUBCONTRACTORS AND SUPPLIERS

### To be incorporated at a later date

The Safety Responsibilities and means for controlling and coordinating subcontractors and suppliers are as described in paragraphs V. c. and XXIII.

CONTRACT NO

PHASE

DATE

ACTIVITY HAZARD ANALYSIS (PREPARATORY INSPECTION)

GENERAL REQUIREMENTS:  FIRE PREVENTION AND PROTECTION

- WHERE WILL FIRE EXTINGUISHERS BE KEPT?

  - A fire extinguisher will be required on each piece of motorized equipment, on each vehicle, at the office, and at fuel storage tanks.

- WHAT TYPE HAZARD IS PRESENT AT EACH LOCATION?

  - Fuels, electrical, grass, and other combustible materials.

- WHAT TYPES OF EXTINGUISHERS WILL BE PROVIDED?

  - Each piece of equipment or vehicle will have a minimum of 5-BC.

  - Each office or building will have a minimum of one extinguisher rated not less than 20-BC.

  - Each fuel truck will have an extinguisher rated at not less than 40-BC.

  - Each service or fueling area shall be provided with at least one extinguisher have a rating of not less than 40- BC and be 50’ - 100’ from each pump or tank.

- WHAT TYPE HEATING DEVICES WILL BE USED?

  - Offices will be heated with electricity. Equipment and vehicles will be heated with factory furnished heaters. No open fires or “fire barrels” will be allowed.

- WHAT FLAMMABLE LIQUIDS WILL BE USED AND HOW WILL THEY BE STORED?

  - Diesel fuel - Stored at the contractor’s staging area in an above ground tank. The tank will be grounded and the tank area diked and PVC lined.

  - Oil / grease - Stored in a metal van a minimum of 50’ from any other building.

- WHAT PRECAUTIONS WILL BE TAKEN WHEN WELDING, CUTTING, HEATING, OR BRAZING AND SOLDERING?

  - There will be a fire watch on standby with a fire extinguisher while this work is being performed. This work will not be performed in tall grass or near combustible material.

PRIME CONTRACTOR C.Q.C.	PRIME CONTRACTOR SUPERINTENDENT

GOVERNMENT REPRESENTATIVE	WORK CREW LEADER OR FORMAN

SUBCONTRACTOR (if applicable)	WORK CREW LEADER OR FORMAN

CONTRACT NO.	FA448420C0006

PHASE:

DATE:

ACTIVITY HAZARD ANALYSIS (PREPATORY INSPECTION)

GENERAL REQUIREMENTS: HOUSEKEEPING

- WHERE WILL TRASH BARRELS BE LOCATED?

  - They will be located at the various work areas as needed to comply with good housekeeping practices.

- WHERE WILL WORKERS EAT LUNCH?  WHERE WILL DRINKING WATER BE LOCATED?

  - They will eat lunch at their workstation, staging area, or other close location. Drinking water will be provided with each crew.

- HOW WILL MATERIAL BE STORED?

  - Material will be stored in trailers or stacked in stockpiles and covered from weather if necessary.

- HOW WILL DEBRIS BE REMOVED FROM THE SITE AND DISPOSED?

  - A trash container will be provided with each crew and will be emptied at the staging daily. Trash from the staging area will be hauled to the dump.

- HOW WILL JOB CLEANUP BE MAINTAINED? HOW WILL SUBCONTRACTORS BE CONTROLLED?

  - Trash will be removed from individual work areas to the staging area daily. Daily inspections will be made.

- HOW WILL THE STORAGE YARD BE MAINTAINED?

  - Trash barrels will be emptied as required for a clean site. If necessary, a dumpster container will be furnished.

PRIME CONTRACTOR C.Q.C.	PRIME CONTRACTOR SUPERINTENDENT

GOVERNMENT REPRESENTATIVE	WORK CREW LEADER OR FORMAN

SUBCONTRACTOR (if applicable)	WORK CREW LEADER OR FORMAN

| Activity/operation to Be performed | Hazards associated with The activity / operation | Method of control Or safeguard |
| --- | --- | --- |
| Fire prevention and protection Smoking and open flames | Fires from flammable combustible material | - Smoking prohibited where flammable combustible material is stored - No smoking when fueling equipment - No open fires or “fire barrels” allowed - Provide fire protection - Post proper signs in fuel areas and on fuel trucks |
| Use of cutting torch or welding, soldering, etc. | Fires or explosions | - When possible, perform all welding/cutting in designated areas - Have a fire watch with a fire extinguisher on hand |
|  | Burns to the eyes or body when welding/cutting | 1. Use proper equipment to protect yourself and others |
| Housekeeping | Fire hazards | - Provide containers for all debris - Daily inspection and clean-up of all areas - Provide fire extinguishers where combustible materials are stored |
| Equipment | Safety hazard | - Provide sufficient personnel and equipment to comply with clean up requirements - Keep all wrecked forms all lumber clear of nails. Do Not leave on the ground for others to step on |
|  | Environment pollution and fuels | - Avoid all spills of toxic chemicals - Burning of debris is not allowed |

CONTRACT NO.

PHASE

DATE

- WHAT EQUIPMENT AND TOOLS WILL BE USED?

1.

2.

3.

4.

5.

6.

7.

8.

- WHO WILL INSPECT THE EQUIPMENT AND TOOLS?

NAME

JOB TITLE

EMPLOYER

- HOW WILL MATERIAL BE HANDLED?

  - HEAVY LOADS WILL BE HANDLED WITH EQUIPMENT.

  - LIGHTER LOADS WILL BE HANDLED BY HAND.

- WHAT PERSONAL PROTECTIVE EQUIPMENT WILL BE REQUIRED?

  - HARD HATS, LONG PANTS, SHIRTS WITH SLEEVES, LEATHER SHOES/BOOTS ARE MINIMUM.

  - SAFETY VEST IF EXPOSED TO VEHICULAR TRAFFIC.

  - FOR SOME OPERATIONS, LEATHER GLOVES, FACES SHIELDS, EAR PLUGS, CHAPS.

PRIME CONTRACTOR C. Q. C.	PRIME CONTRACTOR SUPT.

GOVERNMENT REPRESENTATIVE	WORK CREW LEADER

# Attachment 7

# HOT AND COLD STRESS MONITORING

When climatic conditions cause excessive hot or cold weather, or when there are indications or warnings of impending severe weather (heavy rains, floods, strong winds, tornadoes, etc.), the weather conditions will be closely monitored and appropriate precautions will be taken to protect personnel.

HOT WEATHER

In hot environments, iced drinking water will be made available to personnel. Workers will be encouraged to frequently drink small amounts, e.g., one cup every 15-20 minutes. In situations where heat stress may impact worker safety and health, work-rest regimens shall be established. For example, normal work breaks of 10 minutes per hour are doubled to 10 minutes each one-half hour.

Environmental monitoring of the Wet Bulb Globe Temperature Index shall be conducted and workloads and work/rest regimens established as specified in the American Conference of Governmental Industrial Hygienists (ACGIH) publication “Threshold Limit Values and Biological Exposure Indices “.

COLD WEATHER

- Employees working in air temperatures of -15 degrees F or less shall utilize the work / warm up regimen specified in Table 6-2.

- At air temperatures of 36 degrees or less, workers who become immersed in water or whose clothing becomes wet shall immediately be provided a change of clothing and treated for hypothermia.

- When manual dexterity is not required workers shall be provided gloves at the following temperatures:

  - For sedentary work, 60 F.

  - For light work, 40 F.

  - For moderate and heavy work, 20 F.

- When detail work requiring manual dexterity is required to be performed with bare hands for more than 20 minutes in an environment below 50 degrees F, electric or fueled hand warmers will be provided.

- Metal handles of tools and equipment controls will have thermal insulation at temperatures below 20 F.

COLD WEATHER CLOTHING REQUIREMENTS:

- If wind chill is a factor at a work location, the cooling effect of the wind shall be reduced by shielding the work area or providing employees an outer windbreak layer garment.

- Extremities, ears, toes, and nose shall be protected from extreme cold by protective clothing.

- Employees performing light work and whose clothing may become wet shall wear an outer layer of clothing which is impermeable to water.

- Employees performing moderate to heavy work and whose clothing may become wet shall wear an outer layer of clothing which is water repellent.

- Outer garments must provide for ventilation to prevent wetting of inner clothing by sweat.

- If clothing is wet, the employee shall change into dry clothes before entering a cold environment.

- Due to the added danger of cold injury due to evaporative cooling, workers handling evaporative liquids (such as gasoline, alcohol, or cleaning fluids) at air temperatures below 40 F shall take precautions to avoid soaking of clothing or contact with skin.

- Eyewear providing protection against ultraviolet light, glare, and blowing ice crystals shall be provided to employees in snow and / or ice covered terrain.

ENVIRONMENTAL MONITORING

- At air temperatures of 45 degrees F and falling, the temperature will be monitored each 2 hours.

- When air temperature falls below 30 degrees F, the dry bulb temperature shall be measured at least every 4 hours. In indoor workplaces the wind speed should be measured at least every 4 hours when the rate of air movement exceeds 5 mph: in outdoor work situations the wind speed should be measured and together with the air temperature.  Determine the wind chill per Table 6-3 to take appropriate action.

- Personnel will be excluded from work in cold (30 F or below) if they are suffering from diseases or taking medication which interferes with normal body temperature regulation or reduces tolerance to work in cold environments.

###### Attachment 8

SAFETY POLICY FOR SUBCONTRACTORS

- Our safety policies, procedures and codes of practice are in compliance with Federal OSHA and New Jersey policies (New Jersey Public Employees Occupational Safety and Health (PEOSH) State Plan (pending certification).

- The importance that SAFETY AWARENESS has on our corporate philosophy cannot be overstated. Our approach is quite simple: Substandard Conditions and Substandard Practices will not be tolerated on our project sites.

- Our safety policy is to achieve the lowest possible accident rate, with our goal being zero. Our safety policy will be acknowledged by all of our subcontractors and shall be followed accordingly.

- To meet this goal, we implemented the following requirements for all projects;

  - Basic personal protective equipment consists of construction grade, ANSI approved, hard hat and work boots or shoes. Long pants and T-shirts with sleeves are also the minimum; muscle shirts and shorts are not allowed. Athletic type footwear is also not suitable for our construction sites.

  - Hard hats and proper footwear will be worn at all times while on our projects. Safety glasses, hearing protection and respirators will be employed where hazards to eyes, ears, or respiratory tract exist.

  - Proper fall protection (safety harnesses and lanyards) must be worn where a worker is exposed to a hazard of falling 6 feet or more. Fall protection plans shall be in place on the project for each trade.

  - Where a subcontractor's employee is required to remove a guardrail or protective covering and is exposed to a fall hazard, the worker must be protected against falling by an approved fall protection plan and equipment. If the area is to be left unattended for any period and upon completion of the task, the guardrail or covering must be replaced in a securely fastened state in accordance with applicable OSHA regulations.

  - We believe that regular weekly "job-box" safety meetings/training help reduce accidents. As such all subcontractors are required to conduct their own weekly safety meetings and submit documentation to the MIAMI MEES Site Superintendent. Subcontractor supervisors and all employees shall attend this meeting. The MIAMI MEES Safety Manager or staff may monitor safety meetings/training on an unscheduled basis, as he deems necessary, to confirm compliance with this requirement.

  - In order for us to investigate all accidents, all subcontractors on our project sites shall submit to MIAMI MEES, copies of all accident reports (supervisor and employee) for all injuries and OSHA recordable incidents requiring medical attention or involving property damage within 24 hours of the accident.

  - Each subcontractor is required to supply their own stocked first aid kit and shall have at least one person trained to administer first aid, if the need arises, per guidelines.

  - As required by "Worker Right-to-Know" legislation, all subcontractors must provide current Material Inventory Data with supporting Material Safety Data Sheets (MSDS) for all products to be used on our project sites.       All employees

will be trained on the Hazardous Communication "Right to Know" Program prior to commencing any work on site.

  - Charged 10 lb. fire extinguishers shall be provided in all job-site offices, storage trailers, and contractor vehicles as well as be clearly visible and readily available where any hot work is being performed, fueling operations are in progress, and/or during the use of flammable substance as part of the project.

  - Work areas must be kept clean and free from debris or recyclable materials, including coffee cups and lunch garbage. Unclean areas are substandard and pose a fire and injury hazard. At the end of each shift or day the project site will be cleaned of all construction/demolition debris and common trash.

  - All subcontractors shall be responsible to ensure all their visitors, service, or delivery people meet the minimum needs for personal protective equipment prior to entering the project area.

  - All Subcontractors shall, before commencing any work, provide a written hazard analysis for each position or activity anticipated in the scope of work. Also work procedures for any portion of work that may pose unique or specific hazards to employees performing the work or in the area.

- We have a disciplinary action policy that applies to violations of the codes and standards set forth in the OSHA regulations. Failure to follow these codes and standards as well as our Site Specific Safety Plan may result in disciplinary action from verbal warning up to, and including, removal from project site or dismissal from the project.

- All Subcontractors employers shall be held completely responsible for ensuring their employees fully comply with the OSHA regulations.

- SUBCONTRACTOR SAFETY RECORD

| SUBCONTRACTOR COMPANY NAME |  | DELIVERY ORDER #. |  | PROJECT NAME |
| --- | --- | --- | --- | --- |
| SAFETY POINT OF CONTACT |  | TELEPHONE |  |  |

- List your firm’s insurance interstate Experience Modification Rate (EMR) for the three most recent years.  Use your intrastate EMR if not interstate rated

YEAR:	 YEAR:	 YEAR:

EMR:	 EMR:	 EMR:

- Using your last year’s OSHA No. 300 Log, please provide the following: (As an alternative, you may submit a copy of your OSHA No. 300 Log).

  - Number of lost workday cases (injuries involving time (day lost) away from work; Column 3 of OSHA No. 300 Log). List number of individual cases, not total number of actual days lost (see Paragraph 1 of the Instruction).

  - Number of restricted workday cases (Column 5 of the OSHA No. 300 Log). List number of individual cases, not total number of restricted workdays suffered (See Paragraph II of the Instruction).

  - Number of cases with medical treatment only (Column 6 of the OSHA No. 300 Log). List number of individual cases, taking care to subtract from the total in Column 6 those cases that are actually first aid cases (FAC) (See Paragraphs III and IV of the Instruction).

  - Number of facilities.

- Employees hours worked last year

- Do you hold site safety meetings for:

  - Field Supervisors	Yes

  - Employees	Yes

  - New Hires	Yes

  - Subcontractors	Yes

No   No   No   No

Frequently Frequently Frequently Frequently

- Do you conduct job safety inspections?	Yes

No

Frequently

- Do you have a written safety program?	Yes

No

Copy to	 	  MIAMI MEES

- Lower Tier Subcontractor Safety Records shall be procured by the responsible Subcontractor and attached with their record. These records shall be amended to Attachment 3 of the MIAMI MEES SSHP and kept in the Site Office.

- WARNING Potential Blood borne Pathogen Exposure.  All Subcontractors' are advised to conduct an in-depth review of the project requirements to ensure that all potential Blood borne pathogen exposure (exposure to human blood or other bodily fluids) is identified to their employees. This is especially important when working in or around hospitals, under piers and wharves, on sewage systems and plumbing.

ALLOWABLE CLAIMS with regards to Potential Blood borne Pathogen Exposure

| Preventative Treatment | Payment for inoculation or immunological treatment in cases in which a work-related activity has resulted in probable exposure to a potential infectious occupational disease may be authorized. This may be authorized even if the disease has not been contracted and there is no allowable claim. Routine inoculations without a specific incident of exposure are not payable. If there is no incident of injury, such as laceration, and the disease has not been contracted even though exposure has occurred, the claim is not allowable; however, any inoculation or immunological treatment should be authorized. If there has been an actual injury such as a puncture wound or laceration or if the worker has actually contracted the disease, the claim may be allowable. The definition of the occupational disease or injury has to be satisfied. For occupational diseases, there must be medical proof the condition is related to the exposure on a more probable than not basis. Inoculation against hepatitis should take place within 48 hours of the exposure to be effective. Standard blood serum tests are run to determine if there has been previous infection. For hepatitis B, a follow-up shot is required four weeks after the initial inoculation. All of the above would be payable under the new law even if the claim is rejected. |
| --- | --- |

- OSHA 300 log reporting

RECORDKEEPING GUIDELINES---MEDICAL TREATMENT & FIRST AID

| NATURE OF INJURY | RECORDABLE | NON-RECORDABLE |
| --- | --- | --- |
| CUTS; LACERATIONS; PUNCTURES; ABRASIONS; SPLINTERS | - Sutures (stitches) - Butterfly sutures - Treatment of infection - Application of antiseptic on second or subsequent visit to medical personnel - Removal of foreign bodies requiring skilled services of physician due to depth of embedment, size or shape of object(s), or location of wound - Cutting away dead skin (surgical debridement) | - Bandaging on any visit to medical personnel - Application of antiseptic on first visit to medical personnel - Application of ointments on first or subsequent visits to prevent drying or cracking of skin - Removal of foreign bodies from wound by tweezers or other simple techniques - Removal of foreign bodies in the eye, not embedded by irrigation |
| FRACTURES | - Where X-ray results are positive. Application of a cast or other professional means of immobilizing injured part | - Where X-ray taken as a precaution is negative for fracture |
| STRAINS; SPRAINS; DISLOCATIONS Any strain, sprain, or dislocation is recordable if the worker’s range of motion is affected in a manner which prevents carrying on any of his or her regularly assigned duties, whether or not medical treatment is rendered. | - Application of a cast or other professional means of immobilizing injured part - Use of hot or cold compresses for treatment of strains, sprains and dislocations on second or subsequent visits to medical personnel - Use of diathermy and whirlpool during second or subsequent visits to Doctor - Chiropractic manipulation | - Use of an elastic (Ace) bandage on a strain that is not otherwise recordable, on first visit to doctor or nurse - Use of hot or cold compresses for treatment of a strain on first visit to medical personnel |
| THERMAL OR CHEMICAL BURNS Any burn is recordable if the worker’s range of motion is affected in a manner which prevents carrying on any of his or her regularly assigned duties, whether or not medical treatment is rendered | - Treatment of all second and third degree burns | - Treatment by a doctor or nurse for a first degree burn |
| BRUISES OR CONTUSIONS Any bruise of contusion is recordable if the worker’s range of motion is affected in a manner which prevents carrying on any of his or her regularly assigned duties, whether or not medical treatment is rendered | - Treatment of a bruise by draining collected blood - Soaking or application of cold compresses to a bruise on second or subsequent visits to medical personnel | - Soaking or application of cold compresses to a bruise that is otherwise not recordable, on first visit to medical personnel |

MISCELLANEOUS PROCEDURES

- Medical treatment is only one criterion for determining if an injury is to be recorded. Any injury that required only first aid treatment but also involved lost workdays (other than the day of diagnosis), loss of consciousness, restriction of work or motion, or transfer to another job would also be considered to be recordable.

- Tetanus shots, either initial or boosters, are considered preventative in nature and are not considered medical treatment.

- An X-ray, which is negative for fracture, is not considered medical treatment.

- Hospitalization for observation, where no treatment is rendered other than first aid, is not considered medical treatment. However, most injuries requiring hospitalization will result in lost workdays and will be recordable for that reason.

- The observation of injury by a doctor or nurse on second or subsequent visits is not recordable.

- Use of prescription medicines constitutes medical treatment thus is recordable (except a single dose administered on first visit for minor injury or discomfort).  Recommendation or use of nonprescription medicines is considered first aid only.

- All diagnosed occupational illnesses are recordable.

- All occupational fatalities are recordable.

- A heart attack, if allowed as an industrial claim, is recorded as an illness (column 7g of the log).

- Replacement of broken eyeglasses in itself is not a recordable case.

SUBCONTRACTOR SITE FOREMAN / SUPERVISOR RESPONSIBILITIES

- REVIEW with the Project Manager, Site Superintendent and CQC the Safety Planning Checklist and Job Hazards Analysis prior to the start of each job/project.

- IMPLEMENT the protection requirements dictated in the JHA's and monitor throughout project to assure a safe work environment, proper work practices, and compliance with all applicable laws and regulations.

- CONDUCT and DOCUMENT jobsite safety and operations training for all workers at the start of the project, and subsequent new hires, utilizing the SAFETY CHECKLIST.  Maintain required documentation (attached) at the jobsite and forward to the MIAMI MEES Site Superintendent for filing.

- ENFORCE observance of all site-specific safety rules and regulations.

- MONITOR crew safety performance and behavior to ensure positive safety attitude. Set a personal example of proper safety by requiring compliance with procedures and all applicable rules and regulations as well as common sense safety.

- OPERATE, INSPECT, STORE and MAINTAIN portable and power tools as well as industrial equipment in a safe manner. Provide and assure use of personal protective equipment for all employees, as required.

- CONDUCT weekly safety meetings/training as required by regulation utilizing the SAFETY CHECKLIST.  Discuss and document meeting agenda and training topic.   Maintain copy of minutes at the jobsite and forward to the MIAMI MEES Site Superintendent for filing.

- CONDUCT and DOCUMENT routine safety inspections utilizing the SAFETY CHECKLIST. Review inspection results with crewmembers at regular safety meeting. Attach inspection form to safety meeting minutes. Call MIAMI MEES immediately to report and initiate follow-up action to correct any deficiencies found.

- IMMEDIATELY NOTIFY the MIAMI MEES Office in the advent of an accident, injury and/or incident involving employees, other workers, government and jobsite equipment. Collect the accident reports and commence written investigations. Complete all investigations and forward all reports no later than 24 hours following the incident.

- MAINTAIN current First Aid/CPR certification and a well-stocked First Aid Kit in accordance with guidelines.

- ENSURE adequate 10-lb. fire extinguisher coverage on the project site and for all hot work operations.

- ENSURE adequate sanitary facilities and portable water available at the project site as well as emergency eye wash stations/showers, as appropriate.

- ASSIGN and MONITOR in conjunction with MIAMI MEES Safety Manager, all injured employees who are working temporary modified tasks while recovering from an injury. Tasks must match limits as defined by treating physician. Keep Safety Manager informed regarding employee's progress, attendance, and other relevant matters.

- MONITOR and ENSURE compliance with the posted WORKSITE SAFETY RULES.

- ENSURE that all proper electrical precautions are observed on the project site and ground fault indicators are properly used as appropriate as well as all power cords protected from damage and routinely inspected.

# Attachment 9

# WORKSITE SAFETY RULES/TRAINING

We would like to take this opportunity to welcome you to the MIAMI MEES PROJECT SITE. You are required to read and obey the following safety requirements.

These requirements shall be posted on the job site.

- Accidents or injuries, regardless of their nature, shall be reported to your supervisor for immediate attention.

- Projecting nails shall be turned down or removed from lumber by all employees responsible for eliminating this hazard.

- Hard hats shall be worn on all MIAMI MEES Construction Project Sites and whenever there is potential for head injury from falling objects, shock or impact.

- Appropriate personal protective equipment shall be worn at all times AS REQUIRED.

- Excavations: The sides of all excavations 4’ or more in depth, where there is a danger of slides or cave-ins, shall be shored by substantial sheet piling and bracing, or other effective means, or sloped to the angle of repose. Personnel shall not cross over excavations without proper protection in place.

- Floor and roof openings shall be guarded on all sides except stair openings by a standard railing and toe board or substantial cover.

- Scaffolding shall be constructed of metal supports and scaffold grade lumber. All platforms more than 4' above ground or floor shall have standard guardrails and toe boards on all open sides and ends.

- Ladders shall be constructed of metal or straight-grained lumber, with maximum length of a single ladder not to exceed 30 feet. Ladder rails shall extend at least 36 inches above the roof/landing edge and shall be securely fastened at the top.  Ladders shall be used at a pitch such that the ladder is approximately ¼ of its working length from the supporting wall.

- Hand tools shall be maintained in a safe condition and used only for their intended purpose.

- Power tools shall be maintained in a safe condition with proper guards in place and used only by authorized personnel. Electric powered hand tools shall be grounded or double- insulated type.

- Ground Fault Indicator Circuits shall be utilized for all portable power tool connections. Extension cords shall be only one length and cannot be plugged into one another to extent service without permission.

- Use of any medication, which endangers an employee's ability to maintain a work safe environment, is prohibited at the jobsite. Use of alcohol or illegal drugs before or during work hours will result in immediate dismissal from the Project

- Only employees who have been authorized through their company and who have undergone proper driver training indoctrination are to be allowed to drive on the project site/work area.

- ALL SAFETY RULES SHALL BE OBEYED. Violation of any safety or environmental rule may be cause for immediate dismissal of any employee from the project site.

- HORSEPLAY is not allowed at any time and may be cause for immediate dismissal of any employee.

- All material shall be disposed of in accordance with the Environmental Protection Plan. Violation of these rules may be cause for immediate dismissal of any employee. Willful disregard or intent could be reported to proper authorities for their handling.

- The Project Site shall have SAFE ACCESS to and from the work areas, maintained free of potential accidents such as tripping hazards and falls, adequate project site cleanliness as well as the proper handling and storage of all supplies and material including flammable and combustible liquids and gases.

REPORTING UNSAFE CONDITIONS AND PRACTICES

IF YOU WITNESS ANY EMPLOYEE, MIAMI MEES OR OTHER SUBCONTRACTOR,

VIOLATING A SAFETY POLICY OR DISCOVER A DANGEROUS OR UNSAFE SITUATION, YOU MUST REPORT IT TO EITHER

YOUR SUPERVISOR, MIAMI MEES SITE SUPERINTENDENT or QUALITY CONTROL/SAFETY INSPECTOR IMMEDIATELY.

For confidentiality purposes you may write the incident down on a Hazard Observation Form, below,

and leave it unsigned for the Site Superintendent.

| Date Observed: | Time: | Location: |
| --- | --- | --- |
| Reported by: (optional) |  | Received By: |
| Delivery Order: |  | Project Name: |
| Observation: |  |  |
| Suggested Remedy: |  |  |
| Response: |  |  |

# SAFETY SUGGESTION AND RECOMMENDATION REPORTING

If you have a suggestion or recommendation that you feel would improve the way or manner in which the task is handled or preformed, reduce the potential for injury and/or improve our first aid response, and that could result in a savings of personnel, manpower, time, and equipment, you can report it to the Site Superintendent.

If you would like you can write the suggestion down on a Safety Suggestion Form, below, and leave it unsigned with the Site Superintendent.

| Date Observed: | Time: | Location: |
| --- | --- | --- |
| Reported by: (optional) |  | Received By: |
| Delivery Order: |  | Project Name: |
| Recommendation: |  |  |
| Response: |  |  |

# Site Safety Checklist Page

| - Activity Hazard Assessment | - Site Training |  | - Site Inspection |  |
| --- | --- | --- | --- | --- |
| Site/Project: |  | Conducted By: |  | Date: |

A = YES required	B = NO not expected but possible Blank = NOT APPLICABLE to project

| 1 | Jobsite Information A B |
| --- | --- |
| a. OSHA and other jobsite warning posters posted ...............................................................................................   - Emergency telephone numbers posted: police, fire, doctor, hospital, ambulance, first aid/CPR certified employees - Location of Hospital/Medical Services, first aid equipment and qualified first aiders available onsite................   d. Employee training – initial, weekly, new employee ............................................................................................   - Personal protective equipment requirements, equipment provided, training in use and wear ...........................   - Overview of project scope of work, site operations, emergency & accident procedures discussed...................   - Site Specific programs discussed / training held (Environmental Plan, Right to Know, etc) ..............................   |  |

| 2 | Accident Prevention Program Overview A B |
| --- | --- |
| a. Location of written plan discussed with employees/ workers on-site .................................................................   - PERSONAL SAFETY PARAMOUNT: discuss need to know basics of proper actions to take .........................   - Prepare for known risks: potential injuries, bloodborne pathogens, hazards material present, potential fire hazards - Lines of communication & responsibilities: REPORTING ACCIDENTS/INCIDENTS, Whom to call for assistance - Safety responsibilities: DUTY TO PERFORM JOB SAFELY – disciplinary actions available ...........................   - Reporting and correcting life-threatening hazards immediately, safety suggestions..........................................   - Evacuation Route & muster point, location & direction to Hospital, nearest telephone location .........................   |  |

| 3 | Bloodborne Pathogens Injury & Exposure Control (as needed) A B |
| --- | --- |
| - Employee training provided including scope of work, emergency operations, eye-body wash facilities, first-aid - Accident Prevention Policy - emergency procedures, notification numbers, decontamination clothing removal issues   - First-aid kits stocked with adequate personal protective equipment and disinfectants/antiseptics ...................   - Safe work practices discussed, proper clean-up and disposal practices, decontamination procedures ............   e. Need to immediately report exposures and/or contamination to office ..............................................................   |  |

| 4 | Hazard Communication Program A B |
| --- | --- |
| a. Employee training provided along with other contractor employees training provided.......................................   b. Material Inventory List developed & available on site ........................................................................................   c. Material Safety Data Sheets collected & available on site .................................................................................   - Emergency and First Aid response for Hazardous Substances defined & communicated ................................   - Flammable/Combustible Material stowage requirements defined & communicated, area posted as required ..   f. All containers properly labeled...........................................................................................................................   |  |

| 5 | Sanitation and Housekeeping A B |
| --- | --- |
| a. Working areas maintained neat and clean.........................................................................................................   - Waste and trash properly disposed, waste containers provided and used ........................................................   - Enclosed waste chute provided when material is dropped from heights of over 15 feet ....................................   d. Adequate lighting for all tasks, project site.........................................................................................................   e. Passageways, walkways, stairs & doorways clear, protected from falling objects, handrails provided, non-slip setup f. Sanitary facilities provided and maintained clean ..............................................................................................   g. Potable water available for washing or other non-water means / methods to clean hands and face .................   h. Fresh drinking water available on site, disposable drinking cups used……………………………………………..  |  |

| 6 | Construction Project Personal Protective Equipment A B |
| --- | --- |
| - Work shoes; hard sole, prefer steel toed, leather or strong canvas: NO ATHLETIC SHOES ...........................   - Seat belts required when in moving vehicle, industrial and heavy equipment (as provided) .............................   - Hard Hats will be worn on all sites, at all times (unless special permission is received) ....................................   - Job-specific (circle): Safety Glasses, Face Shield, Hearing Protection, Gloves, Other (specify): .....................   |  |

| 7 | Handling & Storage of Materials A B |
| --- | --- |
| a. Materials properly stored or stacked on firm footings, not too high....................................................................   b. Passageways clear, traffic controlled in the storage area, fire lanes open ........................................................   c. Materials protected against weather conditions and moisture/cold/heat ............................................................   - Flammable & Hazards Materials properly segregated, appropriately sized fire extinguisher available .............   - Gas Cylinders and Bottles properly stored, chained, segregated as required....................................................   |  |

| 8 | Flammable Gases & Liquids A B |
| --- | --- |
| a. All containers approved and clearly identified....................................................................................................   b. Proper storage practices, appropriate segregation requirements observed, all caps in place ..........................   c. Fire hazards checked ........................................................................................................................................   d. Proper type (20lb) and number of fire extinguishers posted nearby ..................................................................   e. Proper method for moving and storing cylinders used.......................................................................................   |  |

| 9 | Fire Prevention A B |
| --- | --- |
| a. Adequate number of fire extinguishers {2 A/ 20 B-C}(10 lb), identified, checked and accessible ......................   b. Fire instructions and training provided to personnel ..........................................................................................   c. Phone number of fire department posted...........................................................................................................   d. Hydrants clear, access open..............................................................................................................................   e. Good housekeeping in evidence .......................................................................................................................   f. NO SMOKING signs posted and enforced where needed .................................................................................   - Flammable Material, Oxygen Bottles, Fuel and Propane; handling, stowage arrangements & segregation discussed - Temporary personnel heating devices safe, access clear and adequate ventilation provided...........................   |  |

| 10 | Barricades A B |
| --- | --- |
| a. Floor and wall openings are substantially planked over or barricaded...............................................................   b. Roadways or walkway hazards effectively barricaded .......................................................................................   c. Barricades are illuminated at night.....................................................................................................................   d. Excavations are properly protected to Class I, II or III levels of protection.........................................................   e. Traffic control devices used when appropriate...................................................................................................   |  |

| 11 | Hand Tools A B |
| --- | --- |
| - Proper tools for the proper job required onsite (no adjustable wrench-hammers, screwdriver pry bars, etc)   - Safe carrying practices used, no throwing tools, free fall to lower floors, lowering by attached electrical cords   - Company and employees' tools inspected daily and properly maintained .........................................................   d. Damaged tools removed from job site or repaired/replaced promptly................................................................   |  |

| 12 | Power Tools A B |
| --- | --- |
| a. Good housekeeping where tools are used ........................................................................................................ …  b. Tools and cords in good condition, proper grounding, plugs and attachment are checked daily .........................   c. Safety goggles or face shields worn as well as hearing protection ......................................................................   d. Employee trained in the proper use and inspection of tools and attached devices............................................   e. All mechanical safeguards and shields in use and properly adjusted ................................................................   f. Tools neatly stored when not in use ..................................................................................................................   - Right tool being used for the job at hand, all bits, blades, etc maintained sharp, straight and true ...................   H. Ground Faulty Interrupter Circuit system being used with all extension cords (as necessary) ..........................   |  |

| 13 | Powder-Actuated Tools A B |
| --- | --- |
| a. All operators have current qualified operator's card...........................................................................................   b. Tools and charges protected from unauthorized use / locked when not in use..................................................   c. Competent instruction, training before use and supervision provided................................................................   d. Tools checked and serviced regularly / tools inspected in good working order..................................................   e. Tools and attaching system used on recommended materials only...................................................................   - Safety goggles and/or face shields worn/ hearing protection worn / area posted hearing protection required ..   - Flying hazards checked by backing up, removal of personnel or use of captive stud tool .................................   |  |

| 14 | Fall Protection A B |
| --- | --- |
| a. Project Specific training held..............................................................................................................................   b. COMPETENT PERSON specified .....................................................................................................................   c. Required when fall hazard of 6 feet or greater exists.........................................................................................   d. Fall restraint types: guardrails, full body harness, warning line system with safety monitor..............................   e. Fall arrest types: full body harness, safety nets, catch platforms ......................................................................   f. Mechanical personnel lifts; safety requirements mobile platforms and aerial lift requirements ..........................   |  |

| 15 | Ladders A B |
| --- | --- |
| a. Ladders inspected and in good condition; removed from project site or tagged if broken/damaged..................   b. Straight ladders tied off at top, set back safe distance from vertical ..................................................................   c. Ladder extended 36" above the top of landing and tied off ................................................................................   d. Ladders properly placed away from support/building.........................................................................................   e. Stepladders fully open when in use ...................................................................................................................   f. Metal ladders not used for electrical work, near energized wiring or electrical hazards.....................................   g. Ladders not painted ...........................................................................................................................................   h. Ladder safety feet in use....................................................................................................................................   i. No standing on top two steps of ladder..............................................................................................................   |  |

| 16 | Scaffolding A B |
| --- | --- |
| a. Erection properly supervised: there are no "temporary" scaffolds, Competent Person identified .....................   b. All structural members meet safety factors, all connections secure...................................................................   c. Scaffold equipment in good working order.........................................................................................................   d. Scaffold tied in to the structure when required...................................................................................................   e. Working areas and walkways to be maintained free of clutter, debris, snow, ice and grease............................   f. Foot sills and mud sills provided ........................................................................................................................   g. Workers protected from falling objects...............................................................................................................   h. Scaffold plumb and square, with cross-bracing, fully supported and sturdy ......................................................   i. Guard rails, intermediate rails and toe boards in place, at proper height ...........................................................   j. Adequate, sound planking provided...................................................................................................................   k. Ropes/cables in good condition; access ladder tied, extended 36" above platform...........................................   |  |

| 17 | Welding & Cutting A B |
| --- | --- |
| a. Operators qualified ............................................................................................................................................   - Goggles, welding helmets, gloves, and clothing used as required. Screens and shields used where needed .   - Equipment in safe operating condition, electrical equipment grounded, power cables and hoses protected.....   d. Fire extinguishers of proper type (20lb) nearby .................................................................................................   - Hot work permit approved, surrounding area inspected for fire hazards, flammable materials protected or removed - Gas cylinders secured upright, cylinder caps in place when not in use, cylinders segregated in storage..........   |  |

| 18 | Demolition A B |
| --- | --- |
| - Demolition plan approved, Survey of project completed, employees trained on hazards expected...................   - Protection of adjacent structures, sidewalk and other public protection provided and barricades installed .......   c. Material chutes used. Floor opening for material disposal barricaded ..............................................................   d. Clear opening space for trucks and other vehicles ............................................................................................   |  |

| 19 | Electrical Installations A B |
| --- | --- |
| a. Assured Grounded Program in-place and followed ...........................................................................................   b. Ground Fault Indicating Circuits used, as necessary .........................................................................................   - Lockout/tagout and disconnect conductors when servicing/maintenance of equipment/machines ....................   - Adequate lighting, insulated and shields in place, protected from damage and weather...................................   - Terminal boxes, switches and breakers equipped with required covers, protected from weather .....................   - Proper training on Hazardous Energy Controls, site specific details discussed.................................................   |  |

| 20 | Motor Vehicles A B |
| --- | --- |
| a. NOTIFY SUPERVISOR & MAIN OFFICE IMMEDIATELY IF ANY PROBLEMS/QUESTIONS .........................   b. Operators qualified for vehicle in use, with valid state license ...........................................................................   c. Facility and state vehicle laws and regulations observed ..................................................................................   d. Weight limits and load sizes controlled ..............................................................................................................   e. SLOW MOVING VEHICLE signs used when required.......................................................................................   BASIC INSPECTION CONDUCTED BY DRIVER: Visual inspection of tires ....................................................................................................................................   All glass/mirrors in good condition .....................................................................................................................   Fluid levels, belts OK .........................................................................................................................................   First aid kit in place, fully stocked ......................................................................................................................   Fire extinguishers installed where required........................................................................................................   Brakes, lights, turn signals, wipers warning devices operative ..........................................................................   Panel warning lights/gauges operative ..............................................................................................................   Back-up signals provided...................................................................................................................................   SEAT BELT USE MANDATORY FOR ALL WHEN VEHICLE MOVING............................................................   |  |

| 21 | Heavy Equipment A B |
| --- | --- |
| a. NOTIFY SUPERVISOR & MAIN OFFICE IMMEDIATELY IF ANY PROBLEMS/QUESTIONS .........................   b. DO NOT OPERATE UNLESS PROPERLY INSTRUCTED FIRST....................................................................   c. Haul roads well maintained and laid out properly ..............................................................................................   d. Equipment is properly secured when not in use ................................................................................................   e. Maintenance records up to date ........................................................................................................................   BASIC INSPECTION CONDUCTED BY OPERATOR: Guards, fenders, tires, glass checked................................................................................................................   Wheels chocked when necessary......................................................................................................................   Test all operating controls before use ................................................................................................................   Back-up alarm/horn, lights, brakes, warning signals operative ..........................................................................   Ladders/stairs/handholds...................................................................................................................................   Proper grounding ...............................................................................................................................................   Shut-off devices on hose air lines, in case of hose failure .................................................................................   Noise arresters in use ........................................................................................................................................   Roll Over Protection System (ROPS) in place ...................................................................................................   Fire extinguisher & first aid kit............................................................................................................................   Seat belt use mandatory when equipment moving on equipment so provided ..................................................   |  |

| 22 | Hoists, Cranes & Derricks A B |
| --- | --- |
| a. Cranes and Derricks must be certified ...............................................................................................................   b. Cables and sheaves inspected? Slings and chains, hooks and eyes inspected before each use .....................   c. Equipment firmly supported. Outriggers in place ..............................................................................................   d. Crane operation area at a safe distance from all power lines ............................................................................   e. Proper loading for capacity at lifting radius. Rated load capacities posted? Load range chart available? .......   |  |

- All equipment properly lubricated and maintained, fluid levels normal, no leaks observed................................ 

- Crane Operator qualifications current, designated crane and rigging crew certified?

Signalmen where needed. ................................................................................................................................ 

- Signals posted, understood and observed Hazard signs posted and visible to operator ................................... 

i.  Inspection and maintenance logs maintained .................................................................................................... 

j.  Critical Lift Plan approved, on-site training scheduled, all members briefed ...................................................... 

k.  All operating controls tested before use. ........................................................................................................... 













| 23 | Industrial Equipment A B |
| --- | --- |
| - CALL SUPERVISOR & MAIN OFFICE IMMEDIATELY IF PROBLEM OR INCIDENT OCCURS .....................   - DO NOT OPERATE UNLESS PROPERLY INSTRUCTED FIRST (Fork lift operators must be trained & authorized)...............................................................................................   - Operators trained to inspect equipment before use at all times, broken welds, frayed cable/cords, fluid leaks………………………………………………………………………………………………………………….   - Rated load capacities posted. Proper loading for capacity at lifting radius known? ..........................................   e. Test all operating controls before use, check all fluid levels normal ..................................................................   f. Watch for hazards/obstructions, customer equipment & property, keep speed under 5 mph ............................   |  |

| 24 | Chain Saws & Cut-Off Saws A B |
| --- | --- |
| a. All operators trained- competent instruction and supervision provided ..............................................................   b. Saws protected from unauthorized use .............................................................................................................   c. Saws checked, in good working order, guards in place, blades tight.................................................................   d. Saws used on recommended materials only .....................................................................................................   e. Personal protective equipment required: safety goggles, hearing protection, gloves, chaps (for chain saws)..   |  |

| 25 | Masonry A B |
| --- | --- |
| a. Scaffolding procedures enforced, inspections by Competent Person................................................................   - Masonry saws properly equipped and grounded, dust protection provided, guards installed, blade tight..........   - Hoisting equipment in safe operating condition and used by qualified personnel ..............................................   d. Limited access zone established .......................................................................................................................   e. Walls over 8 feet in height adequately braced ...................................................................................................   |  |

| 26 | Steel Erection A B |
| --- | --- |
| a. Fall protection provided with safety nets, planked floors and/or personnel restraint devices .............................   b. Hard hats worn as required................................................................................................................................   c. Tools and materials secured from falling ...........................................................................................................   d. Fire hazards at rivet, forge and welding operations eliminated ..........................................................................   e. Floor openings covered or barricaded ...............................................................................................................   f. Unsafe ladders or ladders positioning, stairways, or other unsafe access points eliminated .............................   g. Hoisting apparatus inspected daily ....................................................................................................................   h. Employees prohibited from riding ball or loads ..................................................................................................   |  |

| 27 | Concrete Construction A B |
| --- | --- |
| a. Forms properly installed and braced..................................................................................................................   b. Adequate shoring, plumbed and cross-braced ..................................................................................................   - Shoring remains in place until strength is attained (expected days are = ) ...............................   - Proper curing period and curing procedures discussed and will be followed .....................................................   e. Heating devices checked for fire safety .............................................................................................................   - Mixing and transport equipment supported; traffic planned and routed, barricades in place .............................   - Adequate runways and ramps provided for concrete placement equipment .....................................................   h. Employees protected from cement dust ............................................................................................................   - Hard hats, boots, gloves, eye protection, and skin protection worn at all times .................................................   - Nails bent over or removed and stripped material removed from the area ........................................................   |  |

| 28 | Highway Construction A B |
| --- | --- |
| a. Laws and ordinances observed .........................................................................................................................   b. Competent flaggers properly dressed, instructed, posted..................................................................................   |  |

c.  Adequate traffic control devices used through construction area.......................................................................      

d.  Equipment cleared from right-of-way .................................................................................................................     

e. Adequate marking and maintenance of detours approaching construction .......................................................       f.   Dust controlled...................................................................................................................................................       g.  Adequate lighting for night crews .......................................................................................................................     

| 29 | Pile Driving A B |
| --- | --- |
| a. Stored piles properly secured ............................................................................................................................   b. Unloading only by properly instructed workers ..................................................................................................   c. Steam lines, leads, etc., in safe operating condition ..........................................................................................   d. Pile driving rigs properly supported, outriggers in place ....................................................................................   e. Cofferdams maintained and inspected ..............................................................................................................   f. Adequate pumping available..............................................................................................................................   |  |

| 30 | Excavation/Trenching/Shoring A B |
| --- | --- |
| - Excavation Plan approved, employees trained and safety precautions/procedures discussed .........................   - Competent Person specified, inspections conducted prior to, daily & as conditions change .............................   - Underground utilities located, documented notice to Customer/Client of location, safety precautions observed ........................................................................................................................................   d. Soil type determined (specify): A, B, C, UNKNOWN ........................................................................................   e. Confined space hazard(s) anticipated, ..............................................................................................................   - (Circle): toxicity, flammability, oxygen deficiency, mechanical, electrical, corrosive, temperature, noise, ionizing radiation - Adjacent structures properly shored, roads and sidewalks supported and protected ........................................   - Excavation shored or sides sloped as required if more than 4 feet in depth ......................................................   i. Trench shoring specifications ON SITE .............................................................................................................   - Material stored away from excavations, soils stockpiled away from slope/sides ...............................................   - Excavation barricades and perimeter guards, guardrails, warning signs and lighting adequate ........................   - Ladders provided - maximum lateral travel 25 feet, tied off at top with adequate landing clearance .................   m. Equipment ramps available, designed by engineer ...........................................................................................   n. Equipment a safe distance from edge of excavation .........................................................................................   o. Observer/Slope Monitor provided as required, properly trained.........................................................................   p. Notify Site Supervisor, Main Office if accident or incident occurs ....................................................................   |  |

| 31 | Explosives A B |
| --- | --- |
| a. Qualified operators and supervision during all explosives operations................................................................   b. Proper transport vehicles as required by DOT and regulations ........................................................................   c. State and local laws and regulations observed..................................................................................................   d. Storage magazines constructed per regulations................................................................................................   e. Cases opened with wooden tools only...............................................................................................................   f. NO SMOKING signs posted and observed where appropriate ..........................................................................   g. Detonators tested before each shot...................................................................................................................   h. All personnel familiar with signals, and signals properly used at all times .........................................................   i. Inspection after each shot..................................................................................................................................   j. Proper protection and accounting for all explosives at all times ........................................................................   k. Proper disposition of wrappings, waste and scrap .............................................................................................   l. Nearby residents advised of blasting cap danger ..............................................................................................   m. Radio frequency hazards checked.....................................................................................................................   |  |

| 32 | Confined Space A B |
| --- | --- |
| - Confined Space training accomplished, all employees aware of hazards and dangers associate with spaces on project  - Employee training provided Confined space definition discussed, Permit Required Confined Space Plan discussed   - Permit Required Confined Space Plan approved, Competent Person specified................................................   d. Type(s) of hazards (circle all anticipated): .........................................................................................................   |  |

toxicity, flammability, oxygen deficiency/excess, mechanical, electrical, corrosive, temperature, noise, ionizing

radiation

- Space ventilated prior to entry, continuous flow of air into and out of space...................................................... 

- Exhaust discharges at least 10 feet from space opening, downwind – causes no other health hazard in area. 

- Space isolated, ignition sources controlled, inflow of fluid blocked or discussed ............................................... 

h. Entry controlled by (circle) attendant/observer, permit, other (specify) .............................................................. 

- Attendant/observer to remain in place until all workers out and entry secured .................................................. 

- Personal protective equipment required: hard hat, safety glasses, safety harness with lifeline, air-feed        respirator........................................................................................................................................................    

k.  If hot work, permit ON SITE, special precautions defined ................................................................................. 

l.  If flammable/combustible materials, special precautions defined ...................................................................... 

















| 33 | On-site Fuel Areas, Repair Shops &/or Lay down Areas A B |
| --- | --- |
| a. Fire hazards eliminated .....................................................................................................................................   b. Fuels and lubricants dispensed in a safe location..............................................................................................   c. Good housekeeping maintained ........................................................................................................................   d. Lighting adequate for work tasks .......................................................................................................................   e. Carbon monoxide vented to outside and adequate ventilation provided inside .................................................   f. All fuels and lubricants in proper containers ......................................................................................................   |  |

| 34 | Additional Site Specific Special Precautions/Conditions A B |
| --- | --- |
| a. ...........................................................................................................................................................................   b. ...........................................................................................................................................................................   c. ...........................................................................................................................................................................   d. ...........................................................................................................................................................................   e. ...........................................................................................................................................................................   f. ...........................................................................................................................................................................   g. ...........................................................................................................................................................................   h. ...........................................................................................................................................................................   i. ...........................................................................................................................................................................   j. ...........................................................................................................................................................................   k. ...........................................................................................................................................................................   l. ...........................................................................................................................................................................   |  |

| Signatures below indicate that all appropriate elements of the items checked above have been reviewed to the satisfaction of all parties concerned at the start of the project, and that the Subcontractor accepts responsibility for maintaining a safe working environment. By signing, the Subcontractor acknowledges understanding of and agrees to abide by all laws, regulations, standards, policies and practices as communicated, and that failure to comply may be reason for disciplinary action up to and including removal from the job site. |  |
| --- | --- |
| COMPANY NAME: |  |
| SUPERVISOR SIGNATURE: | DATE: |

NOTES:

# Subcontractor Safety Checklist Information

The following information will assist you in establishing what will be required, reviewed in each section:

Job Hazard Analysis Review: Each employer is required by law to perform a hazard review of each activity/position/job that his or her employees will or are performing. Analysis will define activities being performed, identify the sequences of work, the specific hazards anticipated and the control measures to be implemented to eliminate or reduce each hazard to an acceptable level.

Workmen's Compensation Insurance: Employers are required to provide adequate insurance to support their employees on the project.

Emergency Procedures: Advises your employees and fellow workers that there is a site emergency plan and supply them with the emergency action procedures, preventive measures, and emergency phone numbers.

Employee Orientation: Each and every employee will be given an orientation to familiarize them with the site safety program. Unless otherwise specified, each subcontractor is responsible for the orientation of his or her own workers, replacement workers, and new hires.

Review of Work Plan: If a subcontractor is involved in tasks that subject their workers to serious hazards, the subcontractor will supply us with a written safe work plan that assures protection against the hazard. This plan must be signed by a company representative and communicated to the workers involved in the task.

Visitors & Safety Equipment: Advises that any visitors to the site must be suitably protected from hazards.  They must wear hardhat and proper safety footwear while on the site.

Accident/Incident Investigations: Any injury to any of their workers must be investigated and reported to the Superintendent in writing

Safe Access Requirements: All employers shall provide and ensure safe access throughout the project site is maintained. Advises where temporary removal of guardrails is necessary, that the area around them must be cordoned off with a barrier and workers within the barrier are required to wear and use fall restraint or fall arrest devices. Guardrails must be replaced as soon as possible or whenever the area is left unattended. Advise subcontractor about movement of material on or off landing platforms. All materials must be secured.  Platform gates or chains are to be kept closed at all times when a worker is on a platform.  If not possible the worker is to be tied off with a fall restraint system independent of the platform. Where a subcontractor provides their own platform, ensure that it has engineered drawings on site to show weights and load capacities.

Fall Protection - Scaffolds: Fall restraint or fall arrest protection is required where a fall of 6 feet or more is possible. Where a monitor system is implemented, a written verification of the plan is to be submitted by the subcontractor. Review proper scaffold building requirements: use all braces; full guardrails and toe-boards; tied in 4 times base dimension or use of outriggers or 26 feet vertical and 30 feet horizontal; and engineered over 125 feet in height (standard frame type).

Disciplinary Action Criteria: Explains the "Zero Tolerance" Program.

Tool Box Talks: Each subcontractor is required to conduct weekly or regular safety meetings with their workers and advise the Superintendent that they have been done.

Personal Protective Equipment Review: Advises that all workers are required ANSI Class "B" hard-hats, ANSI "approved construction" footwear, and that eye, ear, and respiratory protection will be required as the task and job hazard analysis dictates.

Housekeeping: Advise subcontractor of daily and as-needed cleanup requirements.

Material Handling/Storage, Flammable Material Storage: Addresses storage requirements and needs. Advises all workers about the storage areas and the proper handling of material so as not to endanger themselves or another worker. Materials shall be properly stacked and must be banded, chained, blocked or otherwise secured.  Access ways shall not be blocked.

GFCI: Advise subcontractor that all tools and cords are required to be grounded. GFCI or an assured grounding program is necessary to meet this action.

Protruding Rebar: Installers of reinforcing steel must protect the protruding hazard or make arrangements to have it protected. Removal of protective coverings is only allowed for task purposes.  Protective covering must be replaced as soon as possible.

Industrial Equipment: The equipment operation manual is required on site at all times and the operators must have received training in the safe operation of the equipment. All protective devices and warning signals work and the proper PPE requirements discussed.

Site Safety Committee: Each major project site will have a safety committee. Where a subcontractor's worker or supervisor is asked to participate, they shall.

Fire Protection: All trades involved in performing hot work of any kind are required to provide fire protection (fire-watch personnel, proper fire extinguisher, clear work area) at the project site.

Task Lighting: Review responsibilities of task-specific lighting and who provides it.

First Aide on Staff, Evacuation Route, and Hospital Location: Each subcontractor is required to have a first aid kit and a qualified first aid/CPR provider. Employer must submit the name of this provider. Advises each employer of the requirement to maintain and as necessary, post the OSHA 300 (series) log and summary. Advises employer to plan evacuation route for project site and the location of nearest medical assistance.

Crane/Riggers Competency: Review crane and rigging requirements for the project. Provide names and proof of qualified operators and riggers.  Certifications/permits on all cranes, types of equipment expected to be used on the project, manufacturer's data, manuals, etc.

Elevating Work Platforms and Aerial Lifts: All boom and scissors style lifts are required to be ANSI approved. The equipment manual is required on site at all times and the operators must have received training in the safe operation of the equipment. All warning devices and alarms are functional.

Hazards Communication Program: Receive verification that all necessary subcontractor workers are trained in the hazard communication program. Supply Material Inventory with supportive MSDS information on all hazardous chemicals and substances that will be brought on to the job site. Have subcontractor submit this information to the Superintendent along with storage locations.

# INITIAL SITE SAFETY ORIENTATION TRAINING

A = Applicable	B = Not Applicable to Project

| # | TOPIC | A B | # | TOPIC | A B |
| --- | --- | --- | --- | --- | --- |
| 1. | Safety Policy and Program - Project overview - Safety program overview - Safety meetings - Safety training requirements - Disciplinary action for safety infractions |   | 7. | Special Hazard Programs Safe Access Requirements Right to Know, HAZCOM Program Injury & exposure control Back safety Handling & storage of materials Hand tools Power tools Powder-actuated tools Jobsite information Fall protection Fire prevention Motor vehicles & training Ladders Scaffolding Barricades Hoists, cranes & derricks Lifts/equipment Heavy equipment & operations Ground Fault Indicator Circuit Assured Electrical Grounding Lockout/tag out Flammable gases & liquids Welding & cutting Chain saw & cut-off saws Parking Lot/Roadway construction Steel erection Concrete construction Masonry Demolition Pile driving Explosives Excavation/trenching/shoring Confined space Repair shops & garages |                                                                             |
| 2. | Personal Protective Equipment (P.P.E.) a. Employee demonstrates an understanding of correct PPE usage: Donning and care of required PPE. Cleaning & inspecting of issued PPE |   |  |  |  |
| 3. | Employee Safety Performance - Project overview - Safety program overview - Safety meetings - Safety training requirements - Disciplinary action for safety infractions |   |  |  |  |
| 4. | Accident & Injury Reporting - Project overview - Safety program overview - Safety meetings - Safety training requirements Disciplinary action for safety infractions |   |  |  |  |
| 5. | Work Rules - Project overview - Safety program overview - Safety meetings - Safety training requirements - Disciplinary action for safety infractions |   |  |  |  |
| 6. | Emergency Action Plan - Exit location & evacuation routes - Fire protection equipment - Location of emergency assembly area |   |  |  |  |
| 7. | Other Requirements |   |  |  |  |
| Signatures below indicate that all appropriate elements of the items checked have been reviewed and discussed. All employees have been made aware of what hazards to expect on the project, all associated site-specific plans and programs for this project. Their responsibility for maintaining a safe working environment. They acknowledge understanding of and agree to abide by all laws, regulations, standards, policies and safety practices. That they understand the required personal protective equipment requirements as communicated, and that failure to comply may be reason for disciplinary action up to and including permanent removal from the project site.. |  |  |  |  |  |
| _____________________________ ____________________________ _____________________________ _____________________________ ____________________________ _____________________________ _____________________________ ____________________________ _____________________________ _____________________________ ____________________________ _____________________________ |  |  |  |  |  |

# PROJECT START INDOCTRINATION CERTIFICATION CHECKLIST

##### SITE SPECIFIC - PERSONAL PROTECTIVE EQUIPMENT CERTIFICATION OF TRAINING / FIT-TEST / KNOWLEDGE

| COMPANY NAME: | DATE OF TRAINING: |
| --- | --- |
| Name & Signature of Supervisor: |  |

TRAINING OUTLINE:

  - All personnel have received initial site safety orientation training and have signed indicating an understanding.

  - A review of the Job Hazards anticipated on this project and the methods chosen to eliminate these hazards and protect the workers. Project anticipated engineering controls and personal protective equipment has been discussed

  - When and where the Personal Protective Equipment will be required.

  - Overviews of how to properly don, doff, adjust, wear and care for the prescribed PPE.

  - The correct procedures to be used/followed in inspection, testing, maintenance, storage as well as disposal of the PPE expected to be used on site.

  - Certification from each employee that he/she has been properly taught the function and purpose of the prescribed PPE. How to don, wear, adjust, inspect and maintained the PPE.

  - Certification that all personal PPE being used on site has been inspected by the Supervisor and found to be in good shape, not damaged, and that the PPE will protect the worker on the project site.

# SAFE ACCESS PLAN TRAINING

| COMPANY NAME: | DATE OF TRAINING: |
| --- | --- |
| Name & Signature of Supervisor: |  |

Scaffold Construction/Inspection Competent Person:

Print Name

TRAINING OUTLINE:

  - All personnel have received an overview of the requirements of the OSHA regulations on Safe Access & Fall Protection

  - Our policy and procedures related to Safe Access, Stairway, Ladders, Scaffolds, Aerial Lifts and Mobile Platforms

  - How to identify all fall hazards in the work area

  - The method(s) of fall arrest &/or fall restraint provided (if applicable)

  - The correct procedures for the assembly, maintenance, inspection and disassembly of the systems used

  - The correct procedures for handling, storing and securing of tools and materials

  - The correct procedures for donning, wearing and removal of fall protection equipment including inspection.

  - Proper construction of and required inspections to be conducted on all scaffold systems. Name of Competent Person.

# EXCAVATION/TRENCHING/SHORING PROGRAM

TRAINING

| COMPANY NAME: | DATE OF TRAINING: |
| --- | --- |
| Name & Signature of Supervisor: |  |

Excavation – Trench – Shoring Competent Person:

Print Name

TRAINING OUTLINE:

  - All personnel have received an overview of the requirements of OSHA Regulations 29 CFR 1926 Subpart P.

  - Our policy and procedures related to EXCAVATION - TRENCHING - SHORING

  - How to identify all hazards in the work area, safe access requirements, and need for personnel egress ladders.

  - The general & site-specific precautions that needs to be followed, including assignment of responsibilities & the designation of competent person.

  - Daily inspections and following major storm effects on upper slope condition and indications of pending slide.

  - The correct procedures for the handling, storing and securing of tools and materials above and in the excavation zone.

  - Methods for prompt and safe removal of injured workers & other emergency procedures.

  - Duty to notify Foreman or Superintendent and be available on the jobsite if an accident occurs.

# HAZARDOUS ENERGY CONTROL (LOCKOUT/TAGOUT)

TRAINING & AWARENESS

| COMPANY NAME: | DATE OF TRAINING: |
| --- | --- |
| Name & Signature of Supervisor: |  |

TRAINING OUTLINE:

  - All personnel have received an overview of the basic requirements of state regulations

  - How to identify all potential for exposure to electrical hazards associated with the project

  - The requirements, precautions and work practices of the energy control policy and procedures

  - The local naval facility outage request format

  - The correct procedures for handling, storing and securing of tools, equipment, supplies and materials

  - The methods of providing protection for workers who may be exposed to electrical hazards on the worksite

  - Review of emergency response and methods for prompt, safe removal of an injured worker

  - Instruction to each worker the requirements and methods of implementing the lockout/tagout controls, use of equipment and how to hand/maintain/remove tags and locks

# LEAD AWARENESS TRAINING

| COMPANY NAME: | DATE OF TRAINING: |
| --- | --- |
| Name & Signature of Supervisor: |  |

TRAINING OUTLINE:

  - All personnel have received awareness training that there is lead painted material present on site and the extent of its affect on the project.

  - The specific nature of the operations (demolition, remodeling, drilling, cutting, etc.)

  - Respiratory protection and its requirements (type of equipment, specific brand, model, size)

  - Personal protective equipment requirements (type of equipment, specific brand, model, size)

  - Engineering controls to be used to limit exposure

  - The Environmental Compliance Plan to be used, methods of disposal and type of disposal containers to be used.

  - The employees right of access to records pertaining to lead exposure

# ASBESTOS AWARENESS TRAINING

| COMPANY NAME: | DATE OF TRAINING: |
| --- | --- |
| Name & Signature of Supervisor: |  |

TRAINING OUTLINE:

  - All personnel have received awareness training that there is asbestos containing material present on site and the extent of its presents as it affects the project.

  - The specific nature of the operation (demolition, remodeling, drilling, cutting, etc.)

  - Respiratory protection and its requirements (type of equipment, specific brand, model, size)

  - Personal protective equipment requirements (type of equipment, specific brand, model, size)

  - Engineering controls to be used to limit exposure

  - The Environmental Compliance Plan to be used, methods of disposal and type of disposal containers to be used.

  - The employees right of access to records pertaining to asbestos exposure

# HAZARDS COMMUNICATION PROGRAM

TRAINING

| COMPANY NAME: | DATE OF TRAINING: |
| --- | --- |
| Name & Signature of Supervisor: |  |

TRAINING OUTLINE:

  - All personnel received an overview of the basic requirements of the "Right to Know" Program, MSDS and Material Inventory information.

  - Our policy and procedures related to Hazards Communication and proper labeling, review of MSDS

  - How to identify, safety precaution associated with, emergency response for all material expected to be used on the project site or in the work area

  - The method(s) of containment, labeling, storage, and handling of expected material.

  - Review of the procedures to be used/followed in the advent of a spill or accident on the project site.

  - The correct procedures for handling, storing and securing of hazardous materials, use of the Material Inventory Sheet.

  - The correct procedures for donning, wearing and removal of proper personal protection equipment including inspection.

  - Methods for prompt, safe removal and treatment of injured workers.

# DEMOLITION PLAN TRAINING

| COMPANY NAME: | DATE OF TRAINING: |
| --- | --- |
| Name & Signature of Supervisor: |  |

TRAINING OUTLINE:

  - All personnel received an overview of the basic requirements for the Project.

  - The method(s) of containment, storage, and handling of expected waste streams and recycling materials.

  - Our policy and procedures related to fences, barricades and warning devices to protect workers and the public

  - The correct procedures to be used/followed in the advent of an unplanned collapse of the structure.

  - Hazards associated with the project.

  - The correct procedures for donning, wearing and removal of proper personal protection equipment including inspection.

  - Methods for prompt, safe removal and treatment of injured workers.

# ENVIRONMENTAL COMPLIANCE PLAN

TRAINING

| COMPANY NAME: | DATE OF TRAINING: |
| --- | --- |
| Name & Signature of Supervisor: |  |

TRAINING OUTLINE:

  - All personnel received an overview of the basic requirements of the Environmental Compliance Plan

  - Our policy and procedures related to Environmental Compliance for this project were discussed

  - How to identify all waste streams expected to be generated on the project site or in the work area

  - The method(s) of containment, storage, and handling of expected waste streams.

  - The correct procedures to be used/followed in the advent of a spill on the project site.

  - The correct procedures for handling, storing and securing of hazardous materials and hazardous waste to include empty containers and rags.

  - The correct procedures for donning, wearing and removal of proper personal protection equipment including inspection expected to be used per the Environmental Compliance Plan.

  - Methods for prompt, safe removal and treatment of injured workers.

# ENVIRONMENT AWARENESS TRAINING

| COMPANY NAME: | DATE OF TRAINING: |
| --- | --- |
| Name & Signature of Supervisor: |  |

TRAINING OUTLINE:

  - All personnel have received an overview of the basic requirements for proper waste management on the project

  - The method(s) of containment, storage, and handling of expected waste streams and recycling materials.

  - Our policy and procedures related to waste handling, disposal, the uncovering of artifacts on customer facilities.

  - The correct procedures to be used/followed in the advent of uncovering artifacts on the project site.

# PROJECT SPECIFIC

NEW HIRE ORIENTATION CHECKLIST

A = Applicable	B = Not Applicable to Project

| # | TOPIC | A B | # | TOPIC |  | A B |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Safety Policy and Program - Project overview - Safety program overview - Safety meetings - Safety training requirements - Disciplinary action for safety infractions |   | 7. | Special Hazard Programs Safe Access Requirements Right to Know, HAZCOM Program Injury & exposure control Back safety Handling & storage of materials Hand tools Power tools Powder-actuated tools Jobsite information Fall protection Fire prevention Motor vehicles & training Ladders Scaffolding Barricades Hoists, cranes & derricks Lifts/equipment Heavy equipment & operations Ground Fault Indicator Circuit Assured Electrical Grounding Lockout/tagout Flammable gases & liquids Welding & cutting Chain saw & cut-off saws Parking Lot/Roadway construction Steel erection Concrete construction Masonry Demolition Pile driving Explosives Excavation/trenching/shoring Confined space Repair shops & garages |  |                                                                             |
| 2. | Personal Protective Equipment (P.P.E.) a. Employee demonstrates an understanding of correct PPE usage: Donning and care of required PPE. Cleaning & inspecting of issued PPE |   |  |  |  |  |
| 3. | Employee Safety Performance - How & when to report unsafe conditions - Safe Access requirements for project. - Wearing required P.P.E on site |   |  |  |  |  |
| 4. | Accident & Injury Reporting - How & when to report injuries - To whom to report injuries - Location of First Aid Supplies - Location of emergency communication systems |   |  |  |  |  |
| 5. | Work Rules - No horseplay - No fighting - Smoking policy - Good housekeeping - Proper lifting for back safety |   |  |  |  |  |
| 6. | Emergency Action Plan Exit location & evacuation routes Fire protection equipment Location of emergency assembly area |   |  |  |  |  |
| 7. | Other Requirements |   |  |  |  |  |
| Signatures below indicate that all appropriate elements of the items checked have been reviewed and discussed. The employee has been made aware of what hazards to expect on the project, all associated site-specific plans and programs for this project. His/her responsibility for maintaining a safe working environment. He/she acknowledges understanding of and agrees to abide by all laws, regulations, standards, policies and safety practices. That he/she understands the required personal protective equipment requirements as communicated, and that failure to comply may be reason for disciplinary action up to and including permanent removal from the project site. NOTE TO EMPLOYEE: Do not sign unless applicable topics are covered and all questions are satisfactorily answered. |  |  |  |  |  |  |
| New Hire Name & Signature: |  |  |  |  | Date of Hire: |  |
| Supervisor Signature: |  |  |  |  | Date of Training |  |

# PROJECT SITE WEEKLY SAFETY MEETING

Conducted By: 		Date & Time Held:

| ATTENDANCE ROOSTER |  |
| --- | --- |
| Print Name: Signature: | Print Name: Signature: |

SUBJECTS TO BE REVIEWED AT EACH SAFETY MEETING

Accident Prevention Program - Emergency Procedures – Safe Access Requirements - Accident Reporting/Investigation/Follow-up

Current Specific Jobsite Hazards – Hazards Materials & Chemicals in use

Discussion on Recent Accidents/Near Misses and Means of Prevention

Review of Monthly Safety Inspection Reports – Discrepancies noted/corrected – OSHA Inspections

Safety Committee Items - Ask for any Safety Suggestions – Safety Concerns – Safety Related Items from Committee Participants

| SELECTED SAFETY SUBJECTS DISCUSSED AT THIS MEETING |  |
| --- | --- |
| Hazard Communication; MSDS, Material Storage/Handling | Fall Protection Plan; Requirements/Equipment/Inspections |
|  | Lockout/Tagout System; Requirements/Lockout/Restoration |
| Personal Protective Equipment; Requirements/Cleaning/Maint |  |
|  | Good Housekeeping; Waste Handling/Site Cleanup |
| Tools & Power Equipment; Maintenance/Inspections | Ergonomic Measures; Back Care & Cumulative Trauma |
| Electrical Safety; GFIC/Assured Grounding/Inspections | First Aid and Fire Protection; Requirements/Inspection |
| Scaffolding and Ladder Safety; Setup/Inspections/Usage | Environmental Guide; Storm water Runoff/Airborne controls |
| Handrails, Guardrails, Toe-boards, Stairs; Setup/Inspection |  |
|  | Job/Activity Hazards Analysis; Review & Updates |

(Copy of Record to be submitted to MIAMI MEES Site Superintendent immediately following meeting)

# EMERGENCY AID & ASSISTANCE POSTING

(Post at Job Site)

{This section requires specific information from individual Delivery Order sites as to routes and phone numbers. Attach maps of area to show routes to Medical Treatment facilities.}

Emergency Telephone Numbers:

Police:	 Fire:	 Ambulance:

Call:

911

Hospital:

| LOCATION OF FIRST AID KITS Main Kit is located in the: Additional kits are located: Location of fire extinguishers: | EMERGENCY Call Check List: Your Name and Telephone #: & Project Location: Site Location of Injured Person: Cause of Accident or Injuries: Extent of Injuries (if known): Contaminated Material Involved: YES NO STAY ON PHONE UNTIL OPERATOR SAYS O.K. |
| --- | --- |

EVACUATION ROUTE MAP:

Project Site

[ATTACH DIRECTIONS AND EVACUATION MAPS FOR PROJECT.]

NAME OF NEAREST URGENT CARE FACILITY & HOSPITAL:

Tab 12.A.	ACTIVITY HAZARD ANALYSIS

Prepared by: 		Approved by:

Date: 		Activity:

| Activity | Potential Safety/Health Hazards | Recommended Controls |
| --- | --- | --- |

| Equipment to be used | Inspection Requirements | Training Requirements |
| --- | --- | --- |

# SAFETY TRAINING

  - The Subcontractor Foreman/Superintendent will conduct safety orientation for all workers on new project sites, any newly hired or reassigned employees and those given non- routine job assignments, utilizing the Safety Orientation Checklist as well as other company forms such as Certification of Training and Weekly Meeting Record.

    - The New Hire Orientation Safety Checklist shall be utilized to provide all new hires the safety awareness and requirements expected of them.

    - Documentation of the Safety Orientation, with signatures of all employee's and person conducting training, will be filed with the Quality Control/Safety Inspector.

  - Initial jobsite safety orientation training will be conducted by the Subcontractor Foreman or Superintendent utilizing the site-specific safety checklist and will include such things as:

    - Job safety hazards to be anticipated on the specific project

    - Environmental Hazards to be expected (Asbestos, Lead, Mercury, Soil)

    - Proper use, care, don and wear of personal protective equipment

    - Safe access requirements for the project

    - The Hazardous Communication Program; specific chemical hazards that the worker may be exposed to and methods of avoiding or neutralizing exposure

    - Review of the accident prevention and accident reporting procedures, emergency response and evacuation plan as well as mustering points.

    - Medical support, location of first aid kit, fire extinguishers, proper storage of material.

    - Documentation of the jobsite safety training, with signatures of employee and supervisor, will be filed with the MIAMI MEES Site Superintendent.

  - Weekly safety meeting (tailgate training, job-box talks, lunchroom briefings) will be conducted by the Subcontractor On-site Foreman or Superintendent

    - Will be held at least weekly in the manner most effective and appropriate for the work operations.

    - This training will be conducted in accordance with OSHA regulations.

    - Documentation of topics covered are to be recorded on the Weekly Safety Meeting form and will be filed with the MIAMI MEES Site Superintendent.

  - Specialized training will be conducted as the need arises in accordance with the regulations, and will be held in the manner most effective and appropriate for work operations. Documentation of training provided will be recorded on the Weekly Safety Meeting Record.

    - Such topics as the Site Posting Information such as the Work Site Safety Rules, Reporting Unsafe Conditions or Safety Suggestion Forms, Emergency Aid & Assistance Posting with evacuation routes and directions to hospital.

    - Utilize the completed site forms, held by the Site Superintendent, for a review of project safety.

# PROJECT SITE WEEKLY SAFETY MEETING

Conducted By: 	Date & Time Held:

| ATTENDANCE ROSTER |  |
| --- | --- |
| Print Name Signature | Print Name Signature |

SUBJECTS TO BE REVIEWED AT EACH SAFETY MEETING

Accident Prevention Program - Emergency Procedures – Safe Access Requirements - Accident Reporting/Investigation/Follow-up

Current Specific Jobsite Hazards – Hazards Materials & Chemicals in use

Discussion on Recent Accidents/Near Misses and Means of Prevention

Review of Monthly Safety Inspection Reports – Discrepancies noted/corrected – OSHA Inspections

Safety Committee Items - Ask for any Safety Suggestions – Safety Concerns – Safety Related Items from Committee Participants

| SELECTED SAFETY SUBJECTS DISCUSSED AT THIS MEETING |  |
| --- | --- |
| Hazard Communication; MSDS, Material Storage/Handling | Lockout/Tagout System; Requirements/Lockout/Restoration |
| Personal Protective Equipment; Requirements/Cleaning/Maint. | Good Housekeeping; Waste Handling/Site Cleanup |

| Tools & Power Equipment; Maintenance/Inspections | Ergonomic Measures; Back Care & Cumulative Trauma |
| --- | --- |
| Electrical Safety; GFIC/Assured Grounding/Inspections | First Aid and Fire Protection; Requirements/Inspection |
| Scaffolding and Ladder Safety; Setup/Inspections/Usage | Machine Guarding - Industrial Equip.; Requirements/Safety |
| Handrails, Guardrails, Toe-boards, Stairs; Setup/Inspection | Environmental Guide; Storm water Runoff/Airborne Controls |
| Fall Protection Plan; Requirements/Equipment/Inspections | Job/Activity Hazards Analysis; Review & Updates |

ITEMS DISCUSSED:

ACTION ITEMS TO BE TAKEN BEFORE NEXT MEETING - COMMENTS

(Copy of Record to be submitted to MIAMI MEES Site Superintendent immediately following meeting)

# WORKSITE SAFETY RULES (TO BE DISCUSSED MONTHLY)

- Accidents or injuries, regardless of their nature, shall be reported to the supervisor for immediate attention.

- Projecting nails shall be turned down or removed from lumber by all employees responsible for eliminating this hazard.

- Hard hats shall be worn on all MIAMI MEES Construction Project Sites and whenever there is potential for head injury from falling objects, shock or impact.

- Appropriate personal protective equipment shall be worn at all times AS REQUIRED.

- Excavations: The sides of all excavations 4’ or more in depth, where there is a danger of slides or cave-ins, shall be supported by substantial sheet piling and bracing, or other effective means, or sloped to the angle of repose. Personnel shall not cross over excavations without proper protection in place.

- Floor openings shall be guarded on all sides except stair openings by a standard railing and toe board or substantial cover.

- Scaffolding shall be constructed of metal or scaffold grade lumber. All platforms more than 4¢ above ground or floor shall have standard guardrails and toe boards on all open sides and ends.

- Ladders shall be constructed of metal or straight-grained lumber, with maximum length of a single ladder not to exceed 30 feet. Side rails shall extend not less than 36 inches above the landings and shall be securely fastened at the top. Ladders shall be used at a pitch so that the horizontal length from the top support to foot of the ladder is about ¼ of the working length of the ladder.

- Hand tools shall be maintained in a safe condition and used only for their intended purpose.

- Power tools shall be maintained in a safe condition with proper guards in place and used only by authorized personnel. Electric powered hand tools shall be grounded or double- insulated type.

- Use of any medication, which endangers an employee's ability to maintain a work safe environment, is prohibited at the jobsite.

- Use of alcohol or illegal drugs before or during work hours will result in immediate dismissal from the Project.

- Only employees who have been authorized through their company and who have undergone proper driver training indoctrination are to be allowed to drive on the project site/work area.

- ALL SAFETY RULES SHALL BE OBEYED. Violation of safety rules may be cause for immediate dismissal of any employee.

- HORSEPLAY is not allowed at any time and may be cause for immediate dismissal of any employee.

- All material shall be disposed of in accordance with the Environmental Protection Plan. Violation of these rules may be cause for immediate dismissal of any employee. Willful disregard or intent could be reported to proper authorities for their handling.

- The Project Site shall have SAFE ACCESS to and from the work areas, maintained free of potential accidents such as tripping hazards and falls, adequate project site cleanliness as well as the proper handling and storage of all supplies and material including flammable and combustible liquids and gases.

# Attachment 10

# PERSONAL PROTECTIVE EQUIPMENT

- GENERAL

  - Based on the hazard assessments, MIAMI MEES and its Subcontractor firms shall make available the prescribed personal protective equipment necessary to protect the employee from the anticipated hazards.

  - MIAMI MEES and its Subcontractor firms shall communicate the necessary PPE requirements to each affected employee and ensure that each person has received adequate training in the donning, wearing and inspection/maintenance of the prescribed PPE as well as had the PPE fit tested to their individual shapes/sizes.

  - All workers shall wear any prescribed PPE that may be required to maintain their exposure within acceptable limits.

  - Employees shall be physically able and medically qualified to use the personal protective equipment that may be required in their duties or for their position/job.

  - All subcontractors shall ensure that their employees required to wear PPE are trained to know:

    - When and what PPE is necessary.

    - How to properly don, doff, adjust, and wear PPE

    - Limitations of the PPE

    - Proper care, inspection, testing, maintenance, storage, useful life and disposal of PPE.

      - Each worker shall have demonstrated an understanding of this training and the ability to properly use the PPE before to working on a MIAMI MEES Project Site.

      - The Subcontractor shall present to the QC/Safety Inspector written certification that all workers understand the basic site PPE requirements as well as individual position/job/activity PPE requirements.

      - When workers are required to provide their own equipment the Subcontractor’s Onsite Superintendent shall present certification that he/she has personally inspected the PPE in question and has determined it to be adequate in protecting the worker against anticipated hazards on-site and that it is adequate state of repair for it function.

- PROJECT SPECIFIC REQUIREMENTS

  - MIAMI MEES requires personal protective equipment (e.g., industrial safety glass with side shields, hard hats, hearing protection, coveralls, boots, steel-toed shoes, aprons, gloves, respirators, etc.) to be used in the performance of work on the

project site and where posted. All equipment will meet the standards specified in OSHA 29 CFR 1900 series copies of which are included for reference purposes only.

  - All Subcontractor Employees, while working on MIAMI MEES Project Sites, shall observe these following rules regarding PPE:

- Hard hats will be worn on all construction sites and when working around cranes, when working in areas where overhead clearance is limited or where there is danger from falling objects.

- Heavy leather work shoes shall be worn on the job site, preference being for safety type footwear. Additional occupational foot protection may be required as identified in the hazard analysis or for specific industrial requirements.

- Shirts, pants, coveralls, sweaters, and jackets will be close fitting with no loose or flowing appendages. Pant legs will extend the full length of the leg and be free of tears. Working without shirts will not be permitted nor are shorts.

- Hands will be protected with mittens, gloves, or hand leathers when handling sharp or rough objects, except when working in close proximity of moving parts of machinery, such as drills, presses, and saws.

- Rings, wrist watches, bracelets, and the like will not be worn while working with moving machinery, on electrical equipment with exposed current-carrying parts, or in materials handling operations.

- Long hair will be appropriately secured.

- Eye protection will be worn when using grinders, drills, saws, hammering metal to include common nails, mowing grass/weed eating, or when there is any danger from flying objects or sparks.

  - All Employees on MIAMI MEES Project Sites shall:

- Use personal protective equipment in accordance with instructions and training received.

- Properly clean, maintain and store equipment to prevent damage and malfunction.

- Inspect personal protective equipment for damage prior to use.

- All respiratory equipment shall be properly fit-tested prior to each use.

- Report any damage or malfunction of safety equipment to the work place supervisor.

- Ensure hearing protection is used, as required.

- Ensure face shield protection is used, as required.

- Obey posted safety equipment requirements.

Attachment 11

# HEARING CONSERVATION PROGRAM

- Occupational Noise Exposure

  - Noise, or unwanted sound, is one of the most pervasive occupational health problems. It is a by-product of many industrial and construction processes. Sound consists of pressure changes in the medium (usually air), caused by vibration or turbulence. These pressure changes produce waves emanating away from the turbulent or vibrating source. Exposure to high levels of noise causes hearing loss and may cause other harmful health effects as well. The extent of hearing damage depends primarily on the intensity of the noise and the duration of the exposure. Noise induced hearing loss can be permanent or temporary. Temporary hearing loss results from short-term exposure to noise, with normal hearing returning after a period rest.  Generally, prolonged exposure to high noise levels over a period of time gradually causes permanent damage.

  - Our hearing conservation program has been designed to protect you, the worker who is working in the MIAMI MEES Project Sites with significant occupational noise exposures, from suffering hearing impairment even if you were subject to such noise exposures continuously over your working lifetime.

  - Warning signs shall be posted outside of the area of high noise levels (>115 dBA), areas where explosive-actuated tools are in use and as deemed necessary by Site Superintendent, Project Manager.

- Monitoring

  - The state regulations governing the hearing conservation program requires employers to monitor noise exposure levels in a manner that will accurately identify employees who are exposed to noise at or above 85 decibels (dB) averaged over 8 working hours or an 8-hour time weighted average (TWA). For those exceptionally noisy projects or project sites we shall contract with an outside professional testing laboratory to monitor several job positions that we assume will have the highest decibel levels. The results of this testing will be posted at the job site for your review. As a result of this testing we will determine what steps are needed to ensure that our workers are adequately protected from occupational hearing loss.

- Audiometric Testing

  - Federal and state regulations require all employees who are exposed to noise levels at or above 85 dB must be given audiometric testing as soon as possible and annually thereafter. This testing is to be provided to the employee at no cost. The basic elements in this type of audiometric testing program is the baseline audiogram, the annual audiogram's, employee training, and proper use of MIAMI MEES required hearing protection.

  - This type of audiometric testing program will not only determine the sharpness and acuity of your ears at the present time but will also allow for monitoring of any changes in hearing capability and the effectiveness of the hearing protection equipment over time.

- Baseline Audiogram's

  - The baseline audiogram is the reference audiogram against which future audiograms are compared. Baseline audiograms must be provided within 6 months of an employee's first exposure at or above an 8 hour TWA of 85 dB. Employees, however, must be fitted with, issued, and required to wear hearing protectors for any period exceeding 6 months after their first exposure until the baseline audiogram is obtained.

  - Baseline audiograms can also be a useful tool to demonstrate to the employee the necessity of wearing his or her hearing protection properly.

- Annual Audiogram's

  - Annual audiograms must be conducted within 1 year of the baseline. It is important to test hearing on an annual basis to identify any deterioration in hearing ability so that protection measures can be evaluated and changes made to prevent further progression of hearing loss.

- Hearing Protection

  - Hearing protection must be available to all workers expected to be exposed to noise levels of 85 dB, in an 8 hour TWA, or above. This requirement is to ensure that employees have access to hearing protection before they experience a loss in hearing.

  - Hearing protection is recommended to be worn at all times on the project site.

  - Hearing protection must be worn by:

    - employees for any period exceeding 6 months from the time they are first exposed to 8 hour TWA noise levels of 85 dB or above or until they receive their baseline audiogram's in situations where baseline audiogram's are delayed

    - employees who have incurred standard threshold shifts since these workers have demonstrated that they are susceptible to noise

    - employees exposed over the permissible exposure limit of 85 dB over an 8 hour TWA.

  - Hearing protection shall be worn by all employees;

    - in posted areas,

    - operating heavy industrial and mechanized equipment,

    - operating portable and stationary power tools, all power saws, pneumatic hammers and chisels

    - mechanics and laborers working around heavy equipment and trucks.

  - Earmuff hearing protection shall be worn by all operators of heavy industrial equipment, explosive-actuated tools and as determined by site management.

    - All employees in the above mentioned work areas must also wear the authorized hearing protection.

  - Failure to comply with the hearing protection requirements shall result in disciplinary action being taken, from an oral reprimand to dismissal from the job site.

- Training

  - Employee training is very important. When workers understand the reasons for the hearing conservation programs' requirements and the need to protect their hearing, they will be better motivated to participate actively in the program and to cooperate by wearing their protectors and taking audiometric tests.

  - Employees exposed to 85 dB and above must be trained at least annually in the effects of noise; the purpose, advantages, and disadvantages of various types of hearing protectors; and the purpose and procedures of audiometric testing.

- Record Keeping

  - Noise exposure measurement records must be kept for 2 years. Records of audiometric test results must be maintained for the duration of employment of affected employee.

- Regulations:  OSHA 29 CFR 1910.95

# Attachment 12

# SAFE ACCESS, FALL PROTECTION, WORK PLATFORMS

In order to comply with federal and state regulations and to ensure that information is available about the dangers of workplace assignments where safe access is required as outlined by the regulations, the following written program has been established. All affected employees will participate in the safe access plan.

- INFORMATION

Quality Control/Safety Manager is responsible for the establishment, implementation and maintenance of all aspects of this Safe Access Plan. Prior to the start of any work performed as part of a project at a specific jobsite, the Project Foreman and Site Superintendent will formulate a Safe Access Plan using the plan format.

- GENERAL

  - Safe access shall be provided to all work areas:

    - where there is a horizontal or vertical break of 19 inches or more in a route of access

    - accesses shall be kept clear of material and services

    - all work areas shall have at least one means of unencumbered access

  - Job-made means of access shall be designed and constructed to support at least four times the maximum intended load.

    - When loaded, planking and decking shall not deflect more than 1/60th of span length.

    - Width of access ways shall be sufficient to allow free passage of personnel and in no case less than 18 inches wide.

    - Supporting members and foundations shall be of sufficient size and strength to safely distribute loading. Foundations shall be firm and smooth to prevent lateral displacement.

    - Vertical members shall be plumb and braced to prevent swaying or displacement.

    - Planks shall be a maximum of 2" by 10" in size.  Planking shall be secured to prevent loosening, tipping or displacement and be supported or braced to prevent excessive spring or deflection. Planking shall be laid close together so that no space is available through which personnel, tools, material could fall.  Each plank shall lap its support by at least 12 inches.

  - Access ways shall be protected if personnel could be exposed to falling objects.

  - Access ways shall be inspected daily by Site Supervisory Personnel and/or Quality Control/Safety Inspector and maintained in a safe manner.

  - Workers shall be protected by standard guardrails, catch platforms, temporary floors, safety nets, personal fall protection devices, or the equivalent in the following situations:

    - on access ways or platforms 6 feet or higher

    - on access ways or platforms over water at any height

    - on runways 4 feet or higher

  - Each employee shall be trained on the hazards to safe access and fall protection needs be expected during performance of this delivery order at the start of work, at new employee indoctrination and when site conditions change.

- STANDARD GUARDRAIL

  - Standard guardrail system shall have a top rail (42 inches from surface), a mid- rail (18 inches from surface) as well as toe boards on all open sides and ends.

  - Standard guardrail shall be constructed of appropriate material of sufficient size to withstand 200 pounds of force in all directions in accordance with the 29 CFR 1926.

- PERSONAL FALL PROTECTION DEVICES AND SAFETY NETS

  - Personal fall protection devices, independently attached or attended, or safety nets shall be used when performing such work as listed below if guardrails can not be employed:

    - work in hoppers, bins, silos, tanks, or other confined spaces

    - work on hazardous slopes, structural steel, poles

    - erection and dismantling of safety nets

    - tying reinforcing bars

    - work from boatswain chair, swinging scaffolds, or other unguarded locations at elevations > 6’

    - work on skips and platforms in shafts

  - Personal fall arrest systems shall be constructed in accordance with OSHA 29 CFR 1926

  - Safety and debris nets shall be inspected and constructed by competent person in accordance with manufacturer recommendations.

    - Nets shall be inspected at time of construction and weekly thereafter. Results of these inspections shall be included in the daily production reports sent to MIAMI MEES.

  - Site Specific Fall Protection Plan

    - Shall identify all fall hazards on site

    - Shall list which system/method of fall restraint or arrest is being utilized.

| - Standard Guardrail - Scaffold with Guardrail | - Full Body Harness - Shock absorbing lanyard | - Mobile Platform - Aerial Lift |
| --- | --- | --- |

| - Warning Line & Monitor - Horizontal Lifeline - Secured to existing structure - Vertical lifeline & grab rope | - Retractable Lanyard - Double Lanyard System - Restraint System - Tie-off Points (5000 lb pull | - Catch Platform - Safety Net - Crane basket - Over water |
| --- | --- | --- |

    - Describe procedures for assembly, maintenance, inspection of system selected.

    - Describe procedures for working from system, use of tools and overhead protection.

    - Describe method for prompt, safe removal of injured workers.

- LADDER

  - All portable ladders shall be of sufficient length and shall be placed so that workers will not stretch or assume a hazardous position.

  - Portable ladders used as temporary access shall extend at least 3 feet past the landing.

  - Portable stepladders shall not exceed 20 feet in length, with a minimum width of 16 inches.

  - Ladders shall be at least 11.5 inches wide, with rungs at least 10 inches to 14 inches apart.

  - Ladders shall be surfaced so as to prevent snags and injury to workers, with slip resistant feet.

  - Wood ladders shall not be painted.

  - Stepladders shall have a spreader bar/locking device to hold ladder legs apart.

  - Ladders shall be inspected daily for any visible defects or damage. Broken or damaged ladders shall be removed from service and tagged.

  - Ladders shall be used for routine or regular maintenance and construction. No work requiring lifting of heavy materials or substantial exertion shall be done from a ladder.

  - Ladders shall not be moved, shifted, or extended while occupied

  - Set-up

    - Ladders shall not be placed in passageways, doorways, drives, or any locations where they may be displaced by traffic unless protected by barricades.

    - Portable ladders shall be used at such a pitch that the horizontal distance from the top support to the foot of the ladder will not be greater than 1/4th the vertical distance between these points.

    - Ladders shall be secured by top, bottom, and intermediate fastenings as required to hold them rigidly in place and to support loads, which will be imposed on them.

    - The rungs of all ladders shall be set to provide at least 7 inches of toe space between the rung and the nearest interference.

    - The top of a non-self-supporting ladder shall be placed with the two rails equally supported unless special ladder support device is used.

  - Portable and Fixed ladders shall comply with the requirements in OSHA 29 CFR 1926.1052 and 29 CFR 1910.27.

- STAIRWAYS

  - On all structures 20 feet or more in height, a stairway shall be provided during construction.

  - Refer to OSHA 29 CFR 1926.1052 for guidance.

- WORK PLATFORMS

  - GENERAL

    - Manufactured work platforms shall be erected, used, inspected, tested, maintained, and repaired in accordance with the manufacturer’s operational manual by trained competent individuals and under the supervision of a designated individual responsible for job safety.

    - Work platforms shall comply with the appropriate requirements for access and fall protection. All requirements for safe access shall apply to work platforms and means of access. Standard railing and handrails for work platforms shall comply with the requirements for guardrails. The requirements for fall protection devices and safety nets shall be in compliance with those of 29 CFR 1926.450

    - Scaffolds and their components shall be capable of supporting without failure at least 4 times the maximum anticipated load. Scaffolds shall be secured to the wall or structure if they exceed four times the minimum scaffold base.

    - Scaffolds shall be plumb and level. Scaffolds shall bear on base plates, upon sills or other adequate foundations.

    - Working levels of work platforms shall be fully planked or decked. Work platforms shall be securely fastened to the scaffold.

    - All planking shall be either overlapped (by 12 inches) or secured from movement. Scaffold planks shall extend over their end supports not less than 6 inches, unless specifically manufactured with restraining hocks or equivalent, nor more than 12 inches. Planking on scaffolds shall extend from the toe boards to not more than 14 inches from the face of the building or structure. Planking shall be supported or braced to prevent excessive spring or deflection. It shall be secured and supported to prevent loosening, tipping, or displacement.

    - A ladder or stairway shall be provided for safe access and shall be affixed or built and so located that when in use it will not have a tendency to tip the scaffold.

  - METAL SCAFFOLDS AND TOWERS

    - Sections of metal scaffolds shall be securely connected and all braces shall be securely fastened.

    - Metal scaffolds and towers shall be constructed in accordance with the guidance provided in OSHA 29 CFR 1926 Subpart L

  - ELEVATING WORK PLATFORMS

    - Elevating work platforms and aerial lifts shall be designed and constructed in accordance with the appropriate American National Standard Institute standard.

    - Elevating work platforms shall be operated, inspected, and maintained as specified in the manufacturer’s operating manual for that equipment and as specified in 29 CFR 1926 Subpart L.

a. Only personnel qualified and certified to operate the equipment shall do so. Certification of all operators shall be provided to MIAMI MEES and maintained on site.

    - Before operating the work platform the operator shall:

- survey the work area for loose or soft ground, ditches, debris, overhead hazards, obstructions, electrical energy sources and other hazards

- ensure the work platform is on level, firm ground

- ensure the elevating platform is loaded properly

- inspect the work platform in accordance with manufacturer’s operational manual; checking brakes, fluid levels, outriggers, wheel stops and chocks

- Records of inspections conducted while the unit is on site shall be maintained on site. Daily inspections shall be indicated on the daily production reports to MIAMI MEES.

    - All boom-supported elevating work platforms shall be equipped with an alarm or other suitable warning device that will sound when the base is

    - The use of personal fall protection devices shall be as specified in the operating manual. If used, personal protective devices may only be secured to manufacturer-approved hard points.

  - VEHICLE-MOUNTED ELEVATING AND ROTATING WORK PLATFORMS AND AERIAL LIFTS

    - Vehicle-mounted elevating and rotating work platforms/aerial lifts shall be designed and constructed in accordance with ANSI standards.

    - Vehicle-mounted elevating and rotating work platforms shall be operated, inspected, and maintained as specified in the manufacturer’s operating manual for that equipment and in accordance with 29 CFR 1926 Subpart L.

- Only personnel qualified and certified to operate the equipment shall do so.

- Certification of all operators shall be provided MIAMI MEES and maintained on site.

    - An aerial lift truck shall not be moved when the boom is elevated to a working height unless specifically designed for such activity and in the process of work.

    - Workers shall always stand firmly on the floor of the basket and shall not site nor climb on the edge of the basket or use planks, ladders or other devices for work positioning.

    - Workers shall wear a full body harness with the lanyard properly attached only to the manufacture-provided harden points. Workers in the basket shall not attach themselves to adjacent structures or equipment.

    - Nothing shall be attached to the sides of the basket, the basket and boom shall not be used in place of a crane or other such lift device. Vehicle-mounted elevating and rotating work platforms shall not be loaded greater than manufacturer’s recommended loading for all phases of operation.

    - Workers shall not exit the basket while it is off the ground by greater than 12 inches.

    - If a problem occurs or the unit appears to be malfunctioning immediately return the aerial lift basket to the ground.

    - Vehicle-mounted elevating and rotating work platforms shall be operated from the basket and not by ground controls unless in an emergency to return the basket to the ground.

- TRAINING

Prior to starting work and thereafter in accordance with the regulations, each new employee will be given a health and safety orientation that includes the following:

- An overview of the requirements of the regulations

- Our policy and procedures related to Safe Access

- How to identify all fall hazards in the work area

- The method(s) of fall arrest &/or fall restraint provided (if applicable)

- The correct procedures for the assembly/disassembly, maintenance, and inspection of the system.

- The correct procedures for handling, storing and securing of tools and materials

- The correct procedures for donning, wearing and removal of fall protection equipment including daily and routine inspections.

- Methods for prompt, safe removal of injured workers

- Duty to notify Site Superintendent and be available on the jobsite if an accident occurs.

All affected employees will receive training on general ladder/stairways requirements to be followed in the ordinary course of work.

In addition, where work is performed as part of a project at a specific jobsite and ladder/stairways will be required, Project Foreman will provide jobsite-specific training prior to the start of the project. This jobsite training will be based upon the plan developed for that specific project referenced above.

- DOCUMENTATION

Where work is performed as part of a project at a specific jobsite, responsibility for maintaining a copy of the site-specific Safe Access Plan and all relevant documentation, shall be assigned to Project Foreman or Site Superintendent for the project.  All documentation will be filed within the MIAMI MEES Office.

Certification of “Competent Person” for scaffold construction, scaffold inspector, instructor for mobile equipment operation shall be in accordance with the applicable OSHA regulations.

# Tab 1

# HAZARD COMMUNICATION PROGRAM

1.1.	Purpose Statement:

It is the intent and commitment of MIAMI MEES, Inc. (MIAMI MEES) to provide a safe and healthy environment for all of our employee’s and subcontractor are alike. It is important that all employees be aware of chemicals or hazardous substances that might affect their health or create hazards, in use for this delivery order. The Hazard Communication Program has been established to insure a safe and healthy environment and to comply with state regulations regarding the handling and stowage of hazardous substances. To ensure that information is available on the dangers of all hazardous chemicals used on JOC work-sites, the following written Hazard Communication Program has been established. All MIAMI MEES, Inc. employees and their subcontractors shall participate in this program.

This written program will be kept at the Project Site as part of the Site Specific Safety Plan and will be available to all workers in accordance with applicable laws and regulations.

  - Company Policy

MIAMI MEES, Inc. is dedicated to the protection of its employees from exposure to hazardous substances. All employees and subcontractor personnel of MIAMI MEES, Inc. have the responsibility to accomplish their work in a safe manner.

It is the responsibility of each Project Manager AND Site Superintendent to implement this hazardous communication plan and ensure that the Material Inventory Sheet is on site prior to commencing work. The Site Superintendent as well as Subcontractor Site Supervisors are all responsible for the continual checks of the work operations and to ensure proper handling and stowage of hazardous substances. Each supervisor is responsible to correct any unsafe acts or conditions immediately. It is the responsibility of each employee to understand and adhere to the proper handling and stowage procedures addressed in this plan and to follow the instructions of the supervisors. Additionally, each employee shall bring to their supervisor’s attention any unsafe or hazardous conditions or acts that may cause injury to either themselves or any other employee.

  - Employee Training and Location of Written Copy:

The Subcontractor Site Foreman, MIAMI MEES Site Superintendent are responsible for the site employee-training program and will ensure that all elements specified below are carried out.

Each current employee and, prior to starting work, each new employee will attend a health and safety orientation session and will receive information and training on hazardous materials used in the work place.

A copy of the Written Hazard Communication Program will be available on the project site for review by any interested employee, following notification to their immediate supervisor.

  - General Information:

The Written Hazard Communication Program establishes a comprehensive program of chemical information dissemination flowing from the manufacturer through our company and ultimately to you the using employee. The Program is intended to cover health hazards and physical hazards of chemicals in use on the project site. All employees and fellow subcontractors, who are exposed or potentially exposed to such hazards, shall be covered under this Program.

The Written Program is part of a comprehensive program of chemical information to provide necessary information to our employees for their protection and well-being.  The program requires the ongoing cooperation and understanding of all employees for its continued success.

The information about hazardous chemicals and substances is provided to all exposed and potentially exposed employees through a variety of measures, including Material Safety Data Sheets (MSDS), chemical lists, container labels, placards, signs, training and education. They are available to employees while they work at our facility. Employees are instructed that questions or concerns about the program be brought to the attention of their immediate supervisor or MIAMI MEES Site Superintendent,

This program contains specific provisions regarding the MSDS system, container- labeling program, and non-routine task hazard warning. It also addresses the information and training methods, which will be used to inform contractors and their employees of hazardous chemicals they may be exposed to while they are in our facility or on our project sites. The program will detail what information is available to employees and where such information is stored.

The Site Foreman and MIAMI MEES Site Superintendent will ensure that any containers are labeled with either the original manufacturer’s label or with generic labels. All labels shall have a block for identity and blocks for the hazard warning for the material. For help in temporary labeling, please contact your supervisor or the Site Superintendent.

The Project Manager and Site Superintendent will ensure that a review of the labeling systems is conducted on a routine basis and updated as necessary.

  - Material Safety Data Sheets (MSDS):

In our efforts to have a Material Safety Data Sheet for every hazardous material or product used on our project sites, the following procedures will be followed:

    - The Project Manager shall be responsible for obtaining and maintaining the MSDS system for the project, in cooperation with the various subcontractors through the submittal program. He/she shall provide this information to the Quality Control/Safety Manager for implementation into the site safety plan.

    - The Quality Control/Safety Manager shall review incoming sheets for new and significant health/safety information. He/she will see that any new information is passed on to all employees on the project site at weekly site safety meetings.

    - Copies of MSDS information for all hazardous chemicals to which employees of the company may be exposed will be kept in the Site Specific Health & Safety Plan maintained on the job site.

    - MSDS will be available for review by all employees. If MSDS are not available or new chemicals in use do not have MSDS, immediately contact your supervisor or the MIAMI MEES Site Superintendent.

    - In the event of an occupational exposure to a hazard, immediately notify your supervisor or the MIAMI MEES Site Superintendent. If you are in a position to administer first aid, always consult the MSDS on the chemical(s) or material(s) involved in the exposure prior to touching the individual.

    - If a customer, or anyone else, requests a copy of a particular MSDS, the Project Manager or Site Superintendent will review and process this request.

  - Information contained in an MSDS:

Section 1:	Product or Material Identification:

Manufacturer's Name; Trade Name; Chemical Name; Synonyms and Family as well as Emergency Telephone numbers.

Section 2:	Hazardous Ingredients:

List of all chemical ingredients used

Section 3:	Physical Data & Chemical Characteristics:

Boil point; Specific gravity; Vapor pressure; Percent volatile; Evaporation rate; Solubility in water; Appearance & Odor

Section 4:	Fire & Explosion Hazard Data:

Flash point; Flammable limits; Extinguishing Agent; Special fire-fighting procedures; Unusual fire & explosion hazards

Section 5:	Reactivity Data:

Stability; Incompatibility; Hazardous decomposition products; Hazardous polymerization

Section 6:       Health Hazard Data:

Threshold limit; Effects of overexposure; First aid procedures

Section 7:	Precautions for Safe Handling and Use: Procedures for Spills; Waste disposal method; Handling and Storing Precautions

Section 8:        Control Measures:

Respiratory Protection; Ventilation; Personal Protective Equipment

  - Labeling:

It is expected that hazardous substances used on the project site will be maintained in the containers as received from manufacturers and material suppliers. Each job-site supervisor shall ensure that, prior to distribution; all containers received are labeled with a copy of the manufacturer’s label. If the need should arise, Site Foremen and Site Superintendent shall assure that all employees know that all secondary containers must be labeled with a copy of the original manufacturer’s label or with labels indicating the chemical identity and the appropriate hazard warning. Quality Control/Safety Manager will review the labeling procedures at the outset of each delivery order and periodically update them as required.

Temporary containers can be utilized for the shift but must be properly disposed of at the end of the workday. They cannot be used to store material from one shift to the next.

  - Orientation and Training:

The Quality Control/Safety Manager shall be responsible to ensure all workers have received site orientation and training. This shall be done prior to starting work for all employees and thereafter in accordance with the regulations for each new employee. This health and safety orientation shall include the following:

- An overview of the requirements of the hazards communication program

- Hazardous chemicals present on the job-site

- How to prevent or reduce exposure to hazardous chemicals through the use of control procedures, work practices and personal protective equipment

- How to read labels and review MSDS to obtain necessary information

- How to check for presence of labels on all containers, put correct labels on secondary labels as required and/or notify the superintendent if questions or problems arise

- Location of MSDS notebook and written program.

Prior to introducing a new chemical or other hazardous substance onto the job-site, each affected worker on the site will be given information and training as outlined above for the new hazard.

It is the responsibility of the Project Manager or Site Superintendent to provide all subcontractors, working on our project sites, with the following information:

- Hazardous chemicals to which they may be exposed while on the job site.

- Precautions their employees may take to lessen the possibility of exposure by usage of appropriate protective measures.

The Project Manager and Site Superintendent will be responsible for contacting each contractor before work is started and establish a meeting to disseminate any safety related information concerning chemical hazards that they might be bringing on to our project site.

  - Material Inventory Sheets:

An inventory list of all Hazardous Substances, expected to be used on the project site by MIAMI MEES or our Subcontractors, shall be included in this section. This Material Inventory shall discuss the material by name, usage, type of container, type of storage as well as any storage requirements in addition to amount on site and the amount expected to be used.

  - Information Updates:

Whenever pertinent and advisable, hazardous materials information updates will be scheduled in the weekly “job-site” training.

  - Reference:

OSHA 29 CFR 1926.59 and 29 CFR 1910.1200

Tab 2

# EXCAVATION, TRENCHING, SHORING PROGRAM

In order to comply with OSHA regulations and to ensure that information is available about the dangers related to working in man-made cuts, cavities, trenches or depressions, the following Excavation, Trenching, Shoring Program has been established. All affected employees of the company will participate in the Excavation, Trenching, and Shoring Program.

INFORMATION

Project Foreman or Site Superintendent is responsible for the establishment, implementation and maintenance of all aspects of the Excavation, Trenching, and Shoring Program. Prior to the start of any major excavation work performed as part of a project at a specific jobsite, the Project Manager and Quality Control/Safety Manager will approve of an Excavation Plan to be presented by a proper excavation firm. The Quality Control/Safety Manager shall review these procedures periodically and update them as required.

GENERAL

  - Prior to any excavation work all underground installations and utilities shall be located, marked and protected from damage or displacement.

  - Means of egress, such as ladder, stairway, or properly configured ramp, shall be provided on all trench excavations that are over 4 feet deep such that it will require no more than 25 feet of unimpeded lateral travel to reach the ladder.

  - When persons are in or around an excavation; the excavation, the adjacent areas, and the protective systems shall be inspected daily, as needed throughout the work shifts, and after every rainstorm or other hazard-increasing occurrence.

  - If evidence of a situation which could result in possible cave-in’s, slides, failure of protective systems, hazardous atmospheres, or other hazardous conditions are identified then all work shall stop, all workers evacuated from the area until all necessary safety precautions have been implemented and the Quality Control/Safety Manager concurrence to proceed.

  - In locations where oxygen deficiency or gaseous conditions are known or suspected, air in the excavations shall be tested prior to the start of work, during random intervals thereafter and a log maintained of the results.

  - The sides of all excavations in which employees are exposed to danger from moving ground shall be guarded by either a support system (shoring), sloping or benching of the ground, or some other equivalent means of protection as described in 29 CFR  1926 Subpart P.

  - When mobile equipment is operated adjacent to the excavation or required to approach the excavation a warning system shall be constructed to warn the operator on the limit of excavation. This warning shall be placed a suitable distance back from the edge to provide adequate response.

  - Employees shall not work in excavations in which there is accumulated water, or in excavations in which water is accumulating unless adequate precautions, such as special support/shield systems, water removal pumps, use of safety harness and lifelines, have been taken to protect employees.

  - Excavations below the level of the base or footing of a foundation, wall or undermining of sidewalks and roadways shall not be allowed without special controls and procedures submitted by a professional engineer.

  - Diversion dikes, ditches, or other means shall be used to prevent surface water from entering the excavation.

  - Excavations below the level of the base or footing of a foundation, wall or undermining of sidewalks and roadways shall not be allowed without special controls and procedures submitted by a professional engineer.

  - Excavated material shall be placed at least 3 feet from the edge of the excavation or shall be retained by devices, which are sufficient to prevent the materials from falling into the excavation.

  - Employees entering excavations classified as confined spaces or which otherwise present the potential for emergency rescue shall wear a harness with a lifeline securely attached to it.

  - Competent person, Quality Control/Safety Inspector, shall inspect all excavations on a routine basis IAW guidelines provided in 29 CFR 1926.261

TRAINING:

Prior to starting work and thereafter in accordance with the regulations, each affected employee will be given a health and safety orientation that includes the following:

    - An overview of the requirements of the regulations

    - Our policy and procedures related to Excavation, Trenching, Shoring Program

    - How to identify all hazards in the work area

    - The general & special precautions to be followed, including assignment of responsibilities & designated competent person

    - The requirements & correct procedures for the use, maintenance, inspection and storage of personal protective equipment

    - The correct procedures for handling, storing and securing of tools and materials

    - Methods for prompt, safe removal of injured workers & other emergency procedures

    - Duty to notify Project Foreman or Site Superintendent and be available on the jobsite if an accident occurs

All affected employees will receive training on general excavation/trenching/ shoring requirements to be followed in the ordinary course of work. In addition, where work is performed as part of a project at a specific jobsite and work in excavations where excavation/trenching/shoring requirements apply, Project Foreman will provide jobsite-specific training prior to the start of the project. This jobsite training will be based upon the plan developed for that specific project referenced above.

SAFE ACCESS

  - Protection shall be provided to prevent personnel, vehicles, and equipment from falling into the excavations

  - Excavations shall be backfilled as soon as possible upon completion of the work.

  - Walkways or bridges with standard guardrail system shall be provided where people or equipment are required to cross over the excavation.

  - Ladders shall be provided for safe access in all excavations over 4 feet in depth.

  - Ladders shall be provided ever 25 feet of unimpeded lateral travel.	If obstructions exist this distance shall be shorten as directed by the Competent Person.

  - Protection shall be provided according to the following:

    - If the excavation is exposed to members of the general public (other individuals not associated with the project) or to vehicular traffic or equipment, then Class I perimeter protection shall be employed.

    - If the excavation does not meet the requirements for Class I but it is:

      - routinely exposed to other employees or

      - either is deeper than 5 feet or contains hazards (impalement hazards, hazardous substances, etc.) then Class II perimeter protection shall be employed.

    - If the excavation does not meet the requirements for either Class I or class II then Class III perimeter protection shall be employed.

  - Class I Perimeter Protection:

    - A standard guardrail system (with upper and mid rails, toe board, proper post spacing) shall be constructed around the excavation to protect other employees from falling into the excavation.

      - For vehicle protection an engineer shall designing a perimeter protection system to withstand the potential forces and bending moments due to impact by traffic.

  - Class II Perimeter Protection

    - Consist of warning barricades or flagging placed at a distance not closer than 6 feet from the edge of the excavation. Warning barricades shall be displayed 3 feet to 4 feet above the ground.

  - Class III Perimeter Protection

    - Consist of warning barricades or flagging placed at a distance not closer than 6 inches or more than 6 feet from the edge of the excavation.

DOCUMENTATION:

Where work is performed as part of a project at a specific jobsite, responsibility for maintaining a copy of the site-specific Excavation, Trenching, Shoring Program and all relevant documentation, shall be assigned to the Project Foreman or Site Superintendent at the project jobsite. Upon completion of the project, all documentation will be filed at MIAMI MEES office. Documentation of new hire and periodic training, and all other program requirements, will be maintained at the at the MIAMI MEES office.

SELECTION OF PROTECTIVE SYSTEMS:

Is the excavation more than 5 feet deep or in stable rock?

If NO

Is there potential for cave-in?

YES	--- Excavation must be sloped, shored, or shielded

NO	--- Excavation can be made with vertical sides

If YES

Is the excavation entirely in stable rock?

NO	--- Excavation must be sloped, shored, or shielded

REFERENCE:

YES	--- Excavation can be made with vertical sides

For additional details on:

- Soil Classification

- Sloping and benching

- Shoring

- Support System Construction Refer to:

(1)   OSHA 29 CFR 1926.652

Tab 3

MACHINERY AND MECHANIZED EQUIPMENT

I.	GENERAL

  - Inspections and test shall be in accordance with the manufacturer’s instructions.

    - All machinery or mechanized equipment before it is placed in use, shall be inspected and tested to confirm proper operation.

    - Daily all machinery or mechanized equipment shall be inspected to ensure continued safe operation.

    - Test shall be made at the beginning of each shift to determine that the brakes and operating system are in proper working order and that all required safety devices are in place and functional.

    - Record of tests and inspections shall be maintained on the Production Reports filed daily with MIAMI MEES Site Superintendent.

  - Whenever any machinery or mechanized equipment is found to be unsafe or defective it shall be placed out of service, tagged as such and its use prohibited until conditions has been corrected. When corrections have been made, the machinery or mechanized equipment shall be retested and inspected before returning it to service.

    - No modifications or additions, which affect the capacity or safe operation of the machinery or equipment, shall be made without the manufacturer’s written approval and recertification.

  - Machinery or mechanized equipment shall be operated only by designated qualified individuals.

    - Machinery or mechanized equipment shall not be operated in a manner that will endanger persons or property nor shall safe operating speeds or loads exceeded.

    - Getting off or on machinery while it is in motions is prohibited. The parking brake shall be applied each time before exiting the vehicle.

    - Machinery or mechanized equipment shall be operated in accordance with manufacturer’s instructions and recommendations.

    - The use of headphones for entertainment purposes while operating equipment is prohibited.

  - Equipment requirements

    - Seats or equal protection must be provided for each person required to ride on equipment.

    - All self-propelled construction and industrial equipment shall have lights, horns, reverse signal backup warning device and an operational service brake system cable of holding a fully loaded equipment on expected grade conditions.

    - All vehicles, which will be parked or operated moving slowly, shall have yellow flashing light visible from all directions.

  - Maintenance and repairs

    - Maintenance, including preventive maintenance and repairs shall be in accordance with manufacturer’s recommendations. These repairs shall be documented in the Daily Production report and made available to designated authority.

    - All machinery or mechanized equipment shall be shut down and positive means taken to prevent its operation while repairs, maintenance, or manual lubrications are being done.

    - Heavy machinery, equipment, or parts thereof which are to be suspended or held apart by slings, hoist, or jacks shall be substantially blocked or cribbed before personnel are permitted to work underneath or between them.

    - Repairs and corrective maintenance shall only be accomplished by properly trained and designated personnel.

  - Bulldozer and scraper blades, end-loading buckets, dump bodies and similar equipment shall be either fully lowered or blocked when being repaired or when not in use.  All controls shall be set in a neutral position, with the engine stopped and brakes applied, unless work being performed requires otherwise.

  - Stationary machinery and equipment shall be placed on a firm foundation and secured before being operated.

  - All mobile equipment and the areas in which they are operated shall be adequately illuminated while work is in progress.

  - Equipment powered by internal combustion engines shall not be operated in an enclosed area unless adequate positive ventilation is provided to ensure the equipment does not generate a hazardous environment.

  - Mechanized equipment shall be shut down before and during fueling operations.

  - All towing devices used on any combination of equipment shall be structurally adequate for the weight drawn and securely mounted.

  - Parking

    - Whenever equipment is parked the parking brake shall be applied.

    - Equipment parked on an incline shall have the wheels chocked or track mechanism blocked in addition to the parking brake being set.

  - All industrial trucks shall meet the requirements of design, construction, stability, inspection, testing, maintenance, and operation as defined in ANSI/ASME B56.1.

  - Lift trucks, stackers, and similar equipment shall have the rated capacity posted on the vehicle so as to be clearly visible to the operator and others. This rating shall not be exceeded.

  - The controls of loaders, excavators, or other similar equipment with folding booms or lift arms shall not be operated from a ground position unless so designed.

  - Personnel shall not work under, pass under or ride in the buckets or booms of loaders in operation.

  - All machinery and mechanized heavy equipment shall be equipped with at least one dry chemical fire extinguisher with a minimum rating of 5-BC as well as a small 5-person first aid kit.

GUARDING AND SAFETY DEVICES

  - A warning device or signal person shall be provided where there is danger to persons from moving equipment, swinging loads, buckets, booms, etc.

  - Guarding

    - All belts, gears, pulleys, sprockets, spindles, drums, flywheels, chains, or other reciprocating, rotating or moving parts of equipment shall be guarded when exposed to personnel or create a hazard.

    - All hot surfaces of equipment shall be protected by guards or insulated to prevent injury and fires.

    - Platforms, foot walks, steps, handholds, guardrails and toe boards provided by the manufacturer shall be designed, constructed, and maintained to provide safe footing and access way for the operator.

    - Substantial overhead protection shall be provided for the operators of forklifts, loaders and similar material handling equipment.

  - Tires shall be mounted, dismounted, inflated by properly trained, designated personnel.

  - No guard, safety appliance, or device shall be removed from machinery or equipment, or made ineffective unless done so during maintenance periods.

All guards, safety appliance, or devices shall be fully operational after such periods.

  - Seatbelts and anchorage’s meeting the requirements of 49 CFR 571 shall be installed and worn in all motor vehicles.

2.5 	Suitable protection against the elements, falling or flying objects, swinging loads and similar hazards shall be provided for operators of all machinery or equipment.  All glass used in equipment shall be a safety glass style.

  - Falling Object Protective structures

    - All bulldozers, tractors, or similar equipment used in clearing operations shall be provided with guards, canopies, or grills to protect operator from falling or flying objects as appropriate to the nature of the clearing operations.

    - Falling object protective structures for other construction, industrial, and grounds-keeping equipment will be furnished when the operator is exposed to falling object hazards.

  - Falling Object Protective structures

    - All bulldozers, tractors, or similar equipment used in clearing operations shall be provided with guards, canopies, or grills to protect operator from falling or flying objects as appropriate to the nature of the clearing operations.

    - Falling object protective structures for other construction, industrial, and grounds-keeping equipment will be furnished when the operator is exposed to falling object hazards.

  - Rollover Protective Structures

    - In addition to seat belts and falling object protection, Rollover Protective Structures shall be installed on:

      - crawler and rubber tire tractors, bulldozers, front-loaders, back- hoes

      - off-the-highway self-propelled pneumatic tired earth movers; trucks, scrapers, end dumps and bottom dumps

      - motor graders, compactors, rollers

      - water tank trucks

    - Modifications, repairs, or field welding on Rollover Protective Structures shall be accomplished only by certified workers.

3.	REFERENCES

OSHA 29 CFR 1926.600

Tab 4

# PRESSURIZED CYCLINDERS AND EQUIPMENT

- GENERAL

  - Check and certification must be done to ensure that state and local controls do not apply to these types of installations.

  - Pressurized equipment and systems shall be inspected and performance tested before being placed in service and after any modification or repair.

  - Portable or temporary pressurized equipment and systems shall be inspected at intervals of not more than six months and permanent installations shall be inspected at least annually.

  - Personnel certified by the ASME Code or the National Board of Boiler and Pressure Vessel Inspectors will perform all inspections and tests.

  - Inspections of pressure vessels shall be in accordance with:

    - (For unfired units prior to operations) AMSE Boiler and Pressure Vessel Code

    - (For operational units) National Board of Boiler and Pressure Vessel Inspectors, “National Board Inspection Code”.

  - Any pressurized equipment or system found to be defective or unsafe shall be tagged “Out of Service” and not used.

  - Pressurized equipment and systems shall be operated and maintained by properly qualified, designated individuals.

  - No safety device or appliance shall be removed or made ineffective while system r component is operational.

  - A pressure gauge, safety and relief valve shall be provided on all pressurized systems and equipment.

  - Quick makeup connections shall be secured with safety lashing or pins.

- COMPRESSED AIR AND GAS SYSTEMS

  - Standards

    - Air receivers shall be constructed in accordance with ASME Code for Unfired Pressure Vessels

    - All safety valves shall be constructed, installed, maintained and tested in accordance with the same code.

  - Setup, operation of compressed air systems and equipment shall in accordance with manufacturer’s technical manual.

  - Compressors and related equipment shall be located to provide safe access for operation and maintenance while at the same time providing some protection to worker in the area in case of failure.

  - Air hoses, piping valves, filters and other fittings shall be pressure rated for the system operating pressure.

  - Hoses shall not be laid over flooring, ladders, steps, scaffolds, or walkways in such a way to create a tripping hazard.

  - Compressed air shall not be used:

    - To clean hands, face, or clothing

    - For any other cleaning purpose unless pressure is reduced to <30 psig.

  - A stop valve shall be provided between the pressure source and each piece of equipment or at each air hose connection.

  - Quick makeup connections shall be secured with safety lashing or pins.

  - No tool change, hose replacement or repair work shall be done until the stop valve in the supply line has been closed.

- BOILERS AND SYSTEMS

  - Boilers and other steam systems shall be operated in accordance with manufacturer’s technical manual.

  - Boilers, which have undergone major repairs or modifications, shall be recertified prior to operation.

- COMPRESSED GAS CYLINDERS

  - All cylinders shall be properly identified as to the gas they contain.

  - Storage

    - Cylinders shall be stored in well-ventilated locations

    - Cylinders containing the same gas shall be stored in a segregated group; empty cylinders shall be stored in the same manner.

    - A distance of 50 ft shall separate cylinders in storage from flammable or combustible materials. Smoking or hot work shall be prohibited within 50 feet wherever cylinders are stored, handled, or used.

    - A distance of 25 feet shall separate cylinders containing oxygen or oxidizing gases from cylinders in storage containing fuel gases.

    - Areas containing hazardous gas in storage shall be appropriately placarded with warnings.

    - Acetylene cylinders shall never be turned or laid on to their side.

    - Compressed gas cylinders shall be stored in their upright position.

  - Compressed gas cylinders shall only be transported in approved cradles, racks, hand trucks

  - Cylinders shall be protected from physical damage by ensuring they are properly supported, chained/retained in the area of the upper half on the cylinder to a solid structure capable of holding the cylinder weight. They shall be protected from electric current and extreme temperatures; the maximum temperature for any cylinder is 130 F.

  - Cylinders containing oxygen and acetylene or other fuel gas shall not be taken into confined spaces.

  - Cylinders in use shall be secured in substantially fixed or portable racks, cradles, or approved hand trucks.

  - Cylinder valves, and valve caps

    - Cylinder valves shall be kept closed when cylinders are not in use.

    - Cylinder valve caps shall be properly screwed into place whenever regulators are not installed.

    - Valve wrench shall be in operating position whenever cylinder is in use.

  - Leaking cylinders shall be moved to an isolated location out of doors, the valve slowly cracked open and gas allowed to escape slowly. The area shall be secured for distance of 50 ft and all personnel and sources of ignition shall be kept clear. The cylinder shall be labeled “defective” and tagged.

  - Oxygen cylinders and fittings shall be kept free of oil and grease.

  - All gas pressure regulators, including their gauges, shall be in proper working order.

Tab 5

# BLASTING PLAN

At the present time we do not anticipate any need for explosives on any of our projects under this contract. When required by the individual Delivery Order, MIAMI MEES, Inc. will employ the services of a qualified Commercial Blasting Operation who will be required to comply with the rules and regulations as discussed in the references listed below.

References:  OSHA 29 CFR 1926 Subpart U

Tab 6

# RESPIRATOR PROTECTION PROGRAM

REFERENCE: OSHA 29 CFR 1926.103

GENERAL INFORMATION

In order to comply with federal and state regulations and to ensure that all employees are protected by the requirements of those regulations the following policy and plan has been established. All employees and subcontractors shall follow the Respirator Program. This program will be part of the Site Specific Safety Plan at the project site and will be available to all workers.

Airborne contaminants that might be hazardous to our workers on the project site are removed from the work site through various means such as natural circulation and mechanical ventilation wherever possible. In some instances, these systems are overcome by the amount of material in the atmosphere. Consequently, it is necessary for workers in these areas to be fitted with and wear respiratory protection against occupational dusts, fumes, gases and vapors. The primary objective is being to prevent excessive exposure to these contaminants by our employees.

The guidelines contained herein form the basis upon which our Respiratory Protection Program is organized. Its purpose is to define the rules under which workers on our job sites will wear and maintain respirators for personal protection against airborne contaminants.

These rules are mandatory and most suited to our company's operations. As such they are required of all workers, both MIAMI MEES, subcontracted firms, and visitors, on our project sites.

MEDICINAL LIMITATIONS

A designated individual, for MIAMI MEES this is the Quality Control/Safety Manager, shall determine whether or not a person may be assigned a task requiring the use of a respirator. Prior to being assigned duties involving the wearing of any respirator, the employee will complete a respirator Medical Screening Form and, as appropriate, shall be screen by a company retained consulting physician.

Persons answering "yes" to any questions contained in the Medical Screening Form shall not be assigned tasks requiring the use of a respirator until a qualified physician has determined that the person is physically able to wear the equipment and perform the task.

On an annual basis, employees assigned tasks involving the wearing of respirators shall complete a new Respirator Medical Screening form and persons answering "yes" to any question shall have his file reviewed and shall again be given a medical examination.

RESPIRATOR SELECTION

Only respirators approved by the National Institute for Occupational Safety and Health (NIOSH) or Mine Safety and Health Administration (MSHA) shall be used. No modification of an approved respirator is permitted.

Employees shall not provide their own respirators but shall use only those provided by their immediate supervisor.

RESPIRATOR PROTECTION PROGRAM

Respirators shall be selected on the basis of the hazards to which the worker is exposed.   Only the respirators approved by MIAMI MEES shall be used on the project site. All subcontractor firms shall submit their choice of respirator to the CQC for this delivery order for approval.

The choice of respirator is dependent upon the airborne contaminant present, the operation performed, and on the basis of comfort and ease of obtaining a proper individual fit. The Company will provide respirators, replacement parts, cartridges and filters as needed. The selected types of respirators are available from the project on-site superintendents.

EMPLOYEE TRAINING

The Designated Individual shall be responsible for providing training to each employee required to wear a respirator to the extent that the person is knowledgeable and proficient with respect to the respirator to be worn.  Refresher training shall be given at least annually.

Training shall cover but is not limited to the following items:

- How to select a respirator

- Proper use of equipment

- Importance of and proper cleaning procedures

- Maintenance of respirators

- Proper storage methods

- Hands on procedures to fit and test face pieces.

FIT TESTING

Each respirator wearer shall ensure his/her respirator is fitted properly before being permitted to enter an area or perform work where airborne hazardous contaminants are present. Fit tests are essential to ensuring that a respirator mask forms a good seal against the wearer's face and prevents contaminants from leaking into the mask.

The respirator should fit snugly, but not so tightly that it's uncomfortable and restricting. The most likely point of failure in a respirator is the seal between the face piece and the wearer's face. Respirator face pieces are made in various sizes to fit a wide variety of face shapes and sizes. Consequently, close attention should be paid to the face seal fit when purchasing and issuing respirators to workers.

Written records of fit testing shall be maintained for each employee assigned a respirator to show:

- The type of respirator being used in fit testing

- Specific make/model of respirator

- Name of person tested

- Name of test operator

- Date of test

- Face dimensions and face piece sizes used for the test.

- Results of test:

  - Success or failure of person to obtain a satisfactory fit (If a qualitative fit test is used)

  - Respirator protection factor based on test results (if a quantitative fit test is used).

The types of fit test are:

- Negative Pressure Fit Test:

This is accomplished when the wearer closes off the respirator inlet and inhales. A vacuum and partial inward collapse of the mask should result. If a vacuum cannot be maintained for at least 10 seconds, readjust the face piece and try again.

- Positive Pressure Fit Test:

When the wearer closes off the exhalation valve and breathes out gently, air should escape through any gaps in the seal.

- Vapor Fit Test:

The respirator is donned and a special smoke tube, containing stannic chloride, is directed at the seals and edges of the respirator as the wear responds to questions and/or reads a prepared statement while simultaneously moving his/her head up, down and side to side as well as deep breathing. If the wearer cannot detect the irritant smoke, the fit is considered satisfactory.

NOTE

The NEGATIVE PRESSURE FIT TEST and the POSITIVE FIT TEST should be made by the wearer when donning the mask each time to ensure a proper fit.

NOTE

Since the proper seal imperative to a respirator program,

ALL EMPLOYEES WEARING RESPIRATORS MUST BE CLEAN SHAVEN

Even beard stubble may prevent a proper seal. Consequently, a daily shave prior to reporting for work is essential. Eyeglass temple pieces shall not interfere with the seal.  Further, goggles, face shields, or welding helmets that must be worn in conjunction with a respirator shall be worn so as not to adversely affect the seal of the face piece to the face.

WARNING

THE WEARER OF A RESPIRATOR SHALL NOT BE ALLOWED TO WEAR CONTACT LENSES IF THE RISK OF EYE DAMAGE IS INCREASED BY THEIR USE.

INSPECTION AND MAINTENANCE

Each person issued a respirator shall inspect the respirator prior to its use to ensure that it is in good operating condition.

Each employee issued a respirator will be responsible to ensure that the following maintenance inspections/task are accomplished and his/her equipment is properly maintained.

  - Cleaning and sanitizing

  - Inspection for defects

  - Repairs accomplished properly

  - Storage

Each respirator shall be properly maintained to retain its original shape and effectiveness.

CLEANING AND SANITIZING

Each respirator, other than disposable, shall be cleaned and sanitized after each use to ensure that the respirator wearer is provided with a clean and sanitized respirator at all times. Cleaning and sanitizing shall be in accordance with the recommendations of the manufacturer. However, in the absence of such instructions, remove filter and/or straps, wash the face piece in a mild soap solution. After washing, the respirator shall be thoroughly rinsed in fresh water to prevent dermatitis from any residue left on the mask. The drying shall take place at room temperature in an atmosphere free of contaminants.

Employees issued personal respirators shall wash, rinse, and dry their assigned respirators regularly to ensure they are maintained in good condition and are clean.

Disposable respirators shall be maintained until passage of air becomes restricted and they properly disposed of in accordance with the environmental plan. No cleaning is authorized; the respirator shall be properly stored it is original bag when not being worn by the individual. The respirator shall not be shared but disposed of and a new item issued.

MAINTENANCE AND REPAIR

Only persons trained in proper respirator assembly and repair shall do replacement of parts or repairs. Replacement parts shall be only those designed for the specific respirator being repaired. When your respirator needs maintenance or repair consult your immediate supervisor.

STORAGE

Respirators can be permanently damaged if they are not stored properly. After having been cleaned, sanitized and dried, they shall be placed in individual resalable plastic bags. Storage shall be in a designated storage area in such manner that they will not be distorted nor have the rubber or other parts damaged.

Individual’s assigned respirators shall not store them in places such as lockers and toolboxes/job boxes unless they are protected from contamination, distortions, and damage.

SURVEILLANCE

Supervisory personnel are tasked to ensure that the right respirator is being used for the task and that respirators are being worn properly by those in the hazardous area. Corrective measures shall be taken immediately when discrepancies are observed.

Should the respirators not be appropriate for the contaminated areas, personnel shall evacuate the area. The task/atmosphere shall be reevaluated and the proper equipment be made available to employees before work is permitted to resume.

The Designated Individual is to ensure equipment is being properly inspected, maintained, sanitized and stored by conducting periodic surveillance. This applies to both the daily issue of respirators as well as those issued permanently to select individual employees.

RESPIRATOR PROGRAM EVALUATION

An appraisal of the program shall be carried out as needed by the MIAMI MEES Quality Control and Safety Manager.  Action shall be taken to correct defects in the program.

42 CFR 84 Standard

The 42 CFR 84 standard enacted by the National Institute of Occupational Safety & Health (NIOSH) requires non-powered, air purifying particulate filter respirators to perform against a 0.3-micron size particle. This new regulation is a revolutionary and far more stringent way to classify filtering efficiency and respirator performance. Previously, respirators were required to screen against 2-micron size and were classified by contaminant types. Research showed that this standard was inadequate against oil and many other harmful particles that were too small to detect.

There are three filter efficiency classes required under the new standard; 95%, 99%, and 99.97%. All these classes are approved to TB protection. Within each class are three divisions based on whether the filter is approved for oil aerosols or not. Such as “N” is not for oil aerosols, “R” is resistant to oil aerosols and “P” is oil aerosol proof.

| CHOOSE FILTER EFFICIENCY |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 95% |  |  |  | 99% |  |  |  |  |  | 99.97% |
| Yes it does or This is Unknown |  |  |  | Does the atmosphere contain oil vapors? |  |  |  |  |  | No it does not |
| YES or Unknown at this time |  |  |  | Will the filter be used | NO |  |  |  |  | Use N Series, R series or P series filter |
|  |  |  |  | hours? |  |  |  |  |  |  |
| Use P series Filter only |  |  |  |  | Use R series or P series Filter |  |  |  |  |  |

more than 8

# MIAMI MEES, LLC 
RESPIRATOR SCREENING FORM

Date:

Employee Name:

S.S.N.:

Position:

| Are you troubled by shortness of breath when hurrying on level ground or walking up a slight hill? | YES | NO |
| --- | --- | --- |
| Do you have asthma? | YES | NO |
| Do you have heart or blood vessel disease? | YES | NO |
| Do you have fits, spells, or seizures? | YES | NO |
| Do you have Chronic Lung Problems? | YES | NO |
| Are you bothered by claustrophobia? | YES | NO |
| Do you have difficulty with taste or smell? | YES | NO |
| Do you have any physical problems or know of any reason why you are not able to wear a respirator? | YES | NO |
| Do you wear contact lenses at work? | YES | NO |

I understand each and every question above and have answered them to the best of my knowledge.

Signed:

MIAMI MEES, LLC

EMPLOYEE RESPIRATOR FIT TEST SCREENING

Employee Name:

SSN:

Make and Model of Respirator Employee was tested on:

Limitations:

None:

Dentures:

Glasses:

Other:

Explain:

Facial dimensions and face piece size utilized for test.

SMALL

Testing:

MEDIUM

LARGE

| PASS | FAIL |  |
| --- | --- | --- |
|  |  | Able to perform both Positive & Negative pressure checks |
|  |  | Smoke Fit Test: Respirator sealed to face properly |
|  |  | Capable of head movements: Up, Down, Side to Side |
|  |  | Capable of several deep breaths during test |
|  |  | Capable of reading passage with normal breathing during test |

Employee Acknowledgment:

I understand that my use of this respirator must be in accordance with company rules, manufactures instructions, and applicable federal and state regulations. That I am only protected if I follow the manufacturer’s instructions presented to me and wear the respirator correctly. Also I will notify my supervisor immediately if any of the above statements change.

Employee Signature: 		Date:

Fit Test Specialist Signature:

Printed Name:

MIAMI MEES,LLC RESPIRATORY PROGRAM

The following individuals have attended and passed the Respiratory Program Course and satisfactorily demonstrated and passed the Fit Test given by the company instructor.

| Name | Initial Training | Fit-Test Complete | Medical Screening | Biannual Training |
| --- | --- | --- | --- | --- |

IRRITANT FUME FIT TEST PROTOCOL

The fit test protocol shall be performed in a location with adequate exhaust ventilation to prevent general contamination of the testing area. Only the Test Subject shall be within 20 feet of the test area.  All others shall stay clear until called.

Respirator style shall be selected with regards to the task it will perform.

Cartridge/Respirator shall be a HIGH EFFICIENCY PARTICULATE & ACID-GAS type.

Test Subject shall don the selected respirator and wear it for a few minutes before testing.

Test Inspector shall review this protocol with subject.

Test Subject shall satisfactorily perform the conventional positive and negative pressure checks.  Inspector shall observe and confirm satisfactory test.

NOTE:

The irritant fume smoke can be irritating to the eyes.

Advice the Test Subject to close his/her eyes for this portion of the test.

Test Inspector shall direct the stream of irritant smoke from the tube towards the face- seal area, moving around the whole perimeter of the mask.

The Test Subject shall perform the following exercises.

NOTE:

Each exercise shall take one minute to accomplish:

- Breathe Normally

- Turn head side to side (slow movements)

- Breathe Deeply (breaths deep and regular)

- Nod head up and down (slow movements)

- Read the rainbow passage.

The Test Subject shall IMMEDIATELY indicate if the irritant smoke is detected.	If smoke is detected the Test Inspector shall immediately stop the test.

If no smoke is detected the Subject has passed the fit test.  Confirm sensitivity by having the Subject smell the irritant smoke with the respiratory mask removed.

COMPETENT PERSON EVALUATION

on: Respirator Protection & Fit-Test Examination

| Employee Name | Position |
| --- | --- |
| Evaluator | Date of Evaluation |

| Does the employee have experience/training in: - Use of Respirator Equipment - Inspection of Respirator Equipment - Maintenance of Respirator Equipment - Storage of Respirator Equipment - Cleaning of Respirator Equipment - Conducting Fit-Test on others - Performing negative/positive seal checks - Qualified CPR and First Aid | Yes         | No         | Does the employee have knowledge about: - Respirator Selection Process - Filter Cartridge Selection Process - Requirements of the standards: OSHA - Toxic & Hazardous Substances - Developing Respirator Protection Work Plans - Emergency Removal | Yes       | No       |
| --- | --- | --- | --- | --- | --- |

| The employee has authority to: - Take prompt corrective measures to eliminate existing and/or predictable hazards. - Stop work and remove all employees (MIAMI MEES and Subcontractor) from the hazardous area. - To direct the corrective measures to eliminate or control the hazards. - To have removed from project site all damaged or hazardous equipment | Yes     | No     |
| --- | --- | --- |

Remarks/Comments:

This employee is considered to be COMPETENT within the requirements of the RESPIRATOR PROTECTION STANDARDS for all MIAMI MEES Projects. Program Manager/Representative Signature:

Tab 7

WELDING, CUTTING AND HOT WORK

- GENERAL

  - Training and Qualifications

- All welders and cutters shall be properly trained in the safe operation of their equipment, safe welding/cutting work practices, and proper fire protection and as needed proper respiratory usage.

- All Subcontractors shall submit this information to the MIAMI MEES office at least 48 hours in advance of all expected welding, cutting, hot work operations.

- All structural welding accomplished on critical items such as scaffolding, shoring, forms, ladders, piling, etc. and as directed by competent government authorities in the Scope of Work shall be accomplished by certified welders utilizing qualified welding procedures.

  - All employees performing welding, cutting or heating shall be protected by PPE appropriate for the hazards, such items as vision, respiratory and skin protection shall be in compliance with applicable standards and requirements.

a. Workers and all others shall be shielded from welding, rays, flashes, sparks, molten metal, and slag.

  - All welding equipment shall be inspected prior to operation and daily thereafter. All defective equipment shall be promptly tagged and removed from service until repaired or replaced.  Re-inspection shall confirm its acceptability for service.

  - Welding cables, hoses, and other equipment shall be kept clear of all passageways, stairs and ladders. Cables shall be secured to permanent structures not lights, ceiling supports, etc. Cables shall be kept clear of all electronic equipment.

  - Welding and cutting on preserved surfaces

- Before welding or cutting on any surface covered by a preservative coating a test shall be made to determine its flammability. Coating, with scrapings that burn rapidly, shall be considered highly flammable and removed before continuing operations.

- Preservative coatings shall be removed a sufficient distance (at least 6 inches) from the area to ensure that any temperature increase of the unstripped metal shall not be appreciable. Artificial cooling can be applied to limit this distance.

- Toxic preservative coatings shall be removed at least 6 inches from the area of heat application. All employees shall be protected by respirators.  If unable to achieve this distance the employee shall be protected by a positive airflow respirator.

  - Before heat is applied to a drum, container, or hollow structure a vent or opening shall be provided for the release of any built-up pressure generated during the application of heat.

  - Arc welding and cutting operations shall be shielded by noncombustible or flameproof screens to protect employees and other workers from direct rays of the arc.

  - Persons not protected by screens shall be protected by filter lenses and welding helmets.

  - Welders and other persons who are exposed to radiation shall be protected so that the skin is covered to prevent burns and other damage by ultraviolet rays. Weld helmets, gloves, hand shields shall be free of leaks, openings, and highly reflective surfaces.

- RESPIRATORY PROTECTION

  - All welding, cutting and other hot work operations shall be ventilated, by either natural or mechanical means, such that personnel exposures to hazardous concentrations of airborne contaminants are within acceptable limits. Most operations can be done with natural air flow without mechanical ventilation or respiratory protective equipment.

  - Local mechanical exhaust ventilation shall be provided whenever welding, cutting or other hot work is performed in a confined space, with its exhaust directed into the open atmosphere external to building or structure.

  - Materials of Toxic Significance.

    - Welding, cutting or other hot work operations which involve or could generate fumes containing any of the heavy metal substances shall be performed utilizing local mechanical exhaust ventilation, with its exhaust located at least 15 feet from other people, and the welder protected by proper respirator PPE.

  - Oxygen cutting using either an iron powder or chemical flux, gas-shielded arc cutting, and plasma cutting shall employ local mechanical exhaust ventilation to remove fumes generated.

  - Other workers exposed to the same atmosphere as the welder/cutter shall be removed or protected in the same manner as the welder or cutter.

- FIRE PROTECTION

  - Before conducting any hot work operations, the immediate vicinity shall be checked to ensure it is free of the following:

- Combustible materials in the proximate vicinity

- The	presence	or	possible	generation	of	potentially	explosive atmospheres, such as natural gases, vapors, liquids, or dust.

- The presence or nature of an oxygen-enriched atmosphere.

  - Before any hot work operation, objects to be affected by the operations shall be:

- Moved to a location free of combustible material

- If necessary, all moveable fire hazards shall be taken to a safe place, at least 35 feet away, or the combustible material shall be protected from the heat, slag, sparks.

- Where hot work is to be done near walls, floors, partitions, ceiling or roof of combustible construction, fire resistant guards shall be provided.

  - When hot work must be done on a location on the Naval facilities, where combustible or flammable materials are located, inspection and written authorization by designated authority shall be required before operations can begin.  The following shall be available for this inspection:

- 2-A/20-BC Fire Extinguisher shall be provided in the immediate vicinity of the hot work.

- A Fire watch: An individual assigned to guard against fire and instructed in anticipated fire hazards and use of fire-fighting equipment.

- Provisions for the Fire watch to remain in the vicinity for 30 minutes after completion of welding and the site has cooled to the touch.

  - When hot work is over combustible flooring material, the combustible material shall be protected with fire-resistant shielding, covered with damp sand or wetted.

  - Noncombustible barriers shall be installed below welding or burning operations in a shaft or raise.

3.6     Opening or cracks in floors, walls, or ducts within 35 feet shall be tightly covered to prevent the passage of sparks into adjacent areas.

3.7. Before welding, cutting on drums, tanks, or other containers and equipment which have contained hazardous materials the containers shall be thoroughly cleaned IAW NFPA 327 and ANSI F4.1.

- OXYFUEL GAS WELDING AND CUTTING

  - Oxygen cylinders and apparatus shall be kept free of oil, grease, and other flammable or explosive substances and shall not be handled with oily hands or gloves.

  - Fuel and oxygen hoses shall be readily distinguishable from each other. Oxy-fuel gas welding and cutting equipment shall be listed by a nationally recognized laboratory.

    - Hoses that show signs of sever wear, damage, or flashback, defective hoses or hoses in doubt shall not be used.

    - Hoses with couplings that disconnect without rotary motion, show signs of leakage, are not securely clamped or fastened to the hose shall be considered damaged and not used.

    - When parallel runs of oxygen and fuel gas hoses are taped together, no more than 4 inches out of every 12 inches shall be taped.

  - Torches shall be inspected for leaking shutoff valves, hose coupling tightness and tip connection tightness. Clogged tips shall be properly cleaned.  Defective or damaged torches shall not be used.

4.4.	Hoses shall be purged individually before lighting the torch for the first time. Purging shall not be done in a confined space or near an ignition source.

  - Torches shall be lighted by friction lighters or other approved devices, not by matches or from hot work.

  - Torch valves shall be tightly closed and gas supply shut off whenever work is suspended.

  - Torch and hoses shall be removed form confined spaces whenever work is suspended.

  - Acetylene regulators shall not be adjusted to permit discharge pressures greater than 15 psig.

  - Gas fuel bottles and oxygen bottles shall be stored upright and securely chained/blocked to prevent their falling over. When not in use the hoses shall be removed and the bottle caps securely and properly installed. Bottles shall not be left uncapped.

- ARC WELDING

  - Cables and Connectors

    - Shall be completely insulated and flexible, capable of handling the maximum current requirements, in good repair.

    - Shall not have repairs or splices within 10 feet of electrode holder.

    - Shall be properly connected using the proper insulated connectors with all exposed metal completely insulated.

    - Welding cables shall not be placed near power supply cables or other high-tension wires.

    - Welding leads shall not be permitted to contact metal parts supports on suspended scaffolds.

    - Coiled welding cables shall be spread out before use

  - Electrode Holders

    - Shall be specifically designed for arc welding and cutting with a rating capable of handling the maximum current for the project.

    - Shall be in good working order, properly insulated to protect the Welder.

  - The frames of the arc welding shall be properly grounded to the power source.

  - Pipe lines carrying gases or flammable liquids shall not be used as a current return ground.

  - Equipment shall be shut down when leads are unattended.

- REFERENCES

OSHA 29 CFR 1926.350

OSHA 29 CFR 1910.251

Tab 8

LEAD PROGRAM

In order to comply with federal and state regulations and to ensure that information is available regarding the hazards of lead in the workplace to all employees the following written lead program has been established.

GENERAL INFORMATION

Quality Control and Safety Manager is responsible for the establishment, implementation and maintenance of all aspects of this Program for Working with Lead in Construction. He/she shall be responsible for considering the safe handling of surfaces coated with a lead-based paint or the safe application of a lead-based paint, where required, in the preparation of estimates or bids. He/she, through the Quality Control/Safety Inspector, will be responsible for ensuring that the requirements outlined in this program are followed in the conduct of the scope of work. Each employee is responsible for compliance with all requirements of this Program.

Methods of Compliance

- Working with Lead in Construction: Employee exposure shall be maintained below the PEL using either engineering controls, personal protection equipment or a combination of both methods.

  - The Permissible Exposure Limit (PEL) to lead exposures in the construction workplace limits is fifty micrograms of lead per cubic meter of air (50 ug/m3), averaged over an 8-hour workday (8-hour Time Weighted Average or TWA).

  - Action level means employee exposure, without regard to the use of respirators, to an airborne concentration of lead of 30 micrograms per cubic meter of air (30 ug/m3) calculated as an 8-hour time-weighted average (TWA).

  - Lead is defined as metallic lead, all inorganic lead compounds, and organic lead soaps.  All other organic lead compounds are excluded.

- Exposure Assessment: Prior to starting work, all lead exposures expected to be encountered during the delivery order construction activity will be identified. Construction activity is defined as work for construction, alteration and/or repair including painting and decorating.  It includes but is not limited to:

  - Demolition or salvage of structures where lead or materials containing lead are present.

  - Removal or encapsulation of materials containing lead

  - New construction, alteration, repair, or renovation of structures, substrates, or portions thereof that contain lead or materials containing lead.

  - Installation of products or material containing lead.

  - Lead contamination/emergency cleanup

  - Transportation, disposal, storage, or containment of lead or materials containing lead on the site or location at which construction activities are performed

  - Maintenance operations associated with the construction activities described in this section.

The Project Manager, working with a qualified lead abatement firm, will formulate a site-specific Lead Abatement Plan using the guidance provided in the submittal to the government. This Plan will contain all documentation related to the identification of the lead material, any project engineering involved as well as work practice controls required to perform all delivery order activities safely. Additionally, this Plan will be the basis for training all employees at the site prior to starting work.

No MIAMI MEES employee shall enter or participate in any lead abatement activities unless he/she is:

- Fully qualified to do so.

- Properly equipped with the necessary personal protective equipment as delineated by the abatement contractor.

- Trained by the Abatement Contractor in his/her Site Specific Health and Safety Plan to included scope of work, good work practices expected operations and anticipated response to accidents and casualties.

- Exposure Management Procedures: Once it has been determined that lead exposure will be part of the scope of work, the site-specific Lead Abatement Plan will specify how the exposure will be managed. More than one of the project engineering items listed below may be needed depending on site and task conditions.

  - Remove, Discard & Replace is the process of taking the lead out of the workplace by the removal of equipment, structures or other fixtures that contain lead to eliminate possible exposure at the site

  - Encapsulation is a method that can be used to prevent the disturbance of any lead in the workplace (e.g., placing a non-hazardous coating over a lead painted surface)

  - Enclosure would isolate the areas or space, confining the lead to a specific and identifiable area for control

  - Stripping means to remove the lead from the material surface by means of:

    - manual or mechanical chipping

    - dry or wet blasting

    - dry or wet scraping or sanding

    - heating to cause the lead to release from the surface

    - using chemicals to remove the lead from the surface.

- Work Practice Controls: Below are several work practice controls that may be utilized to perform work safely. The selection of any or all will depend on the site and task conditions. The approved Lead Abatement contractor will specify the specific work practices to be used in accordance with guidance provided in OSHA 29 CFR 1926.62.

  - Containment of lead will need to be planned and provided to limit the contamination to a controlled and defined area. The containment may need to be provided in the form of a physical barrier. Containment’s may be large and cover large areas or rooms, or they may be small covering only a piece of equipment or small area on equipment.

  - Isolation is somewhat like containment except that you and the client have chosen to isolate an area, object, or equipment until such time as it is convenient to address the lead problem. Isolation requires a certain amount of control or security to ensure exposure or further contamination is impossible.

  - Posted warning signs are required to alert all persons that could inadvertently enter an area contaminated with lead and a potential exposure above the PEL.

  - Signs are to read: WARNING

LEAD WORK AREA POISON

NO SMOKING OR EATING

  - Physical barriers may be needed to prevent people from entering a contaminated area or for preventing contamination from spreading. Containments are usually provided by sheet plastic that surrounds the contaminated area. The plastic provides an economical method of containing any dust, vapor or mist created during the management procedure. Seams and tears can be quickly and easily patched with tape. If security type barriers are needed, then sheet plastic may need to be used with fencing or plywood walls.

  - Decontamination will be part of the daily work routine that will eliminate any lead from worker’s skin, workers clothing, tools, personal protective equipment, or any other materials leaving the containment area. As the containment areas are reduced to elimination, decontamination will need to be planned carefully.

  - Clean rooms or areas need to be provided and maintained free of contamination. Clean rooms are needed for employees to change clothes, eat meals or take breaks. This area is also a safe storage for UN-contaminated personal protective equipment, tools or supplies.

  - Ventilation will need to be provided to all work areas. If not properly planned, the ventilation system can lead to the spread of contamination. Ventilation is a primary concern for the welfare of employees working in adverse conditions, such as wearing respirators, protective clothing.

  - Air monitoring may be needed before, during, and after any work that could release lead into the atmosphere. Air monitoring involves the collection and analysis of air samples to determine the level and/or amount of airborne concentration of lead present in a given area over a period of time.

  - Worker protection equipment will be determined by the exposure management method chosen: hard hats; head covers; eye protection; respiratory protection (full or half-mask, air-feed); gloves; shoe covers; coveralls. Each of these items require the training of employees on the proper method for wearing, use, limitations, cleaning and storing.

WORKER QUALIFICATIONS

All approved lead abatement company employees that will be accessing the work-site shall be properly trained in the following and the Abatement Company shall provide documentation of it in their site-specific health and safety plan.

- Respiratory Protection: (Note: Respirator means a specific brand, model, size. If it is an air purifying type, then it also means a specific filter system.)

- Proper Lead abatement techniques

- Lead Abatement Supervisor Training

- On the Lead Standard and its Appendices

- The scope of work for the project

- Personal protective equipment required (Note: As necessary this could mean a specific brand, model, and size.)

RECORD-KEEPING

All aspects of employee lead exposure must be adequately documented, including but not limited to: exposure assessment; exposure management; project engineering; worker qualification. Sufficient data must be maintained on each of these topics to document that employees were protected and that the physical and administrative controls implemented on the project meet the minimum requirements of the Contract. Provisions will be made by MIAMI MEES and its subcontractor abatement firms to maintain these records for a period of not less than thirty years. All employees shall be authorized to review this record, following notification of request to their supervisor.

REGULATIONS

OSHA 29 CFR 1926.62

TAB 9

# ASBESTOS PROGRAM

References:	Local Air Pollution Control Agency Guidelines OSHA 1926.1101

In order to comply with the above regulations and to ensure that all employees are protected by the requirements of those regulations the following policy and plan has been established. All employees on MIAMI MEES project sites shall adhere to the Asbestos Program requirements to ensure all personnel are not exposed to asbestos fibers.

Per OSHA regulations, MIAMI MEES, as the General Contractor, is responsible for “general supervisory control” of the asbestos abatement. As such all Site Superintendents and Quality Control Inspectors shall be considered a “Competent Person” on the project site. They shall ensure that the asbestos subcontractor is in compliance with this standard, and shall direct immediate actions be taken to correct any deficiencies.

POLICY:

Quality Control/Safety Manager is responsible for establishment, implementation and maintenance of all aspects of this Program. Asbestos is a naturally occurring material once used widely in the construction industry. Its ability to withstand high temperatures and resist many chemicals made it useful for hundreds of applications. However widespread use of asbestos has left a dangerous legacy. The improper handling of asbestos-containing material may release harmful amounts of fiber. When inhaled, asbestos has been shown to cause the following diseases: asbestosis, lung cancer and mesothelioma. If you have any concerns about material that you believe may be asbestos, work activity must stop until positive determination, by competent authority, has been made.

All personnel working around or adjacent to asbestos-containing material shall, daily, ensure the effectiveness of the engineering control methods in place to assure that asbestos fibers do not migrate in to such adjacent areas. IF the effectiveness should be questioned by any employee, all work shall stop until proper assessment can be made.

Quality Control/Safety Manager shall be contacted before any work begins on any project known to involve asbestos. If asbestos-containing material or suspected asbestos-containing material is discovered during the course of any project, all work shall stop immediately. The site shall be secured and all access prevented. The Government shall be contacted and site turned over to them. The circumstance of the project and exposure would dictate the procedures to be followed.

At present we are not certified in asbestos abatement and would refer any concerns to a qualified abatement contractor. The certified asbestos abatement contractor will be required to submit an Abatement Plan along with a Site Specific Health and Safety Plan for asbestos abatement projects.  This plan would then be incorporated into this section.

DEFINITIONS:

- "Asbestos" includes chrysotile, amosite, crocidolite, tremolite asbestos, anthophyllite asbestos, actinolite asbestos, and any of these minerals that has been chemically treated and/or altered. For purposes of this standard, "asbestos" includes PACM, as defined below.

- "Asbestos-containing material (ACM)", means any material containing more than one percent asbestos.

- "Class I asbestos work" means activities involving the removal of TSI and surfacing ACM and PACM.

- "Class II asbestos work" means activities involving the removal of ACM which is not thermal system insulation or surfacing material. This includes, but is not limited to, the removal of asbestos-containing wallboard, floor tile and sheeting, roofing and siding shingles, and construction mastics.

- "Class III asbestos work" means repair and maintenance operations, where "ACM", including TSI and surfacing ACM and PACM, is likely to be disturbed.

- "Class IV asbestos work" means maintenance and custodial activities during which employees contact but do not disturb ACM or PACM and activities to clean up dust, waste and debris resulting from Class I, II, and III activities.

- "Competent person" means, in addition to the definition in 29 CFR 1926.32 (f), one who is capable of identifying existing asbestos hazards in the workplace and selecting the appropriate control strategy for asbestos exposure, who has the authority to take prompt corrective measures to eliminate them, as specified in 29 CFR 1926.32(f): in addition, for Class I and Class II work who is specially trained in a training course which meets the criteria of EPA's Model Accreditation Plan (40 CFR 763) for supervisor, or its equivalent and, for Class III and Class IV work, who is trained in a manner consistent with EPA requirements for training of local education agency maintenance and custodial staff as set forth at 40 CFR 763.92 (a)(2).

- "Critical barrier" means one or more layers of plastic sealed over all openings into a work area or any other similarly placed physical barrier sufficient to prevent airborne asbestos in a work area from migrating to an adjacent area. "Fiber" means a particulate form of asbestos, 5 micrometers or longer, with a length-to-diameter ratio of at least 3 to 1.

- "PACM" means "presumed asbestos containing material". This means thermal system insulation and surfacing material found in buildings constructed no later than 1980.

Tab 10

# HAND AND POWER TOOLS

- GENERAL

  - All power tools used on MIAMI MEES project sites shall be of a manufacturer listed by a nationally recognized laboratory for the specific application for which they are to be used.

  - Inspections

- Hand and power tools shall be used, inspected, and maintained in accordance with the manufactures technical manual. These instructions and recommendations shall be maintained at the job site.

- Hand and power tools shall be inspected, tested, and determined to be in safe operating condition before use. Continued periodic inspections shall be made to assure safe operating conditions, all guards are in place and proper maintenance.

- Hand and power tools shall be in good repair and with all required safety devices/guards installed and properly adjusted. Damaged tools or tools with defects shall be removed from service, tagged “DO NOT USE” until repairs are made and the tool is inspected and tested.

- Only qualified employees shall accomplish repairs to hand or power tools.

- Tools requiring heat treating or redressing shall be tempered, formed, dressed, and sharpened by personnel who are experienced in these operations.

  - Guarding

- Power tools designed to accommodate guards shall be equipped with such guards.

- Reciprocating, rotating, and moving parts of equipment shall be guarded if exposed contact could create a hazard.

  - Throwing tools or materials from one location to another or from one person to another, or dropping them to lower levels shall not be permitted.

  - When working overhead, tools not in use shall be secured or placed in holders.

  - Only non-sparking tools shall be used in locations where sources of ignition could cause a fire or explosion.

  - Electrical power cords shall not be used for hoisting or lowering tools.

- GRINDING AND ABRASIVE MACHINERY

  - Abrasive wheels shall only be used in machines provided with safety guards. Special circumstances allow the use of unguarded wheels in accordance with ANSI B74.2.

- Safety guards shall be strong enough to withstand the effect of a bursting wheel.

- Safety guards shall cover spindle end, nut, and flange projections.

- Safety guards shall maintain proper alignment with rotating wheel

  - All work or tool rest shall be properly adjusted not more than 1/8 inch from wheel.

  - All abrasive wheels shall be closely inspected, cracked or damaged wheels shall not be used. The maximum angular exposure of the grinding wheel shall be not more than 90

  - Grinding wheels shall not be operated in excess of their rated safe speed.

- POWER SAWS AND WOODWORKING MACHINERY

  - Guarding

- Circular saws shall be equipped with guards that automatically and completely enclose the cutting edges, splitters, and anti-kickback devices.

  - Masonry saws shall be equipped with a semicircular enclosure over the blade.

  - Masonry saws/drills shall be equipped with a means to exhaust dust or provided with water to protect personnel from dust.

- All portable power-driven circular saws shall be equipped with guards above and below the base plate. The upper and lower guards shall cover the saw to the depth of the teeth, except for the minimum arc required to permit the base to be tilted for bevel cuts and to allow for proper retraction and contact with the work. When the tool is withdrawn from the work the lower guard shall automatically and instantly return to the covering position.

- Blades of planers and joiners shall be fully guarded and have cylindrical heads with throats in the cylinder.

- Band saw blades shall be fully enclosed except at the point of operation.

- Specialty non-portable woodworking equipment must meet the guarding requirements.

  - Radial arm power saws shall be equipped with an automatic brake. The cutting head shall automatically return to the starting position when released by the operator.

  - The table of radial arm or swing saws shall extend beyond the leading edge of the saw blade.

  - Each hand-fed crosscut or circular ripsaw shall have a spreader to prevent the material from squeezing the saw or being thrown back on the operator.

  - Operating procedures

- A push stick, block, or other safe means shall be utilized on all operations close to high-speed cutting blades/edges.

- The use of cracked, bent, or otherwise defective parts such as saw blades, cutters, knives is prohibited.

- Power saws shall not be left running unattended.

- PNEUMATIC TOOLS

4.1 	Safety clips or retainers shall be installed and maintained on all pneumatic impact tools to prevent dies and tools from being accidentally expelled from the barrel.

  - Impact sockets shall be provided with a locking device for retaining the socket.

  - Pressure shall be shut-off and exhausted from the line before disconnecting

tools.

  - Safety lashing, at least 100-psig tensile strength, shall be provided at connections between hose and tool and at all quick makeup type connections.

  - Hoses shall not be used for hoisting or lowering tools.

- EXPLOSIVE-ACTUATED TOOLS

  - Explosive-actuated (powder-actuated) tools shall meet the design requirements of ANSI A10.3. and shall be provided with the following:

    - A lockable container with the words “Powder-Actuated Tool”

    - Manufactures instruction and service manual

    - Powder load and fastener charts

    - Service tools and accessories.

    - One empty/clean steel gallon container each for used charges and misfires.

  - Only qualified operators shall operate explosive-actuated tools. Certification of this qualification shall be provided MIAMI MEES at least 7 working days before work is to commence.

  - Inspection and Testing

    - Daily	inspection,	cleaning,	and	testing	shall	be	performed	as recommended by the manufacturer.

    - Explosive-actuated tools shall be tested each day, IAW manufacturer’s instructions, before loading to ensure all safety devices are in proper working condition.

    - Explosive-actuated tools shall be inspected, thoroughly cleaned and tested after 1,000 fastenings.

  - Operational Precautions.

    - Operator shall wear proper PPE such as safety glasses and face shield, hearing protection.

    - Surrounding area shall be posted that a “Powder-Actuated Tool is in use”.

    - Hearing and eye protection signs required shall be posted.

    - All flammable material shall be removed or the tool maintained at least 50 feet away. The use of explosive-actuated tools is prohibited in explosive or flammable atmospheres.

    - Explosive-actuated tools shall not be loaded until just before the intended firing time. Hands shall be kept clear of the open barrel. Tool shall be unloaded when not in use.

    - Neither loaded nor empty tools are to be pointed at any employee. If caught the employee shall be reprimanded or possibly removed from the job site.

    - Fasteners shall not be driven into soft or easily penetrable material, very hard or brittle material, steel, tile, etc. Inspection and testing shall be done confirm suitability of the fastening device to be used.

  - Designated governmental authorities shall confirm, in writing, that operation of an explosive-actuated tool on the naval facility is permitted.

    - As necessary special conditions may need to be satisfied prior to bring the tool on site.

    - Less than 5 pounds of explosive charges shall be brought on the project site at any time.

    - Tool and all charges shall be properly stored and locked when not in use.

- CHAIN SAWS

  - All chain saws shall have automatic chain brake or kickback device. Chain shall not move at idle speed.

  - Operators shall wear the following PPE: Safety glasses, face shields, hard hat, ear plugs and muffs, gloves, steel toed safety shoes, and leg protection chaps.

  - Chain saws shall not be fueled while running, hot, nor near open flames. Saws will not be started within 10 feet of a fuel container.

- ABRASIVE BLASTING EQUIPMENT

  - Written operating procedures shall be developed by professional, fully qualified, subcontractor firms for all abrasive blasting operations in accordance with Standards listed in 29 CFR-1910.94(a).

    - Abrasive blasting operations shall be evaluated to determine composition and toxicity of the abrasive and the dust or fumes generated from the blasted material, including surface coatings. This determination shall be included in the written operating procedures and documented on the activity hazards analysis generated for the operation.

  - No employee shall be permitted to work in abrasive blasting operations until trained, medically cleared and wearing proper PPE.

7.3. All connections and nozzles shall be designed to prevent accidental disengagement. All connections shall be equipped with safety lashings of at least 100-psig tensile strength.

  - Nozzle attachments shall be of metal and mount on the outside of the hose. A dead man type control device shall be provided at the nozzle, to cut off the flow if the operator loses control of the hose.

  - Blast cleaning enclosures shall be exhaust ventilated so that a continuous inward flow of air shall be maintained at all openings. The rate of exhaust shall be sufficient to provide prompt clearance of dust-laden air after cessation of blasting.

  - Blast cleaning enclosures shall be so constructed as to prevent the escape of abrasive or dust particles into adjacent areas will be minimized and visible spurts of dust will not be observed.

Tab 11

CONFINED SPACE ENTRY PROGRAM

In order to comply with all applicable regulations and to insure that all employees are protected by the requirements of those regulations, the following policy and plan has been established.   All employees shall follow these requirements. This program will be kept on site and will be available to all employees and workers on site.

REGULATIONS

OSHA 29 CFR 1910.146

ENCLOSURE

CONFINED SPACE ENTRY PERMIT GENERAL INFORMATION

Quality Control/Safety Manager is responsible for the establishment, implementation and maintenance of all aspects of this CONFINED SPACE PROGRAM.

POLICY:

Quality Control/Safety Manager shall be contacted before any work begins on a project known to involve confined space or permit required confined space hazards. If these hazards are discovered during the course of the project, work will be halted, space evacuated and the Quality Control/Safety Manager or his representative will be immediately notified. A special Job Hazards Analysis and Confined Space determination, in accordance with 29 CFR 1910.146, will be made to insure compliance with applicable rules and regulations before work is resumed in these areas.

- QUALITY CONTROL/SAFETY MANAGER

  - Is responsible for the establishment, implementation and maintenance of all aspects of this Confined Space Program.

  - May review all completed entry permits to insure the program is functioning properly (this review is not required prior to entry)

  - Will review these procedures periodically and update them as required.

  - Will insure that all records related to Confined Space Entry are maintained for a minimum of one year.

- PROJECT FOREMAN/SUPERINTENDENT

  - Prior to start of any work performed as part of a project at a specific site, will formulate a CONFINED SPACE PLAN.

  - Is responsible for ensuring that all employee assignments are made within their personal training and skill level.

  - Must approve, prior to entry, all Confined Space Entry Authorization for each employee.

  - Will require an outside attendant during Permit Required Confined Space Entry.

  - Shall be responsible for providing all equipment and tools that are required for the entry. This may involve the rental of equipment such as exhaust or ventilation blowers, safety harness/lanyard/retrieval system, proper lighting and egress means.

- EMPLOYEES

  - Are to apply all of their training and experience in an effort to keep themselves safe.

  - At no time attempt to do something for which they have not been trained and in which they do not feel safe.

  - Are required to follow the Site Specific Confined Space Procedures and obey all restrictions.

DETERMINATION:

- Extra caution must be taken to recognize all confined spaces. The owner or operator may not have identified and/or labeled the area correctly. Use the guidelines provided in 29 CFR 1910.146 to make this determination.

- Use guidelines to determine if the confined space designation can be changed for this project.

- Posted warning signs are required to alert all persons that could encounter a confined space. Signs will read:

DANGER

PERMIT REQUIRED CONFINED SPACE DO NOT ENTER

METHODS OF COMPLIANCE:

- Management

  - Once it has been determined that confined space exists, the Project Manager and Site Superintendent shall pre-plan before entering a confined space. Using the delivery order or other format and working with a specialized firm for this work a confined space plan shall be developed. The plan will specify the management procedures to be followed depending upon the site and task conditions. These may include provisions for the following:

    - The task to be conducted within the confined space

    - Personnel involved and their responsibilities

    - Possible hazards within the space

    - Requirements for making the space safe, include isolation, ventilation, atmospheric monitoring, guarding and fall protection.

    - Equipment needed to enter may include personal protective equipment (PPE) suited to the hazards (encapsulation suits, respirators, gloves, etc.) personal monitors, ventilation equipment, rescue equipment, radios, spark proof tools and lights.

    - Communications between the observer/attendant

    - Emergency procedures to be followed if needed.

- Confined Space

  - The specific requirements for each confined space entry will depend on the site and task conditions. The site-specific CONFINED SPACE PLAN will specify the methods chosen for all phases of the project and will specify who is responsible to:

NOTE:

- Train Employees

- Evaluate rescue operations, develop emergency response checklist.

- Initiate the confined space entry permit and insure that all appropriate items are noted on the permit and emergency retrieval procedures reviewed.

- Conduct the required atmospheric testing, notify all workers, and proceed only if conditions are safe.

- Ensure that initial cleaning and vapor freeing (ventilation) has been done satisfactorily

- Ensure that proper isolation and hazardous energy controls have been accomplished

- Ensure that ventilation in place and the appropriate air monitoring equipment is on hand.

- Assemble all required equipment and tools (Emergency lights, fire extinguishers)

- Ensure all entrants have a properly adjusted full body harness on at entry.

- Assure availability of a lifeline and retrieval system when entering a vertical Confined Space more than 5 feet in depth

- Place an attendant outside the confined space with the capability of maintaining communications with the entrants.

- Don all other personal protective equipment as required

- Gain entry approval from owner/operator, if not on permit

- Enter the confined space

- Ensure that all work conducted within the confined space is done in an alert, cautious manner, always looking for signs of danger

- Evacuate the space if there is any indication of ill effects (dizzy, irritation or excessive odor).

If there is anything that does not appear right, the space will be evacuated immediately.

- Procedures

The Site Specific Confined Space Plan will address these points.

  - Isolation:

Conditions or actions outside of the confined space can create hazards within the space, creating a need for isolating the confined space as a way of protecting the entrants from these remotely caused hazards. For Example:

    - Lockout and tag out of electrical energy

    - Lockout and tag out of mechanical energy

    - Blocking the source of liquids and gases

    - Barriers to keep people and items from accidentally entering the space

  - Ventilation:

Mechanical ventilation is recommended to be set up to ventilate the confined space. Exhaust shall be a minimum of 6 feet or better from the entrance to the space.

    - Prior to entry, ventilation flow will be used to purge the space of any hazardous atmosphere and testing shall be conducted following the purging to confirm this fact.

    - The time required to purge the space will depend on the volume on the confined space and the flow rate of the chosen ventilation.

    - Ventilation, natural or forced, shall be continuous to ensure safe conditions whenever an employee is in a confined space.  (<10% of LEL,

>19.5% oxygen and below all PEL’s)

    - Care must be taken with the operation of the ventilation to avoid problems and maximize effectiveness.

    - Air should enter the space in the vicinity of where the workers will be at.

    - In forced ventilation the number of bends in the tubing shall be kept to a minimum.

    - Evaluate the source of air supply to insure it does not include contaminates, such as engine exhaust

    - Each confined space will be different and the best location for ventilation discharge will be determined during pre-planning

  - Permit:

Prior to entry into the space, the Foreman/Superintendent will complete a Confined Space Entry Permit. This permit will be signed by the Foreman/Superintendent and all Entrants. Each Entrants shall be briefed on the atmospheric conditions and allowed to observe sampling. If conditions remain the same the permit is good for the work shift or eight hours, whichever is shorter.

  - Attendant:

A confined space entry attendant or observer is required for each entry. The attendant must have confined space training and experience and shall:

    - Monitor and protect the confined space entrant(s).

NOTE: This Is THE Primary Duty Of The Attendant

    - Have sufficient knowledge of the hazards of the specific confined space being entered. This includes knowing the potential hazards, signs and symptoms of exposure, and the appropriate emergency procedures to be followed.

    - Communicate with the entrant(s) as necessary to monitor the status and to alert the entrant(s) of the need to evacuate the space if necessary

    - Monitor activities in and around the space to determine if it is safe for entrant(s) to remain within the space.

    - Summon rescue and other emergency services if it determined that the entrant(s) need assistance (non-entry rescues may be performed by the attendant if it can be done safely)

    - Remain outside the confined space and perform no other duties that might interfere with the attendant’s primary duty. Remain in place until relieved by another qualified person.

  - Personal Protection:

    - A comprehensive evaluation will be accomplished for all hazards to be encountered and will be documented as part of the site-specific Confined Space Entry Permit or other approved company form.

    - All reasonable effort will be made to reduce the hazards to safe levels prior to permitting entry into the space.

    - If hazard elimination is not feasible, personal protective equipment will be issued to protect the eyes, face, head, extremities, respiratory tract, skin and other exposed or endangered body parts.

  - Rescue:

Evaluation and preplanning with Rescue Services will include the development and implementation of procedures for summoning rescue services and the Rescue Plan. This Plan will be included in the site-specific Confined Space Plan and will include:

    - The Emergency Rescue Appraisal Checklist

    - The location of emergency equipment

    - The phone number and procedures to summon emergency help (911 may not work at all facilities) in some cases a cell phone may have to be available

    - Training and initial response by Attendant and/or Entrant(s)

    - Learn the facility or base emergency alarm system (what various signals mean, response to or how to activate them)

  - Training/worker qualifications:

This training will be based on the site-specific Confined Space Plan and will be designed to provide all employees with the information and skills needed to recognize potential problems and appropriate actions to take. The training will cover emergency rescue, need for full-body harness and other PPE requirements. It will address the expected work and job hazardous analysis review. The Foreman/Superintendent will provide training and document it on the site-specific Confined Space Plan or approved company form.

  - Record keeping:

All aspects of Confined Space Entry will be adequately documented on the site-specific Confined Space Plan or other approved company form. These records will be kept at the jobsite until completion and then forwarded to the main office for filing. These include but are not limited to:

    - Evaluation assessment forms

    - Site-specific Confined Space Plan and all permits

    - All other plans and documents related to Confined Space Entry

    - Training documentation

# EMERGENCY RESCUE SERVICE APPRAISAL

- Project Description:

Project Location:

- Type of Confined Space and attach a simple diagram showing confined space and entry location:

  - Permit Required,  Confined Space,  Hazardous Substances Diagram attached;  YES	 NO, explain:

- Expected atmosphere:	 Immediately Dangerous to Life & Health or	 Normal space Internal Configuration:  Open	 Obstructed;	 Elevated work area		 Non-elevated work area

Portal Size:  Restricted  Non Restricted; Portal Access:  Horizontal  Vertical & 

Elevated

- Expected hazards to be encountered in the Confined Space and/or as a result of the work ongoing inside the space:

- Emergency Response Team/Company:

Point of Contact:

& address/response telephone number:

- Response time to the MIAMI MEES Project Site is 	.

- Availability of the rescue crew is:  24 hours,	 Normal Working Hours,	 Other, explain:

- Is the Rescue Crew qualified and trained to function appropriately while performing rescue operations from this particular permit space or types of permit spaces as identified above? Are individuals trained on specific rescue related assignments?

  - YES	 NO, explain;

- Does the Rescue Crew have adequate equipment, PPE, Respirators and Communication Devices to perform rescue operations from this particular permit space or types of permit spaces as identified above?

  - YES	 NO, explain;

- Is the Rescue Service familiar with the project site location, the entrance location for the Confined Space?

Is the Rescue Service willing to enter the project site to perform rescues from this particular permit space or types of permit spaces as identified above?

  - YES	 NO, explain;

- Has the Rescue Service developed a Rescue Plan to perform rescue operations from this particular permit space or types of permit spaces as identified above? Can MIAMI MEES receive a copy of this plan for our files?

  - YES	 NO, explain;

- Does the Rescue Service have adequate emergent first aid/CPR support, patient packaging and transport skills? Have they been trained in proper use and operation of all their assigned, or expected to be used, P.P.E.?

  - YES	 NO, explain;

CONFINED SPACE DECISION FLOWCHART

Tab 12

HAZARDOUS ENERGY CONTROL (LOCKOUT/TAGOUT)

In order to comply with federal and state regulations and to ensure that information is available about the dangers of servicing or maintaining any equipment or machines where the unexpected release of stored energy could occur and cause injury, the following Energy Control (Lockout/Tag out) Program has been established. All affected employees will participate in the Energy Control Program. This written program will be kept at the job site and will be available to our employees and to other parties in accordance with applicable laws and regulations.

REGULATIONS

SHA 29 CFR 1910.147

GENERAL INFORMATION

Quality Control/Safety Manager is responsible for the establishment, implementation and maintenance of all aspects of this Energy Control Program. Quality Control/Safety Manager will review these procedures periodically and up-date them as required.

TRAINING

Prior to the start of a project where hazardous energy control is expected, each subcontractor employees shall be given a health and safety orientation that includes the following:

- an overview of the requirements of the regulations

- our policy and procedures related to LOCKOUT/TAGOUT

- when lockout devices are required

- proper use of lockout devices

- overview of alternative safety measures that can be used assure employee protection (e.g., removal of isolating circuit element, opening of an extra disconnecting device, or the removal of a valve handle to reduce the likelihood of an inadvertent energization).

WARNING

DO NOT ATTEMPT TO OPERATE ANY SWITCH, VALVE, BREAKER, ETC. WHEN IT HAS BEEN LOCKED OR TAGGED.

DANGER

DO NOT REMOVE ANY TAG OR LOCKING DEVICE UNLESS SPECIFICALLY AUTHORIZED TO DO SO.

DANGER

INADVERTENT REMOVAL OF SAFETY LOCKOUT DEVICES AND/OR TAGS COULD CAUSE DEATH OR INJURY.

DOCUMENTATION

Documentation of all training, and other program requirements, will be maintained at the Job Site.

POLICY

This policy establishes the minimum requirements for the isolation from all potentially hazardous energy sources whenever maintenance or servicing is done on machines or equipment. It shall be used to ensure that the machine or equipment is stopped, isolated from all potentially hazardous energy sources, and locked and tagged out (or has its conductors disconnected) before employees perform any services or maintenance where the unexpected energization or start-up of the machine or equipment or release of stored energy could cause injury. All employees are required to comply with the restrictions and limitations imposed upon them during the use of this procedure. Only authorized employees shall perform the isolation process in accordance with this procedure.

All employees, upon observing a machine or piece of equipment that is locked, tagged or disconnected to perform servicing or maintenance shall not attempt to start, energize or use that machine or equipment.

Controls that are deactivated during the course of work on energized or de-energized equipment or circuits shall be tagged and locked in the open position.

Equipment and circuits that are de-energized shall be rendered inoperative and have tags and locks attached to all points where such circuits or equipment could be energized.

Tags shall be placed to clearly identify the equipment or circuits being worked.

PROCEDURE

- Sequence for isolation from energy source

  - Notify all affected parties that servicing or maintenance is required on a machine or equipment, and that the machine or equipment shall be shut down and locked out to perform the servicing or maintenance. Affected parties include the Government Inspector/Tenant who is the contact on the service or outage request

  - Obtain information from the Government Inspector/Tenant to identify the type and magnitude of the energy that the machine or equipment utilizes and to know the methods to control the energy

  - Always stop the machine or equipment before lockout/tag out. If in operation, shut it down by normal stopping procedure.

  - Deactivate the energy isolating device so that the machine or equipment is isolated from the energy source (obtain information regarding types and locations of isolating devices from the Government Inspector/Tenant.)

  - Lock out the energy isolating device with assigned individual locks AND affix a tag out device on the energy isolating device.

NOTE:

MACHINE OR EQUIPMENT MUST BE BOTH LOCKED AND TAGGED OR THE CONDUCTORS MUST BE DISCONNECTED PRIOR TO SERVICING OR MAINTENAANCE

  - Dissipate or restrain stored or residual energy (found in capacitors, springs, elevated machine members, rotating flywheels, hydraulic systems, and air, gas, steam or water pressure) by methods such as grounding, repositioning, blocking, or bleeding down.

  - Ensure that the equipment is disconnected from the energy source by first checking that no personnel are exposed, then verify the isolation of the equipment by operating the normal activating device or by testing to make certain the equipment will not operate.

NOTE:

ALWAYS RETURN OPERATING CONTROLS TO NEUTRAL OR OFF POSITION AFTER VERIFYING THE ISOLATION OF THE EQUIPMENT.

- Sequence for restoring equipment to service

  - Check the machine or equipment and the immediate area around the machine to ensure that nonessential items have been removed and that the machine or equipment components are operationally intact.

  - Check the area to assure that all employees have been safely positioned or removed from the area.

  - VERIFY THAT THE CONTROLS ARE IN NEUTRAL OR OFF POSITION

  - Remove the lockout devices and tag out tags (or reconnect conductors to equipment) and reenergize the machine or equipment.

  - Notify affected parties that servicing or maintenance is complete and the machine or equipment is ready for use.

LOCKOUT/TAGOUT FLOWCHART

Tab 13

CRITICAL LIFT, CRANE OPERATION and RIGGING

REGULATIONS

OSHA 29 CFR 1926.550

- CRANES

1.1	Every crane shall have the following documents at all times when it is on the project site.

  - A copy of the manufacturer’s operating manual (developed for that specific make and model of crane).

  - The load-rating chart for that specific crane which shall include:

    - Crane make, model, serial number and year of manufacture

    - Load ratings for all crane-operating configurations, including optional equipment, quadrants of operation

    - Manufacturer’s recommended reeving for the hoist line

      - operating limits in windy or cold weather conditions

  - The Crane Log Book which shall have recorded all operating hours, inspections, tests, maintenance and repairs.

    - This log shall be updated daily, be signed by the operator and supervisor as well as mechanic performing any maintenance or repairs on crane.

  - Responsibilities for crane operations

    - The operator shall not engage in any activity that would divert his/her attention while operating the crane.

    - The operator shall respond to signals from the designated signal individual.

    - Where possible the operator shall have full view of the load and load travel paths at all times the load is rigged to the crane. When this is not possible a properly designated signal person shall assume a position between the operator and the restricted area such that he/she can achieve full view the load and load travel paths at all times.

      - Training shall stress the need for the Signal Person to be observant for adequate safety for both the load and other personnel.

      - No load shall pass over any person, job shack trailer, fueling station, flammable storage area, electrical power station

      - Adequate distance shall be maintained from electrical sources

TABLE A—MINIMUM CLEARANCE DISTANCES

| Voltage (nominal, kV, alternating current) | Minimum clearance distance (feet) |
| --- | --- |
| up to 50 over 50 to 200 over 200 to 350 over 350 to 500 over 500 to 750 over 750 to 1,000 over 1,000 | 10 15 20 25 35 45 (as established by the utility owner/operator or registered professional engineer who is a qualified person with respect to electrical power transmission and distribution). |

Note: The value that follows "to" is up to and includes that value. For example, over 50 to 200 means up to and including 200kV.

      - Adequate clearance (>18") shall be maintained between moving portions of the crane and stationary structures

      - Operational area around the crane shall be barricaded to protect workers from the crane’s rotating superstructure.

    - The government’s designated authority shall be notified at least 72 hours in advance by MIAMI MEES Site Superintendent or Project Manager of all crane operations. Critical lift plans shall be submitted for approval at least 2 days in advance with all supportive documentation.

      - Critical lift plans shall be prepared by the crane operator, lift supervisor, and rigger.

        - It shall specify the exact size and weight of the load

        - It shall include a copy of the maximum load limitations on the crane for the entire range of lifts.

        - It shall specify lift geometry and procedures

        - It shall designate the operator, lift supervisor and rigger and state their qualifications.

        - It shall state expected ground conditions, outrigger and crawler requirements and setup area specifications

        - It should list weather conditions that will affect operations

        - It should specify coordination and communication needs

        - On tandem lifts this would include data on both cranes and need for an equalizer beam

      - A critical lift is a non-routine crane lift requiring detailed planning and additional or unusual safety precautions.

        - Critical lifts include lifts made when the load is 75% or greater than rated capacity of the crane.

        - Lifts made, where part of it is out of operator’s view

        - Lifts made with more than one crane

        - Lifts involving non-routine or technically difficult rigging

        - Hoisting personnel in an approved platform or device

        - Any lift which the operators so deem it to be “critical”

    - Environmental considerations

      - Cranes shall not be operated when wind speeds approach maximum recommendations by the manufacturer.

      - Crane operations shall immediately cease when conditions are such that lightning could occur.

  - Responsibilities in crane operations

    - Before a lift the following items shall be checked

      - The crane is level and blocked

      - Area around crane counterweight swing radius is barricaded, has sufficient clearance and clear of all people.

      - The lift and swing path is clear of obstructions and adequate clearance is maintained from electrical sources and permanent structures.

      - The load is well secured and balanced in the sling or lifting device before it is lifted more than a few inches

  - Operator certifications

    - Cranes and derricks shall only be operated by qualified and certified operators. Certification shall be submitted at least 72 hours in advance of the crane moving onto the project site.

      - Each operator shall be instructed and qualified for each type of crane or derrick he/she is to operate. Certification is to be presented of this fact.

  - A standard signal system shall be used on all crane and derrick operations, (see attached).

    - In situations where the operator cannot see the load, the designated signal person shall utilize standard signals to signal the operator.

    - All personnel on the project site can indicated, by waving of arms, to the crane operator to stop the lift, stop all movement, and engage brake. Once this signal has been given no further operations shall commence until the cause for the signal has been determined and corrected.

  - Inspections	of	cranes	and	derricks	shall	be	in	accordance	with	the manufacturer’s recommendations.

    - A pre-operational inspection shall be conducted by the operator before each operation. The operator shall either use a checklist or so indicate satisfactory inspection results in the operator’s log book.

    - A periodic inspection shall have been accomplished prior to the crane arrival on site by a qualified individual.

    - A written report on the last performance test shall be available on the project site.

  - Cranes and derricks shall be operated, inspected, tested, and maintained in accordance with the manufacturer’s operating manual.

  - Riding on loads, hooks, headache ball, hammers, buckets, material hoist, or other hoisting equipment is prohibited.

  - Tag lines shall be used to control all loads, as deemed necessary by the operator on site.

      - CRAWLER-, TRUCK-, WHEEL- AND RINGER-MOUNTED CRANES

  - All lattice boom and hydraulic mobile cranes shall be equipped with a boom- angle or radius indicator located within operator’s view.

  - All lattice boom and hydraulic mobile cranes shall be equipped with a means for the operator to determine levelness.

  - All lattice boom and hydraulic mobile cranes shall be equipped with a load indicating device or load moment indicator.

  - All lattice boom and hydraulic mobile cranes shall be equipped with an anti-two blocking device that will:

    - stop load-hoisting function at the upper limit before headache ball contacts the boom.

    - boom stop, which will limit the movement of that portion of the boom before it bottoms out.

  - All boom assembly and disassembly shall be conducted by properly trained individuals.  All others shall stand clear of the area.

  - Mobile cranes shall not pick nor swing loads over the side of the crane unless the outriggers are down.

  - Outriggers

    - Shall be fully extended, level, have float securely attached and set to remove the machine weight from the wheels.

    - Blocking under the outriggers shall be placed under the outer bearing surface of the outrigger floats and be of:

- sufficient strength to prevent crushing, bending, or shear failure.

- sufficient size to completely support the entire outrigger, transmit the load to the support surface and prevent shifting, toppling, or excessive settlement under load.

      - RIGGING

  - Rigging equipment shall be inspected before each use and during as necessary to ensure that it is safe. At the minimum rigging shall be inspected for the following failure modes and replaced as necessary:

    - Wire Rope

- Excessively broken wires (over 10% in a three-foot section) per lay or strands per strap or cable

- Severe localized abrasion or scraping

- Kinking, crushing, bird caging, protruding core or any other damage resulting in distortion of the rope structure

- Severe corrosion of the rope or end fittings

- Evidence of electric arc or heat damage

- Excessive pitting, corrosion, cracked, distorted or damaged fittings

- Excessive diameter reduction

- Significant stretching of the wire

- Any signs of metal fatigue or visible damage that raises doubt in the strength of the wire

    - Fiber Rope

      - Broken or cut fibers, either internally or externally

      - Cuts, gouges, abrasions; seriously or abnormally worn fibers

      - Powdered fiber or particles of broken fiber inside the rope between the strands

      - Variations in size or roundness or strands

      - Discoloration or rotting, weakened or brittle fibers

      - Excessive pitting, corrosion, cracked, distorted or damaged fittings

      - Kinks

      - Melting or charring of the rope

      - Other visible damage that raises doubt in the strength of the rope

    - Chain

      - Chains shall be cleaned first and inspected link by link.

      - Excessive wear

      - Excessive stretch (increase in length by 5% shall be cause for rejection)

      - Deformed (twisted or bent) links

      - Cuts, gouges, or nicks

      - Cracks or other visible damage that raises doubt in the strength of the chain

    - Wire Mesh Slings

      - Broken weld or brazed joint along the sling edge

      - Broken wire in any part of the mesh

      - Reduction in wire diameter of 25% due to abrasion or 15% due to corrosion.

      - Lack of flexibility due to distortion of the mesh

      - Distortion of the chocker fitting

      - Distortion of either end fitting so the width of the eye opening is decreased

      - Reduction of the original cross-sectional area of metal at any point around the hook opening

      - Excessive pitting or corrosion of fittings, broken or cracked fittings, distortion of either end fitting out of its plane

      - Visible damage that causes doubt as to the strength of the sling

    - Synthetic Webbing Slings

      - Acid or caustic burns

      - Melting or charring of any part

      - Snags, holes, tears, or cuts

      - Broken or worn stitches

      - Excessive abrasive wear

      - Knot in any part of the sling

      - Wear or excessive elongation exceeding manufacturer limits

      - Excessive pitting, corrosion, cracked, distorted, or broken fittings

      - Other visible damage that cause doubt as to the strength of the sling

    - Attachments

      - Hooks which have been opened more than 15% of the normal throat opening

      - Hooks twisted more than 10% out of plane.

      - Deformed master links and coupling links

      - Assemblies with cracked hooks or other end fittings

      - Excessive pitting, corrosion, distorted or broken fittings

      - Other visible damage that cause doubt as to the strength of the attachment

  - Damaged rigging equipment shall be removed from service, tagged or cut to indicate it unsuitability for use.

  - Hoist rope shall not be wrapped around the load.

  - When hoisting loads, a positive latching device shall be used to secure the load and rigging.

- INSPECTIONS

  - In order to prevent the occurrence of damage to government property and/or injury to government personnel by contractors operating weight handling equipment (cranes), within the confines of the Naval Facility, all contractor cranes shall be inspected in accordance with the attached inspection checklist, as appropriate.

  - Additionally, for all designated critical lifts, the Critical Lift Checklist shall be completed.

# CRITICAL LIFT CHECKLIST

Operator:	 Rigger:	 Lift Supervisor: 	  Government Inspector:

A critical lift is a non-routine crane lift requiring detailed planning and additional or unusual safety precautions. The Critical Lift Plan and Checklist is being developed because the:

  - Lift being made when the load is 75% or greater than rated capacity of the crane.

  - Lift being made, where part of the lift is out of the operator’s view

  - Lift being made with more than one crane

  - Lift involves non-routine or technically difficult rigging

  - Lift involves hoisting personnel in an approved platform or device

  - Operator so deems it to be “critical”. Reason:

_ Government Inspector Notified (three-day advance notice) Date:	Yes 	No

Critical Lift Plan developed and submitted to Government Inspector

- Operator, Lift Supervisor, Rigger, Signalman designated in plan	Yes

  - Crane Operator certification / qualifications determined

  - Qualifications for each person associated with lift established in plan

- Load to be lifted within capacity	Yes

  - Description of load

  - Exact size and weight of load determined.

  - Weight of all crane optional equipment and rigging gear determined

No

No

- Crane’s maximum load limitations, for entire range of lift, specified.	Yes 	No

  - Load ratings for all crane operating configurations

  - Load ratings for all optional equipment

  - Operating quadrants noted on load rating chart

- Plan specifies setup area and establishes necessary conditions.	Yes

  - Crane’s position determined and area outage approved

  - Ground conditions for crane staging area acceptable

  - Area surrounding crane barricaded or warning line determined

  - Area surrounding both pick point and set down area determined

  - Area allows full deployment of outriggers and proper cribbing

No

- Plan specifies lift geometry.	Yes 	No

  - Height of lift determined. Height: 	feet

  - Load radius determined and area barricaded or marked off

  - Boom length and angle determined over entire range of lift

- Plan specifies lift procedures and rigging plan.	Yes 	No

  - All lift points identified

  - All rigging hardware and equipment determined

  - Specifies communication and coordination requirements

  - Position of all personnel associated with lift established

- Plan specifies weather and wind conditions affecting operations.	Yes 	No

  - Operating limits for wind conditions for specific crane

- Specialty lifts and tandem lifts data included on all cranes, equipment, etc.	Yes 	No

  - Equalizer beam data included on all tandem lifts

  - Crawler Track requirements determined in plan.

# CRANE SET-UP INSPECTION (All Sites)

- Manufactures Operator’s Manual is on the crane.	Yes 		No

- Crane’s Log Book

  - Test records from last load test. (4 years) Date:

Yes 		No

  - Daily inspection records and operating hours	Yes 		No		 (Signature of Operator & Supervisor)

  - Maintenance and repair records	Yes 		No		 (Signature of Mechanic & Date)

- Operator Certification for this type of crane.  (3 years)	Yes 		No		 Expiration Date:

Annual Physical Medical Certification Date:

| - Rated capacity posted in Cab, in operator’s view. - Boom Angle (Radius) Indicator and Chart. | Yes Yes |  | No No |  |
| --- | --- | --- | --- | --- |
| a. Load Rating Chart: |  |  |  |  |
| (1) Crane make, model and serial number, year of manufacture | Yes |  | No |  |
| (2) Load ratings for all crane operating configurations | Yes |  | No |  |
| (3) Load ratings for all optional equipment | Yes |  | No |  |
| (4) Recommended reeving for hoist lines | Yes |  | No |  |
| (5) Operating limits for wind conditions | Yes |  | No |  |
| (6) Operating quadrants noted on load rating chart | Yes |  | No |  |
| 6. Crane is level, on firm ground. | Yes |  | No |  |
| 7. Hydraulic fluid level acceptable. (Check with boom in cradle, outriggers up) | Yes |  | No |  |
| 8. Crane Engine Fluid levels acceptable. | Yes |  | No |  |
| 9. Signs of leakage at pump, hoses, connections, fittings, etc. – None | Yes |  | No |  |
| 10. Outriggers are fully deployed; outrigger blocking is adequate, tires off ground. | Yes |  | No |  |
| 11. Windshield wipers and defroster functional. Windshield clean. | Yes |  | No |  |
| 12. Upper limit switch (anti-two-blocking) in place – | Yes |  | No |  |
| 13. Guards on belts, shafts, drums, hot surfaces. | Yes |  | No |  |
| 14. Wire rope reeving in accordance with Manufacturer’s specifications. | Yes |  | No |  |
| 15. Wire even on drum. No fishhooks, birdcages, flat, crushed or damaged wire. | Yes |  | No |  |
| 16. Hooks and hook latches free from defects. | Yes |  | No |  |
| 17. Operator performs routine startup inspection In accordance with manufacturer’s recommendations | Yes |  | No |  |

###### Tab 14

SEVERE WEATHER CONTINGENCY PLAN

All    Job    Sites: High Wind Warnings:

If high wind warnings are posted, greater than 25 mph, all working aloft shall be stopped. This is to include all powered man-lift equipment, all cranes, and extendable boom forklifts.

If wind speeds exceed 10 mph the decision shall be made if work should continue on exposed roof surfaces. The Fall Protection System, in use on the job, shall be reviewed for adequacy and protection with the increased wind speed.

All material, on the roof, shall be properly lashed down and secured when wind speeds exceed 10 mph. This is to include all types of material from 3-tap shingles to metal siding and sealant cans.

Storm Protection:

If warnings of gale force winds are issued, the Contractor shall take precautions to minimize danger to persons and protect work and nearby Government property.

Precautions shall include, but are not limited to:

  - Closure of any openings in work.

  - Removing loose materials that may become missile hazards.

  - Protection of tools and equipment from exposed locations.

  - Removal or securing of scaffolding or temporary work.

  - Crane operations shall be suspended when winds at the top of the crane approach the maximum wind velocity recommended by the manufacturer.

  - When conditions are such that lightning could occur, all crane operations shall cease.

- Floating Plant and Marine Activities:

If we are working over water or off piers all work shall stop once gale warnings are posted. This is to include all work from barges and floating plants. All floating equipment, barges and floats shall be made fast to the pier or additional anchors shall be placed as directed by competent individuals.

During storm warnings, with the increased wind and tidal actions, the work over-water precautions and protective measures shall be reviewed for adequacy and protection especially water rescue. The increased wind and tidal actions may be significant to overcome propulsion means of rescue craft and shallow draft vessels might present a hazard to rescue crew.

- Workers in Cold Environments

If we are working in air temperatures that are less than 35 F, with a wind above 10 mph, all personnel on the project site shall observe the following precautions but are not limited to:

  - Wear warm clothing, adequate for the task at hand. (It is recommended that employees wear at least three layers of clothing when working in air temperatures at or below freezing.

  - Wear proper head and face protection for both the temperature and the weather, (rain, snow, wave action, wind).

  - Wear proper footgear and gloves, insulated to protect against cold and dampness.

  - Engineering controls

    - Shield work areas from drafty or windy conditions

    - Provide an on-site heat source

    - Provide a heated shelter for all employees subject to prolonged exposures to wind-chill temperatures less than 20F or those working over-water, off piers, where the potential exists for water submersion.

    - Ensure the buddy system is in place when working outdoors in low temperatures, off piers, or over-water.

Tab 15

# ACCESS AND HAUL ROAD PLAN

References:    40 CFR 122

29 CFR 1926

Notification:

MIAMI MEES employees and all subcontractor personnel are advised that this project is subject to Federal, State, and local regulatory agency inspections to review compliance with environmental laws and regulations.

MIAMI MEES employees and all subcontractor personnel on the MIAMI MEES project site could be held responsible for damages to persons or property resulting from fault or negligence as well as for the payment of any fines which may be assessed as a result of a violation of any Federal, State or local environmental law or regulation.

All personnel on the MIAMI MEES Project Site shall fully cooperate with any representative from any Federal, State or local regulatory agency who may visit the job site and shall provide immediate notification to the MIAMI MEES Project Manager of the visit and any Notice of Violation (NOV) that might be issued to the Contractor.

Preservation:

- Preserve the natural resources within the project boundaries and outside the limits of permanent work.

- Confine construction activities to within the limits of the work indicated or as specified.

- Conform to the requirements of the Clean Water Act for stream or other water crossings.

- Restore to an equivalent or improved condition any damage or excavated area upon completion of work.

- Except in areas to be cleared, protect existing trees, shrubs and other plants which are to remain, and which may be injured, bruised, defaced, or otherwise damaged by construction operations. Do not fasten or attach ropes, cables, or guys to nearby trees for anchorages. Remove displaced rocks from un-cleared areas.

- All means possible will be taken to prevent oily or other hazardous substances from entering the ground, drainage areas, or local bodies of water.

- Do not alter water flows or otherwise significantly disturb the native habitat adjacent to the project, which might be critical to the survival of fish and wildlife, except as specified.

- Do not disturb fish and wildlife.

- Burn off of the ground cover is not permitted.

Access Route / Haul Road Plan:

- Prior to the start of the project a map showing all access routes and haul roads, needed for the project, will be developed and provided to the Safety Manager.

  - Road layout and widths, curve data and line of sight distances

  - It will detail any rough excavation and grubbing requirements.

  - What type of grading material will be used and how it will be placed?

  - It shall show all stream, drainage ditch, channel crossings that will be encountered.

  - Drainage controls and channels, basins.

- Access and haul roads shall be constructed to widths suitable for safe operation of equipment at speeds no greater than 10 mph. Unless deemed necessary, passing on access and haul roads will not be allowed unless special provisions, such as development of a passing zone, are implemented.

- The maximum allowable grade shall be limited to 10%.

- All curves shall have open sight line and as great a radius as possible.

- Horizontal curves shall consider vehicle speed, roadway width, surfacing material and super elevation.

- Lighting shall be provided as necessary to ensure open line of sight.

- Standard traffic control lights, barricades, road markings, signs and signalperson shall be provided for the safe movement of traffic on the access or haul road.

- The Traffic Signalperson shall be certified and competent per applicable regulations.

- Roadway hardness, smoothness and dust control measures shall be used to maintain the safety of the roadway.

- No employer shall move, or cause to be moved, any equipment or vehicle upon an access or haul road unless the roadway is constructed and maintained to safely accommodate the movement of the equipment or vehicle involved.

- When road levels are above working levels a berm, barricade or curb shall be constructed to prevent vehicles from overrunning edge or embankment.

- Roadway surfaces shall have a crown. Drainage channels and temporary sediment basins shall be constructed as necessary.  These will be inspected and accumulated sediment removed, after each major storm. Use a paved weir or vertical overflow pipe for overflow. Effluent quality monitoring programs shall be instituted to protect surface water.

- The plan shall address the following:

  - Type of vehicle or equipment usage anticipated and traffic density

  - Hours of operation and need for lighting

  - Sign and Signalperson requirements

  - Road markings and traffic control devices

  - Maintenance requirements including roadway hardness, smoothness and dust control measures.

Dust and Debris Control:

- The plan shall address specific means for controlling the dirt and debris that will be carried on to public roadways.

  - It will need to identify the firm subcontracted to clean along the entrance and exit route to remove dirt, dust, and debris from the public roadways.

- Also it will need to address ways to control dust emissions along the access or haul roads to a minimum. Such as through use of water spray, dust suppressant, or by other approved means.

  - It will need to identify the suppressant will be placed. The description of the process and include a copy of the MSDS if any other means except water is used.

  - Also if this task is to be subcontracted, the name of the firm performing the work.

Stream Crossings:

- The preferred method for crossing any live streams or other waterways will be to install a temporary culvert or bridge. This temporary crossing would be removed upon completion of work and the area repaired to its original condition or as specified in contract.

- Specific approval shall be required before any equipment will be permitted to ford live streams. Permission to ford live stream will be an extremely rare occasion and will follow the requirements of the Federal and/or State Clean Water Act.

- Temporary crossing shall be constructed to withstand the potential forces and movements due to impacts of heavy vehicle traffic. It will be constructed with standard guardrail system if pedestrians will also be accessing it.

Grubbing and Earthwork:

- Immediately provide temporary protection on sides and back slopes as soon as rough grading is completed or sufficient soil is exposed to require erosion protection.

  - Plan and conduct earthwork to minimize the duration of exposure of unprotected soils.

  - Protect slopes by accelerated growth of permanent vegetation, temporary vegetation, mulching, or netting. Stabilize slopes by hydro seeding, anchoring

mulch in place, covering with anchored netting, sodding, or such combination of these and other methods necessary for effective erosion control.

- At the completion of work and removal roadway provide new seeding where ground is disturbed. Include topsoil or other nutriments during the seeding operation necessary to reestablish a suitable stand of grass.

Runoff Control:

- Mechanically retard and control the rate of runoff from the roadway or construction site.

  - Such items as construction of diversion ditches, benches, berms, and temporary sediment basins.

  - Use of silt fences and straw bales to retard and divert runoff to protected drainage course.