August 28, 2019

Department of Homeland Security

U.S. Coast Guard Base Miami Beach

100 MacArthur Causeway

Miami Beach, Florida 33139

United States

Re: Machinery Room Drain Pipe Replacement

PROPOSAL

Site Description:

The project site is located at the USCG Miami Beach base, Machinery Room (Building 5). The drain system is receiving direct water from cutting machinery above. The water is then dumped into a receptacle that flows through an 8” sleeve with a 4” inch pipe attached. This pipe is 18” below the ground. The current 4” pipe is under concrete for approximately 25-30 ft leading to an underground water tank located on the outside. The 4” pipe is about 25” deep in the tank.

Scope of Work:

Proposal includes materials, labor, taxes, freight, safety, supervision and equipment to perform the replacement of the drain pipe inside the Machinery Room (Building 5) as per the site visit and requirements discussed 07/31/19. All work will be installed as per local, state and federal codes and regulations.

Scope of Work:

- Use pipe locator equipment to know exactly the location of the pipe underneath the concrete

- Call and set up the utilities company to come out to mark and verify there is no electrical, gas or additional water lines beneath.

- Cut 24” – 36” opening in concrete the length of pipe using a concrete cutter machine.

- Excavate the ground to remove concrete, dirt and sand to expose the current 4” pipe.

- Remove and dispose of debris in an approved waste facility.

- Pump the water out of the tank (into storm drain) to bring water level below the existing 4-inch pipe.

- Remove and replace existing 4” pipe with new 8” PVC pipe starting on the inside of the machinery room at the wall sleeve. New pipe will be installed at a slope of 1/8 inch per foot (recommended) or as per specification to produce a gravity drain.

- Seal around the pipe opening area with concrete mix or alternative product as per requirements.

- Backfill the trench with sand and rebar as per standard specifications.

- Form and pour new concrete replacement slab where existing was removed as per government standards.

NOTE: Excludes

- Laboratory testing for hazardous materials.

- Removal of hazardous materials.

- Night work

- Permits

TOTAL PRICE: 					$ 28,930

Notes:

- Bid is as per plans and specifications except for the above-named exclusions/changes

- Owner/Contractor shall abide by OSHA requirements

Thank you for your attention. If you have any questions, please feel free to contact me.

THE ABOVE PROPOSAL HAS BEEN AUTHORIZED AND APPROVED BY:

Owner:						__ ________________

Date of Acceptance:		 			_____________________________

Accepted by Signature:				_____________________________

**** PROPOSAL EXPIRES IN 60 DAYS ******

Sincerely,

Victory Plumbing Inc.