Accident Prevention Planning - Hazard Control Measures:

Activity Hazard Analyses (Job Hazard Analysis)

F-1 Applicable Outlines (Attach completed copies)

Activity Hazard Analysis

| Activity/Work Task: Unload and PEMB Erection | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: Wichita, KS McConnell AFB | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |
| Contract Number: W912DQ-15-C-4005 | Severity | Probability |  |  |  |  |  |  |
|  |  | Frequent | Likely | Occasional |  | Seldom | Unlikely |  |
| Date Prepared: 08/24/2015 | Catastrophic | E | E | H |  | H | M |  |
|  | Critical | E | H | H |  | M | L |  |
| Prepared by: Eric Coffie | Marginal | H | M | M |  | L | L |  |
|  | Negligible | M | L | L |  | L | L |  |
| Reviewed by: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (see above) |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc. ) PPE: Hard hats, safety glasses, high visibility vest or clothing, steel toed work boots, gloves, hearing protection | “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent, Likely, Occasional, Seldom or Unlikely. |  |  |  | RAC Chart |  |  |  |
|  | “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic, Critical, Marginal, or Negligible |  |  |  | E = Extremely High Risk |  |  |  |
|  |  |  |  |  | H = High Risk |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of the AHA. |  |  |  | M = Moderate Risk |  |  |  |
|  |  |  |  |  | L = Low Risk |  |  |  |

| Activity | Hazard(s) | Controls | EM 385 | RAC |
| --- | --- | --- | --- | --- |
| Hold Steel erection pre-job meeting | None | - Ensure and discuss compliance with plans and specifications - Identify minimum PPE to be worn by all workers - Review fall threshold requirement is 6 feet - Review approved Erection plan - Review Site-Specific Fall protection and Prevention Plan - Ensure workers know all hazards which could be encountered and their location | 05.A.06a 05.E, 05.D 05.F 21.A 27.F.01 21.C.01 | UN L |
| Crane Operations | Operation Failure | - Operators shall be trained, qualified and authorized to operate such equipment. - Crane must be inspected prior to use | 15.B.01 27.F.07 | UM H |
|  | Overload loads | - All crane operators within swing radius of each other will be equipped with radios to assist in the coordination of procedures. - Hand signals for operators must conform to standards | 08.B |  |
| Unload Trucks | Striking workers with loose objects | - Keep area clear of tripping hazards - Locate barrier where truck is to be unloaded prior to removing chains | 14.C.01 | UM L |
|  |  | - Loosen all binders prior to removing chains (at the level to be unloaded) to ensure that the load is stable. - Wear high visibility vests | 05.F |  |
|  |  | - Chock wheels during unloading - Load to be lifted min height necessary to clear ground or obstacles - Do not lift loads over personnel | 18.C.16.a 09.B.23.a 16.S.04 16.S.05 | UM M |
| Staging Material | Falling material | Material to be stacked safely | 14.B.01 | UM M |
|  |  | Materials to be braced, cribbed, or tied to prevent rollover and unbinding |  |  |
| Forklift Use | Equipment failure | - Inspect forklift prior to use each day - Only qualified personnel will operate forklift |  | UM M |
| Rigging Steel for Picking | Under Rated Slings | Verify the weight of the object to be picked up | 16.S | UM M |
|  | Cut Slings | Inspect slings before each pick – turn in cut or frayed slings |  |  |
| Setting structural members into place | Pinch points, striking worker, falling | - Connectors shall position themselves to allow mobility to avoid incoming pieces - Know hand signals, use tag ropes and pay attention - Use spud wrenches, pull pins | 27.F.18 | UM M |
|  |  | Only one connector shall give signals to the crane operator and he must make sure that his partner is clear | 16.G.05 | UM M |
|  |  | Loads must have tag lines to guide load point to connection point when practical | 16.G.07 | UM M |
|  | Member falling | Beams shall be connected with a minimum of two bolts at each prior to releasing hoisting line |  |  |
| Bolting up connections | Falling objects | - Impact wrenches shall have locking devices for retaining sockets. - Any container shall be secured to prevent objects from falling below - When bolts or drift pins are driven out, they shall be prevented from falling | 27.F.18 13.A |  |
| Working in boom lifts | Falling | When in a boom lift, the operator and helpers will be tied off. | 21.A.02 | UM M |
| Drilling, Cutting Steel | Metal in Eyes or Hand | Wear safety glasses and gloves, | 10.A | UM M |

F-2 Operational Equipment Checklist

| Equipment to be Used | Training Requirement/Competent or Qualified Personnel name(s) | Inspection Requirements |
| --- | --- | --- |
| - Boom Truck - Extendable reach fork truck - Scissor Lifts - Boom lifts - Ladder - Extension Cords - Impact Wrenches - Hand tools, Spud wrenches, hammers | - Boom lift operator, fork lift and scissor lift shall be qualified for the type of equipment to be operated - Workers shall be trained in Sub Part R of 1926 steel erection standard - Rigger shall be qualified - Lift operators shall be qualified | - Inspect site prior to set up - Daily inspections shall be completed by the operator of the Boom, Forklift - Lift Operators shall document daily inspection - All tool and equipment shall be inspected prior to each days use |